EU Roaming regulation III Structural Solutions Processes DRAFT Version 0.3 Last Update 18/02/2013

Document History

| History |  |
| --- | --- |
| V 0.1 | Skeleton with first inputs on all process to be included in scope of work. |
| V 0.2 | Updates following Face to face meeting 7th and 8th Feb |
| V 0.3 | Added first versions of processes received prior to conf call 19/2/13 |

Summary

1	Scope	4

1.1	References	4

1.2	Definitions	5

1.3	Abbreviations	7

2	Background	8

3	Single IMSI Processes	8

3.1	DSP’s reference service offer preparation to ARP	8

3.2	Relationship between DSP and ARP	8

3.2.1	DSP to ARP relationship establishment process	8

3.2.2	DSP to ARP testing process	8

3.2.3	DSP to ARP relationship termination	9

3.3	Service activation by the customer	9

3.4	Customer Service query responsibilities	9

3.4.1	DSP	9

3.4.2	ARP	9

3.5	Customer is issued a bill	9

3.6	Customer changes billing basis	9

3.7	Bill-shock measures	9

3.7.1	Send alerts to customer at cost thresholds	10

3.7.2	Notify customer of cessation of service at threshold	10

General on ARP contract termination	10

3.8	Service deactivation at end of contract	10

3.9	Service deactivation by the customer before end of contract	10

3.10	Service deactivation by the ARP before end of contract	10

3.11	Service deactivation by the DSP before end of contact	10

3.12	Customer ports out via MNP during Single IMSI contract	10

3.13	Customer changes UICC card during Single IMSI contract	10

3.14	Customer changes MSISDN during Single IMSI contract	10

3.15	Change of ownership of MSISDN at DSP	11

3.16	Error cases	11

3.17	Open questions	11

4	LBO	11

4.1	Pre-requisite requirements for the DSP and LBO Provider	11

4.1.1	DSP	11

4.1.2	LBO Provider	11

4.2	Service activation by the customer	11

4.3	Customer Service query	12

4.3.1	DSP	12

4.3.2	LBO Provider	12

4.4	Customer is issued a bill	12

4.5	Customer changes billing basis from LBO provider	12

4.6	Bill-shock measures	12

4.6.1	Send alerts to customer at cost thresholds	12

4.6.2	Notify customer of cessation of service at threshold	12

4.7	Customer requests change in terms of contract	12

4.7.1	Change in usage limit	12

4.8	Service deactivation at end of contract	12

4.9	Service deactivation by the customer before end of contract	12

4.10	Service deactivation by the LBO provider before end of contract	12

4.11	Service deactivation by the DSP before end of contract	13

4.12	Customer returns to DSP after LBO contract termination	13

4.13	Customer ports out via MNP during LBO contract	13

4.14	Customer ports-in to new DSP via MNP during LBO contract	13

4.15	Customer changes UICC card during LBO contract	13

4.16	Customer changes MSISDN during LBO contract	13

4.17	Customer changes hardware during contract	13

4.18	Error cases	13

4.18.1	User takes LBO but incorrectly configures APN	13

4.18.2	User attempts to use LBO APN but without a LBO contract with a visited network provider	13

4.18.3	User attempts to use LBO with EU Universal APN, but in country outside of EU	13

4.18.4	Non-EU customer attempts to use LBO	13

5	COMBINE SINGLE imsi/lbo case	13

5.1	Single IMSI subscriber, subsequently subscribes to LBO	13

6	Example  process template	14

6.1	<process name>	14

6.1.1	Process	14

Figures

Figure 1 – Requirement hierarchy & classification	4

# Scope

The objective of this document is to describe the Processes that will be used between the various parties involved in the implementation and operation of roaming unbundling for EU roaming regulation III.  The document will specify the processes needed for;-

- the establishment of a relationship between a DSP and ARP for Single IMSI service.

- the establishment of a relationship between a DSP and LBO Provider for LBO service.

- the actions required by a customer to contract for Single IMSI or LBO-based services and the actions this will trigger within the service providers involved.

- exchanging information for correct operation and billing of service for a customer using Single IMSI or LBO.

- customers to return to their DSP for roaming service either at the end of contract, or in the situation of premature termination of an ARP or LBO contract.

- error cases and any processes to recover such situations.

The processes included vary in nature with some being business processes between service providers to establish relationships between themselves to enable service to be offered to the customer base, while others being processes that relate to an individual customer and enabling that specific customer to receive Roaming services from a party other than their DSP.

Each process will be specified as a series of steps, which will be drawn out as a diagram and then described step-by-step.  In some processes, references will be made to the interfaces defined in High Level Technical Specifications [1].  The interface definitions used within [1] will be referred to where relevant in association with the processes where such interfaces are used.

Out of scope

## References

Editor’s note: These References have been taken directly from the Technical requirements document (with the exception of the addition of a reference to the Technical Requirements document itself) and need to be revisited for relevance to the content of this document once the content of the document is complete.

- EU Roaming Regulation III Structural Solution High Level Technical specifications.

- Regulation (EU) No 531/2012 of the European Parliament and the Council of 13 June 2012 on roaming on public mobile communications networks within the Union

- Regulations, Commission Implementing Regulation (EU) No 1203/2012 of 14 December 2012 on the separate sale of regulated roaming services within the Union

Following documents are for information only (given for supporting the consultation).

No binding requirements should be derived from there.

- BoR (12) 68: ROAMING REGULATION - CHOICE OF DECOUPLING METHOD: A consultation to assist BEREC in preparing advice to the Commission on its forthcoming Implementing Act, June 2012, 72 pages.

- BoR (12) 109: ROAMING REGULATION - CHOICE OF DECOUPLING METHOD, BEREC opinion on article 5 implementing act, 27 Sept 2012, 7 pages

- 3GPP TS 32.240

## Definitions

Editor’s note: These Definitions have been taken directly from the Technical requirements document and need to be revisited for relevance to the content of this document once the content of the document is complete.

Below mentioned definitions are adopted by 3GPP TS 32.240 [5] “Charging architecture and principles” and draft “Implementation Act”

alternative roaming provider: a roaming provider different from the domestic provider;

billing: function whereby CDRs generated by the charging function(s) are transformed into bills requiring payment;

call control function (CCF): CCF is the Call Control Function in the network that provides call/service processing and control (see ITU-T Recommendation Q.1224)

CAMEL: network feature that provides the mechanisms to support operator specific services even when roaming outside HPLMN;

charging data record (CDR): formatted collection of information about a chargeable event (e.g. time of call set-up, duration of the call, amount of data transferred, etc) for use in billing and accounting. For each party to be charged for parts of or all charges of a chargeable event a separate CDR shall be generated, i.e. more than one CDR may be generated for a single chargeable event, e.g. because of its long duration, or because more than one charged party is to be charged;

charging: function within the telecommunications network and the associated OCS/BD components whereby information related to a chargeable event is collected, formatted, transferred and evaluated in order to make it possible to determine usage for which the charged party may be billed (offline charging) or the subscriber’s account balance may be debited (online charging);

circuit switched domain: domain within GSM / UMTS in which information is transferred in circuit switched mode;

domestic provider: an undertaking that provides a roaming customer with domestic mobile communications services;

donor roaming provider: the roaming provider, that is currently providing roaming services to a customer;

EU-Internet access point name (APN): a common identifier set, manually or automatically, in the roaming customer's mobile device and recognised by the home network and visited network to indicate the roaming customer's choice to use local data roaming services;

home network: a public communications network located within a Member State and used by the roaming provider for the provision of regulated retail roaming services to a roaming customer;

home PLMN: PLMN where the Mobile Country Code (MCC) and Mobile Network Code (MNC) of the PLMN identity are the same as the MCC and MNC of the IMSI.

local data roaming service: a regulated data roaming service provided, temporarily or permanently, to roaming customers directly on a visited network, by an alternative roaming provider without the need for roaming customers to change their SIM card or mobile device;

mobile number portability: The ability for a mobile subscriber to change subscription network within the same country whilst retaining their original MSISDN(s).

network barring: a control function used by the home network operator aimed at avoiding the selection of certain visited networks for its roaming customers;

offline charging: charging mechanism where charging information does not affect, in real-time, the service rendered.

online charging system: the entity that performs real-time credit control. Its functionality includes transaction handling, rating, online correlation and management of subscriber accounts/balances.

online charging: charging mechanism where charging information can affect, in real-time, the service rendered and therefore a direct interaction of the charging mechanism with bearer/session/service control is required.

packet switched domain: domain in which data is transferred between core network elements.

real-time: real-time charging and billing information is to be generated, processed, and transported to a desired conclusion in less than 1 second.

recipient roaming provider: a roaming provider, that will provide roaming services instead of roaming services currently provided by the donor roaming provider after the change of roaming provider;

resale of retail roaming services: a provision of regulated roaming services, provided as a bundle, and associated services, such as voice mailbox services, that are usually available to roaming customers, without the need for roaming customers to change their SIM card or mobile device, in accordance with a wholesale agreement concluded between an alternative roaming provider and a domestic provider;

retail charging: see charging

roaming customer: a customer of a roaming provider of regulated roaming services, by means of a terrestrial public mobile communications network situated in the Union, whose contract or arrangement with that roaming provider permits Union-wide roaming

roaming provider: an undertaking that provides a roaming customer with regulated retail roaming services;

roaming: The ability for a user to function in a serving network different from the home network. The serving network could be a shared network operated by two or more network operator.

traffic steering: a control function used by the home network operator aimed at the selection of visited networks for its roaming customers based on a priority list of preferred visited networks;

union-wide roaming: the use of a mobile device by a roaming customer to make or receive intra-Union calls, to send or receive intra-Union SMS messages, or to use packet switched data communications, while in a Member State other than that in which the network of the domestic provider is located, by means of arrangements between the home network operator and the visited network operator;

visited network: a terrestrial public mobile communications network situated in a Member State other than that of the roaming customer’s domestic provider that permits a roaming customer to make or receive calls, to send or receive SMS messages or to use packet switched data communications, by means of arrangements with the home network operator;

To be added:

Wholesale charging

Anti Bill shock

Premium

Public Mobile Network

## Abbreviations

Editor’s note: These Abbreviations have been taken directly from the Technical requirements document and need to be revisited for relevance to the content of this document once the content of the document is complete.

3GPP	Third Generation Partnership Project

APN	Access Point Name

ARP	Alternative Roaming Provider

BEREC	Body of European Regulators for Electronic Communications

CDR	Charging Data Record

CF	Call Forwarding

CS	Circuit Switched

DSP	Domestic Service Provider

EU	Member States of the European Union, the outermost regions of the European Union and countries adopting Regulation

GGSN	Gateway GPRS Service Node

GSM	Global System for Mobile communication

GSMA	GSM Association

HLR	Home Location Register

HPMN	Home Public Mobile Network

IMSI	International Mobile Subscriber Identity

LBO	Local BreakOut

LTE	Long Term Evolution

ME	Mobile Equipment

MMS	Multimedia Messaging Service

MOC	Mobile Originating Call

MTC	Mobile Terminating Call

MNO	Mobile Network Operator

MVNO	Mobile Virtual Network Operator

MVNE	MVNO Enabler

NNI	Network to Network Interface

OCS	On-line Charging System

ORLCF	Optimal Routing Late Call Forwarding

OTA	Over The Air

PDP	Packet Data Protocol

QoS	Quality of Service

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMS	Short Messaging Service

SMSC	Short Message Service Center

TAP	Transferred Account Procedure

TBC	To Be Confirmed

TBD	To Be Defined

VLR	Visited Location Register

VPMN	Visited Public Mobile Network

Important: This document will contain obligation and option.

- “Shall” will describe an obligation

- “May” will describe an option

# Background

# Single IMSI Processes

Some presuppositions…

Within all customer contracts for Roaming there will need to be a clause that states that, unless the customer has an active contract with an ARP, the DSP will provide roaming service to the customer.  This will allow the DSP to take a customer back at the expiry of the ARP contract, and not leave the customer without roaming service.

A customer can only have one active ARP contract at any time.  If a customer requests a new ARP to provide service, part of the activation process of the new contract is the notification of this to the existing ARP by the DSP.

## DSP’s contract template to ARPs

GSMA

DSP’s should prepare a ‘Reference Offer’ that can be used by an ARP as the basis upon which a Single IMSI Roaming offer can be established.  The Reference Offer should include basic information about what the DSP will expose to the ARP in such an agreement.

This should include:-

- Pricing information for the establishment of the agreement.

- Pricing Information for voice, SMS and Data traffic that will be used by an ARP customer whilst roaming, and served by the DSP.

- Coverage of the offer with regard to the EEA countries that the DSP can offer Roaming service within, and the networks within those countries that the DSP has existing Roaming partnership agreements with.

  - Any technical limitations to coverage should be identified.  For example, the absence of CAMEL roaming interfaces between VPLMN and DSP.

- Technical interfaces exposed by the DSP towards the ARP including references, versions of specifications and any relevant implementation details that would be considered specific to that DSP.

- Contact points for an ARP within the DSP to progress with establishing an agreement.

- The Process and timescales associated with the process, for establishing an agreement between the DSP and an ARP.

- Duration of the Reference Offer.

## Relationship between DSP and ARP

GSMA

### DSP to ARP relationship establishment process

Taking the Reference Offer of the DSP as a starting point (where such a reference offer is available), when an ARP approaches the DSP to establish a Single IMSI Roaming relationship, there will need to be additional specific detail added to the Reference Offer.  This should cover as a minimum, the following aspects;-

- All topics covered by the Reference Offer, including variance from the reference offer that is specific to the agreement between the DSP and the specific ARP.

- Definition of the scope of the Agreement with regards to customer’s individual eligibility and any specific exclusions.

- Process for management of modifications to the service.  This should cover modifications made by the DSP that impact the ARP, and modifications made by the ARP that impact the DSP. The process should include periods of notice required prior to a modification being made and mechanisms whereby an existing agreement between DSP and ARP can be modified without the need for a new agreement.

- Testing of interfaces between DSP and ARP prior to service launch or during service, including;-

  - Any required testing and certification of interfaces.

  - Testing for availability of the service.

- Interconnectivity implementation details of interfaces including relevant Addressing details.

- Mechanisms for ensuring the integrity and security of the DSP and ARP networks, and the links interconnecting them.

- Prior to launch, a forecast from the ARP to the DSP of the amount of traffic they expect to bill consumers for and a bank guarantee based on this forecast.

- Details for the settlement of Bills between DSP and ARP including;-

  - Periodicity of payment

  - Schedule of payments from ARP to DSP (e.g. dates of month, day of week)

  - Process for payment of any due refund from DSP to ARP.

  - Penalties and/or interest terms resulting from failure to pay.

  - Liability for Bank Charges

- The responsibilities of the DSP and the ARP for Customer Care, and the definition of any interface between the DSP and ARP to enable each to provide Customer Care.  Particular focus should be given to;-

  - Availability of customer eligibility information.

  - Availability of coverage information

  - Information relating to DSP, ARP and VPLMN network faults.

  - Handling of Lost or Stolen UICC cards

  - Lost, stolen or faulty mobile equipment

  - Billing enquiries.

- Conditions relating to Confidentiality of the Agreement between DSP and ARP, and liabilities of either party found to be in breach of that confidentiality.

- Obligations of each party in relation to Data Privacy.

- Measures that are to be put in place to prevent exposure of either party to fraud.

- Service Level Agreements.

- Conditions under which the service shall be suspended.

- Events and conditions considered to be Force Majeure.

- Conditions under which the service shall be terminated.

- Process and conditions under which changes can be made to the agreement.

- Choice of Law

- Dispute resolution and Arbitration processes.

### DSP to ARP testing process

Editor’s note – think this is within Interface and Protocol group scope – remove?

## Service activation by the customer

Deutsche Telekom

Customer approaches ARP A for service.

ARP A notifies the DSP of request from the customer

- If the customer is already a customer of another ARP B, DSP notifies the ARP B that the customer is moving away.

- If the customer is already within the activation process for an ARP, the request is rejected.

DSP notifies the ARP that the service is provisioned

ARP confirms service to customer

Verification of the request is required but is implementation specific.  It is advised that the single verification mechanism is used within a single country.

ACTION: collect options for verification mechanisms from group participants

## Customer Service query responsibilities

Telekom Austria

Process Description

The customer requests information or complaints about the roaming services to DSP or ARP. DSP and ARP clarify the request and provide the appropriate information to the customer. Customer will get the information from the operator who is responsible for the services customer inquiring about.

Pre-conditions and Trigger

- Customer of DSP using ARP for regulated roaming services

- Customer contacts DSP or ARP to inquire information

Result of Process

- Information is provided to the customer

Assumptions

CRM of DSP needs to provide information that a customer is decoupled, so that contact points are aware if the customer is/has been using the ARP and in what period.

CRM of ARP needs to be maintained to support all customers that have contracts, so that all customer inquiries can be properly handled.

### DSP

Input for Process Step

Customer is/was/will be Single IMSI ARP customer

Customer requests information about services that DSP is responsible for

Only DSP relevant information can be presented to the customer.

Description of the Process Step

- The customer contacts DSP (any touch point)  and requests the following information:

- Specific Single IMSI ARP availability (agreement exists between the ARP and DSP)

- Eligibility status of the customer (e.g. only decision maker SIMs in a multi-SIM contract)

- Subscription to an ARP process -> customer is advised that subscription request needs to be directed to an ARP. All other process related information is to be addresses by the ARP.

- Data cut-off service:  provided ONLY for the services still provided by DSP (for non EU).

- Usage and pricing related to traffic outside of EU - standard procedure

- Data packages usage: does not include Single IMSI and/or LBO data traffic

- Voicemail receipt (free for customer) and retrieval (outgoing call - provided by ARP) tbc

- Information on how to avoid inadvertent roaming in border regions - standard procedure

- Any supplementary services still provided by DSP (Call forward, CLIP, CLIR, multi-call - tbc)

- Information regarding any services by DSP which can affect or are affected when customer is using a Single IMSI ARP (e.g. Voice mail, SMS info, networks available, suspension of roaming, DSP roaming offers, etc.).

- Non-available services in Single IMSI (if applicable to the customer) (tbc which services are those)

- Customer gets explanation about invoice issued by the DSP - DSP is charging only non-EU traffic.

- Necessary actions to come back to DSP. Customer is advised that subscription cancellation needs to be directed to an ARP.

Output/Result of the Process Step

- Customer request is clarified by the DSP.

Remarks

The subscription termination to an ARP has to be triggered by the customer. This cannot be done by DSP on behalf of the customer. Availability of Single IMSI service is depended on availability of domestic and roaming services provided by DSP.

Reference

None

### ARP

Input for Process Step

Customer is/was Single IMSI ARP customer

Customer requests information about services that ARP is responsible for

Only ARP relevant information can be presented to the customer.

Description of the Process Step

- The customer contacts ARP (any available touch point, including a customer care reachable by phone and possibly Internet)  and requests the following information on Single IMSI:

- Subscription processes

- Anything regarding specific offers including prices of services provided by the Single IMSI ARP

- Subscription and contract deactivation

- Single IMSI Contract T&C

- Setup of the data limit with the Single IMSI ARP

- Price notifications

- Service availability and quality inquiries

- Customer complaint about inability to use roaming services in EU with Single IMSI. This could be a result of:

  - Suspension of provided Single IMSI roaming service by Single IMSI ARP

  - Lack of roaming network coverage, which Single IMSI ARP is using

  - Contract terminated with Single IMSI ARP

- Single IMSI invoice/charging/usage

Output/Result of the Process Step

- Customer request is clarified by the ARP.

Remarks

Reference

None

## Customer is issued a bill

GSMA

A customer that has opted to take Single IMSI roaming service from an ARP shall be billed for this service according to the terms of the contract they hold with the ARP.  The customer shall not be charged for Roaming services by the DSP for any period when the customer has a contract with an ARP that is currently active.  If the customer’s ARP contract is cancelled or suspended for any reason (see sections 3.9 to 3.13) and the customer uses roaming services, the DSP will bill the customer for these services.

DSP will continue to charge the customer for domestic services. The DSP shall also charge for roaming services used outside of the EEA markets, unless the ARP contract specifically extends to markets beyond the EEA.

The process by which a bill is generated by and conveyed from an ARP to a customer is specific to the ARP and customer’s contract, and so is not defined.

## Customer changes billing basis

Swap from pre to post paid or vice versa

Wait for resolution of Hybrid Profile discussion.

## Bill-shock measures

Starhome

Retain the capability as today – must be provided by the ARP.

### Send alerts to customer at cost thresholds

### Notify customer of cessation of service at threshold

## Passing of NR-TRDE information to ARP

*new Process* Added based on input from Billing and Provisioning.

## General on ARP contract termination

Condition in contract – customer receives roaming from the DSP unless they take an ARP contract. This would be default roaming solution – does not represent a change to current situation prior to Article 5.

- Close to end of contract ARP may notify customer that contract is about to expire.

- At end of contract ARP may notify customer that contract has expired.

- ARP shall notify DSP that customer is no longer contracted to ARP for roaming service.

## Service deactivation at end of contract

Deutsche Telekom

## Service deactivation by the customer before end of contract

Deutsche Telekom

## Service deactivation by the ARP before end of contract

Deutsche Telekom

## Service deactivation by the DSP before end of contact

Deutsche Telekom

## Customer ports out via MNP during Single IMSI contract

GSMA

Use ‘Service deactivation by the DSP before end of contact’.

## Customer changes UICC card during Single IMSI contract

Depends if IMSI is needed in the ARP – if not then the ARP does not need to be informed of the change.

Await decision within the I&P group on whether IMSI is needed.

## Customer changes MSISDN during Single IMSI contract

Use case is a number change within a single DSP.  Is it the role of the DSP to inform the ARP of the change or should the customer inform the ARP of the change.

Is there a need for an ‘Update’ procedure or does this imply termination of one contract and starting another one.

Process Description

Customer using a Single IMSI ARP changes his/her MSISDN with DSP, but the SIM card (IMSI) and DSP stay the same. Single IMSI ARP might need this information for billing purposes, in which case the DSP will inform the ARP about this change. Precondition for this is that the DSP is aware if MSISDN is used for billing purposes by the ARP.

Pre-conditions and Trigger

- Customer requests from DSP to change his/her MSISDN

- Customer is a Single IMSI ARP customer

Result of Process

- Customer has changed the MSISDN

- ARP is informed about the change

Assumptions

Precondition for the DSP to inform the ARP about MSISDN change is that the DSP is aware that MSISDN is used for billing purposes by the ARP.

### Process

Input for Process Step

- Customer wants to change MSIDSN (e.g. at POS)

- Customer is a Single IMSI ARP customer

Description of the Process Step

- DSP deactivates old MSISDN number.

- DSP assigns customer a new MSISDN (no change on IMSI) per regular procedure.

- Customer is informed about the changes by DSP.

- Change of MSISDN has impact on Single IMSI usage when it is used for invoicing and notifying.

Output/Result of the Process Step

- MSISDN changed

- Single IMSI should work (but is depending on the implementation of the Single IMSI ARP)

- DSP has to take care to transmit the information of the new MSISDN to the Single IMSI ARP, if ARP requires the information for billing reasons.

Remarks

MSISDN is needed as TAP files are also sent to Single IMSI ARP.

Post-paid: Single IMSI should work (but is depending on the implementation of the Single IMSI ARP) - tbc – alignment with Billing and I&P

Pre-paid: Single IMSI only works if the information is transmitted to Single IMSI ARP and he updates his systems. - tbc tbc – alignment with Billing and I&P

Reference

None

## Change of ownership of MSISDN at DSP

Telekom Austria

Process Description

Change of ownership (a.k.a. contract takeover) is made by contract holder (MSISDN, IMSI owner). Because the contracts with DSP and ARP are totally independent it is customer’s responsibility to manage any contract changes with each provider.

Pre-conditions and Trigger

- Customer wants to takeover an existing DSP contract

- Original contract holder is a Single IMSI ARP customer

Result of Process

- DSP contract change of ownership completed

Assumptions

None

### Process

Input for Process Step

- Customer requested contract takeover to another customer

- Old customer: holds DSP contract and Single IMSI contract

- New customer: requests DSP contract takeover from old customer

Description of the Process Step

- New customer is the owner of the existing  contract at the DSP (same MSISDN, IMSI, etc.).

- Single IMSI contract changes must be managed by the customer with the Single IMSI ARP directly, as DSP and ARP contracts are totally independent of each other.

Output/Result of the Process Step

- DSP Contract owner is changed

- Single IMSI ARP contract is unaffected

Remarks

None

Reference

None

## Error cases

## Open questions

# LBO

## Pre-requisite requirements for the DSP and LBO Provider

Telekom Austria

Process Description

Identifying and flagging the customers who are eligible for use data offers provided by an LBO.

Customers are informed about ability to use ARP and LBO providers in EU.

Identifying the roaming partners who are eligible to provide LBO offers to customers include those with whom DSP has EDGE/GRPS/UMTS agreement and is located in EU.

Pre-conditions and Triggers

- Eligibility rules applied to all customers and VPLMNs in EU

- LBO provider must fulfill certain condition in order to offer LBO services

Result of Process

- Eligible VPLMN LBO and eligible customers  are flagged in the DSP systems to allow providing and use of LBO services

- Eligible customers (existing and new ones) are informed about eligibility

- LBO provider is able and prepared to offer LBO services

Assumptions

none

### DSP pre-requisites

Step 1 - Flagging eligible customers and restricting non-eligible

Input for Process Step

- DSP customer base screening

- DSP is to ensure that the following customer eligibility criteria is adhered to:

  - In order to access the LBO offer, the user SHALL have data roaming capabilities.  That is:

    - Roaming option is not blocked due to customer request (High Level Tech document, section 3.3)

    - Data option is not blocked due to customer request (High Level Tech document, section 3.3)

    - Any LBO service is conditional on the customer’s data service: DSP shall not allow LBO usage in any PLMN if the customer is not in receipt of packet switch data services and Internet access (i.e. not restricted to private APN) ( High Level Tech Document, section 3.4.1)

  - Decision maker (DM) SIMs in a multi-user contract, such as business customer contract (only DM SIMs and those which are identified by the DM as eligible for LBO can use an LBO provider)

Description of the Process Step

The customers should be screened, flagged as eligible or restricted to use LBO services in EU in the DSP system(s)

This process should be setup to do continuous eligibility check of the customer base and flagging automatically as the customers’ eligibility is subject to change.

Output/Result of the Process Step

Customer is or is not eligible for using an LBO in EU:

- Customer is eligible for using LBO in EU -> VPAA flag set for EUInternet APN to Yes, valid for customers roaming within the EU

- Customer is NOT eligible for using LBO in EU -> VPAA flag set for EUInternet APN to NO, valid for customers roaming within the EU

Remarks

When customer is reactivated after suspension or the roaming service unbarring takes place, given the compliance to the other eligibility rules, the customer needs to be automatically flagged as eligible.

Physically only one LBO can be used at a time. However multiple LBO contracts can be active at one time. Even in the same country, customer would be able to have multiple LBOs, manually selecting networks and using the EUInternet APN.

Alternatively, customer who has a contract with a Single IMSI provider is free to also choose an LBO provider, having therefore a contract with a Single IMSI and an LBO provider at the same time.

Each SIM (under a MULTI-user contract such as a business customer contract) can have a different LBO provider and offer. We need to disable non-decision maker SIMs from being able to do LBO (not eligible), unless instructed otherwise by the decision-maker.

Reference

EU Reg 531/2012, article 4/3:

The switch to an alternative roaming provider or between roaming providers shall be free of charge for customers and shall be possible under any tariff plan.

Step 2 - Eligibility information to the customer

Input for Process Step

- Customers eligible for LBO port-out

Description of the Process Step

Customer will be informed about her/his ability to use ARPs and LBOs in EU based on the requirements in the regulation. Optionally, DSP could also inform customers about product/device specific conditions applying to customers when using LBO, such as:

- MMS service still provided by DSP

- Services not available when using LBO: Blackberry mail service; Blackberry Enterprise services; VPN service, any IP services

Output/Result of the Process Step

Customer is informed by DSP about her/his ability to use ARPs and LBOs in EU.

Remarks

none

Reference

See Impl reg, L 347/4, par 25 and Article 6

EU Reg, L172/25, article 4, point 4:

Domestic providers shall inform all their roaming customers in a clear, understandable and easily accessible form about the possibility to opt for services referred to in the first subparagraph of paragraph 1.

In particular, at the time of making or renewing a contract on mobile communication services, domestic providers shall provide all their customers individually with full information on the possibility to choose an alternative roaming provider and shall not hinder the conclusion of a contract with an alternative roaming provider. Customers concluding a contract with a domestic provider for regulated roaming services shall explicitly confirm that they have been informed of such possibility. A domestic provider shall not prevent, dissuade or discourage retailers serving as the domestic provider’s points of sale from offering contracts for separate roaming services with alternative roaming providers.

Step 3 - VPLMN LBO eligibility

Input for Process Step

- Screening the list of existing roaming agreements for packet switched data services in EU

Description of the Process Step

The roaming partners should be screened and flagged based on the following rules for eligibility:

- All roaming partners in EU with whom DSP has launched GPRS, EGDE or UMTS (incl. MVPLMNOs hosted by these roaming partners)

- DSP steering mechanism should enable customer to use LBO. Exception: the Technical Requirements Group assumes that network barring is allowed in the event the roaming agreement with the LBO provider is not CAMEL-enabled if delivering voice & SMS to the customer requires CAMEL. (see MoM from Tech Req Group Jan 15/16, point 13)

This process should to be set-up  to do continuous eligibility check and flagging automatically (i.e. in case of newly signed agreements).

Output/Result of the Process Step

VPLMN is flagged as eligible or not eligible for port-out in the systems.

Remarks

It could be that GSMA would build and support a database of operators who are providing LBO services on the territory of EU, which DSP could check if they needed to.

Reference

MoM from Tech Req Group Jan 15/16, point 13:

13. Use of HLR network barring against visited networks offering LBO services shall not be acceptable in the EU due to LBO-related DSP obligations (enabling instantaneous access to LBO services). Note: the Technical Requirements Group assumes that network barring is allowed in the event the roaming agreement with the LBO provider is not CAMEL-enabled if delivering voice & SMS to the customer requires CAMEL.

### LBO pre-requisites

Input for Process Step

LBO provider must be  able and prepared to offer LBO services, meaning:

- LBO provider must be located in EU

- LBO provider must have a roaming agreement for packet switched data services with a DSP whose customers it wants to target

- LBO provider must support EUInternet APN at SGSN and VPAA Flag at SGSN (in MAP messages)

- LBO provider must be able to provide customers with all LBO relevant information, including but not limited to the instructions on manual network selection, APN changes (to EUInternet APN setting and back to original DSP APN settings at the end of contract with LBO), LBO contract and offer conditions including pricing

- LBO must be able to provide customer with data anti-bill shock measures

- LBO must enable and communicate to the customers its touch points for customer service

Description of the Process Step

- LBO provider fulfills the basic criteria to act as an LBO provider

Output/Result of the Process Step

VPLMN is able to provide LBO services.

Remarks

None

Reference

None

## Service activation by the customer

Telekom Austria

Process Description

Customer is changing the roaming provider for data traffic in VPLMN. For all other services responsible is the DSP or an ARP customer has subscribed to.

Pre-conditions and Trigger

- Signed Roaming Agreement for packet switched data services between VPLMN and DSP

- VPLMN LBO is able to provide the LBO offer (all prerequisites are fulfilled)

- Customer is eligible to use the LBO offer

- Visited network (VPLMN) LBO offer is made available to the roaming customer

Result of Process

- Customer is subscribed to an LBO offer

Assumptions

none

### Process

Step 1 - Customer does location update in EU visited LBO network and receives offer from LBO

Input for Process Step

- Customer is registered in VPLMN (manual or automatic network selection).

- See process pre-conditions

Description of the Process Step

- Customer is making location update (LU) and receives LBO offer from the VPLMN.

Output/Result of the Process Step

- LU in VPLMN LBO completed and offer received by customer from VPLMN LBO.

Remarks

None

Reference

Impl reg, L347/3, par 15 to 18

Step 2 - Customer subscribes to the LBO offer

Input for Process Step

- Customer wants to take the LBO offer

- Eligibility restrictions set up in DSP system (and possibly in LBO system) will disable successful LBO offer take up for non-eligible parties

Description of the Process Step

- After customer inserted EUInternet APN and exchanged the necessary data with the LBO provider, he can start using the LBO offer (what information is exchanged and how is up to the LBO provider -> no effect on DSP/ARP)

Output/Result of the Process Step

- Customer activates service by LBO (EUInternet APN is set up to use LBO for data)

- The LBO is responsible for all data related roaming services toward customers, invoicing, CS, etc.

Remarks

- LBO offers data service only locally (in EU country where LBO is located), unless it offers a permanent LBO offer (then customer can continue to use LBO offer in other selected networks covered by the offer as long as the APN settings are correct)

- How the contract is signed and what kind the contracts will be signed, depends on LBO.

Reference

Imp reg, L347/6, article 4, point 4:

The donor roaming provider shall collaborate with the recipient roaming provider in order to ensure that roaming customers who have concluded a contract with a recipient roaming provider for the provision of local data roaming services are able to use the services provided by this provider instantaneously from the moment a recipient roaming provider sends a request to a donor roaming provider.

## Customer Service query

Telekom Austria

Process Description

The customer requests information from HN or complaints about different LBO scenarios. DSP, ARP and LBO clarify the request and provide the appropriate information to the customer for the services they are responsible for.

Pre-conditions and Trigger

- Customer is a DSP customer, could also be using an ARP and using LBO

- Customer requests information at any available DSP or LBO touch point

Result of Process

- Requested information has been given to customer by the responsible party

Assumptions

none

### DSP

Input for Process Step

Customer is/was/will be an LBO customer

Customer requests information about services that DSP is responsible for

Only DSP relevant information can be presented to the customer. DSP is probably unaware if customer is/was an LBO customer, unless customer identifies himself as such.

Description of the Process Step

- The customer contacts DSP (any touch point)  and requests the following information:

  - LBO availability (if DSP has a roaming agreement for data in place with the VPLMN, but this will not confirm if the VPLMN is an LBO provider)

  - His/her ability to subscribe to an LBO offer -> eligibility status

  - Non-available services when using LBO

  - Ability to adjust APN settings with customer's hardware

  - Set-up of the data limit with DSP and its applicability when using an LBO

    - Data cut-off service is  provided by DSP ONLY for the services still provided by DSP

  - Pricing SMS - standard procedure is followed. SMS text adaptation needed: price is valid only using HN

  - Data packages provided by DSP – validity and charging (does not include LBO traffic)

  - Any usage and roaming charges on the invoice issued by DSP (LBO traffic will not be visible)

  - If the customer was charged for data roaming when s/he was using LBO (case of double invoicing) and complains about it to DSP, the incident will be analyzed by DSP.

  - Customer wants to use data in roaming with DSP in current VN (switch back to DSP from an LBO) -> instructions given on APN settings

  - Customer wants to use data in roaming with DSP in new VN (new country or new VN in current country) -> instructions given on APN settings

  - Customer wants to use data national with HN -> instructions given on APN settings

Output/Result of the Process Step

- Customer request is clarified by the DSP.

Remarks

Reference

Impl reg, L 347/4, par 25 and Article 6

Impl reg, L 347/4, par 27:

Roaming providers, including alternative roaming providers offering local data roaming services, should implement transparency and safeguard mechanisms for the data services provided by them, in accordance with Article 15 of Regulation (EU) No 531/2012. In order to ensure transparency, roaming providers should also provide to their roaming customers information on services that may not be available when using local data roaming services, such as proprietary services supported by the domestic network.

EU Reg 531/2012, article 4/5:

…Where the switch (to ARP) does not concern all regulated roaming services, those services which have not been switched shall continue to be provided at the same price and to the fullest extent possible, with the same technical characteristics, incl. quality parameters.

### ARP

Input for Process Step

Customer is/was/will be an LBO customer

Customer requests information about services that ARP is responsible for

Only ARP relevant information can be presented to the customer. ARP is probably unaware if customer is/was an LBO customer, unless customer identifies himself as such.

Description of the Process Step

- The customer contacts ARP (any touch point)  and requests the following information:

  - Non-available services when using LBO

  - Ability to adjust APN settings with customer's hardware

  - Set-up of the data limit with ARP and its applicability when using an LBO

    - Data cut-off service is  provided by ARP only for the services provided by ARP

  - Pricing SMS – still provided by ARP in all of EU, including in the LBO network customer has chosen

  - Offers provided by ARP – validity and charging (does not include LBO traffic)

  - Any usage and roaming charges on the invoice issued by ARP (LBO traffic will not be visible)

  - If the customer was charged for data roaming when s/he was using LBO (case of double invoicing) and complains about it to ARP, the incident will be analyzed by ARP and customer’s DSP.

  - Customer wants to use data in roaming with ARP in current VN (switch back to ARP from an LBO) -> instructions given on APN settings

  - Customer wants to use data in roaming with ARP in new VN in EU (new country or new VN in current country) -> instructions given on APN settings

Output/Result of the Process Step

- Customer request is clarified by the ARP.

Remarks

Reference

### LBO Provider

Input for Process Step

Customer is/was/will be an LBO customer

Customer requests information about services that LBO is responsible for

Description of the Process Step

- The customer contacts DSP (any touch point)  and requests the following information:

- LBO subscription process

- LBO specific offers

- LBO offer and contract activation and deactivation processes

- Setup of the data limit with LBO (still under discussion)

- APN Setup (LBO must provide information on set up of EUInternet APN and reset to the DSP’s APN settings after LBO offer has expired)

- VPLMN selection

- Status of the LBO contract/service

- Non-available services when customer is using an LBO

- LBO related usage and charging, including information on pricing

- Issues with using the LBO network and/or offer, which could be a result of:

- suspension of LBO service by LBO

- deactivation of LBO services by LBO

- lack of coverage by LBO

- contract expiry with LBO

- inability to manually select LBO network

Output/Result of the Process Step

- Customer request is clarified by the LBO

Remarks

- LBO offers data service only locally (in EU country where LBO is located) or in multiple networks in EU via permanent LBO offer.

- All implications of the contract and offer types are a full responsibility of the LBO to properly inform the customer about.

Reference

Impl reg, L 347/4, par 25 and Article 6

## Customer is issued a bill

GSMA

Standard billing from DSP for anything not provided by LBO Provider.

LBO charging as per contract.

## Customer changes billing basis from LBO provider

GSMA

Process is within customer and LBO Provider control – no process required.

## Bill-shock measures

Starhome

### Send alerts to customer at cost thresholds

### Notify customer of cessation of service at threshold

## Customer requests change in terms of contract

Notification of change on (optional) LBO Provider – DSP interface?

### Change in usage limit

## Service deactivation at end of contract

Vodafone

## Service deactivation by the customer before end of contract

Vodafone

## Service deactivation by the LBO provider before end of contract

Vodafone

## Service deactivation by the DSP before end of contract

Vodafone

## Customer returns to DSP after LBO contract termination

Vodafone

## Customer ports out via MNP during LBO contract

Telekom Austria

Process Description

Customer does MNP to a new DSP network (port-out). Old DSP deactivates the SIM after port-out. That customer keeps the same MSISDN, but receives different IMSI. Customer may not be able to use the old LBO offer.

Pre-conditions and Trigger

- Customer wants to port-out MSISDN to a new DSP

- Customer is an LBO customer

Result of Process

- MSISDN is ported out to a new DSP

- LBO (may) not working

Assumptions

Eligibility rules for the customer and LBO may be different at the new DSP (i.e. roaming contract not in existence, roaming services suspension for the customer for new contract, etc.). LBO offer may also be based on customer’s IMSI as well, so with changed information, LBO can’t match customer’s info to continue to provide the service.

### Process

Input for Process Step

- Customer is an LBO customer

- Customer submits MNP port-out request

Description of the Process Step

- Customer receives new SIM card, therefore receives new IMSI, but MSISDN stays the same.

- LBO may not be working after port-out –> see process assumptions.

Output/Result of the Process Step

- MSISDN is ported out to a new DSP

- LBO (may) not working

Remarks

None

Reference

None

## Customer ports-in to new DSP via MNP during LBO contract

Telekom Austria

Process Description

Customer does MNP to a new DSP network (port-in). Old DSP deactivates the SIM after port-out. That customer keeps the same MSISDN, but receives different IMSI. Customer may not be able to use the old LBO offer.

Pre-conditions and Trigger

- Customer wants to port-out MSISDN to a new DSP

- Customer is an LBO customer

Result of Process

- MSISDN is ported out to a new DSP

- LBO (may) not working

Assumptions

Eligibility rules for the customer and VN LBO may be different at the new DSP (i.e. roaming contract not in existence, roaming services suspension for the customer for new contract, etc.). LBO offer may also be based on customer’s IMSI as well, so with changed information, LBO can’t match customer’s info to continue to provide the service.

### Process

Input for Process Step

- Customer is an LBO customer

- Customer submits MNP port-in request

Description of the Process Step

- Customer receives a new SIM card, therefore receives new IMSI, but MSISDN stays the same.

- LBO may not be working after port-out –> see process assumptions.

Output/Result of the Process Step

- MSISDN is ported out to a new DSP

- LBO (may) not working

Remarks

None

Reference

None

## Customer changes UICC card during LBO contract

Telekom Austria

Process Description

Customer changes her/his SIM card. DSP changes the SIM card (IMSI has changed, MSISDN stays the same). All settings (in DSP’s HLR) remain the same for the customer - HLR profile doesn't change.

That means that this could have an impact on LBO usage. This is an identical situation as when customer is doing an MNP port-out to a new DPS (see processes 4.14 and 4.15).

Pre-conditions and Trigger

- Customer wants to change SIM card from DSP

- Customer is LBO customer

Result of Process

- SIM card changed

- LBO could be still working depending on the T&C of the LBO (what customer information is the contract/charging based on)

Assumptions

LBO offer may also be based on customer’s IMSI, so with changed information, LBO can’t match customer’s info to continue to provide the service. If this is the case, LBO offer will not be valid any more for the customer. If IMSI information is not being used by the LBO, there will be no change to the ability to use the LBO offer.

There is no relation between the DSP contract and the LBO contract. Information is about customer’s particulars are not actively exchanged between DSP and the LBO.

4.14.1 Process

Input for Process Step

- Customer is an LBO customer

- Customer comes to DSP TP for SIM replacement: Customer wants to change SIM card (damaged, lost, stolen, micro/nano SIM, etc.)

Description of the Process Step

DSP gives customer a new SIM card with same MSISDN but a different IMSI per regular procedure. Customer bears the responsibility to inform the LBO provider about the IMSI change.

Two scenarios are possible:

a) Customer informs VPLMN LBO about the change and continues to use the existing LBO offer

b) Customer does not inform VPLMN LBO about the change: 1) If LBO contract is based on IMSI information, the customer will not be able to use the LBO offer any more; 2) If LBO contract is independent of the customer’s IMSI, the customer can continue to use the existing LBO offer

Output/Result of the Process Step

(in DSP’s HLR profile of the customer doesn't change)

a) LBO Services remain unchanged, OR

b) LBO Services not working

Remarks

The regulation does not specify any details on the basis for contract (e.g. basis for the LBO contract to be: IMSI, only MSISDN, etc.). It will be up to the LBO provider to identify their customers. If they are not able to identify their customers anymore because of changes (IMSI, MSISDN, etc.) it is up to the customer to provide up-to-date information to the LBO provider. Such cases, where customer information is not accurate anymore, will have to be covered by the LBO T&Cs.

If customer is replacing a stolen SIM card, standard stolen SIM card blocking and replacement procedures take place. Customer would want to communicate the stolen card also to the LBO proider if he believes his LBO credentials may be at risk.

Reference

None

## Customer changes MSISDN during LBO contract

Telekom Austria

Process Description

Customer who is using an LBO changes his/her MSISDN with the DSP, but the SIM card (IMSI) stays the same. That may have influence on LBO usage.

Pre-conditions and Trigger

- Customer wants to change MSISDN from DSP

- Customer is an LBO customer

Result of Process

- MSISDN is changed

- LBO may not be working depending on the T&C of the LBO (what customer information is the contract based on)

Assumptions

LBO offer may be based on customer’s MSISDN, so with changed information, LBO can’t match customer’s info to continue to provide the service. If this is the case, LBO offer will not be valid any more for the customer. If MSISDN information is not being used by the LBO, there will be no change to the ability to use the LBO offer.

There is no relation between the DSP contract and the LBO contract. Information is about customer’s particulars are not actively exchanged between DSP and the LBO.

### Process

Input for Process Step

- Customer wants to change MSIDSN

- Customer is an LBO customer

- Customer comes to DSP POS for replacement

Description of the Process Step

- DSP deactivate old MSISDN number. Old MSISDN goes into quarantine.

- DSP assigns customer a new MSISDN (no change on IMSI) per regular procedure.

- Customer is informed about all changes by DSP.

- HLR settings stays the same. Change of MSISDN has no impact on LBO usage on DSP side.

- On LBO side, customer may not be able to use data, if contract is based on MSISDN information.

Two scenarios are possible:

a) Customer informs VPLMN LBO about the change and continues to use the existing LBO offer

b) Customer does not inform VPLMN LBO about the change: 1) If LBO contract is based on MSISDN information, the customer will not be able to use the LBO offer any more; 2) If LBO contract is independent of the customer’s MSISDN, the customer can continue to use the existing LBO offer

Output/Result of the Process Step

MSISDN is changed (in DSP’s HLR profile of the customer doesn't change)

a) LBO Services remain unchanged, OR

b) LBO Services not working

Remarks

- Contract (SIM) owner still the same, equals to one that signed LBO contract.

- Contract holder can make change of MSISDN.

- See other remarks in the process 4.16

Reference

None

## Customer changes hardware during contract

Telekom Austria

Process Description

Customer changes the hardware and is an LBO user. Changing of hardware means that APN needs to be changed manually in the phone. In DSP’s HLR, customer’s  profile stays the same.

Pre-conditions and Trigger

- Customer has changed the hardware

- Customer is an LBO customer

Result of Process

- If APN setting are set to EUInternet APN settings on the new hardware, services remain unchanged

- If APN settings are set to DSP default settings, LBO service will not work

Assumptions

none

### Process

Input for Process Step

Customer received new hardware and puts current SIM card into the new phone.

Description of the Process Step

Customer starts using new hardware.

a) VPLMN LBO APN is configured correctly (manually by customer)

b) VPLMN LBO APN is not configured correctly for LBO use (i.e. customer has DSP APN settings)

Output/Result of the Process Step

- LBO services remain unchanged if the customer changes the APN settings to the EUInternet APN.

- If not, the customer is not able to make any usage via LBO, but data will be provided by the DSP.

Remarks

APN settings change for usage of LBO services are the responsibility of the LBO provider. See process  4.3.

Reference

None

## Error cases

### User takes LBO but incorrectly configures APN

### User attempts to use LBO APN but without a LBO contract with a visited network provider

### User attempts to use LBO with EU Universal APN, but in country outside of EU

### Non-EU customer attempts to use LBO

# COMBINE SINGLE imsi/lbo case

## Single IMSI subscriber, subsequently subscribes to LBO

# Example  process template

## <process name>

Process Description

High-level description of the should-be process including, but not limited to process goal, purpose, benefits.

Pre-conditions and Trigger

Start of the process with a triggering event and possible pre-conditions. Usually processes start at the customer.

Result of Process

End of the process with a result. Usually Processes end at the customer.

Assumptions

Any key assumptions used for creating a process.

### Process

Include a process diagram? May not be necessary for all process but for complex or lengthy processes, should consider including a process diagram. To discuss in face to face, 7/2/13.

Try this with a ‘well thought’ process and refine.

Step x (repeat for as many steps as included in the process

Include;-

- Description of the step

- Actions taken within the step by parties involved

- References to relevant sections within

  - Technical Requirements document

  - BEREC recommendations

  - EC Implementation Act and/or Regulations

- Decision points (If x, then y, else z).

- Any additional detail that is relevant

- Any open questions (please highlight to show these are open points to be resolved)