Change Request

| Title: | Provide data to the specified PNE(s) in the same Service Group | Public OMA Confidential |
| --- | --- | --- |
| To: | CPNS WG |  |
| Doc to Change: | OMA-AD-CPNS-V1_0-20100111-D |  |
| Submission Date: | 15 Jan 2010 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Clerical |  |
| Source: | Jean-Francois Deprun, LG Electronics, jf.deprun@lge.com Youn-Sung Chu, LG Electronics, ys.chu@lge.com |  |
| Replaces: |  |  |

Reason for Change

This CR tries to specify the scheduling to provide data to the specified PNE(s) in the same Service Group.

The CPNS-HLF-038 requirement is mandatory. “The CPNS Enabler SHALL provide data to the specified PNE(s) in the same Service Group.”

The information about this requirement will be added in the 5.3.1.11 Service/Content Delivery.

The working hypotheses are:

- The SG management is already define

- The SG information is saved in the CPNS enabler and CPNS entities can access to this information.

- In a CPNS enabler it is possible to have several SG. A CPNS entity can belong to

  - several SG,

  - only one SG,

  - no SG.

- The CPNS server manages these SG. It can remove and add SG. It can add and remove CPNS entities to SG.

- In a SG, there is a SG manager (PNE, PN GW or CPNS server).

This CR doesn’t describe the SG management.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

To the group to review this input and agrees to incorporate it in the CPNS AD.

Detailed Change Proposal

Insert function chapter  in section 5.3.2.1

5.3.1.14.1 Service/Content Delivery without SG

5.3.1.14.2 Service/Content Delivery in a SG

5.3.1.14.2.1 A generic representation

A generic representation of a CPNS enabler can be:

Figure a

The PAN Area allows showing the possible communications between several CPNS entities. In this figure, PN GW 1 and PN GW 2 can directly communicate together (with the CPNS-3 interface).

The PNE1 can communicate directly with the GW1 and GW3.

The CPNS Enabler SHALL provide data to the specified PNE(s) in the same Service Group.

This Data can come from:

- An external CPNS entity

- A CPNS entity:

  - A PNE

  - A PN GW

  - A CPNS server

A PN GW has information about the CPNS entities which is in contact. E.g figure a, the GW 3 has information about GW 4, PNE 5, PNE 1, GW 1 and CPNS server:

During the initialization of a SG or during its update, the information about the members of the SG is shared: each member of the SG has the list of all the members of the SG. This allows to the user or to the application to communicate (share data, send data…) with the other member.

- Message 1: the PNE send a request to the PN GW to belong to the Service Group

- Message 2: the PN GW forwards this request to the CPN Server.

- Action A: The CPNS server asks the SG manager to know if it is possible to add this new PNE in the SG. In this figure the answer is yes. The CPNS server updates the SG List.

- Message 3 and 4: the CPNS server sends the answer to  the PNE 1 throught the GW 1

- Message 5: the CPNS sends to the GW belonging to the SG the new SG list (the SG list updated)

- Action B: the GWs update their SG list

- Message 6: the the GWs send to the PNE belonging to the SG the new SG list (the SG list updated).

- Action C: the new member of the SG – PNE1 – creates and updates its SG list.

Each PNE has the list of all the SG members. With the ID of the members, some more information can be added (e.g. name of the user, type of device…). The PNE can have several lists if it belongs to several SG (one list for one SG). The name of this list is SG List.

Each GW has several lists:

- SG Lists: for each Service Group, the GW belonging to a SG has the list of the member of the SG. A GW can have several SG list if it belongs to several SG (one list for one SG). This list can be the same tha the list saved in PNEs. The name of this list is SG List.

- Its Neighbour List; each GW has the list of all the CPNS entities which is in contact (e.g. for the GW1: GW 3, PNE2, PNE 1, CPNS server). The name of this list is N List.

The CPNS server has all the complete SG lists:

- One SG list for one SG. The name of this list is GSG  List (Global SG List).

- The GSG list summarizes all the SG members of the same SG with their information if needed (user name, type of devie…). The graph of the SG is saved in the CPNS server (in a list or other). The graph allows to the CPNS server to find each CPNS SG member (PNE 5 -> GW 3 -> server; PNE 6 ->GW 4 -> Server…)

5.3.1.14.2.1.1 Data comes from Internal CPNS entity

A CPNS Entity can be a PNE, NN GW or CPNS server.

Figure b

Only the scheduling of the process to data comes from PNE is specified because:

- In point of view of PNE and PN GW, the case of CPNS server sends data to other CPNS entities is the same than data comes from External CPNS entity. Then see “5.3.1.14.2.1.2 Data comes from External CPNS entity”

- In case of the data comes from PN GW, the data comes from the PNE Logical Entity inside the PN GW Physical Entity.

To optimize the data transfer in the CPNS enabler, 3 cases are defined:

- No GW in common: The CPNS receiver and the CPNS sender are not in contact with the same PN GW and it is not possible to send data through only PN GW (e.g. PN1 and PN3 figure a).

- One GW in common: The CPNS receiver and the CPNS sender are in contact with the same PN GW (e.g. PN1 and PN2 figure a)

- A GW link: The CPNS receiver and the CPNS sender can communicate through PN GWs (e.g. PN2 and PN6 figure a)

5.3.1.14.2.1.1.1 Data comes from Internal CPNS entity:  First case: No GW in common.

The data can be sent to a single entity or can be shared between all the SG members.

This figure above describes the case when a PNE sends data but the receiver is not in the SG List. This case is out of the scope of this document and it is not described in this part of document.

This figure above describes the case when a PNE sends data and the sender and receiver belong to SG (their ID are in the SG List) (Action A).

Action B: The receiver is not in contact with the GW 1. The GW sends the data to the CPNS server.

Action C: The CPNS server finds the GW in contact with the receiver, thanks to the SG graph. The CPNS server sends the data to this GW (message 3).

Action D: The GW receives the data from the server, identifies the PNE (the GW can use is N List) and sends the date to the PNE.

The next figure describes the case when the data is shared between all the SG entities:

The PNE 1 sends the data which will be shared in the SG to the GW.

Action A: the GW verifies if the sender belongs to the service group. If no, the data is not forwarded and a message is send to the CPNS server. If yes, the data is sent to the CPSN server with the Identity of the sender, and the SG ID.

Action B: the CPNS server identifies all the members of the SG and all the GW in contact with them. The CPSN server sends the data to the GW.

Action C: the GW(s) use the SG List and N List to share the data between the SG PNE.

Action D: in the same time, the first GW uses its N List to identify all the PNE near it and sends the data to them.

5.3.1.14.2.1.1.2 Data comes from Internal CPNS entity: The Second case: GW in common.

In this case, the receiver and the sender are in contact with the same GW.

Figure c

5.3.1.14.2.1.2 Data comes from External CPNS entity

Figure d

5.3.1.14.2.1.3 Data transmission with CPNS server forward

The communication between 2 PNE can be done through GW only:

Figure e

The communication between 2 PNE can be done through GW and PNE only:

Figure f

When the CPNS enabler is done, the CPNS server will ask each GW to know with which CPNS entity they can communicate.

For example in figure e:

- PNE 2:		GW1

- GW1:		PNE2, PNE1, GW 3, CPNS server

- PNE1:		GW 1, GW 2,

- GW 3:		GW 1, PNE 1, PNE 5, GW 4, CPNS server

- PNE 5:		GW 3

- GW 4:		GW 3, PNE 6, CPNS server

- PNE 6:		GW 4

- PNE 6:		External Entity, GW 2

- GW 2:		PNE 6, PNE 4, PNE 3, CPNS server

- PNE 4 :		GW 2

- PNE 3 :		GW 2

With this information the CPNS server will do graph and find the entire possible path between the CPNS entity.

For example, it is possible to have a communication between the PNE2 and PNE 6 through only GW:

PNE 2 – GW 1 – GW 3 – GW 4 – PNE 6.

The CPNS server will send specific information for each vertex of the graph.

In the case above, the GW 1, GW 2 , GW 3 and GW 4 will be receive information than they can transmit information to the PNE 2and PNE 6 without CPNS server..

To implement these features, several lists are needed in each GW :

- SG Lists: for each Service Group, the GW belonging to a SG has the list of the member of the SG. A GW can have several SG list if it belongs to several SG (one list for one SG). This list can be the same tha the list saved in PNEs. The name of this list is SG List.

- Its Neighbour List; each GW has the list of all the CPNS entities which is in contact (e.g. for the GW1: GW 3, PNE2, PNE 1, CPNS server). The name of this list is N List.

These 2 Lists are used to manage and optimize the CPNS enabler.