| Open Connection Manager API Architecture |  |  |  |
| --- | --- | --- | --- |
| Draft Version 1.1 – 25 Mar 2013 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-AD-OpenCMAPI-V1_1-20130325-D |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2013 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

Contents

1.	Scope (Informative)	4

2.	References	5

2.1	Normative References	5

2.2	Informative References	5

3.	Terminology and Conventions	6

3.1	Conventions	6

3.2	Definitions	6

3.3	Abbreviations	6

4.	Introduction (Informative)	8

4.1	Version 1.0	8

4.2	Version 1.1	8

5.	Architectural Model	9

5.1	Dependencies	9

5.2	Architectural Diagram	9

5.3	Functional Components and Interfaces	9

5.3.1	OpenCMAPI Enabler Functional Components	9

5.3.2	Interfaces	10

5.4	Security Considerations	11

5.5	Charging Considerations	12

5.6 Functional Modules	12

5.6.1 Group Management	12

5.6.2 Status Management	12

5.6.3 Statistics Management	12

5.6.4 Power Management	12

Appendix A.	Change History (Informative)	13

A.1	Approved Version History	13

A.2	Draft Version 1.1 History	13

Appendix B.	Flows (informative)	14

Appendix C.	Deployment Examples (Informative)	15

C.1	Deployment example of OpenCMAPI	15

C.2	Deployment Example WebAPI	16

Figures

Figure 1: OpenCMAPI Architectural Diagram	9

Figure 2: OpenCMAPI Deployment Example	15

Figure 3: OpenCMAPI WebAPI Deployment Example	16

# Scope	(Informative)

This document provides the architecture for the OpenCMAPI 1.1 Enabler. This architecture is based on the requirements as listed in the OpenCMAPI Requirement Document [OpenCMAPI-RD].

The objective of the OpenCMAPI Enabler is to the define the OpenCMAPI Connection Management component and the interfaces exposed by this component to applications (CM applications or non CM applications) that rely on OpenCMAPI Enabler to work or to provide additional functionalities.

# References

## Normative References

| [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |
| --- | --- |
| [OpenCMAPI-RD] | “Open Connection Manager API Requirements, Version 1.1”, Open Mobile Alliance™, OMA-RD-OpenCMAPI-V1_1 URL:http://www.openmobilealliance.org/ |
| [OpenCMAPI-TS] | “Open Connection Manager API Technical Specification, Version 1.1”, Open Mobile Alliance™, OMA-TS-OpenCMAPI-V1_1 URL:http://www.openmobilealliance.org/ |

## Informative References

| [OMADICT] | “Dictionary for OMA Specifications”, Version 2.9, Open Mobile Alliance™, OMA-ORG-Dictionary-V2_9, URL:http://www.openmobilealliance.org/ |
| --- | --- |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

## Definitions

| Asynchronous Interface | Is an interface that allows transfer of information, i.e. requests and responses, between entities e.g. applications and OpenCMAPI connection management component, without having to synchronize (e.g with a controlling clock). The requested action may be started in a separate thread of execution and called party, in this case OpenCMAPI connection management component, may respond to the caller usually immediately but anyway before the requested action is complete. |
| --- | --- |
| HotSpot 2.0 | Is a set of specifications from Wi-Fi Alliance. Key features include Wi-Fi roaming and support of seamless connectivity to Wi-Fi Networks. |
| RESTful | Is a Web Service design model that exploits the existing Web technologies and protocols, HTTP and XML to manage the Web content and application behaviour. |
| Synchronous Interface | Is an interface which requires synchronization (e.g. with a controlling clock) and is dependent on the completion of the interaction. Called party, in this case OpenCMAPI connection management component, will keep the caller waiting until the transaction is complete. |

## Abbreviations

| API; Application Programming Interface. CM; Connection Manager. GNSS; Global Navigation Satellite System. D2D; Device to Device. GNSS; Global Navigation Satellite System. LTE; Long Term Evolution. OMA; Open Mobile Alliance. OpenCMAPI; Open Connection Manager (CM) Application Programming Interface (API). P2P; Peer to Peer. PIN; Personal Identification Number. PLMN; Public Land Mobile Network. PUK; Pin Unlocking Key. SIM; Subscriber Identity Module. SMS; Short Message Service. TDD; Time Division Duplex. UICC; Universal Integrated Circuit card. USSD; Unstructured Supplementary Service Data. UTRAN; UMTS Terrestrial Access Network. WCDMA; Wideband Code Division Multiple Access. WLAN; Wireless Local Area Network. |  |
| --- | --- |

# Introduction	(Informative)

This document defines the architecture of the OpenCMAPI 1.1 Enabler based on the OpenCMAPI Requirement Document [OpenCMAPI-RD].

The focus of the OpenCMAPI is the standardization of functional interfaces for access to connection management.

In this context, this Architecture Document defines functional component and interfaces related to the connection management aspects.

## Version 1.0

Version 1.0 of this architecture document covers architectural aspects related to the interfaces for the following areas:

- Security and concurrency control function , e.g. access control and authorization

- Cellular Network Connection Management,

- WLAN connection management,

- Statistics handling

- Information Status handling

- GNSS handling

- Power management

- SMS&USSD management

- PIN/PUK handling

- UICC Interaction

- Push Data service management

## Version 1.1

In addition to Version 1.0, Version 1.1 of this architecture document covers architectural aspects related to the interfaces for the following areas:

- WebAPI (HTTP protocol binding of the interfaces using the RESTful architectural style)

- Phone Book /Contacts management

- WLAN connection management according to Hotspot 2.0

- P2P Direct connection management (for both Wi-Fi and LTE)

- Cellular Network Connection Management of UTRAN Network type (WCDMA TDD)

However, this release of the AD does not address the requirements that were deferred for future release.

# Architectural Model

The OpenCMAPI 1.1 architecture is based on the requirements defined in [OpenCMAPI-RD].

Following sections describe OpenCMAPI architecture diagram, functional components and interfaces.

## Dependencies

There are no dependencies.

## Architectural Diagram

CMAPI-2

OpenCMAPI

Connection

Management

CMAPI-1

Application(s)

Device(s)

Legend:

Components specified by this Enabler

Components not specified by this Enabler

Interface specified by this Enabler

Applications (Browser)

CMAPI-3

Figure 1: OpenCMAPI Architectural Diagram

## Functional Components and Interfaces

### OpenCMAPI Enabler Functional Components

#### OpenCMAPI  Connection Management component

The OpenCMAPI Connection Management component supports the following functions:

- Security and concurrency control functions, e.g. access control and authorization

- Registration management for notifications (i.e. callbacks)

- Notifications (i.e. callbacks)

- Cellular Network Connection Management (including UTRAN Network type (WCDMA TDD))

- WLAN connection management (including Hotspot 2.0),

- Statistics handling

- Information Status handling

- GNSS handling

- Power management

- SMS&USSD management

- PIN/PUK handling

- UICC Interaction

- Push Data service management

- Phone Book /Contacts management

- P2P Direct connection management (Wi-Fi and LTE)

- WebAPI functions

Furthermore, on a given host, only one instance of the OpenCMAPI connection management component SHALL be running at a time. Therefore, all applications, using the OpenCMAPI connection management component, SHALL use and register with this instance.

This component exposes the CMAPI-1, CMAPI-2 and CMAPI-3 interfaces.

#### Entities external to the OpenCMAPI enabler    (Informative)

##### Application(s)

The application(s) use the functional representations defined by the OpenCMAPI enabler.

Two types of applications will be able to use the OpenCMAPI enabler:

- The CM application with the capability to access to all APIs provided by the OpenCMAPI enabler.

- The non-CM application is an external entity that has the capability to access to only a subset of APIs provided by the OpenCMAPI enabler.

In the case of a browser application, it will use CMAPI-3 interface.

##### Device(s)

The OpenCMAPI manages the network access connectivity through the device. The interface between the device and the OpenCMAPI Connection Management component is out of scope for the OpenCMAPI enabler and depends on what the device implementation provides (e.g. device driver calls, AT commands, messages).

In general, when an application invokes an OpenCMAPI enabler function, the OpenCMAPI connection management component translates the function call into an operation on the device interface. The OpenCMAPI enabler facilitates the communication of information from the device to the application, such as success/failure codes, requested information, and asynchronous notifications.

It should be noted that the set of functions exposed by an implementation of the OpenCMAPI Connection Management component is subject to the capabilities of the device(s) being managed.  For example, an implementation managing a device with only cellular connectivity and no WLAN and/or GNSS would support the OpenCMAPI APIs needed to manage the cellular connection, but would not support the APIs specific to WLAN and GNSS.

### Interfaces

Applied to all interfaces stated below, all memory is caller allocated, the callee SHALL NOT modify any memory unless otherwise noted.

#### CMAPI-1

This interface is exposed by the OpenCMAPI Connection Management component and supports the following functions:

- Cellular Network Connection Management (including UTRAN Network type (WCDMA TDD)),

- WLAN connection management (including Hotspot 2.0),

- Statistics handling,

- Information Status handling,

- GNSS handling,

- Power management,

- SMS&USSD management,

- PIN/PUK handling,

- UICC Interaction,

- Push Data service management,

- Security and concurrency control function,

- Phone Book /Contacts management,

- P2P Direct connection management (Wi-Fi and LTE).

The CMAPI-1 interface is mainly a Synchronous Interface with maximum timeout and possibility of cancellation.

However, for long operations (typically more than 7 seconds) before the result is available, Asynchronous versions of the API functions are provided in addition to the Synchronous versions.

#### CMAPI-2

This interface is exposed by OpenCMAPI Connection Management component and supports the following:

- Registration to receive notifications (i.e. callbacks),

- Notifications (i.e. callbacks),

- Security and concurrency control function.

This interface is an Asynchronous Interface.

#### CMAPI-3

This interface is exposed by OpenCMAPI Connection Management component across a WebAPI (i.e. a HTTP protocol binding using the RESTful architectural style).

A subset of functions exposed through CMAPI-1 and CMAPI-2 are supported.

## Security Considerations

The security considerations described in this section apply to any OpenCMAPI enabler implementation, and these considerations may result in different deployment models. Any particular security mechanism relevant to OpenCMAPI enabler is addressed in the [OpenCMAPI-TS].

Any deployment of OpenCMAPI enabler needs to ensure that all the applications requiring access to information provided via the specified API exposed by the OpenCMAPI connection management component are subject to the following security considerations:

- All applications should be authenticated and authorized

- OpenCMAPI implementation decides what level of access applications can have based on some policies, e.g. Service Provider policies.

## Charging Considerations

Not applicable.

## 5.6 Functional Modules

### 5.6.1 Group Management

This function of CMAPI enabler enables a Local Device to manage groups within the P2P context.

Actions performed by the Local Device through this function are as following:

- Creation of group for P2P direct

- Removal of group

- Joining multiple groups

- Leaving a group

- Removal of Remote Devices from groups

- Invite Remote Devices to join groups

- Retrieval of device information belonging to the same groups

- Refusal of invitations from Remote Devices to join groups

- Prevention of Remote Device from joining groups

### 5.6.2 Status Management

Through this function, CMAPI enabler collects, retrieves and provides different status information.

Status information retrieved and provided by this functional module includes the following:

- Connection status, e.g. connected, disconnected, connecting, disconnecting etc.

- Network status, e.g. registered, out of service etc.

- Device status, e.g. on, off, plugged, unplugged, available, unavailable etc.

- PIN status, e.g. enabled, blocked, unblocked etc.

- Roaming status, e.g. roaming, not roaming etc.

### 5.6.3 Statistics Management

Through this function, CMAPI enabler retrieves and provides different statistic information.

Statistic information retrieved and provided by this functional module includes the following:

- Number of kB sent, received, uploaded, downloaded etc.

- Transmission speed, average and maximum speed of upload and download

### 5.6.4 Power Management

Through this function, CMAPI enabler supports some power management aspects that improve battery life and connectivity service

Power information retrieved and provided by this functional module includes the following:

- Power state, e.g. hibernated, standby, on, off

- Battery status, e.g. battery level, charging, not charging

- Power source, e.g. battery, external

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| n/a | n/a | No prior version –or- No previous version within OMA |

Draft Version 1.1 History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-AD-OpenCMAPI-V1_1 | 22 Nov2012 | Baseline | Incorporates the baseline AD based on the AD version 1.0: OMA-CD-OpenCMAPI-2012-0001R01-INP_Architecture_Baseline |
|  | 22 Feb 2013 | ALL | Incorporated the following CRs: OMA-CD-OpenCMAPI-2013-0025R01-CR_ADRR_Editorial OMA-CD-OpenCMAPI-2013-0026R01-CR_ADRR_WebAPI And updated according to resolution of ADRR comments closed on 20130221: A001, A002, A003, A004, A005, A006, A008, A009, A010, A011, A012, A013, A014, A015 |
|  | 25 Mar 2013 | 5.6, App C | Incorporated: OMA-CD-OpenCMAPI-2013-0030-CR_Resolution_of_ADRR_comment_A016 OMA-CD-OpenCMAPI-2013-0029R01-CR_AppendixC_CMAPI3 |

Flows (informative)

Flows can be found in [OpenCMAPI-TS].

Deployment Examples (Informative)

Deployment example of OpenCMAPI

The following diagram shows an example of a possible deployment of the OpenCMAPI.

Non-CM

Application

Proprietary

Implementation

CM

Application

CMAPI2

CMAPI1

CMAPI2

CMAPI1

OpenCMAPI contract

Cellular

Security & Concurrency

Control

WWAN

WLAN

Statistics

OpenCMAPI

Implementation

GNSS

Host

Push Data service

UICC

PIN/PUK

Status

Power

SMS & USSD

Device

Service

Device

Discovery

U2

U1

Driver Layer

U2

U1

Proprietary

Implementation

Device

Device

Device

Proprietary

U1 & U2: Proprietary Device interfaces

CMAPI 1 & 2: OpenCMAPI specified interfaces

Figure 2: OpenCMAPI Deployment Example

In the diagram, the term “Device” could represent different types of devices (such as cellular, WLAN, USB modem, embedded modem …) through which the CMAPI provides network connectivity to the “Host”.

The OpenCMAPI connection management component is the OpenCMAPI implementation on the device side.

The OpenCMAPI connection management component could be logically decomposed in two high level internal functions as follows:

- OpenCMAPI security and concurrency control function

The security and access control function controls the interaction between OpenCMAPI Functional layer and the application layer. It grants full access to connection manager applications and grants only (read only) access to a subset of functions to non-CM applications.

##### OpenCMAPI sub functions

The OpenCMAPI sub functions include: cellular network connection management, WLAN management, PIN/PUK management, etc which can be derived from [OpenCMAPI-RD].

Deployment Example WebAPI

The following diagram shows an example of a possible deployment of the WebAPI.

Host

Proprietary

Implementation

Applications

(Browser)

OpenCMAPI contract

CMAPI3

Driver Layer

Proprietary

Implementation

HTTP Server

Security & Concurrency

Control

Cellular

GNSS

WWAN

WLAN

Statistics

OpenCMAPI

Implementation

Device

Push Data service

UICC

PIN/PUK

Status

Device

Discovery

Device

Service

SMS & USSD

Power

U4

U3

Device

Proprietary

Implementation

Proprietary

U3 & U4: Proprietary Device interfaces

CMAPI 3: OpenCMAPI WebAPI enabler interfaces

Figure 3: OpenCMAPI WebAPI Deployment Example

In the figure, an example is shown where an application within the context of a web browser interfaces with a device via CMAPI3.  In this scenario, typically nothing is installed on the Host.  The driver layer is an inbox operating system WLAN driver.  The OpenCMAPI implementation resides on the device itself and is provided via an embedded HTTP server.  Below the OpenCMAPI implementation on the device there is an interface to proprietary firmware.

The functions in the OpenCMAPI implementation in Figure 3 may be a subset of functions provided via CMAPI1 and CMAPI2 but all functional components are illustrated here.