Change Request

| Title: | CR_TS_Control_Plane_RFC3265_Updates_Pt3 | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA COM PCPS |  |
| Doc to Change: | OMA- PCPS-TS_Control_Plane-V1_0-20140525-D |  |
| Submission Date: | 25 May 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Bob Fredericks – Motorola Solutions; |  |
| Replaces: | n/a |  |

Reason for Change

This change request updates the Control Plane TS with regard to references to obsolete RFC3265.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The PCPS group is recommended to agree to the changes and incorporate it into the corresponding document.

Detailed Change Proposal

- Updating normative reference RFC3265 to RFC6665

From 7.2.1.11.1, pg. 150:

.

.

.

NOTE: 	A local policy, e.g. number of simultaneous subscriptions exceeded, may cause the PoC Server to reject the subscription request.

5. SHALL create a subscription to the Participant Information of the PoC Session according to rules and procedures of [RFC6665] and [RFC4575];

6. SHALL generate a SIP 200 "OK" or a SIP 202 "Accepted" response to the SIP SUBSCRIBE request according to rules and procedures of [RFC6665] and [RFC4575];

7. SHALL set the Contact header of the SIP response to the address of the PoC Server;

8. SHALL include the Authenticated Originator's PoC Address as specified in subclause 5.2 "Authenticated Originator's PoC Address" as follows:

- a) set to the Conference-factory-URI in case of 1-1 PoC Session or Ad-hoc PoC Group Session; or

- b) set to the PoC Group Identity of the PoC Group with the Session Type uri-parameter "session=prearranged" or "session=chat" as specified in E.5.1 "Session Type uri-parameter" in case of a Pre-arranged or Chat PoC Group respectively.

- .

- .

- From 7.2.1.11.2, pg. 151:

##### Generating a SIP NOTIFY request

The PoC Server SHALL generate a SIP NOTIFY request according to rules and procedures of [RFC6665] with the clarifications in this subclause.

If status is "on-hold", "alerting" and "dialing-out" the SIP NOTIFY request SHALL NOT be sent if sending Limited Participating Information only.

- The PoC Server MAY support Limited Participant Information.

- .

- .

- From 7.2.1.11.3, pg. 153:

- .

- .

##### Terminating the subscription

The PoC Server

1. SHALL terminate all subscriptions for Participant Information for the PoC Session when the PoC Session is released and not accept any re-subscriptions;

2. MAY terminate the subscription for a PoC Client when it leaves the PoC Session;

3. for each subscription that shall be terminated the PoC Server:

- a) SHALL generate a SIP NOTIFY request according to rules and procedures specified in [RFC6665];

- b) SHALL include a Subscription-State header with the value of 'terminated' and a reason parameter of 'noresource'; and,

- c) SHALL send the SIP NOTIFY request to the PoC Client according to rules and procedures of SIP/IP Core.

- .

- .

- From 7.2.1.17, pg. 160:

- .

- .

#### 7.2.1.17 Generating a SIP NOTIFY request to the SIP REFER request

When generating a SIP NOTIFY request the PoC Server:

1. SHALL generate a SIP NOTIFY request according to rules and procedures of [RFC6665], [RFC3515] and rules and procedures of the SIP/IP Core;

2. SHALL include in the SIP NOTIFY request a MIME sipfrag body as specified in the [RFC3420], if generating a SIP NOTIFY request as the result of sending a SIP INVITE request, with:

- a) the Status-Line received in the SIP response, (e.g. SIP 180 "Ringing" or SIP 200 "OK"), as specified in [RFC3261]. In case no SIP response is received the SIP 100 "Trying" SHALL be used;

- b) the To header as received in the SIP response;

- .

- .

- From 7.2.1.17, pg. 160:

- .

- .

- c) the Authenticated Originators PoC Address as if received in the SIP response;

- d) the Warning header if it is received in the SIP response;

- e) the P-Answer-State header if it is received in the SIP response from the Invited PoC User; and,

- f) the Contact header if it is received in the SIP response.

3. SHALL include in the SIP NOTIFY request a MIME sipfrag body as specified in the [RFC3420] with the Status-Line received in the SIP response, (e.g. SIP 200 "OK"), as specified in [RFC3261], if generating a SIP NOTIFY request as the result of sending a SIP BYE request. In case no SIP response is received the SIP 100 "Trying" SHALL be used;

4. SHALL include a Resource-Priority header, according to rules and procedures of [RFC4412], if the PoC Server supports 'Official Government Use' QoE Profile, and if a Resource-Priority header was included in the received SIP REFER request; and,

5. SHALL send the SIP NOTIFY request to PoC Client according to rules and procedures of the SIP/IP Core.

The responses to the SIP NOTIFY request SHALL be handled in according to rules and procedures of [RFC6665], [RFC3515], and rules and procedures of the SIP/IP Core.

The PoC Server MAY terminate the implicit subscription created by the SIP REFER request and indicate the termination to the PoC Client according to rules and procedures of [RFC6665] and [RFC3515].

- .

- .

- From 7.2.1.30.1, pg. 172:

- .

- .

Upon reception of a SIP NOTIFY request that is part of the same SIP dialog as the previously sent SIP SUBSCRIBE request from the PoC Server as a result of an action specified in subclause 7.2.2.10.1 "Subscription initiated by PoC Server", the PoC Server:

1. SHALL generate a SIP 200 OK response to the SIP NOTIFY request according to rules and procedures of [RFC3261] and [RFC6665];

2. SHALL send the SIP 200 "OK" response towards the CBUS Server according to rules and procedures of the SIP/IP Core;

3. SHALL check the Subscription-State header value of the received SIP NOTIFY request as specified in [RFC6665] and

- a) if the Subscription-State is 'pending' as specified in [RFC6665] and this is the first SIP NOTIFY request received in this state of the SIP dialog, the PoC Server:

- i. SHALL generate a SIP 183 "Session Progress" response to the SIP INVITE request received for the Dynamic PoC Group Session as specified in the subclause 7.2.1.1 "General" and include the warning text set to '135 Authorization of Dynamic PoC Group ongoing' as specified in subclause 5.6 "Warning header";

- .

- .

- From 7.2.1.30.1, pg. 172:

NOTE 1:	When the PoC Server has initiated a subscription on behalf the Inviting PoC Client the PoC Server waits for the subscription being authorized at an information source beyond the CBUS Server’s authorization.

- b) if the Subscription-State is 'pending' as specified in [RFC6665] and this is not the first SIP NOTIFY request received in this state of the SIP dialog, the PoC Server:

- i. SHALL generate a SIP 183 "Session Progress" response to the SIP INVITE request received for the Dynamic PoC Group Session as specified in the subclause 7.2.1.1 "General" and include the warning text set to '135 Authorization of Dynamic PoC Group ongoing' as specified in subclause 5.6 "Warning header";

- ii. SHALL send a SIP 183 "Session Progress" response towards the Inviting PoC Client according to rules and procedures of the SIP/IP Core;

- c) if the Subscription-State is 'active' as specified in [RFC6665] and this is the first SIP NOTIFY request received in this state of the SIP dialog, the PoC Server:

- .

- .

- From 7.2.1.30.1, pg. 167:

- .

- .

- d) if the Subscription-State is 'active' as specified in [RFC6665] and this is not the first SIP NOTIFY request received in this state of the SIP dialog, the PoC Server:

- i. SHALL perform the following actions, if the notification contains a MIME resource-lists body as specified in [draft-cbus-event] with at least one PoC Address matching the Dynamic PoC Group rules:

- 1. cache the PoC Addresses currently matching the Dynamic PoC Group rules;

- 2. update the list of past Participants as specified in subclause 5.14 "Past Participants", if the PoC Server supports the Ad-hoc PoC Group Session re-initiation functionality;

- 3. invite the PoC Users, as specified in subclause 7.2.2.2 "PoC Session invitation requests" whose PoC Addresses according to the notification matches the Dynamic PoC Group rules and have not previously been invited or expelled;

- 4. remove the Participants, as specified in 7.2.2.4 "Removal of Participant from PoC Session" whose PoC Address according to the notification no longer matches the Dynamic PoC Group rules;

- 5. check the PoC Session release policy as specified in subclause 7.2.1.6 "PoC Session release policy" and if the applied release policy requires that the PoC Session is released, the PoC Server MAY perform the following:

NOTE 4:	When all PoC Users have been invited or joined already once and all Participants have been removed the PoC Session can be released, unless there are Participants left which are not subject to the Dynamic PoC Group rules.

- A. perform for each remaining Participant of the PoC Session the procedures as specified in the subclause 7.2.2.4 "Removal of Participant from PoC Session";

- B. generate a notification of the current state of the PoC Session to the PoC Client(s), which have subscribed to the conference state event package, as specified in subclause 7.2.1.11.2 "Generating a SIP NOTIFY request";

- C. check the subscription termination policy as specified in subclause 7.2.1.11.3 "Termination of subscription" and if the subscription termination policy requires that subscriptions are to be terminated for each PoC Client terminate the existing subscription to the conference state event package;

- D. send the SIP NOTIFY request to the PoC Client(s) according to rules and procedures of the SIP/IP Core; and,

- E. terminate the subscription to the Dynamic Group Member Information of the same SIP dialogue as the received SIP NOTIFY request and indicate it terminated according to rules and procedures of [RFC6665] and do not continue with the rest of the steps.

- .

- .

- From 7.2.1.30.1, pg. 173:

- .

- .

- E. terminate the subscription to the Dynamic Group Member Information of the same SIP dialogue as the received SIP NOTIFY request and indicate it terminated according to rules and procedures of [RFC6665] and do not continue with the rest of the steps.

- e) if the Subscription-State is 'terminated' as specified in [RFC6665], the PoC Server:

- i. SHALL perform the following actions, if Condition Re-evaluation is not applied to the Dynamic PoC Group Session for which the SIP NOTIFY request is received:

- 1. reject the SIP INVITE request with a SIP 480 "Temporarily Unavailable" response with the warning text set to '137 No matching members found' as specified in subclause 5.6 "Warning header", if the request is received for a Dynamic Ad-hoc or Pre-arranged PoC Group and if the notification contains a MIME resource-lists body as specified in [draft-cbus-event] that does not contain any PoC Addresses matching the Dynamic PoC Group rules. Otherwise, continue with the rest of the steps;

- 2. reject the SIP INVITE request with a SIP 480 "Temporarily Unavailable" response with the warning text set to '121 Function not allowed due to <detailed reason>' as specified in subclause 5.6 "Warning header", if the request is received for a Dynamic Chat PoC Group and if the notification contains a MIME resource-lists body as specified in [draft-cbus-event] that does not contain any PoC Addresses matching the Dynamic PoC Group rules. Otherwise, continue with the rest of the steps;

- 3. cache the PoC Addresses matching the Dynamic PoC Group rules, if the notification contains a MIME resource-lists body as specified in [draft-cbus-event] with at least one PoC Address matching the Dynamic PoC Group rules;

- 4. update the list of past Participants as specified in subclause 5.14 "Past Participants", if the PoC Server supports the Ad-hoc PoC Group Session re-initiation functionality; and,

- 5. continue according to the subclause 7.2.1.2 "Ad-hoc PoC Group and 1-1 PoC Session setup request", 7.2.1.3.1 "General", 7.2.1.3.2 "Dispatch PoC Session setup request from PoC Dispatcher" or 7.2.1.5 "Joining Chat PoC Group Session request" initiating the procedures in this subclause.

- ii. SHALL perform the following actions, if Condition Re-evaluation is applied to the Dynamic PoC Group Session for which the SIP NOTIFY request is received:

- 1. check "reason" parameter received in the SIP NOTIFY request and if it is not present or if "reason" parameter is other than 'rejected', 'giveup' or 'noresource' as specified in [RFC6665] the PoC Server SHOULD proceed according to rules and procedures of [RFC6665] and re-subscribe to Dynamic Group Member Information as specified in 7.2.2.10.1 "Subscription initiated by PoC Server" and do not continue with the rest of the steps;

- .

- .

- From 7.2.2.10.1, pg. 188-189:

- .

- .

##### 7.2.2.10.1 Subscription initiated by PoC Server

The procedure is initiated by the PoC Server as the result of an action specified in subclause 7.2.1 "Request terminating at the Controlling PoC Function".

When initiating a subscription request for Dynamic Group Member Information the PoC Server:

1. SHALL generate a SIP SUSCRIBE request according to rules and procedures of [RFC6665] and [draft-cbus-event];

2. SHALL set the Request-URI of the SIP SUBSCRIBE request to the SIP URI identifying the address of the CBUS Server according to local policy;

3. SHALL include the Authenticated Originator's PoC Address as specified in subclause 5.2 "Authenticated Originator's PoC Address" with the URI set to

- a) the PoC Group Identity, if the PoC Session is initiated for a Dynamic Pre-arranged or Chat PoC Group; or,

NOTE 1:	The PoC Server also includes the PoC Group Identity in case of a subscription performed for an entire Dispatch PoC Group for which Dynamic PoC Group rules are defined.

.

.

From 7.2.2.10.1, pg. 190:

.

.

NOTE 3:	The Inviting PoC Client if authorized can override the Dynamic PoC Group rules pre-defined for a Pre-arranged PoC Group by explicitly including rules in the Dynamic PoC Group Session invitation request.

11. SHALL send the SIP SUSCRIBE request towards the SIP/IP Core according to rules and procedures of the SIP/IP Core.

The responses to the SIP SUBSCRIBE request SHALL be handled according to rules and procedures of [RFC6665] and [draft-cbus-event], and rules and procedures of the SIP/IP Core with the clarifications given in this subclause.

.

.

From 7.3.1.15, pg. 222-223:

.

.

#### PoC Session release from User Plane

- Upon receiving a PoC Session release request from the User Plane as specified in [OMA-PoC-UP] "Receive MBCP Media Burst Acknowledgement message (R: MB_Ack)" or "T15 (Connect message re-transmit) timer fired N times" or "Receive PoC Session release indication from PoC Client (R: PoC Session release from PoC Client)", the PoC Server:

1. SHALL remove the Participant referred by User Plane from the PoC Session by performing the procedures as specified in subclause 7.3.2.6.3 "Leaving a PoC Session when using Pre-established Session";

2. SHALL check the PoC Session release policy as specified in subclause 7.2.1.16 "PoC Session release policy" and if the applied release policy requires that the PoC Session is released perform for each Participant of the PoC Session (except for the owner of the Pre-established Session) the procedures specified in subclause 7.2.2.4 "Removal of Participant from PoC Session";

3. SHALL terminate the subscription to the Dynamic PoC Group member information and indicate it terminated according to rules and procedures of [RFC6665] if the subscription is ongoing for the PoC Session and, if the PoC Session is released according to the applied release policy and if the release request is received for a Dynamic PoC Group Session;

.

.