Pre-established Session

Originating flow – Confirmed Indication

PoC Client A invites PoC Client B to a PoC Session by sending a SIP REFER request to the PoC Server. PoC Client A has created a Pre-established Session by using the mechanisms as shown in Figure 4 "PoC Client A initiates a Pre-established Session", and the PoC Client A has learned the URI that identifies this Session. This flow is as shown in Figure 5: "PoC Client A invites another user to a Pre-established Session by sending a SIP REFER request to PoC Server A confirmed case".

Figure 5: PoC Client A invites another user to a Pre-established Session by sending a SIP REFER request to PoC Server A confirmed case

The steps of the flows are as follows:

1.	PoC Client A creates a Pre-established Session

PoC Client A creates a Pre-established Session and learns its URI as shown in Figure 4 "PoC Client A initates a Pre-established Session".

2.	SIP REFER request (from PoC Client A to SIP/IP Core A)

The PoC Client A invites the PoC Client B to the Pre-established Session.

| Request-URI | sip:Pre-establishedSessionIdentityA@PoC-ServerA.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Refer-To: | <sip:PoC-UserB@networkB.net?Reject-Contact=sip.automata%3bsip.actor%3dmsg-taker%3brequire%3bexplicit&Reject-Contact=sip.automata%3bsip.actor%3dprincipal%3bsip.description%3d%22poc%20recording%20device%22%3brequire%3bexplicit&Subject=Let's%20talk&Alert-Info=%3chttp://publicweb.networkB.net/users/PoC-UserA%40networkA.net/myFavouriteRingingTone.wav%3e&Call-Info=%3chttp://publicweb.networkB.net/users/PoC-UserA%40networkA.net/myOwnPicture.jpg%3e%3bpurpose%3dicon> |
| Target-Dialog: | me03a0s09a2sdfgjkl491777; remote-tag=774321; local-tag=64727891 |
| Privacy: | Id |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

NOTE:	For a 1-1 PoC Session the SIP REFER request does not contain a body.

3. 	SIP REFER request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP REFER request to the PoC Server A performing the Participating PoC Function of the PoC Client A according to the SIP/IP Core A routing principles.

| Request-URI | sip:Pre-establishedSessionIdentityA@PoC-ServerA.networkA.net;gr |
| --- | --- |
| Content-Disposition: | "attachment" |
| SIP HEADERS |  |
| Refer-To: | <sip:PoC-UserB@networkB.net?Reject-Contact=sip.automata%3bsip.actor%3dmsg-taker%3brequire%3bexplicit&Reject-Contact=sip.automata%3bsip.actor%3dprincipal%3bsip.description%3d%22poc%20recording%20device%22%3brequire%3bexplicit&Subject=Let's%20talk&Alert-Info=%3chttp://publicweb.networkB.net/users/PoC-UserA%40networkA.net/myFavouriteRingingTone.wav%3e&Call-Info=%3chttp://publicweb.networkB.net/users/PoC-UserA%40networkA.net/myOwnPicture.jpg%3e%3bpurpose%3dicon> |
| Target-Dialog: | me03a0s09a2sdfgjkl491777; remote-tag=774321; local-tag=64727891 |
| Privacy: | Id |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

4.	SIP 200 "OK" response (from PoC Server A to SIP/IP Core A)

The PoC Server A indicates that it has received the SIP REFER request by sending a SIP 200 "OK" response. This means that the PoC Server A has accepted the SIP REFER request and has started to process the request. This does not mean, however, that the PoC Client B (the referred-to resource) has been contacted.

5.	SIP 200 "OK" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the response to the PoC Client A.

6.	SIP INVITE request (from PoC Server A towards PoC Client B)

The PoC Server A invites the PoC User, who is indicated in the Refer-To header of the received SIP REFER request.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy | Id |
| Accept-Contact: | *;+g.poc.talkburst;require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Contact: | <sip:SessionABCDEF@ PoC-ServerA.networkA.net;gr;session1-1>;+g.poc.talkburst;isfocus |
| Supported: | 100rel,timer,norefersub |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| Content-Type: | multipart/mixed |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:premium |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

7.	SIP NOTIFY request (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP NOTIFY request via the SIP/IP Core A towards the PoC Client A to inform about the progress of the SIP REFER request processing. The body of the SIP NOTIFY request contains a fragment of the response as received by the notifying PoC Server A for the request that was initiated due to the SIP REFER request.

| Request-URI | sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | active;expires=60 |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 100 TRYING |  |
| To: | <sip:PoC-UserB@networkB.net> |

8.	SIP NOTIFY request (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the SIP NOTIFY request to the PoC Client A.

| Request-URI | sip:PoC-ClientA.networkA.net |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | active;expires=60 |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 100 Trying |  |
| To: | <sip:PoC-UserB@networkB.net> |

9.	SIP 200 "OK" response (from PoC Client A to SIP/IP Core A)

The PoC Client A acknowledges the SIP NOTIFY request with a SIP 200 "OK" response to the SIP/IP Core A.

10.	 SIP 200 "OK" response (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP 200 "OK" response to PoC Server A.

11.	 PoC Server A Receives Ringing Indication

PoC Server A receives a SIP 180 "Ringing" response.

12.	 SIP NOTIFY request (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP NOTIFY request that indicates that the PoC Client B is being alerted.

| Request-URI | sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | active;expires=60 |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 180 Ringing |  |
| To: | <sip:PoC-UserB@networkB.net> |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Contact: | <sip:PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo |

13.	 SIP NOTIFY request (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the SIP NOTIFY request to the PoC Client A.

| Request-URI | sip:PoC-ClientA.networkA.net |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | active;expires=60 |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 180 Ringing |  |
| To: | <sip:PoC-UserB@networkB.net> |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Contact: | <sip:PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo |

14.	 SIP 200 "OK" response (from PoC Client A to SIP/IP Core A)

The PoC Client A acknowledges the SIP NOTIFY request with a SIP 200 "OK" response to the SIP/IP Core A.

15.	 SIP 200 "OK" response (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP 200 "OK" response to the PoC Server A.

16.	 Referred PoC User accepts the invitation to the PoC Session

The invitation sent towards the referred PoC User has been accepted and the PoC Client B has joined the PoC Session.

A.	MBCP Connect message (PoC Server A to PoC Client A)

In order to inform e.g PoC Session Identity of the established PoC Session the PoC Server A sends the MBCP Connect message to the PoC Client A.

B	MBCP Media Burst Acknowledgement ( PoC Client A to PoC Server A)

The PoC Client A acknowledges the reception of the MBCP Connect message.

17.	 SIP NOTIFY request (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP NOTIFY request that indicates that the PoC Client B is available and capable of receiving Media Streams.

| Request-URI | sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | terminated |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 200 OK |  |
| To: | <sip:PoC-UserB@networkB.net> |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Contact: | <sip:PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo |

18.	 SIP NOTIFY request (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the SIP NOTIFY request to the PoC Client A.

| Request-URI | sip:PoC-ClientA.networkA.net |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | terminated |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 200 OK |  |
| To: | <sip:PoC-UserB@networkB.net> |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Contact: | <sip:PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo |

19.	 SIP 200 "OK" response (from PoC Client A to SIP/IP Core A)

The PoC Client A acknowledges the SIP NOTIFY request with a SIP 200 "OK" response to the SIP/IP Core A.

20.	 SIP 200 "OK" response (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP 200 "OK" response to the PoC Server A.

Terminating flow

NOTE:	Manual answer using a Pre-established Session is not supported in this version of the PoC Enabler.  Manual answer using an On-demand Session is described in the subclause F.5.1 "Terminating flow – Manual Answer".

Originating flow - Unconfirmed Indication

PoC Client A invites PoC Client B to a PoC Session by sending a SIP REFER request to the PoC Server. PoC Client A has created a Pre-established  Session as shown in Figure 4 "PoC Client initates a Pre-established Session", and the PoC Client A has learned the URI that identifies this Session. This flow is as shown in Figure 6 "PoC Client A invites another PoC user to a Pre-established Session by sending a SIP REFER request to PoC Server A unconfirmed case".

Figure 6: PoC Client A invites another PoC User to a Pre-established Session by sending a SIP REFER request to PoC Server A unconfirmed case

The steps of the flows are as follows:

1.	PoC Client A creates a Pre-established Session

PoC Client A creates a Pre-established Session and learns its URI as shown in Figure 4 "PoC Client initates a Pre-established Session".

2.	SIP REFER request (from PoC Client A to SIP/IP Core A)

The PoC Client A invites the PoC Client B to the Pre-established Session.

| Request-URI | sip:Pre-establishedSessionIdentityA@PoC-ServerA.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Refer-To: | <sip:PoC-UserB@networkB.net?Reject-Contact=sip.automata%3bsip.actor%3dmsg-taker%3brequire%3bexplicit&Reject-Contact=sip.automata%3bsip.actor%3dprincipal%3bsip.description%3d%22poc%20recording%20device%22%3brequire%3bexplicit&Priv-Answer-Mode=Auto> |
| Target-Dialog: | me03a0s09a2sdfgjkl491777; remote-tag=774321; local-tag=64727891 |
| Privacy: | id |

NOTE:	For a 1-1 PoC Session the SIP REFER request does not contain a body.

3. 	SIP REFER request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP REFER request to the PoC Server A performing the Participating PoC Function of the PoC Client A according to the SIP/IP Core A routing principles.

| Request-URI | sip:Pre-establishedSessionIdentityA@PoC-ServerA.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Refer-To: | <sip:PoC-UserB@networkB.net?Reject-Contact=sip.automata%3bsip.actor%3dmsg-taker%3brequire%3bexplicit&Reject-Contact=sip.automata%3bsip.actor%3dprincipal%3bsip.description%3d%22poc%20recording%20device%22%3brequire%3bexplicit&Priv-Answer-Mode=Auto> |
| Target-Dialog: | me03a0s09a2sdfgjkl491777; remote-tag=774321; local-tag=64727891 |
| Privacy: | id |

4.	SIP 200 "OK" response (from PoC Server A to SIP/IP Core A)

The PoC Server A indicates that it has received the SIP REFER request by sending a SIP 200 "OK" response. This means that the PoC Server A has accepted the SIP REFER request and has started to process the request. This does not mean, however, that the PoC Client B (the referred-to resource) has been contacted.

5.	SIP 200 "OK" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the response to the PoC Client A.

6.	SIP INVITE request (from PoC Server A towards PoC Client B)

The PoC Server A invites the PoC User, who is indicated in the Refer-To header of the received SIP REFER request.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst;require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Supported: | 100rel,timer,norefersub |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Priv-Answer-Mode: | Auto |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:premium |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 4321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:4321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |

7.	SIP NOTIFY request (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP NOTIFY request via the SIP/IP Core A towards the PoC Client A to inform about the progress of the SIP REFER request processing. The body of the SIP NOTIFY request contains a fragment of the response as received by the notifying PoC Server A for the request that was initiated due to the SIP REFER request.

| Request-URI | sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | active;expires=60 |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 100 TRYING |  |
| To: | <sip:PoC-UserB@networkB.net> |

8.	SIP NOTIFY request (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the SIP NOTIFY request to the PoC Client A.

| Request-URI | sip:PoC-ClientA.networkA.net |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | active;expires=60 |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 100 TRYING |  |
| To: | <sip:PoC-UserB@networkB.net> |

9.	SIP 200 "OK" response (from PoC Client A to SIP/IP Core A)

The PoC Client A acknowledges the SIP NOTIFY request with a SIP 200 "OK" response to the SIP/IP Core A.

10.	 SIP 200 "OK" response (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP 200 "OK" response to PoC Server A.

11.	 Invited PoC User accepts the invitation to the PoC Session

The invitation sent towards the referred PoC User has been accepted and the PoC Client B has joined the PoC Session.

A. 	MBCP Connect message (PoC Server A to PoC Client A)

The PoC A Server sends the MBCP Connect message to the PoC Client A.

12.	 SIP NOTIFY request (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP NOTIFY request that indicates that the PoC Client B is available and capable of receiving Media Streams.

| Request-URI | sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | terminated |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 200 OK |  |
| To: | <sip:PoC-UserB@networkB.net> |
| P-Asserted-Identity | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Contact: | <sip:PoC-ServerB.networkB.net;b2bua>;+g.poc.talkburst;+g.poc.fdcfo |
| P-Answer-State: | Unconfirmed |

B.	MBCP Media Burst Acknowledgement ( PoC Client A to PoC Server A)

The PoC Client A acknowledges the reception of the MBCP Connect message.

13.	 SIP NOTIFY request (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the SIP NOTIFY request to the PoC Client A.

| Request-URI | sip:PoC-ClientA.networkA.net |
| --- | --- |
| SIP HEADERS |  |
| Event: | refer |
| Subscription-State: | terminated |
| Content-Type: | message/sipfrag |
| MIME BODY |  |
| SIP/2.0 200 OK |  |
| To: | <sip:PoC-UserB@networkB.net> |
| P-Asserted-Identity | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Contact: | <sip:PoC-ServerB.networkB.net;b2bua>;+g.poc.talkburst;+g.poc.fdcfo |
| P-Answer-State: | Unconfirmed |

14.	 SIP 200 "OK" response (from PoC Client A to SIP/IP Core A)

The PoC Client A acknowledges the SIP NOTIFY request with a SIP 200 "OK" response to the SIP/IP Core A.

15.	 SIP 200 "OK" response (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP 200 "OK" response to the PoC Server A.

C.	MBCP Media Burst Granted message (from PoC Server A to PoC Client A)

The PoC Server X sends a MBCP Media Burst Granted message to the PoC Server A.

D.	RTP Media from PoC Client A to PoC Server A

The PoC Client A sends the RTP Media to the PoC Server A.

Terminating flow – Unconfirmed Indication

PoC Server X invites the PoC Client B to a PoC Session by sending a SIP INVITE request to the terminating SIP/IP Core. The PoC Client B has created a Pre-established Session as shown in Figure 4"PoC Client A initates a Pre-established Session".

QoE Profiles feature is used in this flow. Premium QoE Profile is assigned to the PoC Session and PoC Client B also created the Pre-established Session using Premium QoE Profile. PoC Client B participates in the PoC Session with Premium Local QoE Profile.

Figure 7: PoC Session invitation with automatic answer – Pre-established Session case.

This flow shows a PoC Server X inviting a PoC User to a PoC Session. The flow is as shown in Figure 7 "PoC Session invitation with automatic answer – Pre-established Session case". The invitation may be as a result of another PoC User sending a SIP REFER request to the PoC Server X as shown in Figure 6 "PoC Client A invites another user to a Pre-established Session by sending a SIP REFER request to PoC Server A unconfirmed case". Note that the PoC Server A as shown in Figure 6 "PoC Client A invites another user to a Pre-established Session by sending a SIP REFER request to PoC Server A unconfirmed case " has changed its role and acts as the PoC Server X in this flow.

The steps of the flows are as follows:

1.	Session Invitation has been received to invite PoC Client B

2.	SIP INVITE request (PoC Server X to SIP/IP Core X)

The PoC Server X invites the PoC Client B to a PoC Session. This may happen as a result of another PoC User referring the PoC Client B to the PoC Server X. The PoC Server X sends a SIP INVITE request via the SIP/IP Core X to the PoC Client B home network. The SIP INVITE request contains an SDP offer with the media information (e.g IP address and port number(s)) of the PoC Server X) and the Media Burst Control Protocol. In this example PoC Server X supports and offers both the AMR and EVRC codecs.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Contact: | <sip: PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr; session=1-1>;+g.poc.talkburst;isfocus |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Priv-Answer-Mode: | Auto |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:premium |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 4321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:4321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |

3.	SIP 100 "Trying" response (SIP/IP Core X to PoC Server X)

The SIP/IP Core X responds to the SIP INVITE request with a SIP 100 "Trying" provisional response.

4.	SIP INVITE request (SIP/IP Core X to SIP/IP Core B)

The SIP/IP Core X resolves the SIP/IP Core B address of the PoC Client B and forwards the SIP INVITE request to the SIP/IP Core B. The SIP INVITE request contains the same PoC specific elements as the SIP INVITE request sent by PoC Server X to SIP/IP Core X.

5.	SIP 100 "Trying" response (SIP/IP Core B to SIP/IP Core X)

The SIP/IP Core B responds to the SIP INVITE request with a SIP 100 "Trying" provisional response.

6.	SIP INVITE request (SIP/IP Core B to PoC Server B)

The SIP/IP Core B forwards the SIP INVITE request to the PoC Server B. The SIP INVITE request contains the same PoC specific elements as the SIP INVITE request sent by PoC Server X to SIP/IP Core X.

7.	SIP 100 "Trying" response (PoC Server B to SIP/IP Core B)

The PoC Server B responds to the SIP INVITE request with a SIP 100 "Trying" provisional response.

8.	SIP 200 "OK" response (PoC Server B to SIP/IP Core B)

The PoC Server B receives the SIP INVITE request, identifies that auto answer is defined for the PoC Client B and that the PoC Client B has already a Pre-established Session established. Therefore the PoC Server B sends a SIP 200 "OK" final response to the SIP INVITE request to the SIP/IP Core B. The SIP 200 "OK" response is sent along the signalling path. The SIP 200 "OK" response contains the SDP answer including the accepted media information (e.g. Codecs, IP address and port number(s) of the PoC Server B) and accepted Media Burst Control Protocol. In  this example PoC Server B knows that PoC Client B only supports the EVRC codec and so only returns EVRC in the SDP answer and PoC Server B uses the same port for RTCP and the Media Burst Control Protocol.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB.networkB.net> |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-ServerB@networkB.net;b2bua >;+g.poc.talkburst;+g.poc.fdcfo |
| Session-Expires: | 1800;refresher=uas |
| P-Answer-State: | Unconfirmed |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,MESSAGE,OPTIONS |
| Accept-Language: | en |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:uu vv |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:vv |
| a= | a=upcc:0 |
| m= | message 43821 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[99055::fff:aaa:eee:bbb]:43821/xk83a45;tcp |
| a= | max-size:10000 |

A.	MBCP Connect message (PoC Server B to PoC Client B)

In order to inform about the invitation to a PoC Session the PoC Server B sends the MBCP Connect to the PoC Client B. The message includes the PoC Session Identity.

B.	MBCP Media Burst Acknowledgement ( PoC Client B to PoC Server)

The PoC Client B acknowledges the reception of the MBCP Connect message.

9.	SIP 200 "OK" response (SIP/IP Core B to SIP/IP Core X)

The SIP/IP Core B forwards the SIP 200 "OK" final response to the SIP/IP Core X. The SIP 200 "OK" response contains the same PoC specific elements as the SIP 200 "OK" response sent by PoC Server B to SIP/IP Core B.

10.	SIP 200 "OK" response (SIP/IP Core X to PoC Server X)

The SIP/IP Core X forwards the SIP 200 "OK" response to the PoC Server X. The SIP 200 "OK" response contains the same PoC specific elements as the SIP 200 "OK" response sent by PoC Server B to SIP/IP Core B.

11.	The invitation towards the PoC Client B has been accepted

The PoC Server X sends a notification towards the PoC Client A that the Invited PoC User is available and capable of receiving the Media Streams.

12.	SIP ACK request (PoC Server X to SIP/IP Core X)

The PoC Server X acknowledges the SIP 200 "OK" response with a SIP ACK request sent to the SIP/IP Core X.

13.	SIP ACK request (SIP/IP Core X to SIP/IP Core B)

The SIP/IP Core X forwards the SIP ACK request to the SIP/IP Core B.

14.	SIP ACK request (SIP/IP Core B to PoC Server B)

The SIP/IP Core B forwards the SIP ACK request to the PoC Server B.

C.	MBCP Media Burst Taken (PoC Server X to PoC Server B)

The PoC Server X sends the MBCP Media Burst Taken to the PoC Server B. Unless privacy is required the MBCP Media Burst Taken message includes the Sender Identity.

D.	MBCP Media Burst Taken (PoC Server B to PoC Client B)

The PoC Server B sends the MBCP Media Burst Taken to the PoC Client B. Unless privacy is required the MBCP Media Burst Taken message includes the Sender Identity The PoC Server B includes a request to acknowledge the message.

E.	MBCP Media Burst Acknowledgement message (PoC Client B to PoC Server B)

The PoC Client B acknowledges the reception of the MBCP Media Burst Taken message.