Backward compatibility

This subclause shows examples of cooperation of PoC Server compliant to the PoC version 2.0 specification with the PoC Server compliant only to the PoC version 1.0 specification.

The flows in this subclause do not relate to each other.

Backward compatibility towards Controlling PoC Function

This subclause describes the PoC Session invitation message flow when the PoC Client A with the PoC Server A performing the Participating PoC Function compliant to the PoC version 2.0 specification attempts to join a Chat PoC Group Session hosted by the PoC Server X performing the Controlling PoC Function compliant only to the PoC version 1.0 specification.

The PoC Client A does not support the FDCFO and Discrete Media sent over the SIP MESSAGE request. The PoC Client A does not request anonymity.

The PoC Server A stays in the Media path.

The flow is as shown in Figure 33 "Backward compatibility with the PoC Server performing the Controlling PoC Function compliant only to the PoC version 1.0 specification". The SIP 100 "Trying" responses to SIP INVITE requests are not shown in the figure.

Figure 33: Backward compatibility with the PoC Server performing the Controlling PoC Function compliant only to the PoC version 1.0 specification

- SIP INVITE request (from PoC Client A to SIP/IP Core A)

| Request-URI | sip:ChatPoCGroupXXXX@networkX.net |
| --- | --- |
| SIP HEADERS |  |
| P-Preferred-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-client/OMA2.0 |
| Contact: | <PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+g.poc.talkburst+sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>"; |
| Supported: | timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,OPTIONS,MESSAGE |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:premium |
| m= | audio 3456 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 5560 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 5678 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| m= | message 12345 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5555::aaa:bbb:ccc:ddd]:12345/kjhd37s2s20w2a;tcp |
| a= | max-size:10000 |

- SIP INVITE request (from SIP/IP Core A to PoC Server A)

| Request-URI | sip:ChatPoCGroupXXXX@networkX.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-client/OMA2.0 |
| Contact: | <PoC-USerA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>";+g.poc.talkburst |
| Supported: | timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,OPTIONS,MESSAGE |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:premium |
| m= | audio 3456 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 5560 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 5678 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| m= | message 12345 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5555::aaa:bbb:ccc:ddd]:12345/kjhd37s2s20w2a;tcp |
| a= | max-size:10000 |

- SIP INVITE request (from PoC Server A to SIP/IP Core A)

| Request-URI | sip:ChatPoCGroupXXXX@networkX.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Contact: | <sip:sessionabcdef@PoCServerA.networkA.net;gr;b2bua>;+g.poc.talkburst |
| Supported: | timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,OPTIONS,MESSAGE |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5055::ccc:ddd:aaa:bbb |
| a= | poc-qoe:premium |
| m= | audio 5468 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:5469 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 5469 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 5678 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| m= | message 23456 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5055::ccc:ddd:aaa:bbb]:12345/xxxkjhd37s2s20w2a;tcp |
| a= | max-size:10000 |

- SIP INVITE request (from SIP/IP Core A to SIP/IP Core X)

The PoC specific elements are the same as in the previous message.

- SIP INVITE request (from SIP/IP Core X to PoC Server X)

The PoC specific elements are the same as in the previous message.

- SIP 200 "OK" response (from PoC Server X to SIP/IP Core X)

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:ChatPoCGroupXXXX@networkX.net;session=chat> |
| Server: | PoC-serv/OMA1.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=chat>;+g.poc.talkburst;isfocus |
| Require: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 5777::eee:fff:aaa:bbb |
| m= | audio 57786 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:57787 |
| m= | application 57787 udp TBCP |
| a= | fmtp:TBCP queuing=1;tb_priority=2;timestamp=1 |
| m= | video 0 RTP/AVP 99 |
| m= | message 0 TCP/MSRP * |

- SIP 200 "OK" response (from SIP/IP Core X to SIP/IP Core A)

The PoC specific elements are the same as in the previous message.

- SIP 200 "OK" response (from SIP/IP Core A to PoC Server A)

The PoC specific elements are the same as in the previous message.

- SIP 200 "OK" response (from PoC Server A to SIP/IP Core A)

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:ChatPoCGroupXXXX@networkX.net;session=chat> |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:sessionabcdef@PoCServerA.networkA.net;gr;session=chat>;+g.poc.talkburst;isfocus |
| Require: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER |
| Content-Type: | application/sdp |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 5055::ccc:ddd:aaa:bbb |
| m= | audio 6654 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:6655 |
| m= | application 6655 udp TBCP |
| a= | fmtp:TBCP queuing=1;tb_priority=2;timestamp=1 |
| m= | video 0 RTP/AVP 99 |
| m= | message 0 TCP/MSRP * |

- SIP 200 "OK" response (from SIP/IP Core A to PoC Client A)

The PoC specific elements are the same as in the previous message.

- SIP ACK request (from PoC Client A to SIP/IP Core A)

- SIP ACK request (from SIP/IP Core A to PoC Server A)

- SIP ACK request (from PoC Server A to SIP/IP Core A)

- SIP ACK request (from SIP/IP Core A to SIP/IP Core X)

- SIP ACK request (from SIP/IP Core X to PoC Server X)

Backward compatibility towards terminating Participating PoC Function

This subclause describes the PoC Session invitation message flow when the PoC Server X performing the Controlling PoC Function invites to a PoC Session a PoC Client B which is served by the PoC Server B performing the Participating PoC Function which is compliant only to the PoC version 1.0 specification.

The PoC Client B does not request anonymity.

The PoC Server B stays in the Media path.

The flow is as shown in Figure 34 "Backward compatibility with the PoC Server performing the terminating Participating PoC Function compliant only to the PoC version 1.0 specification". The SIP 100 "Trying" responses to SIP INVITE requests are not shown in the figure.

Figure 34: Backward compatibility with the PoC Server performing the terminating Participating PoC Function compliant only to the PoC version 1.0 specification

- SIP INVITE request (from PoC Server X to SIP/IP Core X)

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkbust; isfocus |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5777::eee:fff:aaa:bbb |
| a= | poc-qoe:professsional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5777::eee:fff:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

- SIP INVITE request (from SIP/IP Core X to SIP/IP Core B)

The PoC specific elements are the same as in the previous message.

- SIP INVITE request (from SIP/IP Core B to PoC Server B)

The PoC specific elements are the same as in the previous message.

- SIP INVITE request (from PoC Server B to SIP/IP Core B)

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA1.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerB.networkB.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Supported: | norefersub,timer |
| Session-Expires: | 1800 |
| Answer-Mode: | Manual;Require |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 6033::ddd:ccc:aaa:bbb |
| m= | audio 63776 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:63090 |
| m= | application 63090 udp TBCP |
| a= | fmtp:TBCP queuing=1;tb_priority=2;timestamp=1 |

- SIP INVITE request (from SIP/IP Core B to PoC Client B)

The PoC specific elements are the same as in the previous message.

- SIP 180 "Ringing" response (from PoC Client B to SIP/IP Core B)

| SIP HEADERS |  |
| --- | --- |
| P-Preferred-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Require: | timer |
| Server: | PoC-client/OMA1.0 |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+g.poc.talkburst |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER |

- SIP 180 "Ringing" response (from SIP/IP Core B to PoC Server B)

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Require: | timer |
| Server: | PoC-client/OMA1.0 |
| Contact: | <sip:PoC-UsertB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>; +g.poc.talkburst |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER |

- SIP 180 "Ringing" response (from PoC Server B to SIP/IP Core B)

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Require: | timer |
| Server: | PoC-serv/OMA1.0 |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>; +g.poc.talkburst |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER |

- SIP 180 "Ringing" response (from SIP/IP Core B to SIP/IP Core X)

The PoC specific elements are the same as in the previous message.

- SIP 180 "Ringing" response (from SIP/IP Core X to PoC Server X)

The PoC specific elements are the same as in the previous message.

- SIP 200 "OK" response (from PoC Client B to SIP/IP Core B)

| SIP HEADERS |  |
| --- | --- |
| P-Preferred-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Require: | timer |
| Server: | PoC-client/OMA1.0 |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+g.poc.talkburst |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER |
| Session-Expires: | 1800;refresher=uas |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 5569::fff:ddd:aaa:bbb |
| m= | audio 43772 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53090 |
| m= | application 53090 udp TBCP |
| a= | fmtp:TBCP queuing=1;tb_priority=2;timestamp=1 |

- SIP 200 "OK" response (from SIP/IP Core B to PoC Server B)

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Require: | timer |
| Server: | PoC-client/OMA1.0 |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+g.poc.talkburst |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER |
| Session-Expires: | 1800;refresher=uas |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 5569::fff:ddd:aaa:bbb |
| m= | audio 43772 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53090 |
| m= | application 53090 udp TBCP |
| a= | fmtp:TBCP queuing=1;tb_priority=2;timestamp=1 |

- SIP 200 "OK" response (from PoC Server B to SIP/IP Core B)

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Require: | timer |
| Server: | PoC-serv/OMA1.0 |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+g.poc.talkburst |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER |
| Session-Expires: | 1800;refresher=uas |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 6033::ddd:ccc:aaa:bbb |
| m= | audio 55886 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:44229 |
| m= | application 44229 udp TBCP |
| a= | fmtp:TBCP queuing=1;tb_priority=2;timestamp=1 |
| m= | video 0 RTP/AVP 99 |
| m= | message 0 TCP/MSRP * |

- SIP 200 "OK" response (from SIP/IP Core B to SIP/IP Core X)

The PoC specific elements are the same as in the previous message.

- SIP 200 "OK" response (from SIP/IP Core X to PoC Server X)

The PoC specific elements are the same as in the previous message.

- SIP ACK request (from PoC Server X to SIP/IP Core X)

- SIP ACK request (from SIP/IP Core X to SIP/IP Core B)

- SIP ACK request (from SIP/IP Core B to PoC Server B)

- SIP ACK request (from PoC Server B to SIP/IP Core B)

- SIP ACK request (from SIP/IP Core B to PoC Client X)