ERRATA to OMA-WAP-TS-ProvCont-V1_1-20090728-A.pdf (Additions are in RED)

Summary : PXADDRTYPE to accommodate IPv4v6 option

Change 1 : Section 4.6.2

    - 4.6.2 Parameters for PXPHYSICAL characteristics

The PXPHYSICAL characteristic is only allowed to be used within the characteristic of a logical PROXY. Note that the parameters listed below are described in the scope of this particular characteristic.

<….. Note the contents are not replicated here for keeping the ERRATA small …..>

# PXADDR

The PXADDR can store addresses of different kinds, for example an IP address or a SME number. The type of address in the field is defined by the PXADDRTYPE parameter. This parameter MUST be supported by the WAP client.

| PXADDRTYPE | Content of PXADDR |
| --- | --- |
| IPV4 | An IPv4 address [RFC791] represented in decimal format with dots as delimiters |
| IPV6 | An IPv6 address [RFC2373] represented as hexadecimal numbers with colons as delimiters or as a combination of hexadecimal and decimal numbers with dots and colons as delimiters |
| E164 | A phone number according to the E164 scheme [GENFORM] |
| ALPHA | Generic alphanumeric address (as defined by alphanum in [RFC2396]) |
| IPV4V6 | This represents dual stack IPv4 and IPv6 supported by corresponding entities |

PXADDRTYPE (0 or 1 entries)

The PXADDRTYPE indicates the format and the interpretation of the PXADDR attribute. The PXADDRTYPE can indicate an IP address, a phone number according to the E164 scheme, or a generic alphanumeric address format. If the parameter is not present or if no value is given, the default value IPV4 should be assumed. This parameter MUST be supported by the WAP client.

Value

IPV4 (default)

IPV6

E164

ALPHA

IPV4V6

Change 2 : Section 4.6.2

B.1.6. Characteristic PXPHYSICAL

| Item | Function | Ref. | Status | Requirement |
| --- | --- | --- | --- | --- |
| ProvCont-CPP-C-001 | Support for the parm PHYSICAL-PROXY-ID | 4.6.2 | M | ProvCont-MLP-C-018 |
| ProvCont-CPP-C-002 | Support for the parm PXADDR | 4.6.2 | M | ProvCont-MLP-C-013 |
| ProvCont-CPP-C-003 | Support for the parm PXADDRTYPE | 4.6.2 | M | ProvCont-CPP-C-009 OR ProvCont-CPP-C-010 OR ProvCont-CPP-C-011 OR ProvCont-CPP-C-012 |
| ProvCont-CPP-C-004 | Support for the parm TO-NAPID | 4.6.2 | M |  |
| ProvCont-CPP-C-005 | Support for the parm DOMAIN | 4.6.2 | O | ProvCont-MLP-C-006 |
| ProvCont-CPP-C-006 | Support for the parm WSP-VERSION | 4.6.2 | O |  |
| ProvCont-CPP-C-007 | Support for the parm PUSHENABLED | 4.6.2 | O |  |
| ProvCont-CPP-C-008 | Support for the TO-NAPID value “INTERNET” | 4.6.2 | O |  |
| ProvCont-CPP-C-009 | Support for PXADDRTYPE value “IPV4” | 4.6.2 | O |  |
| ProvCont-CPP-C-010 | Support for PXADDRTYPE value “IPV6” | 4.6.2 | O |  |
| ProvCont-CPP-C-011 | Support for PXADDRTYPE value “E164” | 4.6.2 | O |  |
| ProvCont-CPP-C-012 | Support for PXADDRTYPE value “ALPHA” | 4.6.2 | O |  |
| ProvCont-CPP-C-013 | Support for PORT characteristic within PXPHYSICAL | 4.5.4 | M | ProvUAB-UDP-C-006 |
| ProvCont-CPP-C-014 | Support for multiple PORT characteristics within PXPHYSICAL | 4.5.4 | O |  |
| ProvCont-CPP-C-015 | Support for multiple TO-NAPID within one PXPHYSICAL | 4.6.2 | O |  |
| ProvCont-CPP-C-016 | Support for the parm PXADDR-FQDN | 4.6.2 | O | ProvCont-MLP-C-022 AND ProvUAB-UDP-C-005 |
| ProvCont-CPP-C-017 | Support for the parm PULLENABLED | 4.6.2 | O |  |
| ProvCont-CPP-C-018 | Support for PXADDRTYPE value “IPV4V6” | 4.6.2 | O |  |