Change Request

| Title: | Fix_Bootstrap Commands | Public OMA Confidential |
| --- | --- | --- |
| To: | DM LightweightM2M WG |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0-20160308-D |  |
| Submission Date: |  |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thierry Garnier, Gemalto N.V, thierry.garnier@gemalto.com |  |
| Replaces: | n/a |  |

Reason for Change

- Reference to the CoAP Mapping Chapter 8, is helpful for better understanding

- The BOOTSTRAP REQUEST and BOOTSTRAP FINISH are missing in the command section

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and agree this CR

Detailed Change Proposal

Reference to The CoAP mapping chapter 8 is useful + the 2 missing Bootstrap commands (Request/Finish) have been added

### Bootstrap Commands

The mapping to CoAP Methods of the LWM2M Bootstrap Interface operations specified in this section, is detailed in chapter 8 of  the present document (Transport Layer Binding and Encodings).

#### BOOSTRAP WRITE

The “Write” operation in Bootstrap Interface is different from the “Write” Operation in Device Management and Service Enablement interface; the LWM2M Client MUST write the value included in the payload regardless of an existence of the targeting Object Instance(s) or Resource(s).

The Write operation can be sent multiple times.

Only in Bootstrap Interface, the “Write” MAY target just an Object ID, which will allow a Bootstrap Server in using a TLV or JSON formatted payload, to populate a LWM2M Client in a single message containing several Instances of the same Object.

The Bootstrap “Write” operation has the following parameters:

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | No | - | Indicates the Object Instance to write. If no Object Instance ID is indicated , Object Instance(s) MUST be specified in the TLV or JSON payload |
| Resource ID | No | - | Indicates the Resource to write. The payload is the new value for the Resource. If no Resource ID is indicated, then the value included payload is an Object Instance containing the Resource values. |
| New Value | Yes | - | The new value included in the payload to update the Object Instance(s) or Resource. |

#### BOOTSTRAP DELETE

The Delete operation targets an Object Instance and can be sent multiple times.

Only in Bootstrap Interface, the Delete operation MAY targets “/” URI. In that case, the operation MUST delete all the existing Object Instances - except LWM2M Bootstrap Server Account - in the LWM2M Client; this functionality could be used for initialization purpose before LWM2M Bootstrap Server sends Write operation(s) to the LWM2M Client.

The Delete operation has the following parameters:

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | No | - | Indicates the Object from which Object Instance will be deleted; if no Object ID is indicated, all existing Object Instances (except the LWM2M Bootstrap Server Account one) in the LWM2M Client will be deleted |
| Object Instance ID | No | - | Indicates the Object Instance to delete (Object ID MUST be provided) |

#### BOOTSTRAP REQUEST

The Bootstrap Request operation is only performed to initiate the Bootstrap Sequence in the “Client Initiated Bootstrap” mode.

The Bootstrap Request operation has the following parameter:

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| /bs?ep={Endpoint Client Name} | Yes | - | Indicates the LWM2M Client’s “Endpoint Name” in order to allow the LWM2M Bootstrap to provision the Bootstrap Information for the LWM2M Client |

#### BOOTSTRAP FINISH

The Bootstrap  Finish operation is only performed to terminate the Bootstrap Sequence previously initiated either in “Client Initiated Bootstrap” mode or in “Server Initiated Bootstrap” mode.

This command informs the LWM2M Client, that all the Bootstrap Information have been provided by the LWM2M Bootstrap Server

The Bootstrap Finish operation has the following parameter:

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| /bs | Yes | - |  |