Change Request

| Title: | Portfolio Basics | Public OMA Confidential |
| --- | --- | --- |
| To: | DM LightweightM2M WG |  |
| Doc to Change: | OMA-TS-LWM2M_PortfolioObj-V1_0-20160407-D |  |
| Submission Date: | 19/04/2016 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thierry Garnier, Gemalto N.V, thierry.garnier@gemalto.com |  |
| Replaces: | n/a |  |

Reason for Change

- Baseline must be improved in order to properly introduce the Portfolio extended capabilities (Use case II)

- Use case 1 addressing GSMA CLP03 requirement must be detailled

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and agree this CR

Detailed Change Proposal

- Portfolio Baseline  improved

# Scope

This document defines the technical specification for an Object to be used in conjunction with the Lightweight M2M enabler  in order to extend the data storage capability of other Object Instances in the system.

Moreover, such data extension shall come with the support of new functionalities as strong data authentication (which confidence can be granted to a data likely to represent a personal ID), with respect to the appropriate level of  privacy protection.

# References

## Normative References

| [LWM2M] | “OMA LightweightM2M”, Version 1.0, Open Mobile Alliance™, URL:http://www.openmobilealliance.org/ |
| --- | --- |
| [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |

## Informative References

| [OMADICT] [GSMA] | “Dictionary for OMA Specifications”, Version x.y, Open Mobile Alliance™, OMA-ORG-Dictionary-Vx_y, URL:http://www.openmobilealliance.org/ “GSM Association CLP.03 –IOT Device Connection Efficiency Guidelines Version 1.1 30 January 2015” |
| --- | --- |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

## Definitions

| Tamper-resistant component | A hardware component used to store and process private or sensitive information, such as private keys. To prevent an attacker from retrieving or modifying the information, the component is designed so that the information is not accessible through external means and can be accessed only by the embedded software, which should contain the appropriate security device that consumes low power and is limited in its CPU, memory, I/O for processing of requests. |
| --- | --- |

## Abbreviations

| OMA | Open Mobile Alliance |
| --- | --- |

# Introduction and Scope

Some LWM2M Applications may require extended  data storage capacity  over what is available with existing LWM2M Object Instances.  This storage capacity extension can  be used to manage information from a device that is closely related to a LWM2M Client even if this information is not directly used by the LWM2M Client itself (e.g. the identity of a Host Device having an embedded module containing a LWM2M Client).

Moreover, these extended data may be associated with specific constraints. For example, some data could be used to univoically identify a person or a device; in that case appropriate authentication services MUST be proposed to fulfill that need.

The Portfolio Object specification defines a mechanism for extending the data storage capability of other Object Instances in the LWM2M system, as well asthe services which may be used to authenticate and to protect privacy of data contained in those extensions. In addition a data encryption service is also defined.

# Portfolio Object Functionality

The Portfolio Object offers the possibility to extend the data storage capability of another LWM2M Object instance by attaching one of the Portfolio Object Instances to the set of resources of the original LWM2M Object Instance.

Moreover, a second capability of the Portfolio Object is to provide  services for applications associated with this data extension. These services can be used to authenticate identities and to encrypt data.

## 5.1 Portfolio Data Storage Extension

A Portfolio Object Instance and its Resources MUST be fully dedicated to a given LWM2M Object Instance.

Identity Resource (Resource 0 of the Portfolio Object definition) is a mandatory resource that maybe instantated multiple times.

Appendix B provides one example targeting the GSMA CLP0.3 requirement :  a Portfolio Object Instance (e.g. Instance 0) is attached to the LWM2M Device Object Instance (LWM2M Object ‘3’). This Portfolio Object Instance defines four Identity resource instances fitting with the GSMA requirements related to the Host Device information.

- Use Case 1 (GSMA Requirement) detailed

- Portfolio Object Usage Examples

## B.1   IIlustration I   : Current GSMA CLP.03 requirement [GSMA]

Figure 1 :  GSMA CLP.03 Requirement (IoT Device Connection Efficiency Guidelines)

In the GSMA CLP.03 document, there are specific requirements (DID4-7) related to the mandatory capability of the Device Management Technology. LWM2M TS 1.0 using Portfolio Object functionality is able to cover such a requirement as illustrated below :

| DID4 | The following LWM2M resource has been defined to specify information related to the manufacturer of the IoT Host Device, this field will need to match the IoT Device Host manufacturer name that is referenced in the Mobile Network Operator lab certification of the IoT Device. Type: Host Device Manufacturer Occurrence: One Format: String Name: /16/x/0/0 (HostMan) Access Type: READ The IoT Device Host manufacturer will be maintained in this resource by the Communications Module LWM2M client. |
| --- | --- |
| DID5 | The following LWM2M resource has been defined to specify the Model name/number of the IoT Device Host. This shall match the model name/number used in the certification of the IoT Device. Type: Host Device Model Occurrence: One Format: String Name: : /16/x/0/1 (HostMod) Access Type: READ The IoT Host Device model will be maintained in the node by the Communication Module LWM2M client. |
| DID6 | The following LWM2M resource has been defined to specify the software version of the IoT Device Host, this information shall be populated by the IoT Device Host manufacturer, shall match the version of SW certified by PTCRB and must be updated whenever the SW is updated on the device. Type: Host Device Software Version Occurrence: One Format: String Name: : /16/x/0/2 (HostSwV) Access Type: READ The IoT Host Device software version will be maintained in this resource by the Communication Module LWM2M client |
| DID7 | The following OMA-DM node has been defined to specify the unique ID allocated to the IoT Device Host by the certifying Mobile Network Operator. Mobile Network Operators’ may decide to include this field if they need a way to monitor for uncertified devices used on the network. Type: Host Device Unique ID Occurrence: One Format: Alphanumeric String Name: : /16/x/0/3 (HostUniqueID) Access Type: READ The IoT Device Host Unique ID is assigned by the Mobile Network Operator and will be stored in this resource. |

In a given LWM2M Client implementation, the graphical data representation of the GSMA-compatible Connection Module can be illustrated as follows ( e.g. the GSMA Portfolio Object Instance ID is assigned to /16/1).

Figure 2 :  LWM2M Object Device (ID:3) extended in using a link to a dedicated GSMA Portfolio Object Instance