Change Request

| Title: | Portfolio Extended Capability | Public OMA Confidential |
| --- | --- | --- |
| To: | DM LightweightM2M WG |  |
| Doc to Change: | OMA-TS-LWM2M_PortfolioObj-V1_0-20151212-D |  |
| Submission Date: | 19/04/2016 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thierry Garnier, Gemalto N.V, thierry.garnier@gemalto.com David Smith, AT&T, DS9930@att.com |  |
| Replaces: | n/a |  |

Reason for Change

- The Portfolio Extended Capability is specified

- The Portfolio Extended Use Case  (Use case II)  is detailed

- R02 Editorial Changes. OMNA portfolio registration is now 14

- R03  few Editorial Changes

- R04 changed back to ID 16

- R05 Identity is a mandatory resource

- R06 add a co-signature

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and agree this CR

Detailed Change Proposal

The Portfolio Extended Capability is specified

# Portfolio Object Functionality

.

.

.

.

## Portfolio Services

### Resources description

The Portfolio Object defines three Resources to support Services that can be used in conjunction with the Identity resource:  GetAuthData, AuthData, AuthStatus.

- GetAuthData:   defines a executable Resource which is used to trigger the process of generating  data according to the set of arguments provided with the LWM2M Server request.

- Argument  0 :  ‘Service’ :  the type of Service required. Two Services are specified in section 5.2.2 : Data Authentication and Data encryption. Services

0: SRV_AUTHENT: Data Authentication  Service

1: SRV_ENCRYPT:  Data Encryption Service

- Argument 1:   Identity Instance ID  :  determins which Identity Resource Instance is concerned by the GetAuthData request

- Argument 2:  Challenge :  a random value generatedby the LWM2M server to increase the response entropy.

- Argument 3 :  Key Index : when needed, used to select the key which has to be used for the cryptographic operation

- AuthData:   defines a String Resource which receives the data generated according to the GetAuthData request. This resource is a Read-Only resource.

- when AuthBuffer resource receives valid information, the AuthStatus MUST be turned to DATA_AVAIL_STATE

- when AuthBuffer resource is accessed through a LWM2M READ request or a LWM2M NOTIFY, the AuthStatus MUST be reset to IDLE_STATE

- AuthStatus:  defines a Resource which contains the state related to the process triggered by GetAuthData request.

0 :  IDLE_STATE                   :  the  buffer AuthData doesn’t contain any valid data

1 :  DATA_AVAIL_STATE  :  the  buffer AuthData  contains valid data

2 :  ERROR_STATE               :  the process triggered by GetAuthData request incurred an error

  - the AuthStatus MUST be set to IDLE_STATE  when a GetAuthData  request is triggered

  - the AuthStatus MUST be set to DATA_AVAIL_STATE, when AuthData is filled up with valid data

  - the AuthStatus MUST be set to ERROR_STATE when the process triggered by a GetAuthData  request incurred an error

For supporting such Services, the Portfolio Objects resources of this sub-section MUST be bound to the interface of a Tamper-resistant component (e.g. embedded SE as defined in [GP]). This binding is implementation specific.

The Tamper-resistant component MUST be provisioned with a dedicated cryptographic key set for each Portfolio Object Instance Service. The cryptographic key sets MUST NOT be shared by the different Portfolio Object Instances.

### Services Description

- Data Authentication:  this Service will be used when an Identity Resource Instance of a Portfolio Object Instance must be certainly authenticated and must be guaranteed against cloning threats.

Strong authentication requires the cryptographic capability provided by a Tamper-resistant component which MUST be provisioned according to the security policy rules of the Application Provider.

At least two data items MUST be provisioned in the Tamper-resistant component:

- a Hash of the identity to authenticate

- a cryptographic keyset shared with the Server intended to authenticate the identity above.

Cryptographic Process: when a GetAuthData request is forwared to the Tamper-resistant component, this cryptographic element will process the request to generate a Signature of the specified Identity Resource Instance value which MUST be of the following form:

Signature = SIGKn (Hash(Identity) + Challenge)

Where :

- SIG is a signature function shared between the Tamper-resistant component and the Server qualified to authenticate the Identity

- Kn is one cryptographic key of the Data Authentication Service keyset provisioned in the Tamperresistant component and selected by the Argument 3 of the GetAuthData request.

- Hash(Identity) is the data provisioned in the Tamper-resistant component and selected by the Argument 1 of the request

- Challenge is a random value provided by the Server in Argument 2 of the GetAuthData request

- Signature is the result of this cryptographic process which will be stored in the AuthData resource, and will be transported by the LWM2M protocol for authentication by the qualified Server.

Note : the properties of the Hash and Signature functions as well as the cryptographic key set, only have to be known between the Tamper-resistant component and the Server intended to authenticate the provided Identity; that’s why these elements are out of the scope of the Portfolio Object Specification(e.g. Hash function can be based on SHA-2 algorithm, and the Signature function can use the AES-CMAC algorithm specified in [RFC4493]).

2 :  Data Encryption : this Service will be used when a data contained in an Identity Resource Instance of a Portfolio Object Instance needs to be encrypted.

At least one cryptographic key must be provisioned in the Tamper-resistant component

Cryptographic Process: when a GetAuthData request is forwared to the Tamper-resistant component, this cryptographic element will process the request to cipher the specified Identity Resource Instance value. The cipher function MUST be of the form:

Encryption = ENCRYPTKn ( data +Challenge)

Where :

- ENCRYPT is a cryptographic function (e.g. AES-128 function) shared between the Tamper-resistant component and the Server intended to decrypt the data

- Kn is one cryptographic key of the Data Encryption Service key set provioned in the Tamper-resistant component and selected by the Argument 3 of the GetAuthData request.

- data  is the content of the Identity Resource Instance as selected by the Argument 1 of the GetAuthData request

- Challenge is a random value provided by the Server in Argument 3 of the GetAuthData request

- Encryption is the result of this cryptographic process which will be stored in the AuthData resource.

# LWM2M Object: Portfolio

Description

This section formalizes the Resources definitions of the Portfolio Object described in section 5 of this document.

Object definition

Name; Object ID; Instances; Mandatory; Object URN. Portfolio; 16; Multiple; Optional; urn:oma:lwm2m:oma:16.

Resource definition ID; Name; Operations; Instances; Mandatory; Type; Range or Enumeration; Units; Description. 0; Identity; RW; Multiple; Mandatory; String; Data Storage extension for other Object Instances. e.g for [GSMA] : 0: Host Device ID, 1: Host Device Manufacturer 2: Host Device Model 3: Host Device Software Version, This Resource contains data that the GetAuthData executable Resource can work with.. 1; GetAuthData; E; Single; Optional; none; Executable resource to trigger Services described in Section 5.2.2 Arguments definitions are described in Section 5.2.1 as well as in table 2 of this document. 2; AuthData; R; Multiple; Optional; String; Buffer which contains the data generated by the process triggered by a GetAuthData request. 3; AuthStatus; R; Single; Optional; Integer; [0-2]; This Resource contains the state related to the process triggered by GetAuthData request. 0 : IDLE_STATE : AuthData doesn’t contain any valid data 1 : DATA_AVAIL_STATE : AuthData contains valid data 2 : ERROR_STATE : an error occurred This state is reset to IDLE_STATE, when the executable resource “GetAuthData” is triggered or when the AuthData resource has been returned to the LWM2M Server (READ / NOTIFY) ..

Table 1 : Portfolio Object Resources Definition

| Execution Resource Arguments definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ID | Resource Name | Order | Name | Type | Range or Enum | Unit | Description |  |
|  | 1 | GetAuthData | 0 | Service | Integer | [0-1] |  | Data Security Services 0: SRV_AUTHENT: Authentication 1: SRV_ENCRYPT: Encryption |  |
|  |  |  | 1 | Identity Instance Id | Integer | - | - | Identity Resource Instance ID of the current Portfolio Object Instance |  |
|  |  |  | 2 | Challenge | Integer |  |  | A random provided by the Server |  |
|  |  |  | 3 | Key Index | Integer |  |  | To select the cryptographic key in the Service dedicated key set provisioned in the Tamper-resistant component |  |
|  |  |  | 4..9 | - | String |  |  | Application dependent arguments |  |

Table 2: GetAuthData Executable Resourse Arguments Definition

Use case II (Extended Capability) is detailed

- Portfolio Object Usage Examples

## B.2  IIlustration II  :  Extended usage

The use cases below provide only few examples on how the association of the LWM2M Client, the Portfolio Object and a Tamper-resistant component can work together. Itis left to implementation to further enhance the capabilities of such an association.

Figure 2 :  LwM2M Client coupled with a Tamper-resistant component

Coupled with a Tamper-resistant component, the LWM2M Client and the Portfolio Object may support Authentication and end-to-end data encryption services exposed to applications.

Use Case 1 :  Vehicle (Host Device)  Authentication

A Car Manufacturer Server decides to contact a vehicle (Host Device) and would like to rely on the Host Device ID managed by the LWM2M Client of the vehicle to authenticate that car

The LWM2M Client and its association with a Tamper-resistant component have been configured according to an out-of-band process specific to the Car Manufacturer security policy. Typically, the Tamper-resistant component has to be provisioned with the Hash of the Host Device ID and a key shared with the LWM2M Server of the Car Manufacturer.

Following the LWM2M process with the GSMA-based Portfolio functionality guaranties that the Host Device ID is really unique on the network.

Process Illustration for Host Device Authentication

Host Device Authentication Process

Initial Step : Provisioning : in a secure environment according to the Car Manufacturer security policy,  the Tamper-resistant component is provisioned with:

- a key- K0 - shared with the Car Manufacturer LWM2M Server

- a hash of the Host Device ID : Hash(Host Device ID)

Step 1)  Host Device Secured Identification Request

a) An initial request is initiated by the LWM2M Server in addressing the Executable resource GetAuthData of the Portfolio Object Instance with the proper parameters (the DataStatus resource is reset to IDLE_STATE):

e.g. LWM2M command:   EXECUTE /146/0/1   0=SRV_AUTHENT,1=3,2=0x2345178,3=0

- argument 0 :  An Authentication process is requested

- argument 1 : the Identity Resource Instance 3  is  the target  (e.g. Unique Host Device ID)

- argument 2 : 0x2345178 :  a Challenge

- argument 3 :  the cryptographic key 0 of the Authentication key set must be used

            - Information is exchanged between the Tamper-resistant component and the LWM2M Client:

In the Tamper Resistant Component a signature is generated in using the pre-registered key 0 and the hash of the Host Device ID.The challenge is used for introducing a random  in the signature.

Step 2)  the LWM2M Server is polling the AuthStatus Resource or is waiting for a notification related to the availability of the AuthData Resource

Step 3)  when available, a Signature of the requested Identity Resource Instance  [ Sig (Hash(Host Device ID)+ Challenge ) ] is delivered by the Tamper-resistant component into the AuthData Resource of the Portfolio Object Instance.

Step 4) when the Signature is available in the Portfolio Instance (DataStatus Resource is set to DATA_AVAIL_STATE), either a notification is sent to the LWM2M Server with such data or the LWM2M Server will perform a READ request on the AuthData resource(s) of the Portfolio Instance. In both cases the DataStatus Resource is reset to IDLE-STATE.

When receiving this Signature, the LWM2M Server which initiated the request can authenticate the identity of the vehicle with the highest degree of confidence.

Use Case 2 :  Encryption of sensitive data from the Portfolio Object Instance

Very similar to the “Use Case 1” process, this new use case is addressing the need to answer to the Server request for returning sensitive data in encrypted form. The initial data is present in a Portfolio Object Instance (i.e. in one Instance of the Identity multi-instance Resource).

LWM2M Server  request  :     EXECUTE  /16/0/GetAuthData   0=SEC_ENCRYPT, 1=2, 2=0x2345178,3=1

- argument 0 :  the data encryption service is requested (SEC_ENCRYPT)

- argument 1 :  the Identity Instance of ID=2  is the target  (e.g.  Host Device Model)

- argument 2 : 0x2345178 : the Challenge

- argument 3 :  the cryptographic key 1 of  the Authentication key set must be used.

When generated by the Tamper-resistant component, the AuthData Resource will contain an encrypted value for e.g. the Host Device Model data