Change Request

| Title: | Smartcard Bootstrap | Public OMA Confidential |
| --- | --- | --- |
| To: | DM LightweightM2M WG |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0-20161007-D |  |
| Submission Date: | 18/10/2016 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Mohammed Dadas, mohammed.dadas@orange.com Thierry Garnier, Thierry.GARNIER@gemalto.com Nicolas Damour, ndamour@sierrawireless.com |  |
| Replaces: | n/a |  |

Reason for Change

During the Bootstrap, whatever is the mode used, when the device supports a Smartcard, it must retrieve and process the bootstrap data from the Smartcard.

In some cases, the bootstrap information of the Smartcard is not the one needed by the client and it should be possible of not using it.

This CR proposes a solution to allow the client to choose the Bootstrap information it wants to use.

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

It is recommended that OMA DM group to review and agree to the changes presented in this change request.

Detailed Change Proposal

Section 5. Bootstrap from Smartcard

#### Bootstrap from Smartcard

When the Device supports a Smartcard, the LWM2M Client MUST retrieve and process the bootstrap data contained in the Smartcard as described in Appendix F. When the bootstrap data retrieval is successful, the LWM2M Client MUST process the bootstrap data from the Smartcard and MAY apply the Bootstrap Information to its configuration.

Due to the sensible nature of the Bootstrap Information, a secure channel SHOULD be established between the Smartcard and the LWM2M Device.

When such a secure channel is established between the Smartcard and the LWM2M Device, this secure channel MUST be based on [GLOBALPLATFORM] procedure, mainly described in Appendix G.

In this Bootstrap mode, if the LWM2M Client intends to apply the Bootstrap Information to its configuration, it MUST also ensure that the Bootstrap Information previously retrieved from the Smartcard is unchanged within the Smartcard. If it is changed, and if the previous Bootstrap Information was applied from Smartcard, this Bootstrap Information MUST be disabled in the LWM2M Client and the LWM2M Client MUST apply the new Bootstrap Information from Smartcard to its configuration.

Disabling the Smartcard (e.g. removing the Smartcard) within the LWM2M Client requires the Bootstrap Information created from the bootstrap data of the previous Smartcard MUST be deleted.

Checking for Smartcard change and disabling MUST be performed by LWM2M Client, each time a “Register” or “Update” operation take place, with a LWM2M Server provisioned from Smartcard. As usual, the Bootstrap security rules (5.2.4) then apply.

NOTE: Bootstrap Information in Smartcard can be updated by using Smartcard OTA protocol as specified in ETSI TS 102 225 [ETSI TS 102.225] / TS 102 226 [ETSI TS 102 226] and extensions such as 3GPP TS 31.115 [3GPP TS 31.115] / TS 31.116 [3GPP TS 31.116] and 3GPP2 C.S0078-0 [3GPP2 C.S0078-0] / C.S0079-0 [3GPP2 C.S0079-0].