## Attributes

### Attributes Definitions and Rules

Attributes are metadata which can be attached to an Object, an Object Instance or a Resource. The value of an Attribute is LwM2M Server specific. These attributes can fulfil various roles: from just carrying information (e.g., Discover), up to containing parameters for Notification for example.

Attributes attached to Object, Object Instance, Resource, are respectively named O-Attribute, OI-Attribute, R-Attribute.

These Attributes MAY be carried in the message payload of Registration and Discover operations; they also MAY be updated - when writable - through the “Write-Attributes” operation.

Regardless to the LwM2M entity a given Attribute is attached to, the value of such an Attribute can be set at various levels: Object, Object Instance, Resource levels. Additionally, precedence rules apply when the same Attribute receives a value at different levels.

Following rules governs usage of LwM2M Attributes

- The value of an O-Attribute MAY only be set at the Object level.

- The value of an OI-Attribute MAY be set at the Object Instance level, and also at the Object level.

precedence rules:

- MUST Rule 1: When set at both levels, the value of the OI-Attribute set at Object Instance level will prevail.

- MUST Rule 2: When the Attribute value is set at the Object level, the scope of the OI-Attribute value extends to all the Instances of that Object, as long as the Rule 1 is respected.

- An R-Attribute MAY be set at 3 different levels: the Resource level, the Object Instance level and the Object level.

precedence rules:

- MUST Rule 3: When set at the Resource level, the value of an R-Attribute prevails for that Resource whatever a value for this R-Attribute is also specified at an upper level (Object or Object Instance level).

- MUST Rule 4: When set at the Object Instance level, the scope of an R-Attribute value extends to all the Resources of that Object Instance as long as the Rule 3 is respected.

- MUST Rule 5: When set at the Object level, the scope of an R-Attribute value SHALL extend to all the resources of any Instance of that Object, as long as the Rule 4 is respected.

An attribute is fully determined by several characteristics which are listed in the table below:

| Attribute characteristics | Description |
| --- | --- |
| Name | Attribute Name used to reference a specific Attribute in that Enabler (e.g., “Minimum Period”) |
| CoRE Link Param | the string used when this Attribute is transferred to CoAP as a CoRE link parameter (ex pmin) |
| Attachment | The Object, Object Instance or Resource, to which an Attribute applies |
| Assignation Level | The Level (Object, Object Instance, Resource) where the value of the Attribute MAY be set. |
| Class | Attributes are organized according to their purpose; 2 Class of Attributes are supported in LwM2M TS 1.0 <NOTIFICATION> gather Attributes regarding Notify operations parameters <PROPERTIES> gather Attributes regarding general information |
| Access Mode | R, W, RW: operation allowed by the LwM2M Server. |
| Applicability | Condition to fulfil for allowing to attach such an Attribute |
| Default Value | <value> or “-” or “ ” |
| Value Type | Data Type (Refer Appendix C) |
| Value | The Value carried by this Attribute : its data type must be of “Value Type” |

Table 2: Attribute Characteristics

Some Attributes can be exposed to the LwM2M Server in the payload response to a “Discover” command (Section 5.4.2).

The value of some Attributes can be changed by the LwM2M Server in using the “Write-Attributes” command (Section 5.4.4); which Attribute are concerned are marked as “W” (writable) in the table of the next Section.

Note: A payload response to a “Discover” command is a list of application/link-format CoRE Links [RFC6690] which will include the LwM2M Attributes