# Appendix B :

## B.0  use case and high level requirement – all

| ID | Use Case | High level requirement | Reference | Presented Company(s) | Version planned |
| --- | --- | --- | --- | --- | --- |
| B1 | 3GPP CIoT and emergence of LPWAN technologies to be supported by LwM2M releases (NB-IoT and LTE CAT-M) |  | 2017_INP91 | Vodafone, Nokia, ARM, Orange, Gemalto | 1.x |
|  |  | Support of connection management and 3GPP CIoT parameters in LwM2M |  |  |  |
|  |  | Support of SCEF path in 3GPP CIoT |  |  |  |
|  |  | Support of IP path inside 3GPP CIoT |  |  |  |
|  | LoRA & LTE-Cat M Support | A general framework for supporting LPWAN (non IP) in LwM2M is expected in the field | 2017 _INP60 | ORANGE*, Gemalto | 1.x |
| B2 | Maintenance and upgrade of constrained devices would be necessary in the field once deployed. This should avoid unnecessary overheads in terms of configuration and reconfigurations. |  | 2017_INP91 | Nokia | 1.x |
|  |  | Upgrading firmware preserves pre-upgrade settings and device comes online without bootstrapping |  | Nokia |  |
|  |  | Reverting to previous firmware is made possible and device comes online without bootstrapping |  | Nokia |  |
| B3 | When supporting multiple LWM2M server instances it is sometime necessary to have a configurable timer (seconds) with which the client waits after being bootstrapped to register the first time with each to the server instances. This provides an opportunity for a provisioning system to learn of the deviceID and bs/ created credentials and provision them to the proper server instances prior to client registering |  | 2017_INP91 | Nokia | 1.x |
| B4 | In TS 1.0 handling multiples resource instances is painful to use (In field scenarios without having ability to address resources directly in the LwM2M command it creates multiple handshake’s) | Extended addressing for multiple resource instances | 2017 _INP60 | Gemalto*, Sierra Wireless, ARM, ORANGE, Nokia | 1.x |
|  |  | ACL to be extended for resource level. | 2017_INP91 | (Nokia) |  |
| B5 | OMA-JSON has good capability but compactness is far from the OMA-TLV. In order to reduce the size of payload further CBOR can be utilized in LwM2M to achieve better compression as well utilize all the functionalities provided by JSON. | OMA-CBOR compact Media Type introduction | 2017 _INP60 | Gemalto*, Sierra Wireless ARM ORANGE Ericsson Nokia | 1.x |
| B6 | Remove the last limitation for Incremental Bootstrap capability | Bootstrap Enhancement | 2017 _INP60 | Gemalto*, ORANGE | 1.x |
| B7 | An Instance of a virtual object refers resources of various Object Instances. | Virtual Object Concept | 2017 _INP60 | Gemalto*, ORANGE, U-Blox, HUAWEI | 1.x |
| B9 | Generalized framework for extending LwM2M Security (SE, eUICC-M2M support) | Secure Element Support | 2017 _INP60 | Gemalto*, Ericsson, ORANGE, Nokia | 1.x |
| B10 |  |  |  |  |  |