# Appendix B :

## B.0  use case and high level requirement – all

| ID | Use Case | High level requirement | Reference | Presented Company(s) |  | Version planned |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1 | 3GPP CIoT and emergence of LPWAN technologies to be supported by LwM2M releases (NB-IoT and LTE CAT-M) |  | 2017_INP91 | Vodafone, Nokia, ARM, Orange, Gemalto, Ublox |  | 1.x |  |
|  |  | Support of connection management and 3GPP CIoT parameters in LwM2M |  |  |  |  |  |
|  |  | Support of SCEF path in 3GPP CIoT |  |  |  |  |  |
|  |  | Support of IP path inside 3GPP CIoT |  |  |  |  |  |
|  | LoRA & LTE-Cat M Support | A general framework for supporting LPWAN (non IP) in LwM2M is expected in the field | 2017 _INP60 | ORANGE*, Gemalto |  | 1.x |  |
| B2 | Maintenance and upgrade of constrained devices would be necessary in the field once deployed. This should avoid unnecessary overheads in terms of configuration and reconfigurations. |  | 2017_INP91 | Nokia |  | 1.x |  |
|  |  | Upgrading firmware preserves pre-upgrade settings and device comes online without bootstrapping |  | Nokia |  |  |  |
|  |  | Reverting to previous firmware is made possible and device comes online without bootstrapping |  | Nokia |  |  |  |
| B3 | When supporting multiple LWM2M server instances it is sometime necessary to have a configurable timer (seconds) with which the client waits after being bootstrapped to register the first time with each to the server instances. This provides an opportunity for a provisioning system to learn of the deviceID and bs/ created credentials and provision them to the proper server instances prior to client registering |  | 2017_INP91 | Nokia |  | 1.x |  |
| B4 | In TS 1.0 handling multiples resource instances is painful to use (In field scenarios without having ability to address resources directly in the LwM2M command it creates multiple handshake’s) | Extended addressing for multiple resource instances | 2017 _INP60 | Gemalto*, Sierra Wireless, ARM, ORANGE, Nokia |  | 1.x |  |
|  |  | ACL to be extended for resource level. | 2017_INP91 | (Nokia) |  |  |  |
| B5 | OMA-JSON has good capability but compactness is far from the OMA-TLV. In order to reduce the size of payload further CBOR can be utilized in LwM2M to achieve better compression as well utilize all the functionalities provided by JSON. | OMA-CBOR compact Media Type introduction | 2017 _INP60 | Gemalto*, Sierra Wireless ARM ORANGE Ericsson Nokia, u-blox |  | 1.x |  |
| B6 | Remove the last limitation for Incremental Bootstrap capability | Bootstrap Enhancement | 2017 _INP60 | Gemalto*, ORANGE, u-blox |  | 1.x |  |
| B7 | An Instance of a virtual object refers resources of various Object Instances. | Virtual Object Concept | 2017 _INP60 | Gemalto*, ORANGE, u-blox, HUAWEI |  | 1.x |  |
| B9 | Generalized framework for extending LwM2M Security (SE, eUICC-M2M support). | Secure Element Support | 2017 _INP60 | Gemalto*, Ericsson, ORANGE, Nokia |  | 1.x |  |
| B10 | 1) Aggregation of service and DM data in a single message 2) Observing multiple resources with a single notify | Define New LwM2M operations with binding to CoAP FETCH and PATCH methods as specified in RFC 8132 | 2017_INP89 |  | u-blox, Huawei, Vodafone |  | 1.x |
| B10’ | When a LwM2M server needs to get/observe resource data from the different objects at the same time, a mechanism that reports the data in a single message instead of in several messages is desirable to reduce the number of message exchanges, so as to save the power consumption and the bandwidth of the constrained device/network. | Support aggregation of resource data from different objects in a single reporting message. | 2017_INP74 | Huawei |  | 1.x |  |
| B10” | Make registration interface more efficient | Add support for Patch | 0121-INP_patch_support (note also 0089R01-INP_Binding_to_CoAP_FETCH_PATCH) | ARM |  | 1.x |  |
| B11 | When a LwM2M server needs to issue the same commands or data to a group devices, a group multicast mechanism can increase transmission efficiency and save the bandwidth. | Support group multicast (RFC7390) for issuing the same commands or data to a group of devices. | 2017_INP83 | Huawei u-blox |  | 1.x |  |
| B12 | Extended support of the PKI infrastructure | Add new certificate provisioning types, and discuss secure time and revocation strategy | OMA-DM- 2017-0042- INP_security_ features | ARM |  | 1.x |  |
| B13 | Alignment with IoT DTLS/TLS security recommendations |  | OMA-DM- 2017-0043- INP_rfc7925 _support | ARM, u-blox |  | 1.x |  |
| B14 | Adding support for user identity management |  | OMA-DM- 2017-0044- INP_user_ identity_ management | ARM |  | 1.x |  |
| B15 | Support for multi-tenancy | Add additional identifier to indicate customer | OMA-DM-2017-0045-INP_multi_tenancy | ARM |  | 1.x |  |
| B16 | Make communication with Bootstrap Server more robust in error situations. | Provide more details on when a LwM2M has to re-connect to the bootstrap server to recover from error conditions | 0114-INP_bootstrap_reconnect | ARM, u-blox |  | 1.x |  |
| B17 | Support non-LwM2M legacy devices (behind a gateway) | Extend the data model so that a LwM2M server can interface with a LwM2M client running on a gateway to interface non-LwM2M-enabled devices. | 0115-INP_legacy_gateway | ARM |  | 1.x |  |
| B18 | Make queue mode more efficient by taking the resource volatility into account. | Support resource volatility | 0117-INP_volatility | ARM |  | 1.x |  |
| B19 | Make LwM2M robust in environments where firewalls block UDP traffic. | Support CoAP over TCP specification | 0118-INP_coap_over_tcp | ARM |  | 1.x |  |