| ## Test |  |
| --- | --- |
| ### Description |  |
| Support for ETS testcases |  |
| ### Object definition |  |
| Name; Object ID; Object Version; LWM2M Version. Test; 1023; 1.0; 1.0. Object URN; Instances; Mandatory. Multiple; Optional. |  |
| ### Resource definitions |  |
| ID; Name; Operations; Instances; Mandatory; Type; Range or Enumeration; Units; Description. 0; RW String; RW; Single; Mandatory; String; Capable to hold 16-character string.. 1; RW Integer; RW; Single; Mandatory; Integer. 2; RW Float; RW; Single; Mandatory; Float. 3; RW Boolean; RW; Single; Mandatory; Boolean. 4; RW Opaque; RW; Single; Mandatory; Opaque; Capable to hold 16 bytes of data.. 5; RW Time; RW; Single; Mandatory; Time. 6; RW Object link; RW; Single; Mandatory; Objlnk. 7; Read Only; R; Single; Mandatory; Integer; Value set to 1207.. 8; Write Only; W; Single; Mandatory; Integer. 9; Clear All; E; Single; Mandatory; Opaque; Reset value of resource 0 to "", 3 to false, 4 to a 0-length data, 6 to 0:0 and resources 1, 2 and 5 to 0.. 10; Multiple; RW; Multiple; Mandatory; Integer. |  |