Change Request

| Title: | CoAP Blockwise Transfer for Non-IP | Public OMA Confidential |
| --- | --- | --- |
| To: | DM WG |  |
| Doc to Change: | OMA-TS-LightweightM2M_Transport-V1_1-20170915-D |  |
| Submission Date: | 2017-12-10 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Sandeep Akhouri, Ericsson, Sandeep.akhouri@ericsson.com |  |
| Replaces: | n/a |  |

Reason for Change

Appendix M: LwM2M over NB-IoT specifies that CoAP block transfer MAY be used for CoAP over NB-IoT in both IP and non-IP mode. However, this is not reflected with the TS in the appropriate sections.

IETF has defined segmentation handling at the CoAP layer for large file transfer e.g. firmware updates. Blockwise transfers in CoAP.

CoAP block transfer MAY be used with for carrying CoAP over NB-IoT. This option can be used with IP-mode and non-IP-mode.

Existing Text in TS:

CoAP Blockwise Transfer MUST be supported by the LwM2M Client when the Firmware Update Object (ID:5) is iimplemented by the client and MUST be supported by the LwM2M Server.

CoAP Header itself does NOT contain the length information, Blockwise Transfer relies upon the UDP Header. The maximum UDP packet size is 64KB.

Impact on Backward Compatibility

None

Impact on Other Specifications

Additional requirement

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

To review and accept the proposed requirements

Detailed Change Proposal

Requirements (section 6 Requirements)

# Transport Layer Bindings and Encoding

## Required Features

Additional Proposed Text:

CoAP Blockwise Transfer can also be benefitial for transmission of large payloads over non-IP.

References