| # Enabler Validation Plan for BCAST |  |  |  |
| --- | --- | --- | --- |
| Draft Version 1.1 – 23 Jun 2010 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-EVP-BCAST-V1_1-20101108-D |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2010 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

Contents

1	Scope	5

1.1	Assumptions	5

1.2	Exclusions	5

2	References	6

2.1	Normative References	6

2.2	Informative References	7

3	Terminology and Conventions	8

3.1	Conventions	8

3.2	Definitions	8

3.3	Abbreviations	8

4	Enabler Validation Description	10

5	TestFest Activities	11

5.1	Enabler Test Guidelines	11

5.1.1	Minimal Test Configuration	11

5.1.2	Minimal Participation Guidelines	12

5.1.3	Optimal TestFest Achievement Guidelines	12

5.2	Enabler Test Requirements	18

5.2.1	Test Infrastructure Requirements	18

5.2.2	Test Restrictions	19

5.2.3	Service Guide Related Flows	20

5.2.4	File Distribution Related Flows	23

5.2.5	Stream Distribution Related Flows	27

5.2.6	Service & Content Protection Related Flows	31

5.2.7	Interaction Channel Related Flows	33

5.2.8	Service Provisioning Related Flows	34

5.2.9	Notification Function Related Flows	38

5.2.10	Terminal Provisioning Related Flows	41

5.2.11	Audience Measurement Function Related Flows	41

5.2.12	Test Content Requirements	42

5.2.13	Test Limitations	42

5.2.14	Test Restrictions	43

5.2.15	Test Tools	43

5.2.16	Resources Required	43

5.3	Tests to be Performed	44

5.3.1	Minimal Pre-Test Guidelines	44

5.3.2	Configuration of Pre-Testing Phase	46

5.3.3	Testing to be Performed at TestFest	47

5.4	Enabler Test Reporting	48

5.4.1	Problem Reporting Requirements	48

5.4.2	Enabler Test Requirements	48

6	Alternative Validation Activities	49

7	Approval Criteria	50

7.1	Enabler Validation Test Cases	50

7.2	Non Covered ETR Requirements	53

Appendix A.	Change History (Informative)	56

A.1	Approved Version History	56

A.2	Draft/Candidate Version 1.1 History	56

Tables

Table 2: Enabler Validation Test Cases	44

Table 3: Non-Covered ETR Requirements	44

# Scope

This document details the Validation plan for the BCAST 1.1 Enabler Release.  The successful accomplishment of the validation activities will be required for the Enabler to be considered for Approved status.

Mobile Broadcast Services 1.1 (BCAST 1.1) provides 3 new BDSs (DVB-SH, Forward Link Only-IP, and WiMAX), enhancement of existing functions and new functions such as Audience Measurement, Rich Media Solution and Smartcard/Terminal Provisioning over Broadcast.

The validation plan for the BCAST 1.1 Enabler Release specifications is based on testing expectations in the Enabler Test Requirements (ETR). While the specific test activities to be performed are described in the Enabler Test Specification (ETS) the test environment is described in this plan. This test environment details infrastructure, operational and participation requirements identified for the needed testing activities.

## Assumptions

None

## Exclusions

None

# References

## Normative References

| [BCAST1.1-FLO-IP Adaptation] | "Broadcast Distribution System Adaptation –Forward Link Only", Open Mobile Alliance™, OMA-TS-BCAST_FLO_Adaptation-V1_1, URL: http://www.openmobilealliance.org/ |
| --- | --- |
| [BCAST1.1WiMAX-Adaptation] | "Broadcast Distribution System Adaptation – WiMAX", Open Mobile Alliance™, OMA-TS-BCAST_WiMAX_Adaptation-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-BCMCS-Adaptation] | "Broadcast Distribution System Adaptation – 3GPP2/BCMCS", Open Mobile Alliance™, OMA-TS-BCAST_BCMCS_Adaptation-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-Distribution] | "File and Stream Distribution for Mobile Broadcast Services", Open Mobile Alliance™, OMA-TS-BCAST_Distribution-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-DVBH-IPDC-Adaptation] | "Broadcast Distribution System Adaptation – IPDC over DVB-H", Open Mobile Alliance™, OMA-TS-BCAST_DVB_Adaptation-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-DVBSH-IPDC-Adaptation] | "Broadcast Distribution System Adaptation – IPDC over DVB-SH", Open Mobile Alliance™, OMA-TS-BCAST_DVBSH_Adaptation-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-ETR] | “Enabler Test Requirements for Mobile Broadcast Services”, Open Mobile Alliance™, OMA-ETR-BCAST-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-MBMS-Adaptation] | "Broadcast Distribution System Adaptation – 3GPP/MBMS", Open Mobile Alliance™, OMA-TS-BCAST_MBMS_Adaptation-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-Requirements] | "Mobile Broadcast Services Requirements", Open Mobile Alliance™, OMA-RD-BCAST-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-ServContProt] | "Service and Content Protection for Mobile Broadcast Services", Open Mobile Alliance™, OMA-TS-BCAST_SvcCntProtection-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-Services] | "Mobile Broadcast Services", Open Mobile Alliance™, OMA-TS-BCAST_Services-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-SG] | "Service Guide for Mobile Broadcast Services", Open Mobile Alliance™, OMA-TS-BCAST_ServiceGuide-V1_1, URL: http://www.openmobilealliance.org/ |
| [DRM20-Broadcast-Extensions] | "OMA DRM v2.0 Extensions for Broadcast Support", Open Mobile Alliance™, OMA-TS-DRM_XBS-V1_1-20091027-D, URL: http://www.openmobilealliance.org/ |
| [DRM-v2.0] | “DRM Specification V2.0”, Open Mobile Alliance™, OMA-DRM-DRM-V2_0, URL: http://www.openmobilealliance.org/ |
| [IOPPROC] | “OMA Interoperability Policy and Process”, Version 1.1, Open Mobile Alliance™, OMA-IOP-Process-V1_1, URL: http://www.openmobilealliance.org/ |
| [OMA DM 1.2] | “Enabler Release Definition for OMA Device Management v1.2”, OMA-ERELD-DM-V1_2_0, Open Mobile Alliance™, URL: http://www.openmobilealliance.org/ |
| [OMA DM 1.3] | “Enabler Release Definition for OMA Device Management v1.3”, OMA-ERELD-DM-V1_3_0, Open Mobile Alliance™, URL: http://www.openmobilealliance.org/ |
| [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL: http://www.ietf.org/rfc/rfc2119.txt |
| [SCRRULES] | “SCR Rules and Procedures”, Open Mobile Alliance™, OMA-ORG-SCR_Rules_and_Procedures, URL: http://www.openmobilealliance.org/ |

## Informative References

| [BCAST11-Architecture] | "Mobile Broadcast Services Architecture", Open Mobile Alliance™, OMA-AD- BCAST-V1_1, URL: http://www.openmobilealliance.org/ |
| --- | --- |
| [BCAST11-EICS-Client] | "Client Enabler Implementation Conformance Statement”, Version 1.1, Open Mobile Alliance™, OMA-EICS-BCAST_Client-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-EICS-Server] | "Server Enabler Implementation Conformance Statement”, Version 1.1, Open Mobile Alliance™, OMA-EICS-BCAST_Server-V1_1, URL: http://www.openmobilealliance.org/ |
| [BCAST11-ETS-IOP] | "Interoperability Enabler Test Specification for BCAST", Version 1.1, Open Mobile Alliance™, OMA-ETS-BCAST_IOP-V1.1, URL: http://www.openmobilealliance.org/ |
| [OMADICT] | “Dictionary for OMA Specifications”, Open Mobile Alliance™. Version 2.7, OMA-ORG-Dictionary-V2_7, URL: http://www.openmobilealliance.org/ |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope”, are normative, unless they are explicitly indicated to be informative.

## Definitions

| Application | A value-added data service provided to a Client. The application may utilise BCAST and/or other ways of data transfer to deliver content |
| --- | --- |
| Bearer Network | A network used to carry the messages of a transport-layer protocol between physical devices. Multiple bearer networks may be used over the life of a single push session. |
| Client | A client is a device or service that expects to receive BCAST content from a server. |
| Content | Subject matter (data) stored or generated at an origin server. Content is typically displayed or interpreted by a user agent on a client. |
| Device | Is a network entity that is capable of sending and/or receiving packets of information and has a unique device address. A device can act as either a client or a server within a given context or across multiple contexts. For example, a device can service a number of clients (as a server) while being a client to another server. |
| End-user | See "user" |
| Server | A device (or service) which accepts or rejects a connection request from a client. A server may initiate a connection to a client or many clients as part of a service. |
| User | A user is a person who interacts with a user agent to view, hear, or otherwise use a rendered content. Also referred to as end-user. |
| User agent | A user agent (or content interpreter) is any software or device that interprets resources. This may include multimedia players, news tickers, etc. |

## Abbreviations

| BDS | Broadcast Distribution System |
| --- | --- |
| ETR | Enabler Test Requirements |
| ETS | Enabler Test Specification |
| GPRS | General Packet Radio Service |
| HTTP | Hypertext Transfer Protocol |
| IP | Internet Protocol |
| MS | Mobile Station |
| MSISDN | Mobile Station International Subscriber Directory Number |
| OMA | Open Mobile Alliance |
| QoS | Quality of Service |
| RFC | Request For Comments |
| TCP | Transmission Control Protocol |
| UDP | User Datagram Protocol |
| URI | Uniform Resource Identifier |
| URL | Uniform Resource Locator |

# Enabler Validation Description

A full description of BCAST 1.1 can be found in the BCAST11-ERELD, ETR, TS and other documents mentioned in the references. For details see chapter 5.

# TestFest Activities

## Enabler Test Guidelines

### Minimal Test Configuration

The tests will be done with servers and matching clients i.e. testing a MBMS server against IPDC clients will not be planned.

Server 
(BDS agnostic)

BDS agnostic Client 
(if suitable network interface exists)

Server 
(BDS specific)

Client 
(BDS specific)

Mode 2

Mode 1

In order to minimize the number of test cases the tests will be defined in four sets (BDS agnostic, IPDC, MBMS and BCMCS) so that the BDS agnostic parts don't need to be tested four times.

BDS dependent (6 separate sets of test cases: IPDC, MBMS, BCMCS, WiMAX, FLO, DVBSH)

BDS specific tests can start at (2) if the BDS agnostic tests (between 1 and 2) have been passed.

### Minimal Participation Guidelines

Minimum

- three client implementations

and

- three BCAST servers

are needed.

The clients and the servers have to match. Having e.g. one IPDC client, one MBMS client and one MBMS server is not sufficient.

In case a test fest is desirable to happen with less servers or clients such requests will be handled as exceptions. Privacy regarding test results could be handled in that way that test results are not published. Details will be negotiated with the test event organizer and the participants. Such exceptions have happened at test fests before.

### Optimal TestFest Achievement Guidelines

The TestFest participants are expected to do as much testing as possible during the dedicated test slot. Full list of interoperability test cases can be found from the BCAST Interoperability Enabler Test Specification [BCAST11-ETS_IOP]. The list of test cases executed during a TestFest may vary depending on the special characteristics of the test environment (e.g. the used BDS or the available content protection methods).

In case there is a need to prioritize testing during the TestFest, due time constraints or otherwise, the following prioritization should be used as described in Table 1.

#### Service Provisioning (SP)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.1.1 Service bootstrap and single content | BCAST-1.1-DIST-int-101 | Low (pre-test) |
| 5.1.2 Web-based Service Provisioning | BCAST-1.1-DIST-int-102 | Low |
| 5.1.3. Service Guide discovery using DNS | BCAST-1.1-DIST-int-115 | High |

#### Service Guide (SG) - Broadcast

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.2.1 Service Guide update (same fragment id, Lower version number) | BCAST-1.1-DIST-int-103 | Low (pre-test) |
| 5.2.3 Service Guide Update (new fragment id) – Broadcast Channel | BCAST-1.1-DIST-int-105 | Low |
| 5.2.5 GZIP compression of Service Guide Delivery Unit | BCAST-1.1-DIST-int-107 | Low (pre-test) |
| 5.2.6 Content hierarchy | BCAST-1.1-DIST-int-108 | Low |
| 5.2.7 PreviewData and Service | BCAST-1.1-DIST-int-109 | Low |
| 5.2.9 Select language specific access parameters | BCAST-1.1-DIST-int-111 | Low |
| 5.2.10 Subscription of Service | BCAST-1.1-DIST-int-112 | Low |
| 5.2.11 Select language specific Service Guide elements | BCAST-1.1-DIST-int-113 | Low |

#### Service Guide (SG) - Interactive

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.2.2 Service Guide update (same fragment id, Lower version number) – Interaction Channel | BCAST-1.1-DIST-int-104 | Low (pre-test) |
| 5.2.4 Service Guide Update (new fragment id) – Interaction Channel | BCAST-1.1-DIST-int-106 | Low |
| 5.2.5 GZIP compression of Service Guide Delivery Unit | BCAST-1.1-DIST-int-107 | Low (pre-test) |
| 5.2.6 Content hierarchy | BCAST-1.1-DIST-int-108 | Low |
| 5.2.7 PreviewData and Service | BCAST-1.1-DIST-int-109 | Low |
| 5.2.9 Select language specific access parameters | BCAST-1.1-DIST-int-111 | Low |
| 5.2.10 Subscription of Service | BCAST-1.1-DIST-int-112 | Low |

#### File Distribution (FD)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.3.1.1 Support of ALC protocol and delivery of meta-data in the Service Guide | BCAST-1.1-DIST-int-201 | Low |
| 5.3.1.2 Support of in-band delivery of meta-data and FLUTE | BCAST-1.1-DIST-int-202 | Low (pre-test) |
| 5.3.1.3 Support the delivery using HTTP over Interaction Channel | BCAST-1.1-DIST-int-203 | Low |
| 5.3.1.4 Support of FEC RAPTOR | BCAST-1.1-DIST-int-204 | Low |
| 5.3.1.5 Support of the post-delivery repair of files | BCAST-1.1-DIST-int-205 | Low |
| 5.3.1.6 Support of reception report | BCAST-1.1-DIST-int-206 | Low |
| 5.3.1.7 Support of Flute Session Setup and Control with RTSP | BCAST-1.1-DIST-int-207 | Low |
| 5.3.1.8 Support of hybrid broadcast/interactive file distribution – FLUTE as fallback | BCAST-1.1-DIST-int-212 | High |
| 5.3.1.9 Support of hybrid broadcast/interactive file distribution – HTTP as fallback | BCAST-1.1-DIST-int-213 | High |

#### Stream Distribution (SD)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.3.2.1 Support of RTP for stream distribution over the broadcast channel | BCAST-1.1-DIST-int-208 | Low (pre-test) |
| - Support of RTP for stream distribution over the interactive channel using SDP | BCAST-1.1-DIST-int-209 | Low |
| - Support of RTP for stream distribution over the interactive channel using HTTP with out-of-band signalling | BCAST-1.1-DIST-int-210 | Low |
| - Support of streaming associated procedure | BCAST-1.1-DIST-int-211 | Low |
| 5.3.2.5 Support of hybrid broadcast/interactive stream distribution | BCAST-1.1-DIST-int-214 | High |
| 5.3.2.6 Support of advisable time ranges for access switch | BCAST-1.1-DIST-int-213 BCAST-1.1-DIST-int-215 | High |

#### Service Interaction (SI)  - Broadcast

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.4.1 XHTML MP Interactivity | BCAST-1.1-DIST-int-301 | Low (pre-test) |
| 5.4.3 SMS interactivity | BCAST-1.1-DIST-int-303 | Low |
| 5.4.5 MMS Interactivity | BCAST-1.1-DIST-int-305 | Low |
| 5.4.7 Select language specific Interactivity | BCAST-1.1-DIST-int-307 | Low |

#### Service Interaction (SI) - Interactive

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.4.2 XHTML MP Interactivity – Interaction Channel | BCAST-1.1-DIST-int-302 | Low |
| 5.4.4 SMS interactivity – Interaction Channel | BCAST-1.1-DIST-int-304 | Low |
| 5.4.6 MMS Interactivity – Interaction Channel | BCAST-1.1-DIST-int-306 | Low |
| 5.4.7 Select language specific Interactivity | BCAST-1.1-DIST-int-307 | Low |

#### Service Protection (ServProt) - DRM

| Description | Test Case Id | Priority |
| --- | --- | --- |
| - Delivery of IPSec protected stream | BCAST-1.1-DIST-int-401 | Low (pre-test) |
| - Delivery of SRTP protected stream | BCAST-1.1-DIST-int-402 | Low (pre-test) |
| - Delivery of ISMACrypt protected stream | BCAST-1.1-DIST-int-403 | Low (pre-test) |

#### Roaming (R)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.6.1 Availability of Roaming and Showing SG of visited service provider | BCAST-1.1-MORO-int-101 | Low |

#### Parental Control for Service Ordering (PCSO)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.7.1 Parental control for service ordering using the Generic Solution | BCAST-1.1-PCSO-int-102 | High |

#### Parental Control of Unicast Services (PCUS)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 5.8.1 Parental control of unicast service consumption – RTSP service | BCAST-1.1-DIST-int-114 BCAST-1.1-PCUS-101 | High |
| 5.8.2 Parental control of unicast service consumption – HTTP service | BCAST-1.1-DIST-int-115 BCAST-1.1-PCUS-102 | High |

#### Service Protection (ServProt) - SmartCard

| Description |  | Test Case Id |  | Priority |  |
| --- | --- | --- | --- | --- | --- |
| 5.5.2.1.1 GBA-U Bootstrapping USIM / BSM with success |  | BCAST-1.1-DIST-int-404 |  | Low (pre-test) |  |
| 5.5.2.1.2 GBA-U Bootstrapping USIM / BSM with synchronization error |  | BCAST-1.1-DIST-int-405 |  | Low |  |
| 5.5.2.1.3 GBA_U: Expired Bootstrapping data |  | BCAST-1.1-DIST-int-406 |  | Low |  |
| 5.5.2.1.4 GBA_U: Different Key K on Client and Server |  | BCAST-1.1-DIST-int-407 |  | Low |  |
| 5.5.2.1.5 Deregistration |  | BCAST-1.1-DIST-int-408 |  | Low |  |
| 5.5.2.1.6 Deregistration with bootstrapping |  | BCAST-1.1-DIST-int-409 |  | Low |  |
| 5.5.2.1.7 Subscriber Key Establishment for (R-)UIM/CSIM |  | BCAST-1.1-DIST-int-410 |  | Low (pre-test) |  |
| 5.5.2.2.1.1 LTKM without SPE, without consumption reporting, MBMS only card |  | BCAST-1.1-DIST-int-411 |  | Low |  |
| 5.5.2.2.1.2 LTKM without SPE, Without consumption reporting, BCAST card |  | BCAST-1.1-DIST-int-600 |  | Low |  |
| 5.5.2.2.1.3 LTKM with SPE, MBMS only card |  | BCAST-1.1-DIST-int-601 |  | Low |  |
| 5.5.2.2.1.4 LTKM with SPE, BCAST card |  | BCAST-1.1-DIST-int-602 |  | Low |  |
| 5.5.2.2.1.5 LTKM request from the terminal, LTKM reception at the terminal / smartcard |  | BCAST-1.1-DIST-int-412 |  | Low |  |
| 5.5.2.2.1.6 BSM solicited pull procedure |  | BCAST-1.1-DIST-int-413 |  | Low |  |
| 5.5.2.2.1.7 BSM solicited pull procedure initiation over SMS bearer |  | BCAST-1.1-DIST-int-414 |  | Low |  |
| 5.5.2.2.1.8 BSM solicited pull procedure to initiate the Registration Procedure |  | BCAST-1.1-DIST-int-603 |  | Low |  |
| 5.5.2.2.1.9 LTKM Replay Detection in secure function, failure case |  | BCAST-1.1-DIST-int-604 |  | Low |  |
| 5.5.2.2.2.1 Set of Live_ppt_purse associated with a key group; SPE=0x00 |  | BCAST-1.1-DIST-int-605 |  | Low |  |
| 5.5.2.2.2.2 Test of set mode for the live_ppt_purse associated with a key group, SPE=0x00 |  | BCAST-1.1-DIST-int-606 |  | Low |  |
| 5.5.2.2.2.3 Test of add mode for the live_ppt_purse associated with a key group, SPE=0x00 |  | BCAST-1.1-DIST-int-607 |  | Low |  |
| 5.5.2.2.2.4 Test of overflow for the live_ppt_purse associated with a key group, SPE=0x00 |  | BCAST-1.1-DIST-int-608 |  | Low |  |
| 5.5.2.2.2.5 Set of Playback_ppt_purse associated with a key group; SPE=0x01 |  | BCAST-1.1-DIST-int-609 |  | Low |  |
| 5.5.2.2.2.6 Test of set mode for the Playback_ppt_purse associated with a key group, SPE=0x01 |  | BCAST-1.1-DIST-int-610 |  | Low |  |
| 5.5.2.2.2.7 Test of add mode for the Playback_ppt_purse associated with a key group, SPE=0x01 |  | BCAST-1.1-DIST-int-611 |  | Low |  |
| 5.5.2.2.2.8 Test of overflow for the Playback_ppt_purse associated with a key group, SPE=0x01 |  | BCAST-1.1-DIST-int-612 |  | Low |  |
| 5.5.2.2.2.9 Set of user_purse associated with a NAF/SMK ID |  | BCAST-1.1-DIST-int-613 |  | Low |  |
| 5.5.2.2.2.10 Test of set mode for the user_purse associated with a NAF/SMK ID |  | BCAST-1.1-DIST-int-614 |  | Low |  |
| 5.5.2.2.2.11 Test of add mode for the user_purse associated with a NAF/SMK ID |  | BCAST-1.1-DIST-int-615 |  | Low |  |
| 5.5.2.2.2.12 Test of overflow for the user_purse associated with a NAF/SMK ID |  | BCAST-1.1-DIST-int-616 |  | Low |  |
| 5.5.2.2.2.13 Set of playback counter associated with a SEK/PEK ID, SPE=0x07 |  | BCAST-1.1-DIST-int-617 |  | Low |  |
| 5.5.2.2.2.14 Test of set mode for playback counter associated with a SEK/PEK ID, SPE=0x07 |  | BCAST-1.1-DIST-int-618 |  | Low |  |
| 5.5.2.2.2.15 Test of add mode for playback counter associated with a SEK/PEK ID, SPE=0x07 |  | BCAST-1.1-DIST-int-619 |  | Low |  |
| 5.5.2.2.2.16 Test of overflow of playback counter associated with a SEK/PEK ID, SPE=0x07 |  | BCAST-1.1-DIST-int-620 |  | Low |  |
| 5.5.2.2.2.17 Set of TEK counter associated with a SEK/PEK ID |  | BCAST-1.1-DIST-int-621 |  | Low |  |
| 5.5.2.2.2.18 Test of set mode for TEK counter associated with a SEK/PEK ID |  | BCAST-1.1-DIST-int-622 |  | Low |  |
| 5.5.2.2.2.19 Test of add mode for TEK counter associated with a SEK/PEK ID |  | BCAST-1.1-DIST-int-623 |  | Low |  |
| 5.5.2.2.2.20 Test of overflow of TEK counter associated with a SEK/PEK ID |  | BCAST-1.1-DIST-int-624 |  | Low |  |
| 5.5.2.2.3 SPE value not supported by the card |  | BCAST-1.1-DIST-int-625 |  | Low |  |
| 5.5.2.3.1 Correct STKM parsing by a BCAST Smartcard |  | BCAST-1.1-DIST-int-430 |  | Low (pre-test) |  |
| 5.5.2.3.2 Correct STKM parsing by MBMS Smartcard |  | BCAST-1.1-DIST-int-431 |  | Low |  |
| 5.5.2.3.3 Incorrect STKM generation – inexistent SEK/PEK (wrong key domain ID) |  | BCAST-1.1-DIST-int-432 |  | Low |  |
| 5.5.2.3.4 STKM processing – inexistent SEK/PEK (wrong SEK ID) |  | BCAST-1.1-DIST-int-433 |  | Low |  |
| 5.5.2.3.5 STKM processing, Key Validity data check |  | BCAST-1.1-DIST-int-626 |  | Low |  |
| 5.5.2.3.6 Key deletion from server |  | BCAST-1.1-DIST-int-439 |  | Low |  |
| 5.5.2.3.7 SPE deletion from server |  | BCAST-1.1-DIST-int-627 |  | Low |  |
| 5.5.2.3.8.1 STKM Processing when LTKM SPE=00; Testing Live_ppt_purse |  | BCAST-1.1-DIST-int-628 |  | Low |  |
| 5.5.2.3.8.2 STKM Processing when LTKM SPE=0x01; Testing Playback_ppt_purse |  | BCAST-1.1-DIST-int-629 |  | Low |  |
| 5.5.2.3.8.3 STKM Processing when LTKM SPE=02; Testing user_purse |  | BCAST-1.1-DIST-int-630 |  | Low |  |
| 5.5.2.3.8.4 STKM Processing when LTKM SPE=0x07; Testing playback_counter |  | BCAST-1.1-DIST-int-631 |  | Low |  |
| 5.5.2.3.8.5 STKM Processing when LTKM SPE=0C; Testing TEK_counter |  | BCAST-1.1-DIST-int-632 |  | Low |  |
| 5.5.2.3.9.1 Testing SPE priorities: live content with subscription |  | BCAST-1.1-DIST-int-633 |  | Low |  |
| 5.5.2.3.9.2 Testing SPE priorities: live content without subscription |  | BCAST-1.1-DIST-int-634 |  | Low |  |
| 5.5.2.3.9.3 Testing SPE priorities: playback modes including SPE=0x05 |  | BCAST-1.1-DIST-int-635 |  | Low |  |
| 5.5.2.3.9.4 Testing SPE priorities: playback modes without SPE=0x05 |  | BCAST-1.1-DIST-int-636 |  | Low |  |
| 5.5.2.3.9.5 Testing KV priorities when several LTKM available with same SPE |  | BCAST-1.1-DIST-int-637 |  | Low |  |
| 5.5.2.3.10 STKM processing when sent to different SPE sharing the same user purse |  | BCAST-1.1-DIST-int-638 |  | Low |  |
| 5.5.2.3.11 STKM reception with parental control without PIN defined in the card |  | BCAST-1.1-DIST-int-456 |  | Low |  |
| 5.5.2.3.12 STKM reception with parental control and with PIN defined in the card |  | BCAST-1.1-DIST-int-457 |  | Low |  |
| 5.5.2.3.13 Multiple streams protected with same STKM stream |  | BCAST-1.1-DIST-int-458 |  | Low |  |
| 5.5.2.3.14 Multiple streams protected with Different STKM stream |  | BCAST-1.1-DIST-int-459 |  | Low |  |
| 5.5.2.4.1 Delivery of IPSec protected stream |  | BCAST-1.1-DIST-int-460 |  | Low |  |
| 5.5.2.4.2 Delivery of SRTP protected stream |  | BCAST-1.1-DIST-int-461 |  | Low |  |
| 5.5.2.4.3 Delivery of ISMACrypt protected stream |  | BCAST-1.1-DIST-int-462 |  | Low |  |

#### Smartcard Broadcast Provisioning (SCBP)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 6.2.1 Sending a file to a group of Smartcards through Smartcard Broadcast provisioning using ENVELOPE technology | BCAST-1.1-SCBP-int-101 | High |
| 6.2.2 Sending a file to a specific Smartcard through Smartcard Broadcast provisioning using ENVELOPE technology | BCAST-1.1-SCBP-int-102 | High |
| 6.2.3 Sending a file to a set of Smartcards in a group through Smartcard Broadcast provisioning using ENVELOPE technology | BCAST-1.1-SCBP-int-103 | High |
| 6.2.4 Sending a file to a group of Smartcards through Smartcard Broadcast provisioning using SCWS technology | BCAST-1.1-SCBP-int-104 | High |

#### Parental Control for Service Ordering (PCSO)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 6.3.1 Parental control for service ordering using the Smartcard Profile Extension: Service provisioning protection enabled | BCAST-1.1-PCSO-int-101 | High |
| 6.2.2 Sending a file to a specific Smartcard through Smartcard Broadcast provisioning using ENVELOPE technology | BCAST-1.1-PCSO-int-103 BCAST-1.1-PCSO-int-102 | High |
| 6.3.3 Parental control for token purchase using the Smartcard Profile Extension: No local protection | BCAST-1.1-PCSO-int-103 | High |

#### Smartcard-Centric Audience Measurement (SAM)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 7.1.1.1 Registration Process and Opt_in trigger process using SMS-PP bearer | BCAST-1.1-AM-int-101 | High |
| 7.1.1.2 Configuration process, activation and reporting using SMS-PP bearer in Push Mode | BCAST-1.1-AM-int-102 | High |
| 7.1.1.3 Opt_in Message using SMS-PP bearer | BCAST-1.1-AM-int-103 | High |
| 7.1.1.4 Configuration process, activation and reporting using SMS-PP bearer in Push Mode with additional metrics | BCAST-1.1-AM-int-104 | High |
| 7.1.1.5 Reporting using SMS-PP bearer in Pull Mode without additional metrics | BCAST-1.1-AM-int-105 | High |
| 7.1.1.6 Registration Process and Opt_in trigger process using security at application level | BCAST-1.1-AM-int-106 | High |
| 7.1.1.7 Configuration process, activation and reporting using HTTP bearer in Push Mode using security at applicative level | BCAST-1.1-AM-int-107 | High |
| 7.1.1.8 Audience Measurement disallowed for a specific encrypted content | BCAST-1.1-AM-int-108 | High |

#### Smartcard-Centric Audience Measurement (STAM)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 8.1.1.1 STKM-based and Event signalling-based Audience Measurement for clear to air services | BCAST-1.1-AM-int-109 | High |

#### Terminal-Centric Audience Measurement (TCAM)

| Description | Test Case Id | Priority |
| --- | --- | --- |
| 9.1.1 AM campaign initiated over broadcast channel (SG-based trigger) with scheduled AM data reporting | BCAST-1.1-TCAM-int-101 | High |
| 9.1.2 AM campaign initiated over broadcast channel (SG-based trigger) with user rejecting the Opt-in | BCAST-1.1-TCAM-int-102 | High |
| 9.1.3 AM campaign initiated over interaction channel (SMS -based trigger) with AM Report Trigger initiated AM data reporting | BCAST-1.1-TCAM-int-103 | High |
| 9.1.4 Server initiated silent Opt-out for an active AM campaign (SMS triggered) | BCAST-1.1-TCAM-int-104 | High |
| 9.1.5 Blocking of Audience Measurement for a particular service/content | BCAST-1.1-TCAM-int-105 | High |

## Enabler Test Requirements

### Test Infrastructure Requirements

The testing shall be performed as end-to-end testing. Most likely the client participants will be in one place, while the participant servers will be located in member companies premises, accessible to the rest of the test fest environment. Such a “distributed” test fest environment puts effort on the test fest host and requires detailed documented configuration.  It is also possible that server providers bring their respective servers to the test site.

The Network Elements involved in BCAST testing are:

- 3G network

- MBMS network

- DVB-H network

- BCMCS network

- WiMAX network

- FLO network

- DVBSH network

- Internet

A Network Analyzer (e.g. Wireshark) for Network Monitoring/Protocol Analyzing is also useful.

A requirement for a test fest host is that one BDS must be available. At least one of the following BDS:

- IPDC

- MBMS

- BCMCS

- WiMAX

- FLO

- DVBSH

### Test Restrictions

For the different services that comprise BCAST it can be found below the different execution flows.

### Service Guide Related Flows

#### Service Guide Generation and Delivery over Broadcast channel

#### Service Guide Generation and Delivery over Interaction channel

#### Service Guide Delivery to BDS Service Distribution

### File Distribution Related Flows

#### File Distribution over Broadcast channel

#### File Distribution in a Hybrid Broadcast/Unicast Environment

#### File transmission or repairing over Interaction Channel when BDS Service Distribution exists

#### File Delivery to BDS Service Distribution

### Stream Distribution Related Flows

#### Stream Distribution over Broadcast channel

#### Stream Delivery BDS Service Distribution

#### Stream Distribution in a Hybrid Broadcast/Unicast Environment

### Service & Content Protection Related Flows

#### Service Protection Related Flows

##### Service Protection Function Flows for Terminal with Interaction Channel

##### Service Protection Function Flows for Broadcast-only Terminal

#### Content Protection Function Flows

##### Content Protection Function Flows for Terminal with Interaction Channel

##### Content Protection Function Flows for Broadcast-only Terminal

### Interaction Channel Related Flows

#### Interactive Service example Flow – Case1

### Service Provisioning Related Flows

#### Service Provisioning Function Related Flows (DRM based solution)

##### Unsuccessful Service ordering

##### Successful Service ordering

#### Service Provisioning Function Related Flows (GBA based solution)

##### Option 1:  BSP-M Serves as NAF Function

##### Option 2:  BSD/A serves as NAF Function, BSP-M serves as BSF function

### Notification Function Related Flows

#### Notification Generation and Delivery over Broadcast Channel by OMA BCAST

#### Notification Delivery over Broadcast channel by BDS

#### Notification Delivery over Interaction Channel by OMA BCAST

### Terminal Provisioning Related Flows

#### Terminal Provisioning over Broadcast Channel

#### Terminal Provisioning over Interaction Channel

### Audience Measurement Function Related Flows

#### The Overall Flow for Audience Measurement

### Test Content Requirements

For the interoperability tests there is no particular requirement for the test content. The test content has to be adapted to the interoperability test cases.

### Test Limitations

#### Physical

When running BDS specific tests then radio equipment is required and terminals with a radio unit must stay in range of the signal.

#### Resources

It might be required to have technicians for operating the BDS specific network or for configuring the DRM server, supplying encryption keys and other support services.

It might be necessary that a network operator or Telco supplier provides BDS specific hardware (e.g. a network in a lab or a demo network). Contributions are welcome and should be announced early so that tests fests can be planned properly.

### Test Restrictions

#### Test Session Entrance Criteria

The test session entry criteria are defined by the Test-Fest Participation Guidelines of the IOP WG.

#### Technical Prerequisites

- Client settings shall be in accordance with the network parameters provided by the test fest host.

- Gateway and proxy configuration shall be in accordance with the information provided by the test fest host for serving all clients participating in the test fest.

### Test Tools

There are no test tool requirements.

#### Existing Tools to be used

BCAST 1.0 TTCN-3 test cases  can be used for conformance tests to assess client functionality. The main areas for server tests are the service guide, boot strapping and service protection.

It is optional but in case of problems recommended to use a network analyzer like Wireshark (formerly called Ethereal) to create traces for trouble shooting.

In the backend of the conformance test tool or server implementations a collection of existing tools might be required (DRM, codecs, content, keys).

#### Conformance Test Case Priorities

No BCAST 1.1 conformance test cases have been defined.

#### Test Tool Requirements

There are no test tool requirements.

### Resources Required

No estimate so far – depends highly on the maturity of individual enabler implementations. For a good estimate in this chapter input from implementation providers is required later on.

## Tests to be Performed

The following sections describe the tests related to the formal TestFest validation activities.

### Minimal Pre-Test Guidelines

This section details a set of test cases that the Trusted Zone may require participating companies to perform during the Pre-Test day. The order of the listed tests is only intended as advice to participating teams and not necessarily to indicate that the sequence of execution is mandatory.

The results will be considered across all Pre-Test sessions that a team will perform during the Pre-Test day and will NOT be included in any Product Test Report or Enabler Test Report generated as a result of participation in the event.

If the team is unable to present passing results for the Pre-Test test cases allocated by the Trusted Zone (according to the EICS for that implementation) and if the failures are due to team’s implementation, the team shall not be allowed to proceed with regular test sessions. The host shall provide adequate space for the teams to sit separately and improve their implementations. The teams should validate the implementation with the passing teams at the passing teams’ discretion. Once passing results for all Pre-Test test cases are presented, the team should be allowed to proceed with regular testing.

#### Service Guide (SG)

The purpose is to check reception and handling of SG, not method of delivery. In case of broadcast terminals BCAST-1.1-DIST-int-103 is executed. Terminals implementing delivery over interaction channel are required to execute BCAST-1.1-DIST-int-104

| Test Case Id | Test Case Description |
| --- | --- |
| BCAST-1.1-DIST-int-103 | Updating description of content. This test case also tests that the update of the SG is performed correctly. |
| BCAST-1.1-DIST-int-104 | Updating description of content. This test case also tests that the update of the SG is performed correctly. |

#### Stream Distribution (SD)

| Test Case Id | Test Case Description |
| --- | --- |
| BCAST-1.1-DIST-int-208 | The purpose of this test is to test the supports of RTP as a transport protocol for streaming distribution over the broadcast channel. |

#### File Distribution (FD)

| Test Case Id | Test Case Description |
| --- | --- |
| BCAST-1.1-DIST-int-202 | To test the support of the in-band delivery of the metadata associated with file distributed using FLUTE. |

#### Service Protection (ServProt)

DRM

Only one of the recommended test cases need to be executed depending upon the type of encryption implemented by the participant.

| Test Case Id | Test Case Description |
| --- | --- |
| BCAST-1.1-DIST-int-401 | Opening an Ipsec encrypted stream with key material associated to the subscription |
| BCAST-1.1-DIST-int-402 | Opening an SRTP encrypted stream with key material associated to the subscription |
| BCAST-1.1-DIST-int-403 | Opening an ISMACrypt encrypted stream with key material associated to the subscription. |

SmartCard

Test cases BCAST-1.1-DIST-int-404 and BCAST-1.1-DIST-int-430 must be executed by all USIM implementations.

| Test Case Id | Test Case Description |
| --- | --- |
| BCAST-1.1-DIST-int-404 | Test that GBA bootstrapping with the BSM is successfully achieved. Test that the SRK is correctly generated in the terminal. |
| BCAST-1.1-DIST-int-430 | Test that the Smartcard correctly parses STKMs. |

Test cases BCAST-1.1-DIST-int-410 and BCAST-1.1-DIST-int-430 must be executed by all (R-)UIM/CSIM implementations.

| Test Case Id | Test Case Description |
| --- | --- |
| BCAST-1.1-DIST-int-410 | Test that SMK and SRK derivation from pre-provisioned SCK in the terminal are successful. |
| BCAST-1.1-DIST-int-430 | Test that the Smartcard correctly parses STKMs. |

#### Content Protection (ContProt)

DRM

No explicit test cases.

SmartCard

No explicit test cases.

#### Service Provisioning (SP)

| Test Case Id | Test Case Description |
| --- | --- |
| BCAST-1.1-DIST-int-101 | Bootstrapping a service with content. Associating content with service. This test case also tests that the reception of the SG is performed correctly. |

#### Service Interaction (SI)

| Test Case Id | Test Case Description |
| --- | --- |
| BCAST-1.1-DIST-int-301 | Associating content with interactivity. Reception of InteractivityMediaDocuments over broadcast file distribution. XHTML MP as an interaction method. |

#### Notification (N)

No test cases available.

#### Terminal Provisioning (SP)

No test cases available.

### Configuration of Pre-Testing Phase

The section provides configuration information for pre-testing in order to optimize the progress of the test fest activities. These settings shall not be used in the regular test sessions during the test fest where implementations shall use the discovery and configuration procedures as described by the BCAST 1.1 specification, when applicable.

#### BCAST NAF Configuration Parameters

| NAF |  |
| --- | --- |
| BSF Server IP Address | IPAddress of BSF |
| Registration | http://<IPAddressNAF>/keymanagement?requesttype=register |
|  | http://<IPAddressNAF>/keymanagement?requesttype=deregister |
|  | http://<IPAddressNAF>/keymanagement?requesttype=msk-request |
| UDP Port | 4359 |
| NAFID | <IPAddressNAF> |
| ServiceID | cannot be fix at the moment |
| IDI of the LTKM | Must be the IP Address of the NAF, i.e. <IPAddressNAF> |

| SG |  |
| --- | --- |
| Service Guide Rights Issuer URL | Must be the IP Adress of the NAF, i.e. <IPAddressNAF> |

Note: Although the specs mentions using FQDN of the NAF, for simplicity we use the IP Address

#### Smartcard Configuration Parameters

The following parameters shall be used by the GBA-BSF when simulating the HLR function during pre-test.

| HLR/HSS Simulation - MILENAGE (standard) |  |
| --- | --- |
| Input Values (standards) |  |
| Ki | 00 11 22 33 44 55 66 77 88 99 AA BB CC DD EE FF |
| OP | 00 11 22 33 44 55 66 77 88 99 AA BB CC DD EE FF |
| Ri | 40 00 20 40 60 |
| Ci1 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 |
| Ci2 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01 |
| Ci3 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 02 |
| Ci4 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 04 |
| Ci5 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 08 |
| SQN | 00 00 00 00 00 00 |
| Expected Value |  |
| OPC | 62 E7 5B 8D 6F A5 BF 46 EC 87 A9 27 6F 9D F5 4D |

| Input Values (standards) |  |
| --- | --- |
| RAND | 00 11 22 33 44 55 66 77 88 99 AA BB CC DD EE FF |
| SQN | 00 00 00 00 00 00 |
| AMF | 00 00 |
| Expected Authentication Vectors |  |
| XRES | 700EB2300B2C4798 |
| CK | B379874B3D183D2A21291D439E7761E1 |
| IK | F4706F66629CF7DDF881D80025BF1255 |
| AUTN | DE656C8B0BCE0000803CE765D94CAF16 |

### Testing to be Performed at TestFest

The testing at the BCAST Test Fests is expected to be executed according to the  BCAST Interoperability Enabler Test Specification [BCAST11-ETS_IOP]. See also chapter 5.1.3.

## Enabler Test Reporting

### Problem Reporting Requirements

Normal Reporting, no special reporting required.

### Enabler Test Requirements

Normal Reporting, no special reporting required

# Alternative Validation Activities

Any results from bi/multi-lateral testing where OMA BCAST 1.1 test cases have been used can be used to validate the enabler.

# Approval Criteria

The BCAST 1.1 Enabler can be put in the Approved state when:

- The Enabler has been tested successfully at 3 Test Fests or

- 3 Companies have successfully run BI-lateral tests sessions towards a BCAST server fulfilling the requirements of the generic adaptation layer and have reported results and issues to OMA.

- No open PRs exist.

## Enabler Validation Test Cases

The following table should list the set of tests that are used for enabler validation.

| Test Case Id | ETR Requirement Id | ETR Status | Notes |
| --- | --- | --- | --- |
| BCAST-1.1-DIST-int-115 | SG-041 | O |  |
|  | SG-056 | O |  |
| BCAST-1.1-DIST-int-212 | FD-001 | O |  |
|  | FD-002 | O |  |
|  | FD-003 | O |  |
|  | FD-004 | O |  |
|  | FD-032 | O |  |
|  | FD-035 | O |  |
| BCAST-1.1-DIST-int-213 | FD-001 | O |  |
|  | FD-002 | O |  |
|  | FD-003 | O |  |
|  | FD-004 | O |  |
| BCAST-1.1-DIST-int-214 | SD-001 | O |  |
|  | SD-002 | O |  |
|  | SD-003 | O |  |
|  | SD-004 | O |  |
| BCAST-1.1-DIST-int-213 BCAST-1.1-DIST-215 | SD-034 | O |  |
|  | SD-045 | O |  |
| BCAST-1.1-PCSO-int-102 | SPR-010 | O |  |
|  | SPR-015 | O |  |
| BCAST-1.1-DIST-int-114 BCAST-1.1-PCUS-101 | SD-035 | O |  |
|  | SD-046 | O |  |
|  | SD-047 | O |  |
| BCAST-1.1-DIST-int-115 BCAST-1.1-PCUS-102 | SD-035 | O |  |
|  | SD-046 | O |  |
|  | SD-047 | O |  |
| BCAST-1.1-SCBP-int-101 | SCBP-001 | O |  |
|  | SCBP-002 | O |  |
|  | SCBP-003 | O |  |
|  | SCBP-004 | O |  |
| BCAST-1.1-SCBP-int-102 | SCBP-001 | O |  |
|  | SCBP-002 | O |  |
|  | SCBP-003 | O |  |
|  | SCBP-004 | O |  |
| BCAST-1.1-SCBP-int-103 | SCBP-001 | O |  |
|  | SCBP-002 | O |  |
|  | SCBP-003 | O |  |
|  | SCBP-004 | O |  |
| BCAST-1.1-SCBP-int-104 | SCBP-001 | O |  |
|  | SCBP-002 | O |  |
|  | SCBP-003 | O |  |
|  | SCBP-004 | O |  |
| BCAST-1.1-PCSO-int-101 | SPR-016 | O |  |
|  | SPR-019 | O |  |
|  | SPR-012 | O |  |
| BCAST-1.1-PCSO-int-103 BCAST-1.1-PCSO-int-102 | SPR-016 | O |  |
|  | SPR-019 | O |  |
|  | SPR-012 | O |  |
| BCAST-1.1-PCSO-int-103 | SPR-016 | O |  |
|  | SPR-019 | O |  |
|  | SPR-012 | O |  |
| BCAST-1.1-AM-int-101 | AMS-001 | O |  |
|  | AMS-002 | O |  |
|  | AMS-004 | O |  |
|  | AMS-009 | O |  |
|  | AMS-011 | O |  |
| BCAST-1.1-AM-int-102 | AMS-001 | O |  |
|  | AMS-005 | O |  |
|  | AMS-006 | O |  |
|  | AMS-008 | O |  |
|  | AMS-012 | O |  |
|  | AMS-013 | O |  |
|  | AMS-015 | O |  |
| BCAST-1.1-AM-int-103 | AMS-001 | O |  |
|  | AMS-003 | O |  |
|  | AMS-010 | O |  |
| BCAST-1.1-AM-int-104 | AMS-001 | O |  |
|  | AMS-005 | O |  |
|  | AMS-006 | O |  |
|  | AMS-008 | O |  |
|  | AMS-012 | O |  |
|  | AMS-013 | O |  |
|  | AMS-015 | O |  |
| BCAST-1.1-AM-int-105 | AMS-001 | O |  |
|  | AMS-005 | O |  |
|  | AMS-007 | O |  |
|  | AMS-012 | O |  |
|  | AMS-014 | O |  |
| BCAST-1.1-AM-int-106 | AMS-001 | O |  |
|  | AMS-002 | O |  |
|  | AMS-004 | O |  |
|  | AMS-009 | O |  |
|  | AMS-011 | O |  |
| BCAST-1.1-AM-int-107 | AMS-001 | O |  |
|  | AMS-005 | O |  |
|  | AMS-006 | O |  |
|  | AMS-008 | O |  |
|  | AMS-012 | O |  |
|  | AMS-013 | O |  |
|  | AMS-015 | O |  |
| BCAST-1.1-AM-int-108 | AMS-001 | O |  |
|  | AMS-005 | O |  |
|  | AMS-007 | O |  |
|  | AMS-012 | O |  |
|  | AMS-014 | O |  |
| BCAST-1.1-AM-int-109 | AMS-001 | O |  |
|  | AMS-005 | O |  |
|  | AMS-007 | O |  |
|  | AMS-012 | O |  |
|  | AMS-014 | O |  |
| BCAST-1.1-TCAM-int-101 | AMT-001 to AMT-014 | O |  |
| BCAST-1.1-TCAM-int-102 | AMT-001 to AMT-006 | O |  |
|  | AMT-008 to AMT-012 | O |  |
|  | AMT-014 | O |  |
| BCAST-1.1-TCAM-int-103 | AMT-001 to AMT-014 | O |  |
| BCAST-1.1-TCAM-int-104 | AMT-001 to AMT-014 | O |  |
| BCAST-1.1-TCAM-int-105 | AMT-003 | O |  |
|  | AMT-008 | O |  |
|  | AMT-014 | O |  |

Table 2: Enabler Validation Test Cases

## Non Covered ETR Requirements

Any restrictions, limitations and/or infeasibility of testing of the ETR requirements should be stated in this section.

If new information about limitations and/or infeasibility of testing of any of the ETR requirements is discovered, this section should be updated accordingly.

| ETR Requirement Id | ETR Status | Notes |
| --- | --- | --- |
| RM-006 | O | Rich Media Solution |
| RM-007 | O | Rich Media Solution |
| RM-008 | O | Rich Media Solution |
| RM-009 | O | Rich Media Solution |
| RM-010 | O | Rich Media Solution |
| RM-011 | O | Rich Media Solution |
| RM-012 | O | Rich Media Solution |
| RM-013 | O | Rich Media Solution |
| RM-014 | O | Rich Media Solution |
| RM-015 | O | Rich Media Solution |
| RM-016 | O | Rich Media Solution |
| RM-017 | O | Rich Media Solution |
| RM-018 | O | Rich Media Solution |
| SPR-013 | O | Support of Pause & Resume of Subscription Period messages |
| SPR-017 | O | Support of Pause & Resume of Subscription Period messages |
| SPR-014 | O | Support of User Defined Bundle related provisioning messages |
| SPR-018 | O | Support of User Defined Bundle related provisioning messages |
| SPR-009 | O | Support for Related Contents Request messages |
| SPR-011 | O | Support for Related Contents Request messages |
| TP-011 | O | Broadcast Terminal Provisioning signalling |
| TP-012 | O | Broadcast Terminal Provisioning signalling |
| TP-013 | O | Broadcast Terminal Provisioning signalling |
| SCPD-013 | O | SPCP Backend Message for DRM Profile |
| SCPD-014 | O | SPCP Backend Message for DRM Profile |
| SCPD-015 | O | SPCP Backend Message for DRM Profile |
| SCPD-016 | O | SPCP Backend Message for DRM Profile |
| SCPD-028 | O | SPCP Backend Message for DRM Profile |
| SCPS-013 | O | SPCP Backend Message for Smartcard Profile |
| SCPS-014 | O | SPCP Backend Message for Smartcard Profile |
| SCPS-015 | O | SPCP Backend Message for Smartcard Profile |
| SCPS-016 | O | SPCP Backend Message for Smartcard Profile |
| SG-040 | O | Support RMS template retrieval signalling in SGDD |
| SG-044 | O | Support RMS template signalling in SGDD |
| SG-045 | O | Declaration of different entry points in SGDD |
| CODEC-004 | M | Forward Link Only Adaptation |
| FD-034 | M | Forward Link Only Adaptation |
| SD-036 | M | Forward Link Only Adaptation |
| SD-037 | M | Forward Link Only Adaptation |
| SG-046 | M | Forward Link Only Adaptation |
| SG-047 | M | Forward Link Only Adaptation |
| SI-006 | M | Forward Link Only Adaptation |
| SPR-020 | M | Forward Link Only Adaptation |
| TP-008 | M | Forward Link Only Adaptation |
| SD-038 | M | Forward Link Only Adaptation |
| SD-039 | M | Forward Link Only Adaptation |
| FD-033 | O | Forward Link Only Adaptation |
| SG-048 | O | Forward Link Only Adaptation |
| TP-009 | O | Forward Link Only Adaptation |
| TP-010 | O | Forward Link Only Adaptation |
| SG-049 | M | WiMAX Adaptation |
| SG-050 | M | WiMAX Adaptation |
| SI-007 | M | WiMAX Adaptation |
| SG-051 | M | WiMAX Adaptation |
| SG-052 | M | WiMAX Adaptation |
| SI-008 | M | WiMAX Adaptation |
| SI-009 | M | WiMAX Adaptation |
| SCPD-027 | O | WiMAX Adaptation |
| SCPS-023 | O | WiMAX Adaptation |
| SCPD-029 | O | WiMAX Adaptation |
| SCPS-024 | O | WiMAX Adaptation |

Table 3: Non-Covered ETR Requirements

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| OMA-EVP-BCAST-V1_1 | n/a | No prior version –or- No previous version within OMA |

Draft/Candidate Version 1.1 History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-EVP-BCAST_INT-V1_1 | 23 Jun 2010 | all | First Agreed draft baseline |
|  | 08 Nov 2010 | 5.1.3, 5.2.4.2, 5.2.5.3, 5.2.11, 7.1, 7.2 | Inclusion of new BCAST 1.1 test scenarios and test cases. CR OMA-IOP-BRO-2010-0085-CR_BCAST_1.1_EVP_updated |