Change Request

| Title: | CPM Session History Storage Conversation TS | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_Conversation_Function-V2_0-20121106-DD |  |
| Submission Date: | 25 October 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thinh Nguyenphu, NSN; thinh.nguyenphu@nsn.com |  |
| Replaces: | N/A |  |

Reason for Change

R00: Initial draft, file corrupted

R01: re-upload the file

R02 – online edits during LA meeting

The purpose of this CR is to align the agreed change (CR59R03) of delivery notifications.

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM Conversation Function TS

Detailed Change Proposal

PF Procedure @ the Original Network for 200 OK

Upon receiving an incoming SIP NOTIFY request that is part of the same SIP dialog as a previously sent SIP SUBSCRIBE request for subscribing to participation information, the CPM Client:

- SHALL handle the request according to the rules and procedures of [RFC3265] and [RFC4575];

- MAY display the Participants of the CPM Group Session by that are included in the chat Blacklist URI-list stored in [OMA-XDM-List].

## CPM File Transfer

### CPM File Transfer Session Initiation

When a CPM Client needs to send or receive one or more files, the CPM Client:

- SHALL follow the procedures defined in 7.3.1.1 “Initiating a CPM 1-1 Session” if the CPM File Transfer is to one recipient or in 7.3.1.2 “Initiating a CPM Group Session for a CPM Ad-hoc Group” if the CPM File Transfer is to a list of recipients, or in 7.3.1.3 “Initiating a CPM Group Session for a CPM Pre-defined Group” if the CPM File Transfer is to a pre-defined group and SHALL follow the rules and procedures of [RFC5547] with the following clarifications:

  - SHALL include an Accept-Contact header with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.filetransfer’ according to the rules and procedures of [RFC3841] instead of the 3gpp-service.ims.icsi.oma.cpm.session’ used in those procedures;

  - SHALL include a Contact header with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.filetransfer’ according to the rules and procedures of [RFC3841] instead of the 3gpp-service.ims.icsi.oma.cpm.session’ used in those procedures;

  - SHALL add the relevant media attributes to the SDP as specified in [RFC5547], with the following additional considerations:

    - If the request is to send (push) a file, the “name”, “size” and “type” sub-attributes of the “file-selector” attribute SHALL be included;

    - If the request is to receive (pull) a file, as many of the “file-selector” sub-attributes as are known SHALL be included;

  - In the case of a CPM File Transfer to a single recipient and more than one file is to be sent, each set of file descriptors SHALL be sent in separate m-lines as described in [RFC5547];

  - If the request is to send one or more files, the CPM Client SHALL validate that the size of a file being sent does not exceed the provisioned Max File Transfer Size value before continuing with the above procedures.

  - If the recipient’s capability indicates thumbnail support and the sending CPM Client supports it as well, , the sending CPM Client SHALL include a thumbnail in the SDP of the SIP INVITE request by using the file-icon attribute as defined in [RFC5547].

  - When requesting a disposition notification, the CPM Client SHALL set the disposition notification according to the rules and procedures as described in section 5.4 “Disposition Notification”.

NOTE: the method to determine the thumbnail capability by the CPM Clients is out of scope of this specification

NOTE: If there is more than one recipient, to transfer multiple files is not supported.

- When the MSRP 200 “OK” response for the last MSRP SEND is received, or when an MSRP REPORT is received when a success report was requested:

  - The CPM Client MAY check if there are other files to be transferred to the same destination;

  - If there are additional files to be transferred and the CPM Client wants to reuse m-lines for transferring them as described in [RFC5547], the CPM Client SHALL send re-INVITE with the following clarifications:

    - The CPM Client SHALL reuse the needed m lines by creating new relevant attributes for the new files according to procedures of [RFC5547];

    - if more m lines were used and are no longer needed, the CPM Client SHALL set the port in the m-line to zero (i.e. m=0) to close the MSRP session for  the particular files that has been transferred according to procedures of [RFC5547];

  - Otherwise, the CPM Client SHALL follow the procedures defined in 7.4.3 “CPM File Transfer Session Release”;

- If an MSRP error response or an MSRP REPORT with a failure report is received, the CPM Client SHALL indicate to the CPM User that the CPM File Transfer has failed.

### Receiving a CPM File Transfer Request

Upon receiving a SIP INVITE request containing a CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.filetransfer’ included in the Accept-Contact header, the CPM Client:

- SHOULD render to the CPM User the file description parameters received as well as the identity of the original sender of the request if available;

- SHALL accept from the CPM User input regarding the file(s):

  - which he is willing to accept in case of a push request, or,

  - which he is willing to send, in the case of a pull request;

- SHALL inspect the SDP attributes relating to the file(s) to be transferred, and if the request is for receiving one or more files, the CPM Client shall validate whether the “file-selector:size” attribute exceeds the provisioned value for this CPM Client.

- If the CPM User accepts at least one file, SHALL return a SIP 200 “OK” response with an SDP answer containing the file identities of the files as described in [RFC5547], and otherwise SHALL return a 603 “Decline” response.

- SHALL execute the Media Plane procedures as described in section 7.3.9.1 “MSRP-based Media Streams” to receive the files.

- After completing the MSRP procedures for each transferred file,  the CPM Client SHALL check whether the CPM File Transfer Request contains the request for delivery notification. If true, the CPM Client SHALL send the appropriate delivery notification as described in Section 5.4.1 “Generate Delivery Notification”.