| # OMA CPM Charging Specification |  |  |  |
| --- | --- | --- | --- |
| Candidate Version 2.2 – 26 Sep 2017 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-TS-CPM_Charging_Specification-V2_2-20170926-C |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2017 Open Mobile Alliance All Rights Reserved.
Used with the permission of the Open Mobile Alliance under the terms set forth above.

# Contents

1.	Scope	5

2.	References	6

2.1	Normative References	6

2.2	Informative References	6

3.	Terminology and Conventions	7

3.1	Conventions	7

3.2	Definitions	7

3.3	Abbreviations	7

4.	Introduction	8

5.	CPM Charging Architecture	9

6.	CPM charging principles and scenarios	10

6.1	CPM charging principles	10

6.1.1	Charging principles for CPM Events	10

6.2	CPM Offline Charging Scenarios	10

6.2.1	Basic principles	10

6.2.2	Offline Charging for CPM Events	10

6.2.2.1	Receiving a Pager Mode CPM Standalone Message	10

6.2.2.2	Deliver a Pager Mode CPM Standalone Message	11

6.2.2.3	Receiving a Large Message Mode CPM Standalone Message	12

6.2.2.4	Deliver a Large Message Mode CPM Standalone Message	12

6.2.2.5	Receiving a CPM File Transfer	13

6.2.2.6	Deliver a CPM File Transfer	14

6.2.2.7	Receiving a CPM Chat Message	15

6.2.2.8	Deliver a CPM Chat Message	16

6.3	CPM online charging scenarios	17

6.3.1	Basic principles	17

6.3.2	Online Charging for CPM Events	18

6.3.2.1	Receiving a Pager Mode CPM Standalone Message	18

6.3.2.2	Deliver a Pager Mode CPM Standalone Message	18

6.3.2.3	Receiving a Large Message Mode CPM Standalone Message	19

6.3.2.4	Deliver a Large Message Mode CPM Standalone Message	20

6.3.2.5	Receiving a CPM File Transfer	21

6.3.2.6	Deliver a CPM File Transfer	22

6.3.2.7	Receiving a CPM Chat Message	23

6.3.2.8	Deliver a CPM Chat Message	24

7.	Definition of charging information	26

7.1	Mapping of CPM parameters to OMA charging data elements	26

Appendix A.	Change History (Informative)	29

A.1	Approved Version History	29

A.2	Draft/Candidate Version 2.2 History	29

# Figures

Figure 1: Charging architecture for CPM charging	9

Figure 2: Offline Charging for receiving a Pager Mode Message	11

Figure 3: Offline Charging for sending a CPM Pager Mode Message	11

Figure 4: Offline Charging for receiving a Large Message Mode CPM Standalone Message	12

Figure 5: Offline Charging for sending Large Message Mode CPM Standalone Message	13

Figure 6: Offline Charging for receiving a CPM File Transfer	14

Figure 7: Offline Charging for sending CPM File Transfer	15

Figure 8: Offline Charging for receiving a CPM Chat Message	16

Figure 9: Offline Charging for sending CPM File Transfer	17

Figure 10: Online Charging for receiving a Pager Mode Message	18

Figure 11: Online Charging for sending a CPM Pager Mode Message	19

Figure 12: Online Charging for receiving a Large Message Mode CPM Standalone Message	20

Figure 13: Online Charging for sending Large Message Mode CPM Standalone Message	21

Figure 14: Online Charging for receiving a CPM File Transfer	22

Figure 15: Online Charging for sending CPM File Transfer	23

Figure 16: Online Charging for receiving a CPM Chat Message	24

Figure 17: Online Charging for sending CPM File Transfer	25

# Tables

Table 1: Charging Request Message triggered by SIP Methods or MSRP Messages for a CPM Server	10

Table 2: The Charging Request messages triggered by SIP Methods or MSRP Messages for CPM Server	18

Table 3: Structure of the CPM Charging Information	28

# Scope

This document specifies the use of OMA Charging Enabler to realise the offline and online charging requirements of OMA CPM Enabler. The OMA Charging Enabler defines a set of interfaces that are utilised by the other Enablers to fulfil their charging requirements. The interfaces are specified in [CHRG_AD11] and the parameter definitions in [CHRG_DDS]. This document defines how, when and by which entities charging is triggered and which function invokes charging over the OMA Charging Enabler interfaces. This document also defines the data that will be exchanged during the process.

This document specifies in detail:

- The charging models for the OMA CPM enabler,

- The logical messages and message types used on CH-1 and CH-2 interfaces,

- The flow of messages between the Charging Enabler User and the Charging Enabler with regard to the CPM Service and applicable CPM scenarios,

- Mapping of the CPM parameters to the OMA Charging Data Elements

# References

## Normative References

| [CHRG_AD10] |  | “Charging Architecture”, Open Mobile Alliance™. OMA-AD-Charging-V1_0, URL: http://www.openmobilealliance.org/ |  |
| --- | --- | --- | --- |
| [CHRG_AD11] |  | “Charging Architecture”, Open Mobile Alliance™. OMA-AD-Charging-V1_1, URL: http://www.openmobilealliance.org/ |  |
| [CHRG_DDS] |  | “Charging Data”, Open Mobile Alliance™. OMA-DDS-Charging_Data-V1_0, URL: http://www.openmobilealliance.org/ |  |
| [OMA ONLINE CHG] |  | “OMA Online Charging Interface”, Open Mobile Alliance™. OMA-TS-Charging_Online-V1_1, URL: http://www.openmobilealliance.org/ |  |
| [OMA OFFLINE CHG] |  | “OMA Offline Charging Interface”, Open Mobile Alliance™. OMA-TS-Charging_Offline-V1_1, URL: http://www.openmobilealliance.org/ |  |
| [RFC2119] |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |  |
| [SCRRULES] |  | “SCR Rules and Procedures”, Open Mobile Alliance™, OMA-ORG-SCR_Rules_and_Procedures, URL:http://www.openmobilealliance.org/ |  |

## Informative References

| [OMA-DICT] | “Dictionary for OMA Specifications”, Version 2.9, Open Mobile Alliance™, OMA-ORG-Dictionary-V2_9, URL:http://www.openmobilealliance.org/ |
| --- | --- |
| [RFC3455] | “Private Header (P-Header) Extensions to the Session Initiation Protocol (SIP) for the 3rd-Generation Partnership Project (3GPP)”, January 2003, URL: http://www.ietf.org/rfc/rfc3455.txt |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

## Definitions

| CH-1 EventRequest | Refers to the EventRequest defined in [OMA OFFLINE CHG] |
| --- | --- |
| CH-1 Response | Refers to the Charging Response Message defined in [OMA OFFLINE CHG] |
| CH-2 Initial Request | Refers to the Initial Request defined in [OMA ONLINE CHG] |
| CH-2 Response | Refers to the Charging Response Message defined in [OMA ONLINE CHG] |
| CH-2 Termination Request | Refers to the Termination Request defined in [OMA ONLINE CHG] |
| CPM CPM Server | Refers to a CPM Participating Function, CPM Controlling Function, CPM Interworking Function or CPM Interworking Selection Function supporting a Charging Enabler User |
| CPM Entity | Refers to a CPM Client, CPM Participating Function, CPM Controlling Function, CPM Interworking Function or CPM Interworking Selection Function |

## Abbreviations

| CPM | Converged IP Messaging |
| --- | --- |
| OMA | Open Mobile Alliance |
| SIP | Session Initiation Protocol |

# Introduction

This document describes the charging aspects of the CPM enabler.

# CPM Charging Architecture

The OMA CPM service architecture is described in [CPM-AD]. Figure 1 depicts the high-level OMA CPM charging architecture.

# Figure 1: Charging architecture for CPM charging

In this figure:

- CH-1 is Offline charging reference point between CPM Server and the Offline Function of the Charging Enabler.

- CH-2 is Online charging reference point between CPM Server and the Online Function of the Charging Enabler.

The OMA CPM Server MAY comprise the Charging Enabler User, an integrated function that monitors and generates the charging events, sends the charging requests to the Charging Enabler and receives the responses from the Charging Enabler, over the CH-1 or the CH-2 interface, or both.

# CPM charging principles and scenarios

OMA CPM charging is based on the concept of charging for the transmission of messages and files. OMA CPM charging SHALL support the Event-based charging model that charges for messages and transfer of files, applicable for both online and offline charging procedures.

The CPM Server may process a Large message or a message within an CPM session which MAY be broken down into smaller chunks for the purposes of transmission. In such cases, the CPM Server may receive multiple intermediate MSRP 200 OK messages; however, these MSRP 200 OK messages shall NOT trigger a charging request message. In this case, the CPM Server SHALL trigger a charging request message only after receiving the last MSRP 200 OK, i.e. on determining that the transmission of the chunked messages is completed.

## CPM charging principles

### Charging principles for CPM Events

The OMA Charging Enabler User SHALL use Event-based charging to enable charging for the following CPM Events

- Sending or receiving of a Pager Mode CPM Standalone Message to a single user or to/from a predefined/ad-hoc group or another CPM entity. Deferred messages shall be considered as successful. Unsuccessful Pager Mode CPM Standalone Messages sent to a single user or to a predefined/ad-hoc group SHALL or another CPM entity SHALL be reported.

- Sending or receiving of Large Message Mode CPM Standalone Message to a single user or to a predefined/ad-hoc group or another CPM entity. Unsuccessful Large Message Mode CPM Standalone Message to a single user or to a predefined/ad-hoc group or another CPM entity SHALL be reported.

- Sending or receiving of CPM File Transfer to a single user or another CPM entity. Unsuccessful CPM File Transfer to a single user or another CPM entity SHALL be reported.

- Sending or receiving of a CPM Chat Message to a single user or another CPM entity in a CPM 1-1 Session or in a CPM Group Session. Unsucessful transfer CPM Chat Messages SHALL be reported.

## CPM Offline Charging Scenarios

### Basic principles

The CPM Events described in section 6.1.1. SHALL be charged using the Event-based messages (EventRequest).

The charging request messages sent from a OMA CPM Server are described in the following table. The table summarises the SIP methods and MSRP messages which may trigger the charging messages.

| OMA Charging message | Triggering SIP Method /MSRP message |
| --- | --- |
| EventRequest | SIP 200 OK acknowledging a SIP MESSAGE is successfully accepted for a Pager Mode CPM Standalone Message |
|  | The final MSRP 200 OK acknowledging a MSRP SEND of a Large Message Mode CPM Standalone Message. |
|  | The final MSRP 200 OK acknowledging a MSRP SEND of a CPM File Transfer. |
|  | The final MSRP 200 OK acknowledging a MSRP SEND of a CPM Chat Message. |

Table 1: Charging Request Message triggered by SIP Methods or MSRP Messages for a CPM Server

### Offline Charging for CPM Events

#### Receiving a Pager Mode CPM Standalone Message

Figure 2shows the charging request transactions between the Charging Enabler User in the CPM Server and the Charging Enabler for the reception of a Pager Mode CPM Standalone Message.

Figure 2: Offline Charging for receiving a Pager Mode Message

- The CPM Server receives a SIP MESSAGE from the originating CPM Entiy.

- The CPM Server returns a SIP 200 OK or any SIP error to the originating CPM Entity.

- The Charging Enabler User SHALL trigger a CH-1 EventRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-1 Response message.

#### Deliver a Pager Mode CPM Standalone Message

Figure 3 shows the charging request transaction between the Charging Enabler User in the CPM Server and the Charging Enabler for the delivery of a Pager Mode CPM Standalone Message.

# Figure 3: Offline Charging for sending a CPM Pager Mode Message

- The CPM Server sends a SIP MESSAGE to the terminating CPM Entiy.

- The CPM Server receives a SIP 200 OK or any SIP error from the terminating CPM Entity.

- The Charging Enabler User SHALL trigger a CH-1 EventRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-1 Response message.

#### Receiving a Large Message Mode CPM Standalone Message

Figure 4 shows the charging request transaction between the Charging Enabler User in the CPM Server and the Charging Enabler for the reception of a Large Message Mode CPM Standalone Message.

# Figure 4: Offline Charging for receiving a Large Message Mode CPM Standalone Message

- The CPM Server receives a SIP INVITE for a Large Message Mode CPM Session from the originating CPM Entiy.

- The CPM Server returns a SIP 200 OK to the originating CPM Entity.

- The CPM Server receives a MSRP SEND request from the originating CPM Entity.

- The CPM Server returns a MSRP 200 OK or any MSRP error to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-1 EventRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-1 Response message.

#### Deliver a Large Message Mode CPM Standalone Message

Figure 5hows the charging request transactions between the Charging Enabler User in the CPM Server and the Charging Enabler for the delivery of a Large Message Mode CPM Standalone Message.

# Figure 5: Offline Charging for sending Large Message Mode CPM Standalone Message

- The CPM Server sends a SIP INVITE for a Large Message Mode CPM Session to the terminating CPM Entiy.

- The CPM Server receives a SIP 200 OK from the terminating CPM Entity.

- The CPM Server sends a MSRP SEND request to the terminating CPM Entity.

- The CPM Server returns a MSRP 200 OK or any MSRP error to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-1 EventRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-1 Response message. CPM online charging scenarios

#### Receiving a CPM File Transfer

Figure 6 shows the charging request transaction between the Charging Enabler User in the CPM Server and the Charging Enabler for the reception of a CPM File Transfer.

# Figure 6: Offline Charging for receiving a CPM File Transfer

- The CPM Server receives a SIP INVITE for a CPM File Transfer from the originating CPM Entiy.

- The CPM Server returns a SIP 200 OK to the originating CPM Entity.

- The CPM Server receives a MSRP SEND request from the originating CPM Entity.

- The CPM Server returns the final MSRP 200 OK or any MSRP error to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-1 EventRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-1 Response message.

#### Deliver a CPM File Transfer

Figure 7 shows the charging request transactions between the Charging Enabler User in the CPM Server and the Charging Enabler for the delivery of a CPM File Transfer.

# Figure 7: Offline Charging for sending CPM File Transfer

- The CPM Server sends a SIP INVITE for a CPM File Transfer to the terminating CPM Entiy.

- The CPM Server receives a SIP 200 OK from the terminating CPM Entity.

- The CPM Server sends a MSRP SEND request to the terminating CPM Entity.

- The CPM Server returns the final MSRP 200 OK or any MSRP error to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-1 EventRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-1 Response message.

#### Receiving a CPM Chat Message

Figure 8 shows the charging request transaction between the Charging Enabler User in the CPM Server and the Charging Enabler for the reception of a CPM Chat Message.

# Figure 8: Offline Charging for receiving a CPM Chat Message

- The CPM Server receives a SIP INVITE for a CPM 1-1 Session or a CPM Group Session from the originating CPM Entiy.

- The CPM Server returns a SIP 200 OK to the originating CPM Entity.

- The CPM Server receives a MSRP SEND request from the originating CPM Entity.

- The CPM Server returns a MSRP 200 OK or any MSRP error to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-1 EventRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-1 Response message.

#### Deliver a CPM Chat Message

Figure 9 shows the charging request transactions between the Charging Enabler User in the CPM Server and the Charging Enabler for the delivery of a CPM Chat Message.

# Figure 9: Offline Charging for sending CPM File Transfer

- The CPM Server sends a SIP INVITE for a CPM 1-1 Session or a CPM Group Session to the terminating CPM Entiy.

- The CPM Server receives a SIP 200 OK from the terminating CPM Entity.

- The CPM Server sends a MSRP SEND request to the terminating CPM Entity.

- The CPM Server returns a MSRP 200 OK or any MSRP error to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-1 EventRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-1 Response message.

## CPM online charging scenarios

### Basic principles

The CPM Events described in section 6.1.1. SHALL be charged using the Event-based charging (Event Charging with reservation).

The charging request messages sent from a OMA CPM Server are described in the following table. The table summarises the SIP Methods and MSRP messages which may trigger the charging messages.

| OMA Charging message | Triggering SIP Method/MSRP Message |
| --- | --- |
| CH-2 Initial Request | SIP MESSAGE for a Pager Mode CPM Standalone Message |
|  | MSRP SEND for a Large Message Mode CPM Standalome Message |
|  | SIP INVITE for a CPM File Transfer. |
|  | MSRP SEND for a CPM Chat Message. |
| CH-2 Termination Request | SIP 200 OK acknowledging a SIP MESSAGE is successfully accepted for a Pager Mode CPM Standalone Message |
|  | The final MSRP 200 OK acknowledging a MSRP SEND of a Large Message Mode CPM Standalone Message. |
|  | The final MSRP 200 OK acknowledging a MSRP SEND of a CPM File Transfer. |
|  | The final MSRP 200 OK acknowledging a MSRP SEND of a CPM Chat Message. |

Table 2: The Charging Request messages triggered by SIP Methods or MSRP Messages for CPM Server

### Online Charging for CPM Events

#### Receiving a Pager Mode CPM Standalone Message

Figure 10 shows the charging request transactions between the Charging Enabler User in the CPM Server and the Charging Enabler for the reception of a Pager Mode CPM Standalone Message.

# Figure 10: Online Charging for receiving a Pager Mode Message

- The CPM Server receives a SIP MESSAGE from the originating CPM Entiy.

- The Charging Enabler User SHALL trigger a CH-2 InitialRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

- The CPM Server processes the message request and returns a SIP 200 OK to the originating CPM Entity.

- The Charging Enabler User SHALL trigger a CH-2 TerminationRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

#### Deliver a Pager Mode CPM Standalone Message

Figure 11 shows the charging request transaction between the Charging Enabler User in the CPM Server and the Charging Enabler for the delivery of a Pager Mode CPM Standalone Message.

# Figure 11: Online Charging for sending a CPM Pager Mode Message

- The Charging Enabler User SHALL trigger a CH-2 InitialRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

- The CPM Server sends a SIP MESSAGE to the terminating CPM Entiy.

- The CPM Server receives a SIP 200 OK from the terminating CPM Entity.

- The Charging Enabler User SHALL trigger a CH-2 TerminationRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

#### Receiving a Large Message Mode CPM Standalone Message

Figure 12 shows the charging request transaction between the Charging Enabler User in the CPM Server and the Charging Enabler for the reception of a Large Message Mode CPM Standalone Message.

# Figure 12: Online Charging for receiving a Large Message Mode CPM Standalone Message

- The CPM Server receives a SIP INVITE for a Large Message Mode CPM Session from the originating CPM Entiy.

- The CPM Server returns a SIP 200 OK to the originating CPM Entity.

- The CPM Server receives a MSRP SEND request from the originating CPM Entity.

- The Charging Enabler User SHALL trigger a CH-2 InitialRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

- The CPM Server returns a MSRP 200 OK to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-2 TerminationRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

#### Deliver a Large Message Mode CPM Standalone Message

Figure 13 shows the charging request transactions between the Charging Enabler User in the CPM Server and the Charging Enabler for the delivery of a Large Message Mode CPM Standalone Message.

# Figure 13: Online Charging for sending Large Message Mode CPM Standalone Message

- The CPM Server sends a SIP INVITE for a Large Message Mode CPM Session to the terminating CPM Entiy.

- The CPM Server receives a SIP 200 OK from the terminating CPM Entity.

- The Charging Enabler User SHALL trigger a CH-2 InitialRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

- The CPM Server sends a MSRP SEND request to the terminating CPM Entity.

- The CPM Server returns a MSRP 200 OK to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-2 TerminationRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

#### Receiving a CPM File Transfer

Figure 14 shows the charging request transaction between the Charging Enabler User in the CPM Server and the Charging Enabler for the reception of a CPM File Transfer.

# Figure 14: Online Charging for receiving a CPM File Transfer

- The CPM Server receives a SIP INVITE for a CPM File Transfer from the originating CPM Entity.

- The Charging Enabler User SHALL trigger a CH-2 InitialRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

- The CPM Server returns a SIP 200 OK to the originating CPM Entity.

- The CPM Server receives the MSRP SEND request from the originating CPM Entity.

- The CPM Server returns the final MSRP 200 OK to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-2 TerminationRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

#### Deliver a CPM File Transfer

Figure 15 shows the charging request transactions between the Charging Enabler User in the CPM Server and the Charging Enabler for the delivery of a CPM File Transfer.

# Figure 15: Online Charging for sending CPM File Transfer

- The Charging Enabler User SHALL trigger a CH-2 InitialRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

- The CPM Server sends a SIP INVITE for a CPM File Transfer to the terminating CPM Entiy.

- The CPM Server receives a SIP 200 OK from the terminating CPM Entity.

- The CPM Server sends a MSRP SEND request to the terminating CPM Entity.

- The CPM Entity returns the final MSRP 200 OK to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-2 TerminationRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

#### Receiving a CPM Chat Message

Figure 16 shows the charging request transaction between the Charging Enabler User in the CPM Server and the Charging Enabler for the reception of a CPM Chat Message.

# Figure 16: Online Charging for receiving a CPM Chat Message

- The CPM Server receives a SIP INVITE for a CPM 1-1 Session or a CPM Group Session from the originating CPM Entiy.

- The CPM Server returns a SIP 200 OK to the originating CPM Entity.

- The CPM Server receives a MSRP SEND request from the originating CPM Entity.

- The Charging Enabler User SHALL trigger a CH-2 InitialRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

- The CPM Server returns a MSRP 200 OK to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-2 TerminationRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

#### Deliver a CPM Chat Message

Figure 17 shows the charging request transactions between the Charging Enabler User in the CPM Server and the Charging Enabler for the delivery of a CPM Chat Message.

# Figure 17: Online Charging for sending CPM File Transfer

- The CPM Server sends a SIP INVITE for a CPM 1-1 Session or a CPM Group Session to the terminating CPM Entiy.

- The CPM Server receives a SIP 200 OK from the terminating CPM Entity.

- The Charging Enabler User SHALL trigger a CH-2 InitialRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

- The CPM Server sends a MSRP SEND request to the terminating CPM Entity.

- The CPM Server returns a MSRP 200 OK to the originating CPM Entity

- The Charging Enabler User SHALL trigger a CH-2 TerminationRequest message towards the Charging Enabler.

- The Charging Enabler acknowledges with a CH-2 Response message.

# Definition of charging information

## Mapping of CPM parameters to OMA charging data elements

CPM specific charging information is provided within the Service Information. The detailed structure and mapping of the Service Information which includes the CPM Information can be found in [CHRG_DDS].

[Editor's note: it requires changes to CHRG_DDS to introduce the CPM service context and the CPM context specific parameter definitions.]

| CPM Field Name |  | Category |  | Type |  | Description |  | OMA Charging Data Element |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value: CPM@openmobilealliance.org |  | OM |  | String |  | Fixed value to identify the service specification in the context of which the charging events must be interpreted. |  | Service Context Id |  |
| CPM Server Role |  | OM |  | Enumerated |  | Identifies the CPM Server function as participating function (0), controlling function (1), interworking function (2), interworking selection function (3). |  | Role of node |  |
| CPM User Role |  | OC |  | Enumerated |  | Identifies if the CPM user role in the CPM request as sender (0) or the receiver (1) |  | Role of User |  |
| CPM Messaging Service |  | OM |  | Enumerated |  | Identifies the type of the CPM service as: Pager mode (0), Large message mode (1), 1-1 Session (2). Group Session (3), File Transfer (4) |  | Service Identifier |  |
| CPM Message Service Type |  | OC |  | Enumerated |  | Identifies the type of the service as Sending (0), Receiving (1) |  | Application Service Type |  |
| Number of Participants |  | OC |  | Integer |  | Indicates the number of invited parties of the CPM session when included in the initial charging request message. |  | Number Of Participants |  |
| List Of Participants |  | OC |  | String |  | Holds the information for participants, e.g., the addresses. |  | Participant Group |  |
| Called Party Address |  | OC |  | String |  | The address of the receiver. It can be of the following type: - SIP URI - TEL URI |  | Called Party Address |  |
| Calling Party Address |  | OC |  | String |  | The address of the sender. The address of the receiver. It can be of the following type: - SIP URI - TEL URI |  | Calling Party Address |  |
| CPM Server Identity |  | OC |  | String |  | Identifies the CPM Server |  | Application Server Id |  |
| CPM Group Name |  | OC |  | String |  | Identifies a pre-defined group |  | Group Name |  |
| CPM Session Id |  | OC |  | Integer |  | Uniquely identifies an CPM session. All events using the same session SHALL have the same Session ID |  | Session Id |  |
| Served Party |  | OM |  | String |  | Holds the identity of the party that the charging information relates to. |  | Subscription Id |  |
| Inter Operator Identifier |  | OC |  | Group |  | The IOI identifies both originating and terminating networks involved in a session/transaction. The IOI may be generated from each side of session/transaction to identify the home networks associated with each side. The orig-ioi and term-ioi parameters of P-Charging-Vector represent the originating and terminating operator identifiers. For further information regarding the composition and usage of the orig-ioi and term-ioi parameters refer to RFC 3455 [RFC3455] |  | Inter-Operator Id |  |
| Orig IOI |  | OC |  | String |  | This field holds the IOI for the originating network as generated by the IMS network element which takes responsibility for populating this parameter in a SIP request. |  | Originating IOI |  |
| Term IOI |  | OC |  | String |  | This field holds the IOI for the terminating network as generated by the IMS network element which takes responsibility for populating this parameter in a SIP request. |  | Terminating IOI |  |
| Transit IOI |  | OC |  | String |  | This parameter holds the identification of the involved transit networks as exchanged via SIP signalling if available. |  | TransitIOI List |  |
| Interface-ID |  | OC |  | String |  | The parameter indicates the CPM Server interface on which the event occurred, values are UNI, NNI. |  | Interface-ID |  |
| Access Network Identifier |  | OC |  | String |  | This identifies the access network and may be populated if available. |  | Access Network Charging Identifier Value |  |
| Content Type |  | OC |  | String |  | Identifies the type of content carried in the CPIM or MSRP body. The value is taken from the MSRP or CPIM content-type header or CPIM Payload-Typeheader as applicable. |  | Content Type |  |
| Message Size |  | OC |  | Integer |  | Identifies the content summary delivered in CPIM, except for system content such as Group State Object and Conference Information Objects. Applicable to CPM Standalone Messages and CPM Chat Messages. |  | Content Length |  |
| File-size |  | OC |  | Integer |  | For CPM File Transfer it SHALL include the ‘file-size’ of the file transferred. |  | Content Length |  |
| Delivery status |  | OC |  | String |  | Identifies if the message is successfully or unsuccessfully sent/delivered |  | Delivery Status |  |
| Message-ID |  | OC |  | Integer |  | Identifies the message being charged |  | Message-ID |  |
| Charging Correlation Identifier |  | OC |  | Integer |  | Identifies the correlation information to correlate with bearer network. It can be populated from 'icid-value', a mandatory part of the P-Charging-Vector and coded as a text-based UTF-8 charset (as are all SIP messages). For further information regarding the composition and usage of the P-Charging-Vector refer RFC 3455 [RFC3455] |  | Application Charging Identifier |  |
| Service Request Time Stamp |  | OM |  |  |  | This field contains the time stamp which indicates the time at which the service was requested. |  | Event Timestamp |  |
| Service Reason Return Code |  | OM |  |  |  | This parameter provides the returned status code for the service request for the successful and failure case, |  | Cause Code |  |

Table 3: Structure of the CPM Charging Information

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| n/a | n/a | No previous version within OMA |

Draft/Candidate Version 2.2 History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-TS-CPM_Charging_Specification-V2_2 | 09 May 2017 | all | First draft |
|  | 22 Jun 2017 | 7.1 | Incorporated CR: OMA-COM-CPM-2017-0082-CR_Content_Type_in_CPM_Charging_Requests |
| Candidate Version OMA-TS-CPM_Charging_Specification-V2_2 | 26 Sep 2017 | n/a | Status changed to Candidate by TP TP Ref # OMA-TP-2017-0040-INP_CPM-V2_2_ERP_for_1st_Candidate_Approval |