Change Request

| Title: | Changes in content adaptation sections: Voice-to-Text and Text-to-Speech | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA EVVM |  |
| Doc to Change: | OMA-ER-EVVM-V1_0-20120518-D |  |
| Submission Date: | 25.05. 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thomas Scheerbarth, DTAG; thomas.scheerbarth@telekom.de Jindrich Kubelik, jindrich.kubelik@t-mobile.cz |  |
| Replaces: | n/a |  |

Reason for Change

This change request proposes additional / edited text for the content adaptation part for the EVVM client as well as the EVVM server section of the OMA-ER-EVVM-V1_0-20120518-D document.

Furthermore the closure of the following CONRR comments is proposed:

| A054 | 2012.04.13 | T | 6.1.4 | Source: Research In Motion Ltd. Form: Doc #0067 Comment: EVVM-VMC-001 is not covered in the ER. None of the procedures include descriptions about on-the-fly transcoding of text to audio. As such, features that depend on it (textual greeting) will ultimately fail. Proposed Change: Considering that the ‘Transcoder’ has been postponed, the specification of Greetings (including textual greetings) is missing as a whole and the specification of these procedures might take a long time, we propose to postpone these requirements and remove the bits and pieces that exist in the specification, ultimately leaving all transcoding-related issues as an implementation issue in EVVM 1.0 (same as OMA-STI currently). | Status: OPEN |
| --- | --- | --- | --- | --- | --- |

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Review and agree the proposed changes.

Detailed Change Proposal

Change in chapter 7.6. Procedures at the EVVM client, chapter 7.6.7 Content adaptation

### Content adaptation

#### Text to speech conversion

The text to speech conversion is requested by the EVVM Client according to the EVVM user preferences (see chapter 7.3.2.3.12 “Text to speech conversion”).

The text to speech content adaptation is used for generating audio parts (synthesized speech) for text based greetings or messages.

A greeting or message containing only text can be created and edited on the client, using existing composition procedures. When such a greeting or message is stored on the EVVM server the EVVM server triggers the text to speech conversion to get the corresponding synthesized audio. The resulting synthesized audio part is attached to the text based greeting or message and replaces that greeting or message,

Only if the EVVM user settings for text to speech conversion are switched on (see chapter 7.3.2.3.12 “Text to speech conversion”) the EVVM Client is allowed to store the text based greeting in the greeting folder.

Only if the EVVM user settings for text to speech conversion are switched on (see chapter 7.3.2.3.12 “Text to speech conversion”) the EVVM Client is allowed to store messages which contain only textual part into outbox / draft folder.

#### Voice to text conversion

The voice to text conversion is requested by the EVVM Client according to the user preferences (see chapter 7.3.2.3.13 “Voice to text conversion”).

The user preferences, described in chapter 7.3.2.3.13 “Voice to text conversion”, relate to start / stop procedures.

The voice to text content adaptation is used for generating text from voicemails.

If the user requests an on demand transcription then it will be realized via CNS4 XTRANSCRIBE_ command with parameter UID as described in [OMTP VVM]. EVVM Client MAY support CNS4 XTRANSCRIBE_ command as defined in [OMTP VVM].

[Editor’s NOTE 4/19/2012]: comverse will come back with an answer if CNSx is the command or a sequence number.

If the CNS4 XTRANSCRIBE_ command is supported the EVVM Client SHALL support CNS5 XTRANSCRIPTION_SERVICE_ command with parameter STATE and its values START/STOP as defined in [OMTP VVM].

To identify the original voice message of transcription EVVM Client SHALL support X-Original-Msg-UID and X-Transcription message headers as defined in [OMTP VVM].

Change in chapter 7.7. Procedures at the EVVM Server, chapter 7.7.7 Content adaptation

### 7.7.7 Content adaptation

#### 7.7.7.1 Text to speech conversion

The text to speech conversion is performed by the EVVM Server itself or by delegation to a Transcoder entity.

If the EVVM user setting for text to speech conversion is switched on (see chapter 7.3.2.3.12“Text to speech conversion”), the text part of the text based greeting or text based message will be converted into synthesized audio and attached to the greeting or message and subsequently synchronized with the EVVM Client.

#### 7.7.7.2 Voice to text conversion

The voice to text conversion is performed, if supported, by the EVVM Server itself or by delegation to a Transcoder entity.

The user preferences, described in chapter 7.3.2.3.13 “Voice to text conversion”, relate to start / stop procedures.

An on demand transcription will be realized via CNS4 XTRANSCRIBE_ command with parameter UID as described in [OMTP VVM]. The EVVM Server SHALL support CNS4 XTRANSCRIBE_ command as defined in [OMTP VVM].

For backward compatibility with VVM 1.3 clients the EVVM Server SHALL support CNS5 XTRANSCRIPTION_SERVICE_ command with parameter STATE and its values START/STOP as defined in [OMTP VVM].

To identify the original voice message of transcription EVVM Server SHALL support X-Original-Msg-UID and X-Transcription message headers as defined in [OMTP VVM].

Change in chapter 7.3. 2 Preferences , chapter 7.3.2.3.12. Text to speech conversion and 7.3.2.3.13 Voice to text conversion

##### 7.3.2.3.12 	Text to speech conversion

The demand for text to speech conversion can be set by the user and will be stored in EVVM user preferences.  The text to speech conversion is used for generating audio parts (synthesized speech) for text based greetings or messages.

The following settings define the user’s request to perform text to speech conversion.

When ‘greetings’ flag is set to ’true’, the text based greeting, stored in the Greetings ‘folder’ will be converted into synthesized speech.

When ’voicemails’ flag is set to ’true’, the text based message stored in the Outbox or Draft ‘folder’ will be converted into synthesized speech.

Corresponding preferences:

- userprefs/prefs/tts@greetings

- userprefs/prefs/tts@voicemails

##### 7.3.2.3.13  Voice to text conversion

The preferences for voice to text conversion are set by the user and will be stored in EVVM user preferences.

The following settings define the user’s request to perform voice to text conversion for received voice messages

When ‘active’ flag is set to ‘true’ received voicemail messages will be converted into text, which will be delivered to the EVVM Client. If it is set to ‘false’ only the original voicemail will be delivered to the EVVM Client.

When ‘includeoriginalvm’ flag is set to ‘true’ the original voicemail will be sent to the EVVM Client in addition to the resulting text. The ‘includeoriginalvm’ flag has no effect if ‘active’ flag is set to ‘false’.

Corresponding preferences:

- userprefs/prefs/v2t@active

- userprefs/prefs/v2t@includeoriginalvm