Change Request

| Title: | CR_TS _ Control_Plane_Compatibility _Updates | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA COM PCPS |  |
| Doc to Change: | OMA-PCPS-TS-Control_Plane-V1_0-20140903-D |  |
| Submission Date: | 14 September 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Bob Fredericks – Motorola Solutions; |  |
| Replaces: | n/a |  |

Reason for Change

This change request updates the Control Plane specification backward compatibility sections

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The PCPS group is recommended to agree to the changes and incorporate it into the corresponding document.

Detailed Change Proposal

- Updating the Control Plane specification backward compatibility sections.

.

.

.

From 2.1 “Normative References”:

.

.

.

| [OMA-POC-1-CP] | "OMA PoC Control Plane", Version 1.0, Open Mobile Alliance™, OMA-TS-PoC_ControlPlane-V1_0_1, URL: http://www.openmobilealliance.org/ |
| --- | --- |
| [OMA-POC-1-UP] | "PoC User Plane", Version 1.0, Open Mobile Alliance™, OMA-TS-PoC_UserPlane-V1_0_1. URL: http://www.openmobilealliance.org/ |
| [OMA-POC-2-CP] | "OMA PoC Control Plane", Version 2.0, Open Mobile Alliance™, OMA-TS-PoC_ControlPlane-V2.0, URL: http://www.openmobilealliance.org/ |
| [OMA-POC-2_1-CP] | "OMA PoC Control Plane", Version 2.1, Open Mobile Alliance™, OMA-TS-PoC_ControlPlane-V2.1, URL: http://www.openmobilealliance.org/ |
| [OMA-PoC-2-SD] | "OMA PoC System Description", Version 2.0, Open Mobile Alliance, OMA-TS-PoC_System_Description-V2_0, URL: http://www.openmobilealliance.org/ |

### .

### .

### .

### From section 6.1a “Backward compatibility”:

When PoC Client is configured according to the parameters specified in [OMA-PoC1.0-CP] "The parameters to be provisioned for PoC service" the PoC Client SHALL perform procedures specified in [OMA-PoC-1-CP] "Procedures at the PoC Client" instead of the procedures specified in this document.

NOTE 1:	OMA PoC 1 CP doesn't specify how to handle the PoC 2 specific PoC Service Settings.

When PoC Client is configured with a “version number” parameter as specified in [OMA-PoC-2 -CP] , [OMA-PoC-2_1 –CP] or this specification  "The parameters to be provisioned for PoC service" which indicates its Home PoC Server is compliant to the PoC 2 specification,  the PoC Client SHALL perform procedures specified in [OMA-PoC-2-CP] "Procedures at the PoC Client" instead of the procedures specified in this document.

NOTE 2:	OMA PoC 2 CP doesn't specify how to handle the PoC 2.1 specific PoC Service Settings.

When PoC Client is configured with a “version number” parameter as specified in [OMA-PoC-2 -CP] , [OMA-PoC-2_1 –CP] or this specification  "The parameters to be provisioned for PoC service" which indicates its Home PoC Server is compliant to the PoC 2.1 specification,  the PoC Client SHALL perform procedures specified in [OMA-PoC-2_1-CP] "Procedures at the PoC Client" instead of the procedures specified in this document.

.

.

.

### From section 7.2.2a “Backward compatibility”:

When PoC Server performing Controlling PoC Function sends a SIP request towards the terminating PoC Network, the PoC Server SHALL perform the actions according to subclause 7.2.2 "Request initiated by the Controlling PoC Function", if the PoC Server does not know the version of OMA PoC specification supported by the terminating PoC Server or the version supported is the one specified in this specification.

When PoC Server performing Controlling PoC Function has learned by means which are out of scope of this specification that the terminating PoC Server supports only the PoC 1.0 specifications the PoC Server SHOULD perform the actions according to [OMA-PoC-1-CP] "Request initiated by the Controlling PoC Function", when sending a SIP request towards the terminating PoC Network.

When PoC Server performing Controlling PoC Function has learned by means which are out of scope of this specification that the terminating PoC Server supports only the PoC 2.0 specifications the PoC Server SHOULD perform the actions according to [OMA-PoC-2-CP] "Request initiated by the Controlling PoC Function", when sending a SIP request towards the terminating PoC Network.

When PoC Server performing Controlling PoC Function has learned by means which are out of scope of this specification that the terminating PoC Server supports only the PoC 2.1 specifications the PoC Server SHOULD perform the actions according to [OMA-PoC-2_1-CP] "Request initiated by the Controlling PoC Function", when sending a SIP request towards the terminating PoC Network.

When sending subsequent SIP requests inside an existing SIP dialog the PoC Server performing the Controlling PoC Function SHALL perform actions according to the rules and procedures of [OMA-PoC-1-CP] "Request initiated by the Controlling PoC Function", if the terminating PoC Server indicated in the User-Agent or Server headers of the previous SIP transaction that the terminating PoC Server supported only the PoC 1 specifications.

When sending subsequent SIP requests inside an existing SIP dialog the PoC Server performing the Controlling PoC Function SHALL perform actions according to the rules and procedures of [OMA-PoC-2-CP] "Request initiated by the Controlling PoC Function", if the terminating PoC Server indicated in the User-Agent or Server headers of the previous SIP transaction that the terminating PoC Server supported only the PoC 2.0 specifications.

When sending subsequent SIP requests inside an existing SIP dialog the PoC Server performing the Controlling PoC Function SHALL perform actions according to the rules and procedures of [OMA-PoC-2_1-CP] "Request initiated by the Controlling PoC Function", if the terminating PoC Server indicated in the User-Agent or Server headers of the previous SIP transaction that the terminating PoC Server supported only the PoC 2.1 specifications.

When the SIP/IP Core corresponds with 3GPP/3GPP2 IMS, the PoC Server SHALL use 3GPP/3GPP2 IMS mechanisms according to rules and procedures of [3GPP TS 24.229] / [3GPP2 X.S0013.4] with the clarifications given in this subclause.

.

.

.

### From 7.3.1a “Backward compatibility”:

When PoC Server performing Participating PoC Function sends a SIP request towards the terminating PoC Network, the PoC Server SHALL perform the actions according to subclause 7.3.1 "Request initiated by the served PoC User", if the PoC Server does not know the version of OMA PoC specification supported by the terminating PoC Server or the version supported is the one specified in this specification.

When PoC Server performing Participating PoC Function in the originating PoC Network has learned by means which are out of scope of this specification that the terminating PoC Server supports only the PoC 1 specifications the PoC Server SHOULD perform the actions according to [OMA-PoC-1-CP] "Requests initiated by the served PoC User", when sending a SIP request towards the terminating PoC Network.

When PoC Server performing Participating PoC Function in the originating PoC Network has learned by means which are out of scope of this specification that the terminating PoC Server supports only the PoC 2.0 specifications the PoC Server SHOULD perform the actions according to [OMA-PoC-2-CP] "Requests initiated by the served PoC User", when sending a SIP request towards the terminating PoC Network.

When PoC Server performing Participating PoC Function in the originating PoC Network has learned by means which are out of scope of this specification that the terminating PoC Server supports only the PoC 2.1 specifications the PoC Server SHOULD perform the actions according to [OMA-PoC-2_1-CP] "Requests initiated by the served PoC User", when sending a SIP request towards the terminating PoC Network.

When sending a SIP request on behalf of a served PoC Client which indicated in the User-Agent header of the SIP PUBLISH request setting the PoC Service Settings that the PoC Client supports only the PoC 1 specifications, the PoC Server performing the Participating PoC Function SHALL perform actions according to the rules and procedures of [OMA-PoC-1-CP] "Request initiated by the Served PoC User".

When sending a SIP request on behalf of a served PoC Client which indicated in the User-Agent header of the SIP PUBLISH request setting the PoC Service Settings that the PoC Client supports only the PoC 2.0 specifications, the PoC Server performing the Participating PoC Function SHALL perform actions according to the rules and procedures of [OMA-PoC-2-CP] "Request initiated by the Served PoC User".

When sending a SIP request on behalf of a served PoC Client which indicated in the User-Agent header of the SIP PUBLISH request setting the PoC Service Settings that the PoC Client supports only the PoC 2.1 specifications, the PoC Server performing the Participating PoC Function SHALL perform actions according to the rules and procedures of [OMA-PoC-2_1-CP] "Request initiated by the Served PoC User".

When sending subsequent SIP requests inside an existing SIP dialog the PoC Server performing the Participating PoC Function SHALL perform actions according to the rules and procedures of [OMA-PoC-1-CP] "Request initiated by the Served PoC User", if the terminating PoC Server indicated in the User-Agent or Server headers of the previous SIP transaction that the terminating PoC Server supported only the PoC 1 specifications.

When sending subsequent SIP requests inside an existing SIP dialog the PoC Server performing the Participating PoC Function SHALL perform actions according to the rules and procedures of [OMA-PoC-2-CP] "Request initiated by the Served PoC User", if the terminating PoC Server indicated in the User-Agent or Server headers of the previous SIP transaction that the terminating PoC Server supported only the PoC 2.0 specifications.

When sending subsequent SIP requests inside an existing SIP dialog the PoC Server performing the Participating PoC Function SHALL perform actions according to the rules and procedures of [OMA-PoC-2_1-CP] "Request initiated by the Served PoC User", if the terminating PoC Server indicated in the User-Agent or Server headers of the previous SIP transaction that the terminating PoC Server supported only the PoC 2.1 specifications.

When the SIP/IP Core corresponds with 3GPP/3GPP2 IMS, the PoC Server SHALL use 3GPP/3GPP2 IMS mechanisms according to rules and procedures of [3GPP TS 24.229] / [3GPP2 X.S0013.4] with the clarifications given in this subclause.

.

.

.

### From 7.3.2a “Backward compatibility”:

When PoC Server performing Participating PoC Function in the terminating PoC Network sends a SIP request towards the terminating PoC Clients, the PoC Server SHALL perform the actions according to the [OMA-PoC-1-CP] "Requests terminated by the served PoC User", if the User-Agent header received from the terminating PoC Client, when setting the PoC Service Settings, indicates the support only to the PoC 1.0 specifications.

When PoC Server performing Participating PoC Function in the terminating PoC Network sends a SIP request towards the terminating PoC Clients, the PoC Server SHALL perform the actions according to the [OMA-PoC-2-CP] "Requests terminated by the served PoC User", if the User-Agent header received from the terminating PoC Client, when setting the PoC Service Settings, indicates the support only to the PoC 2.0 specifications.

When PoC Server performing Participating PoC Function in the terminating PoC Network sends a SIP request towards the terminating PoC Clients, the PoC Server SHALL perform the actions according to the [OMA-PoC-2_1-CP] "Requests terminated by the served PoC User", if the User-Agent header received from the terminating PoC Client, when setting the PoC Service Settings, indicates the support only to the PoC 2.1 specifications.

The PoC Server performing Participating PoC Function SHALL indicate in the Server header the OMA PoC release version supported by the Invited PoC Client, when sending SIP responses back to the PoC Server performing Controlling PoC Function.

When the SIP/IP Core corresponds with 3GPP/3GPP2 IMS, the PoC Server SHALL use 3GPP/3GPP2 IMS mechanisms according to rules and procedures of [3GPP TS 24.229] / [3GPP2 X.S0013.4] with the clarifications given in this subclause.

.

.

.

From Appendix B. “Static Conformance Requirements (SCR) 	(Normative)”

.

.

.

### NOTE: The following convention applies for the labels regarding the PoC version:

- PoCv1.0 - SCR Item that is the same in PoCv2.0, PoCv2.1, and PCPSv1.0 as it is in PoCv1.0.
- PoCv2.0 - SCR Item that is the same in PoCv2.1 and PCPSv1.0 as it is in PoCv2.0.
- PoCv2.1 - SCR Item that is the same in PCPSv1.0 as it is in PoCv2.1.

- PCPSv1.0 – SCR Item that is new in PCPSv1.0.
- PoCv1.0mod - SCR Item that exists in PoCv1.0, but modifications in any or all of PoCv2.0, PoC v2.1 and  PCPSv1.0.
- PoCv2.0mod - SCR Item that exists in PoCv1.0, but modifications in PoC v2.1 and/or PCPSv1.0.

- PoCv2.1mod - SCR Item that exists in PoCv1.0, but modifications in PCPSv1.0.

From B.1.1 “PoC Client”:

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| PoC_CP-GEN-C-001-M | Backward compatibility with the mandatory client functions PoCv2.0mod | 6.1A, [OMA-POC-1-CP], [OMA-POC-2-CP] [OMA-POC-2_1-CP] | [OMA-POC-1-CP]:MCF [OMA-POC-2-CP]:MCF [OMA-POC-2_1-CP]:MCF |
| PoC_CP-GEN-C-002-O | Backward compatibility with the optional client functions PoCv2.0mod | 6.1A [OMA-POC-1-CP], [OMA-POC-2-CP] [OMA-POC-2_1-CP] | [OMA-POC-1-CP]:OCF [OMA-POC-2-CP]:OCF [OMA-POC-2_1-CP]:OCF |

.

.

.

From B.1.2 “PoC Server”:

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| PoC_CP-GEN-S-001-M | Backward compatibility with the mandatory server functions PoCv2.0mod | 7.2.2a 7.3.1a, 7.3.2a, [OMA-POC-1-CP], [OMA-POC-2-CP] [OMA-POC-2_1-CP] | [OMA-POC-1-CP]:MSF [OMA-POC-2-CP]:MSF [OMA-POC-2_1-CP]:MSF |
| PoC_CP-GEN-S-002-O | Backward compatibility with the optional server functions PoCv2.0mod | 7.2.2a 7.3.1a, 7.3.2a, [OMA-POC-1-CP], [OMA-POC-2-CP] [OMA-POC-2_1-CP] | [OMA-POC-1-CP]:OSF [OMA-POC-2-CP]:MSF [OMA-POC-2_1-CP]:OSF |