Change Request

| Title: | CR_TS_Multicast_C_1_1_Flow_Updates | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA COM PCPS |  |
| Doc to Change: | OMA-PCPS-TS-Multicast-V1.0-20140922-D |  |
| Submission Date: | 28 September 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Bob Fredericks – Motorola Solutions; |  |
| Replaces: | n/a |  |

Reason for Change

This change request updates the Appendix C.1.1 “Starting a Multicast PoC Channel” flow diagram and associated steps to be illustrative for LTE networks as well as GPRS networks.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The PCPS group is recommended to agree to the changes and incorporate it into the corresponding document.

Detailed Change Proposal

- Updates the Multicast TS Appendix C.1.1 “Starting a Multicast PoC Channel” flow diagram and associated steps to be illustrative for LTE networks as well as GPRS networks.

.

.

.

# From Appendix C “Examples of signalling flows	(Informative)”

  - From C.1 “MBMS message flows”

This clause describes example message flows for Multicast PoC using MBMS as the radio access technology.

In order to get an overview of the Multicast PoC message flows, the messages and associated step descriptions are shown below. However, only the messaging between the Radio Access Network and the PoC Server and PoC Client are shown. Note that the message flows are intended to be applicable to both EPS/eUTRAN and GPRS/UTRAN systems.

Only some RRC messages are described in this section; in reality, many other RRC messages will appear between the PoC Client and the Radio Network Controller.

The parameter coding in this appendix is informative and not intended to be used when implementing the Multicast PoC. For detailed coding see the appropriate specification.

    - From C.1.1 “Starting a Multicast PoC Channel”

This subclause shows how a PoC Server performing the Participating PoC Function acting as a BM-SC starts a Multicast PoC Channel during an ongoing PoC Session and how information about the ongoing PoC Session and Multicast PoC Channel details are broadcast to the PoC Client.

The PoC Server is performing the role of a BM-SC.

The PoC Client is performing the role of a MBMS Client.

Figure 2 "Starting a Multicast PoC Channel" shows the message flow for the scenario.

Figure 2: Starting a Multicast PoC Channel.

During an ongoing PoC Session the PoC Server performing the Participating PoC Function serving PoC User A at PoC Client A decides to start a Multicast PoC Channel.

The steps of the flow are as follows:

1. [DIAMETER] CER (from PoC Server A to Core Network)

The PoC Server A (BM-SC) sends a [DIAMETER] CER message to Radio Access Network A.

2. [DIAMETER] CEA (from Core Network to PoC Server A)

The Radio Access Network A sends a [DIAMETER] CEA message to the PoC Server A (BM-SC).

3. [DIAMETER] RAR (Start) (from PoC Server A to Core Network)

The PoC Server A (BM-SC) sends a [DIAMETER] RAR (Start) to the Radio Access Network A.

| DIAMETER HEADERS | Value |
| --- | --- |
| Framed-Interface-Id Framed –Interface-IPv6-Prefix | 0:0:0:1F FD01:0:0:0/64 |
| Framed-IP-Address AVP | 224.0.0.106 |
| MBMS-StartStop-Indication | 0 |
| MBMS-Service-Area | 67 |
| Allocation/Retention Priority | '3' |
| QoS Profile | The quality of service information element is coded as shown in figure 10.5.138/3GPP TS 24.008 and table 10.5.156/3GPP TS 24.008. |
| MBMS-Session-Duration | '000' |
| MBMS-Service-Type | 1 NOTE: this information element only present for GPRS systems |
| MBMS-Counting-Information | 1 NOTE: this information element only present for GPRS systems |
| MBMS-Session-Identity | 4 |
| MBMS-Session-Repetition-number | '0'??? |
| TMGI | MBMS Serv Id: 24 MCC:310 MNC:66 |
| 3GPP-SGSN-Address | 33.0.0.106 |
| 3GPP-SGSN-IPv6-Address | 0222:0:0:1F00 |
| MBMS-2G-3G-Indicator (GPRS) | 2 |
| MBMS-ACCESS-Indicator (EPS) | 2 |
| MBMS-User-Data-Mode-Indication | 1 |

4. [DIAMETER] RAA (start) (from Radio Access Network A to PoC Server A)

The Radio Access Network A sends a [DIAMETER] RAA (Start) to the PoC Server (BM-SC).

| DIAMETER HEADERS | Value |
| --- | --- |
| Experimental-Result-Code | DIAMETER_SUCCESS 2001 |

5a. [RRC] MBMS MODIFIED SERVICE INFO (from Radio Access Network A to PoC Client) – (for GPRS/UTRAN)                OR

5b. MBSFN Configuration INFO (from Radio Access Network A to PoC Client) – (for EPS/eUTRAN)