Ad-hoc and 1-1 PoC Session establishment On-demand Session

Terminating flow – Manual Answer

PoC Server X invites the PoC Client B to a PoC Session by sending a SIP INVITE request to the terminating SIP/IP Core.

The signaling sequences in this subclause show the establishment of the Ad-hoc or 1-1 PoC Session using on-demand signaling in the terminating side. As a prerequisite for the on-demand signaling, the PoC Client needs to be registered.

This scenario applies for the On-demand Session case.

QoE Profiles feature is used in this flow, QoE Profile assigned to the PoC Session is Professional QoE Profile and PoC Client B is subscribed and selects Premium as Local QoE Profile.

This flow shows the signaling sequence for the manual answer case. The flow is as shown in Figure 10 "PoC Session invitation with manual answer".

Figure 10: PoC Session invitation with manual answer

The steps of the flow are as follows:

1.	Session invitation has been received to invite PoC Client

2.	SIP INVITE request (from the PoC Server X to SIP/IP Core X)

PoC Server X sends the SIP INVITE request to SIP/IP Core X.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkbust; isfocus;+g.poc.discretemedia |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professsional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

3.	SIP INVITE request (from SIP/IP Core X to SIP/IP Core B)

SIP/IP Core X forwards the SIP INVITE request to SIP/IP Core B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professsional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

4.	SIP INVITE request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP INVITE request to PoC Server B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | poc-qoe:professional |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

5.	SIP INVITE request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP INVITE request to SIP/IP Core B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerB.networkB.net;gr;session=1-1>;+g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | norefersub,timer |
| Session-Expires: | 1800 |
| Answer-Mode: | Manual;Require |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| c= | IN IP6 60333::ddd: ccc:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 63776 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:63090 |
| i= | speech |
| a= | label:al |
| m= | Application 60000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:al ak |
| m= | video 47566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:ak |
| a= | a=upcc:0 |
| m= | message 44321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[60333::ddd: ccc:aaa:bbb]:44321/gsgsdfg4;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:anonymous@anonymous.invalid" cc:copyControl="to" cc:count="2"/> </list> </resource-lists> |

6.	SIP INVITE request (from SIP/IP Core B to PoC Client B)

SIP/IP Core B forwards the SIP INVITE request to PoC Client B.

| Request-URI | sip:PoC-ClientB@PoCClientB.networkB.net |
| --- | --- |
| SIP HEADERS |  |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerB.networkB.net;gr;session=1-1>; +g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | norefersub,timer |
| Session-Expires: | 1800 |
| Answer-Mode: | Manual;Require |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| c= | IN IP6 60333::ddd: ccc:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 63776 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:63090 |
| i= | speech |
| a= | label:al |
| m= | Application 60000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:al ak |
| m= | video 47566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:ak |
| a= | a=upcc:0 |
| m= | message 44321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[60333::ddd: ccc:aaa:bbb]:44321/gsgsdfg4;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:anonymous@anonymous.invalid" cc:copyControl="to" cc:count="2"/> </list> </resource-lists> |

7.	SIP 180 "Ringing" response (from PoC Client B to SIP/IP Core B)

PoC Client B sends a SIP 180 "Ringing" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Preferred-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Contact: | <sip:PoC-UserB@networkB.net;gr= urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>";+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |

8.	SIP 180 "Ringing" response (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP 180 "Ringing" response to PoC Server B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Contact: | <sip:PoC-UserB.@networkB.net;gr= urn:uuid:00000000-0000-1000-8000-000A95A0E128>; +sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>";+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |

9.	SIP 180 "Ringing" response (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP 180 "Ringing" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,PRACK, UPDATE,MESSAGE,OPTIONS |

10.	SIP 180 "Ringing"  response (from SIP/IP Core B to SIP/IP Core X)

SIP/IP Core B forwards the SIP 180 "Ringing" response to SIP/IP Core X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr>;+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,PRACK,UPDATE,MESSAGE,OPTIONS |

11.	SIP 180 "Ringing" response (from SIP/IP Core X to PoC Server X)

SIP/IP Core X forwards the SIP 180 "Ringing" response to PoC Server X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr>;+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,PRACK,UPDATE,MESSAGE,OPTIONS |

12.	SIP 200 "OK" response (from PoC Client B to SIP/IP Core B)

PoC Client B sends a SIP 200 "OK" response to SIP/IP Core B. PoC User B is not allowed to use Professional QoE Profile because he is subscribed to Premium QoE Profile.

| SIP HEADERS |  |
| --- | --- |
| P-Preferred-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-UserB@networkB.net;gr= urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>";+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 75555::eee:aaa:fff:bbb |
| a= | poc-qoe:premium |
| m= | audio 75575 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:75000 |
| i= | speech |
| a= | label:bl |
| b= | AS:25.4 |
| m= | application 75590 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:bl bk |
| m= | video 47888 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:bk |
| a= | a=upcc:0 |
| m= | message 22335 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[75555::eee:aaa:fff:bbb]:22335/45yzxcasd;tcp |
| a= | max-size:20000 |

13.	SIP 200 "OK" response (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP 200 "OK" response to PoC Server B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-UserB@networkB.net;gr= urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>";+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 75555::eee:aaa:fff:bbb |
| a= | poc-qoe:premium |
| m= | audio 75575 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:75000 |
| i= | speech |
| a= | label:bl |
| b= | AS:25.4 |
| m= | application 75590 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:bl bk |
| m= | video 47888 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:bk |
| a= | a=upcc:0 |
| m= | message 22335 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[75555::eee:aaa:fff:bbb]:22335/45yzxcasd;tcp |
| a= | max-size:20000 |

14.	SIP 200 "OK" response (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP 200 "OK" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:uu vv |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:vv |
| a= | a=upcc:0 |
| m= | message 43821 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[99055::fff:aaa:eee:bbb]:43821/xk83a45;tcp |
| a= | max-size:10000 |

15.	SIP 200 "OK" response (from SIP/IP Core B to SIP/IP Core X)

SIP/IP Core B forwards the SIP 200 "OK" response to SIP/IP Core X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:uu vv |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:vv |
| a= | a=upcc:0 |
| m= | message 43821 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[99055::fff:aaa:eee:bbb]:43821/xk83a45;tcp |
| a= | max-size:10000 |

16.	SIP 200 "OK" response (from SIP/IP Core X to PoC Server X)

SIP/IP Core X forwards the SIP 200 "OK" response to PoC Server X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1;tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:uu vv |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:vv |
| a= | a=upcc:0 |
| m= | message 43821 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[99055::fff:aaa:eee:bbb]:43821/xk83a45;tcp |
| a= | max-size:10000 |

A.	MBCP Media Burst Taken message (from PoC Server X to PoC Server B).

The MBCP Media Burst Taken message is sent to inform the PoC Client B that another PoC Client has been granted to send a Media Burst. The message contains the identity of the PoC User that has been granted to send a Media Burst.

B.	MBCP Media Burst Taken message (from PoC Server B to PoC Client B).

The MBCP Media Burst Taken message is sent to inform the PoC Client B that another PoC Client has been granted to send a Media Burst. The message contains the identity of the PoC User that has been granted to send a Media Burst.

17.	SIP ACK request (from PoC Server X to SIP/IP Core X)

PoC Server X sends a SIP ACK request to SIP/IP Core X.

18.	SIP ACK request (from SIP/IP Core X to SIP/IP Core B)

SIP/IP Core X forwards the SIP ACK request to SIP/IP Core B.

19.	SIP ACK request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP ACK request to PoC Server B.

20.	SIP ACK request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP ACK request to SIP/IP Core B.

21.	SIP ACK request (from SIP/IP Core B to PoC Client B)

SIP/IP Core B forwards the  SIP ACK request to PoC Client B.

Terminating flow – Automatic Answer

This flow shows the automatic answer case for the On-demand Session case.

QoE Profiles feature is used in this flow, QoE Profile assigned to the PoC Session is Professional QoE Profile and PoC Client B is subscribed and selects Premium as Local QoE Profile.

The flows in Figure 11 "PoC Session invitation with automatic answer" shows the message flow for the scenario.

Figure 11: PoC Session invitation with automatic answer

The steps of the flow are as follows:

1.	Session invitation has been received to invite PoC Client

2.	SIP INVITE request (from PoC Server X to SIP/IP Core X)

PoC Server X sends the SIP INVITE request to SIP/IP Core X.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=adhoc>;+g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserC@networkC.com;uriusage=user" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserD@networkD.com" cc:copyControl="to" cc:anonymize="false"/> </list> </resource-lists> |

3.	SIP INVITE request (from SIP/IP Core X to SIP/IP Core B)

SIP/IP Core X forwards the SIP INVITE request to SIP/IP Core B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=adhoc>; +g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserC@networkC.com;uriusage=user" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserD@networkD.com" cc:copyControl="to" cc:anonymize="false"/> </list> </resource-lists> |

4.	SIP INVITE request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP INVITE request to PoC Server B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=adhoc>; +g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserC@networkC.com;uriusage=user" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserD@networkD.com" cc:copyControl="to" cc:anonymize="false"/> </list> </resource-lists> |

5.	SIP INVITE request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP INVITE request to SIP/IP Core B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerB.networkB.net;gr;session=adhoc>; +g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | norefersub,timer |
| Session-Expires: | 1800 |
| Answer-Mode: | Auto |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER.MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| c= | IN IP6 60333::ddd: ccc:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 63776 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:63090 |
| i= | speech |
| a= | label:al |
| m= | Application 60000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:al ak |
| m= | video 47566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:ak |
| a= | a=upcc:0 |
| m= | message 44321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[60333::ddd: ccc:aaa:bbb]:44321/gsgsdfg4;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserD@networkD.com" cc:copyControl="to" cc:anonymize="false"/> <entry uri="sip:anonymous@anonymous.invalid" cc:copyControl="to" cc:count="2"/> </list> </resource-lists> |

6.	SIP INVITE request (from SIP/IP Core B to PoC Client B)

SIP/IP Core B forwards the SIP INVITE request to PoC Client B.

| Request-URI | sip:PoC-ClientB@PoCClienB.networkB.net |
| --- | --- |
| SIP HEADERS |  |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerB.networkB.net;gr;session=adhoc>; +g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | norefersub,timer |
| Session-Expires: | 1800 |
| Answer-Mode: | Auto |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| c= | IN IP6 60333::ddd: ccc:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 63776 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:63090 |
| i= | speech |
| a= | label:al |
| m= | Application 60000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:al ak |
| m= | video 47566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:ak |
| a= | a=upcc:0 |
| m= | message 44321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[60333::ddd: ccc:aaa:bbb]:44321/gsgsdfg4;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: Content-Disposition: | text/directory;profile="vcard";charset=UTF-8 "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserD@networkD.com" cc:copyControl="to" cc:anonymize="false"/> <entry uri="sip:anonymous@anonymous.invalid" cc:copyControl="to" cc:count="2"/> </list> </resource-lists> |

7.	SIP 183 "Session Progress" response (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP 183 "Session Progress" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | 100rel |
| Server: | PoC-Serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr>;+g.poc.talkburst |
| P-Answer-State: | Unconfirmed |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE, REFER,MESSAGE,OPTIONS |

8.	SIP 183 "Session Progress" response (from SIP/IP Core B to SIP/IP Core X)

SIP/IP Core B forwards the SIP 180 "Session Progress" response to SIP/IP Core X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-tUserB@networkB.net> |
| Privacy: | id |
| Require: | 100rel |
| Server: | PoC-Serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr>; +g.poc.talkburst |
| P-Answer-State: | Unconfirmed |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE, REFER,MESSAGE, OPTIONS |

9.	SIP 183 "Session Progress" response (from SIP/IP Core X to PoC Server X)

SIP/IP Core X forwards the SIP 183 "Session Progress" response to PoC Server X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | 100rel |
| Server: | PoC-Serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr>; +g.poc.talkburst |
| P-Answer-State: | Unconfirmed |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE, OPTIONS |

10.	SIP PRACK request (from PoC Server X to SIP/IP Core X)

If PoC Server B requested SIP 183 "Session progress" response to be sent reliably, PoC Server X sends a SIP PRACK request to the SIP/IP Core X.

| Request-URI | sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr |
| --- | --- |

11.	SIP PRACK request (from SIP/IP Core X to SIP/IP Core B)

SIP/IP Core X forwards the SIP PRACK request to SIP/IP Core B.

| Request-URI | sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr |
| --- | --- |

12.	SIP PRACK request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards SIP PRACK request to PoC Server B.

| Request-URI | sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr |
| --- | --- |

13.	SIP 200 "OK" response to the SIP PRACK request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP 200 "OK" response to SIP/IP Core B.

14.	SIP 200 "OK" response to the SIP PRACK request (from SIP/IP Core B to SIP/IP Core X)

SIP/IP Core B forwards the SIP 200 "OK" response to SIP/IP core X.

15.	SIP 200 "OK" response to the SIP PRACK request (from SIP/IP Core X to PoC Server X)

SIP/IP Core X forwards the SIP 200 "OK" response to PoC Server X.

16.	SIP 200 "OK" response to the SIP INVITE request (from PoC Client B to SIP/IP Core B)

PoC Client B sends a SIP 200 "OK" response to SIP/IP Core B. PoC User B is not allowed to use Professional QoE Profile because he is subscribed to Premium QoE Profile.

| SIP HEADERS |  |
| --- | --- |
| P-Preferred-Identity: | "PoC User B"<sip:PoC-ClientB.networkB.net> |
| Privacy: | id |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Contact: | <sip:PoC-UserB@networkB.net;gr= urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>";+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Require: | timer |
| Session-Expires: | 1800;refresher=uas |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| SDP PARAMETERS |  |
| c= | IN IP6 75555::eee:aaa:fff:bbb |
| a= | poc-qoe:premium |
| m= | audio 75575 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:75000 |
| i= | speech |
| a= | label:bl |
| b= | AS:25.4 |
| m= | application 75590 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:bl bk |
| m= | video 47888 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:bk |
| a= | a=upcc:0 |
| m= | message 22335 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[75555::eee:aaa:fff:bbb]:22335/45yzxcasd;tcp |
| a= | max-size:20000 |

17.	SIP 200 "OK" response to the SIP INVITE request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP 200 "OK" response to PoC Server B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Contact: | <sip:PoC-UserB@networkB.net;gr= urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>";+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Require: | timer |
| Session-Expires: | 1800;refresher=uas |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 75555::eee:aaa:fff:bbb |
| a= | poc-qoe:premium |
| m= | audio 75575 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:75000 |
| i= | speech |
| a= | label:bl |
| b= | AS:25.4 |
| m= | application 75590 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:bl bk |
| m= | video 47888 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:bk |
| a= | a=upcc:0 |
| m= | message 22335 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[75555::eee:aaa:fff:bbb]:22335/45yzxcasd;tcp |
| a= | max-size:20000 |

18.	SIP 200 "OK" response to the SIP INVITE request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP 200 "OK" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Session-Expires: | 1800;refresher=uas |
| Server: | PoC-Serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:uu vv |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:vv |
| a= | a=upcc:0 |
| m= | message 43821 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[99055::fff:aaa:eee:bbb]:43821/xk83a45;tcp |
| a= | max-size:10000 |

19.	SIP 200 "OK" response to the SIP INVITE request (from SIP/IP Core B to SIP/IP Core X)

SIP/IP Core B forwards the SIP 200 "OK" response to SIP/IP Core X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Session-Expires: | 1800;refresher=uas |
| Server: | PoC-Serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:uu vv |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:vv |
| a= | a=upcc:0 |
| m= | message 43821 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[99055::fff:aaa:eee:bbb]:43821/xk83a45;tcp |
| a= | max-size:10000 |

20.	SIP 200 "OK" response to the SIP INVITE request (from SIP/IP Core X to PoC Server X)

SIP/IP Core X forwards the SIP 200 "OK" response to PoC Server X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Session-Expires: | 1800;refresher=uas |
| Server: | PoC-Serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:uu vv |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:vv |
| a= | a=upcc:0 |
| m= | message 43821 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[99055::fff:aaa:eee:bbb]:43821/xk83a45;tcp |
| a= | max-size:10000 |

A.	MBCP Media Burst Taken message (from the PoC Server X to the PoC Server B).

The MBCP Media Burst Taken message is sent to inform the PoC Client B that another PoC Client has been granted to send a Media Burst. The message contains the identity of PoC User at the granted PoC Client.

B.	MBCP Media Burst Taken message (from the PoC Server B to the PoC Client B).

The MBCP Media Burst Taken message is sent to inform the PoC Client B that another PoC Client has been granted to send a Media Burst. The message contains the identity of PoC User at the granted PoC Client.

21.	SIP ACK request (from PoC Server X to SIP/IP Core X)

PoC Server X sends a SIP ACK request to SIP/IP Core X.

22.	SIP ACK request (from SIP/IP Core X to SIP/IP Core B)

SIP/IP Core X forwards the SIP ACK request to SIP/IP Core B.

23.	SIP ACK request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the  SIP ACK request to PoC Server B.

24.	SIP ACK request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP ACK request to SIP/IP Core B.

25.	SIP ACK request (from SIP/IP Core B to PoC Client B)

SIP/IP Core B forwards the  SIP ACK request to PoC Client B.