Leaving/Disconnecting from a PoC Session

This subclause describes the scenario when a PoC Client leaves a PoC Session.

Originating flow – On-demand Session case

This subclause shows the procedures for leaving PoC Session establishment on the originating side for the On-demand case when the PoC Server performing the Participating PoC Function acts as a B2BUA.

Figure 16 "Leaving a PoC Session, originating flow – On-demand Session case" shows the message flow for the scenario.

Figure 16: Leaving a PoC Session establishment, originating flow – On-demand Session case

The steps of the flows are as follows:

1.	On-going PoC Session

An on-going PoC Session exists and two or more PoC Participants is connected.

2.	SIP BYE request (from the PoC Client A to the SIP/IP Core A)

The PoC Client initiates release of the SIP session by sending the SIP BYE request to the SIP/IP Core A.

| Request-URI | sip:TokenA%PoC-Session-Identity1@PoC-ServerX.networkX.net%@PoC-ServerA.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Privacy: | id |

3.	SIP BYE request (from the SIP/IP Core A to the PoC Server A)

The SIP/IP Core A sends the SIP BYE request to the PoC Server A.

| Request-URI | sip:TokenA%PoC-Session-Identity1@PoC-ServerX.networkX.net%@PoC-ServerA.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Privacy: | id |

4.	SIP 200 "OK" response (from the PoC Server A to the SIP/IP Core A)

The PoC Server A sends the SIP 200 "OK" response to the SIP/IP Core A.

5.	SIP 200 "OK" response (from the SIP/IP Core A to the PoC Client A)

The SIP/IP Core A sends the SIP 200 "OK" response to the PoC Client A.

6.	SIP BYE request (from the PoC Server A to the SIP/IP Core A)

The PoC Server A sends the SIP BYE request to the  SIP/IP Core A.

| Request-URI | sip: PoC-Session-Identity1@PoC-ServerX.networkX.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Privacy: | id |

7.	SIP BYE request (from the SIP/IP Core A to the SIP/IP Core X)

The PoC Server A sends the SIP BYE request to SIP/IP Core A.

| Request-URI | sip: PoC-Session-Identity1@PoC-ServerX.networkX.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Privacy: | id |

8.	SIP BYE request (from the SIP/IP Core X to the PoC Server X)

The SIP/IP Core X sends the SIP BYE request to the PoC Server X.

| Request-URI | sip: PoC-Session-Identity1@PoC-ServerX.networkX.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Privacy: | id |

9.	SIP 200 "OK" response (from the PoC Server X to the SIP/IP Core X)

The PoC Server X sends the SIP 200 "OK" response to the SIP/IP Core X.

10.	SIP 200 "OK" response (from the SIP/IP Core X to the SIP/IP Core A)

The SIP/IP Core X sends the SIP 200 "OK" response to the SIP/IP Core A.

11.	SIP 200 "OK" response (from the SIP/IP Core A to the PoC Server A)

The SIP/IP Core A sends the SIP 200 "OK" response to the PoC Server A.

12.	Terminating the PoC Session

Depending on the PoC Session release policy the PoC Server X disconnects all PoC Participants from the PoC Session as shown in Figure 18 "Disconnecting a PoC Participant from a PoC Session, terminating flow – On-demand Session case" or Figure 19 "Disconnecting a PoC Participant from a PoC Session, terminating flow – Pre-established Session case".

Originating flow – Pre-established Session case

This subclause shows the procedures for leaving PoC Session establishment on the originating side for the Pre-established case.

Figure 17 "Leaving a PoC Session, originating flow – Pre-established Session case" shows the message flow for the scenario.

Figure 17: Leaving a PoC Session establishment, originating flow – Pre-established Session case

The steps of the flows are as follows:

1.	On-going PoC Session

An on-going PoC Session exists and two or more PoC Participants is connected.

2.	SIP REFER request (from the PoC Client A to SIP/IP Core A)

The PoC Client A sends a REFER request to the SIP/IP Core A.

| Request-URI | sip:Pre-establishedSessionIdentityA@PoC-ServerA.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Refer-To: | sip:PoC-Session-Identity1@PoC-ServerX.networkX.net;method=BYE |
| Target-Dialog | me03a0s09a2sdfgjkl491777; remote-tag=774321; local-tag=64727891 |
| Privacy: | id |
| Contact: | sip:PoC-UserA@ networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |

3.	SIP REFER request (from SIP/IP Core A to the PoC Server A)

The SIP/IP Core A sends the REFER request to the PoC Server A.

| Request-URI | sip:Pre-establishedSessionIdentityA@PoC-ServerA.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| Refer-To: | sip:PoC-Session-Identity1@PoC-ServerX.networkX.net;method=BYE |
| Target-Dialog: | me03a0s09a2sdfgjkl491777; remote-tag=774321; local-tag=64727891 |
| Privacy: | id |
| Contact: | sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |

4.	SIP 20O "OK" response (from the PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP 200 "OK" response to the SIP/IP Core A.

5.	SIP 200 "OK" response (from SIP/IP Core A to the PoC Client A)

The SIP/IP Core A sends the SIP 200 "OK" response to the PoC Client A.

6.	SIP NOTIFY request (from the PoC Server A to SIP/IP Core A)

The PoC Server sends a SIP NOTIFY request towards the PoC Client A and at the same time a SIP BYE request (see step 10) towards the PoC Server performing the Controlling PoC Function.

| Request-URI | sip:PoC-UsertA@networkA.net; gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |
| --- | --- |
| SIP HEADERS |  |
| Subscription-State | active |
| Content-Type | message/sipfrag;version=2.0 |
| MIME BODY |  |
| SIP/2.0 100 Trying |  |

7.	SIP NOTIFY request (from SIP/IP Core A to the PoC Client A)

The SIP/IP Core A sends the NOTIFY request to the PoC Client A.

| Request-URI | sip:PoC-ClientA@PoCClientA.networkA.net |
| --- | --- |
| SIP HEADERS |  |
| Subscription-State | active |
| Content-Type | message/sipfrag;version=2.0 |
| MIME BODY |  |
| SIP/2.0 100 Trying |  |

8.	SIP 200 "OK" response (from the PoC Client A to SIP/IP Core A)

The PoC Client A sends the SIP 200 "OK" response to the SIP/IP Core A.

9.	SIP 200 "OK" response (from SIP/IP Core A to the PoC Server A)

The SIP/IP Core A sends the SIP 200 "OK" response to the PoC Server A.

10.	SIP BYE request (from the PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP BYE request to the SIP/IP Core A.

| Request-URI | sip:PoC-Session-Identity1@PoC-ServerX.networkX.net;gr |
| --- | --- |

11.	SIP BYE request (from SIP/IP Core A to SIP/IP Core X)

The SIP/IP Core A sends the SIP BYE request to the SIP/IP Core X.

| Request-URI | sip:PoC-Session-Identity1@PoC-ServerX.networkX.net;gr |
| --- | --- |

12.	SIP BYE request (from SIP/IP Core X to the PoC Server X)

The SIP/IP Core X sends the SIP BYE request to the PoC Server X.

| Request-URI | sip:PoC-Session-Identity1@PoC-ServerX.networkX.net;gr |
| --- | --- |

13.	SIP 200 "OK" response (from the PoC Server X to SIP/IP Core X)

The PoC Server X sends the SIP 200 "OK" response to SIP/IP Core X.

14.	SIP 200 "OK" response (from SIP/IP Core X to SIP/IP Core A)

The SIP/IP Core X sends the SIP 200 "OK" response to the SIP/IP Core A.

15.	SIP 200 "OK" response (from SIP/IP Core A to the PoC Server A)

The SIP/IP Core A sends the SIP 200 "OK" response to the PoC Server A.

16.	SIP NOTIFY request (from the PoC Server A to the SIP/IP Core A)

The PoC Server A sends a SIP NOTIFY request towards the PoC Client A.

| Request-URI | sip:PoC-UserA@networkA.net; gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |
| --- | --- |
| SIP HEADERS |  |
| Content-Type: | message/sipfrag;version=2.0 |
| Subscription-State: | terminated;reason=noresource |
| MIME BODY |  |
| SIP/2.0 200 OK |  |

17.	SIP NOTIFY request (from the SIP/IP Core A to the PoC Client A)

The SIP/IP Core A sends the NOTIFY request to the PoC Client A.

| Request-URI | sip:PoC-ClientA@PoCClientA.networkA.net |
| --- | --- |
| SIP HEADERS |  |
| Content-Type: | message/sipfrag;version=2.0 |
| Subscription-State: | terminated;reason=noresource |
| MIME BODY |  |
| SIP/2.0 200 OK |  |

18.	SIP 200 "OK" response (from the PoC Client A to the SIP/IP Core A)

The PoC Client A sends the SIP 200 "OK" response to the SIP/IP Core A.

19.	SIP 200 "OK" response (from the SIP/IP Core A to the PoC Server A)

The SIP/IP Core A sends the SIP 200 "OK" response to the PoC Server A.

20.	Terminating the PoC Session

Depending on the PoC Session release policy the PoC Server X disconnects all PoC Participants from the PoC Session as shown in Figure 18 "Disconnecting a PoC Participant from a PoC Session, terminating flow – On-demand Session case" or Figure 19 "Disconnecting a PoC Participant from a PoC Session, terminating flow – Pre-established Session case".

Terminating flow – On-demand Session Case

This subclause shows the procedure for disconnecting a PoC Participant in an on-going PoC Session for the On-demand Session case and when the PoC Server performing the Participating PoC Function acts as a B2BUA.

NOTE:	In the case more than one PoC Participant is connected the procedure is repeated for each PoC Participant.

Figure 18 "Disconnecting a PoC Participant from a PoC Session, terminating flow – On-demand Session case" shows the signalling flow for the scenario.

Figure 18: Disconnecting a PoC Participant from a PoC Session, terminating flow – On-demand Session case

The steps of the flows are as follows:

1.	Release of PoC Session initiated

The PoC Server X (controlling) initiates the release of the PoC Session, e.g. when a PoC Participant leaves the PoC Session. Figure 16 "Leaving a PoC Session establishment, originating flow" Figure 17 "Leaving a PoC Session establishment, originating flow – Pre-established Session case" shows the signalling flow for the originating procedure.

2.	SIP BYE request (from the PoC Server X to the SIP/IP Core X)

The PoC Server X sends the SIP BYE request to SIP/IP Core X.

| Request-URI | sip:xyz@PoC-ServerB.networkB.net;gr |
| --- | --- |

3.	SIP BYE request (from the SIP/IP Core X to the SIP/IP Core B)

The SIP/IP Core X sends the SIP BYE request to SIP/IP Core B.

| Request-URI | sip:xyz@PoC-ServerB.networkB.net;gr |
| --- | --- |

4.	SIP BYE request (from the SIP/IP Core B to the PoC Server B)

The SIP/IP Core B sends SIP BYE request to the PoC Server B.

| Request-URI | sip:xyz@PoC-ServerB.networkB.net;gr |
| --- | --- |

5.	SIP 200 "OK" response (from the PoC Server B to the SIP/IP Core B)

The PoC Server B sends the 200 "OK" response to SIP/IP Core B.

6.	SIP 200 "OK" response (from the SIP/IP Core B to the SIP/IP Core X)

The SIP/IP Core B sends the 200 "OK" response to SIP/IP Core X.

7.	SIP 200 "OK" response (from the SIP/IP Core X to the PoC Server X)

The SIP/IP Core X sends the 200 "OK" response to the PoC Server X.

8.	SIP BYE request (from the PoC Server B to the SIP/IP Core B)

The PoC Server B sends the SIP BYE request to SIP/IP Core B.

| Request-URI | sip:PoC-UsertB@networkB.net; gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |
| --- | --- |

9.	SIP BYE request (from the SIP/IP Core B to the PoC Client B)

The SIP/IP Core B sends the SIP BYE request to the PoC Client B.

| Request-URI | sip:PoC-ClientB@PoCClientB.networkB.net |
| --- | --- |

10.	SIP 200 "OK" response (from the PoC Client B to the SIP/IP Core B)

The PoC Client B sends the 200 "OK" response to SIP/IP Core B.

11.	SIP 200 "OK" response (from the SIP/IP Core B to the PoC Server B)

The SIP/IP Core B sends the 200 "OK" response the PoC Server B.

Terminating flow – Pre-established Session Case

This subclause shows the procedure for disconnecting a PoC Participant in an on-going PoC Session for the Pre-established Session case.

NOTE:	In the case more than one PoC Participant is connected the procedure is repeated for each PoC Participant.

Figure 19 "Disconnecting a PoC Participant from a PoC Session, terminating flow – Pre-established Session case" shows the signalling flow for the scenario.

Figure 19: Disconnecting a PoC Participant from a PoC Session, terminating flow – Pre-established Session case

The steps of the flows are as follows:

1.	Release of PoC Session initiated

The PoC Server X (controlling) initiates the release of the PoC Session, e.g. when a PoC Participants leaves the PoC Session. Figure 16 "Leaving a PoC Session establishment, originating flow – On-demand Session case" or Figure 17 "Leaving a PoC Session establishment, originating flow – Pre-established Session case" shows the signalling flow for the originating procedure.

2.	SIP BYE request (from the PoC Server X to the SIP/IP Core X)

The PoC Server X sends the SIP BYE request to the SIP/IP Core X.

| Request-URI | sip:PoC-UserB-1@PoCServerB.networkB.net;gr |
| --- | --- |

3.	SIP BYE request (from the SIP/IP Core X to the SIP/IP Core B)

The SIP/IP Core X sends the SIP BYE request to the SIP/IP Core B.

| Request-URI | sip:PoC-UserB-1@PoCServerB.networkB.net;gr |
| --- | --- |

4.	SIP BYE request (from the SIP/IP Core B to the PoC Server B)

The SIP/IP Core B sends the SIP BYE request to the PoC Server B.

| Request-URI | sip:PoC-UserB-1@PoCServerB.networkB.net;gr |
| --- | --- |

5.	SIP 200 "OK" response (from the PoC Server B to the SIP/IP Core B)

The PoC Server B sends the SIP 200 "OK" response to the SIP/IP Core B.

6.	SIP 200 "OK" response (from the SIP/IP Core B to the SIP/IP Core X)

The SIP/IP Core B sends the SIP 200 "OK" response to the SIP/IP Core X.

7.	SIP 200 "OK" response (from the SIP/IP Core X to the PoC Server X)

The SIP/IP Core X sends the SIP 200 "OK" response to the PoC Server X.

A.	MBCP Disconnect message (from the PoC Server B to the PoC Client B)

The PoC Server B sends the MBCP Disconnect message to the PoC Client B.

B.	MBCP Media Burst Acknowledgment message (from the PoC Client B to the PoC Server B)

The PoC Client B sends theMBCP Media Burst Acknowledgment message to the PoC Server B.