Group Advertisement

This subclause shows an example of how one PoC User can send a Group Advertisement to PoC Group. The example includes authorization and Access Control. The example also includes sending of text content, a reference to media content and media content included in MIME body.

Originating flow

Terminating flow

This subclause describes the message flow between the receiver of the Group Advertisement, i.e. the PoC Client B and the PoC Server B in the PoC Client B Home Network.

In this example the PoC Server B acts as proxy according to [RFC3261] and is not allowed to change the value of the headers User-Agent and Server, as specified in [RFC3428].

The flow is as shown in Figure 28 "PoC Client B receives a Group Advertisement from PoC Client A".

Figure 28: PoC Client B receives a Group Advertisement from PoC Client A

The steps of the flows are as follows:

1.	The receival of a Group Advertisement (PoC Server X)

The PoC Server X receives an authorized request from a PoC User A to send a Group Advertisement to a PoC Group that includes PoC User B.

2.	SIP MESSAGE request (from PoC Server X to SIP/IP Core X)

The PoC Server X sends a SIP MESSAGE request to the SIP/IP Core X. The Request-URI includes the PoC Address of the PoC User B and an Accept-Contact header with the PoC feature tag '+g.poc.groupad'.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.groupad; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Content-Type: | multipart/mixed |
| Subject: | A message from Your travel guide! |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| XML MIME BODY |  |
| Content-Type: | application/vnd.poc.group-advertisement+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <group-advertisement xmlns="urn:oma:xml:poc:group-advertisement" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="urn:oma:xml:poc:group-advertisement"> <note>I set this group up for us golfers</note> <group type="dialed-in"> <display-name>Golf Buddies</display-name> <uri>sip:OMA-Golf-buddies@networkX.net </uri> <supported-services> <service enabler=”poc”> <group-media> <poc-speech> </group-media> </service> </supported-services> </group> </group-advertisement> |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:Tom Howard n:Howard;Tom org:Albatross Tours EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

3.	SIP MESSAGE request (from SIP/IP Core X to SIP/IP Core B)

The SIP/IP Core X sends the SIP MESSAGE request to the SIP/IP Core B based on the PoC Address of the PoC User B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.groupad; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Content-Type: | multipart/mixed |
| Subject: | A message from Your travel guide! |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| XML MIME BODY |  |
| Content-Type: | application/vnd.poc.group-advertisement+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <group-advertisement xmlns="urn:oma:xml:poc:group-advertisement" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="urn:oma:xml:poc:group-advertisement"> <note>I set this group up for us golfers</note> <group type="dialed-in"> <display-name>Golf Buddies</display-name> <uri>sip:OMA-Golf-buddies@networkX.net </uri> <supported-services> <service enabler=”poc”> <group-media> <poc-speech> </group-media> </service> </supported-services> </group> </group-advertisement> |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:Tom Howard n:Howard;Tom org:Albatross Tours EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

4.	SIP MESSAGE request (from SIP/IP Core B to PoC Server B)

The SIP/IP Core B sends the SIP MESSAGE request to the PoC Server B based on the PoC feature tag '+g.poc.groupad' in the Accept Contact.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.groupad; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Content-Type: | multipart/mixed |
| Subject: | A message from Your travel guide! |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| XML MIME BODY |  |
| Content-Type: | application/vnd.poc.group-advertisement+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <group-advertisement xmlns="urn:oma:xml:poc:group-advertisement" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="urn:oma:xml:poc:group-advertisement"> <note>I set this group up for us golfers</note> <group type="dialed-in"> <display-name>Golf Buddies</display-name> <uri>sip:OMA-Golf-buddies@networkX.net </uri> <supported-services> <service enabler=”poc”> <group-media> <poc-speech> </group-media> </service> </supported-services> </group> </group-advertisement> |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:Tom Howard n:Howard;Tom org:Albatross Tours EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

5.	SIP MESSAGE request (from PoC Server B to SIP/IP Core B)

The PoC Server B performs Access Control and determines that the PoC User A at the PoC Client A is authorized to send a Group Advertisement to the PoC User B. The PoC Server B sends the SIP MESSAGE request to the SIP/IP Core.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.groupad; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Content-Type: | multipart/mixed |
| Subject: | A message from Your travel guide! |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| XML MIME BODY |  |
| Content-Type: | application/vnd.poc.group-advertisement+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <group-advertisement xmlns="urn:oma:xml:poc:group-advertisement" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="urn:oma:xml:poc:group-advertisement"> <note>I set this group up for us golfers</note> <group type="dialed-in"> <display-name>Golf Buddies</display-name> <uri>sip:OMA-Golf-buddies@networkX.net </uri> <supported-services> <service enabler=”poc”> <group-media> <poc-speech> </group-media> </service> </supported-services> </group> </group-advertisement> |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:Tom Howard n:Howard;Tom org:Albatross Tours EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

6.	SIP MESSAGE request (from SIP/IP Core B to PoC Client B)

The SIP IP/Core B sends the SIP MESSAGE to the PoC Client B based on information stored during registration.

| Request-URI | sip:PoC-ClientB@PoCClientB.networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.groupad; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Content-Type: | multipart/mixed |
| Subject: | A message from Your travel guide! |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| XML MIME BODY |  |
| Content-Type: | application/vnd.poc.group-advertisement+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <group-advertisement xmlns="urn:oma:xml:poc:group-advertisement" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="urn:oma:xml:poc:group-advertisement"> <note>I set this group up for us golfers</note> <group type="dialed-in"> <display-name>Golf Buddies</display-name> <uri>sip:OMA-Golf-buddies@networkX.net </uri> <supported-services> <service enabler=”poc”> <group-media> <poc-speech> </group-media> </service> </supported-services> </group> </group-advertisement> |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:Tom Howard n:Howard;Tom org:Albatross Tours EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

7.	SIP 200 "OK" response (from PoC Client B to SIP/IP Core B)

The PoC Client B sends a SIP 200 "OK" response to the SIP/IP Core B in order to acknowledge that the Group Advertisement was received. The SIP 200 "OK" response is sent along the signalling path to the PoC Server A.

| SIP HEADERS |  |
| --- | --- |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |

8.	SIP 200 "OK" response (from SIP/IP Core B to PoC Server B)

The SIP/IP Core B forwards the SIP 200 "OK" response to PoC Server B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-ClientB@networkB.net> |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |

9.	SIP 200 "OK" response (from PoC Server B to SIP/IP Core B)

The PoC Server B forwards the SIP 200 "OK" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-ClientB@networkB.net> |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.010 |

10.	SIP 200 "OK" response (from SIP/IP Core B to SIP/IP Core X)

The SIP/IP Core B forwards the SIP 200 "OK" response SIP/IP Core X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-ClientB@networkB.net> |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |

11.	SIP 200 "OK" response (from SIP/IP Core to PoC Server X)

The SIP/IP Core X forwards the SIP 200 "OK" response to PoC Server X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-ClientB@networkB.net> |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |

12.	The sending of the acknowledgement (PoC Server X to PoC Client A)

The PoC Server X sends the acknowledgment to the PoC Client A as shown in Figure 27 "PoC Client A sends a Group Advertisement to a PoC Group".