Full Duplex Call Follow-on Proceed

This subclause shows an example of how one Participant can ask other Participants to switch from the PoC Session to a full duplex call.

Originating flow

This subclause describes the Full Duplex Call Follow-On Proceed message flow between the PoC Client A participating in the PoC Session Q and the PoC Server X performing the Controlling PoC Function of the PoC Session Q.

The PoC Client A is sending the Full Duplex Call Follow-on Proceed request to all the Participants of the PoC Session Q whose PoC Clients support Full Duplex Call Follow-on Proceed.

The PoC Server A is a SIP B2BUA in the PoC Session Q.

The SIP MESSAGE request is sent within the SIP dialog of the PoC Session.

NOTE:	The Request URI is set according to the SIP dialog.

The flow is as shown in Figure 31 "PoC Client A sends the Full Duplex Call Follow-on Proceed request".

Figure 31: PoC Client A sends the Full Duplex Call Follow-on Proceed request

1.	The PoC Client A participates in the PoC Session Q.

2.	SIP MESSAGE request (from PoC Client A to SIP/IP Core A)

| Request-URI | sip:PoCSessionQ%%networkX.net@networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| P-Preferred-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Content-Type: | application/vnd.poc.fdcfo+xml |
| User-Agent: | PoC-client/OMA2.0 vendorOfPoCClientA |
| XML MIME BODY |  |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:fdcfo="urn:oma:xml:poc:fdcfo" > <list name="My conference"> <entry uri="sip:myconference@conferencebridge.com"/> <entry uri="tel:+15555555555"> <fdcfo:postd>23456#</fdcfo:postd> </entry> </list> </resource-lists> |

3.	SIP MESSAGE request (from SIP/IP Core A to PoC Server A)

| Request-URI | sip:PoCSessionQ%%networkX.net@networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Content-Type: | application/vnd.poc.fdcfo+xml |
| User-Agent: | PoC-client/OMA2.0 vendorOfPoCClientA |
| XML MIME BODY |  |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:fdcfo="urn:oma:xml:poc:fdcfo" > <list name="My conference"> <entry uri="sip:myconference@conferencebridge.com"/> <entry uri="tel:+15555555555"> <fdcfo:postd>23456#</fdcfo:postd> </entry> </list> </resource-lists> |

4.	SIP MESSAGE request (from PoC Server A to SIP/IP Core A)

| Request-URI | sip:PoCSessionQ@networkX.net;gr |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Content-Type: | application/vnd.poc.fdcfo+xml |
| User-Agent: | PoC-serv/OMA2.0 vendorOfPoCServerA |
| XML MIME BODY |  |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:fdcfo="urn:oma:xml:poc:fdcfo" > <list name="My conference"> <entry uri="sip:myconference@conferencebridge.com"/> <entry uri="tel:+15555555555"> <fdcfo:postd>23456#</fdcfo:postd> </entry> </list> </resource-lists> |

5.	SIP MESSAGE request (from SIP/IP Core A to SIP/IP Core X)

The PoC specific elements are the same as in the previous message.

6.	SIP MESSAGE request (from SIP/IP Core X to PoC Server X)

The PoC specific elements are the same as in the previous message.

7.	PoC Server sends a Full Duplex Call Follow-on Proceed request to each Participant whose PoC Client supports the Full Duplex Call Follow-on Proceed.

8.	SIP 200 "OK" response (from PoC Server X to SIP/IP Core X)

Upon receiving the first SIP 2xx response for the SIP MESSAGE requests sent in previous step, a SIP 200 "OK" response to the SIP MESSAGE request is sent to towards the PoC Client A.

| SIP HEADERS |  |
| --- | --- |
| Server: | PoC-serv/OMA2.0 vendorOfPoCServerX |

9.	SIP 200 "OK" response (from SIP/IP Core X to SIP/IP Core A)

The PoC specific elements are the same as in the previous message.

10.	SIP 200 "OK" response (from SIP/IP Core A to PoC Server A)

The PoC specific elements are the same as in the previous message.

11.	SIP 200 "OK" response (from PoC Server A to SIP/IP Core A)

| SIP HEADERS |  |
| --- | --- |
| Server: | PoC-serv/OMA2.0 vendorOfPoCServerA |

12.	SIP 200 "OK" response (from SIP/IP Core A to PoC Client A)

The PoC specific elements are the same as in the previous message.

13.	PoC Client A leaves the PoC Session

14.	UE of the PoC User A joins the full duplex call at one of the addresses listed in the sent Full Duplex Call Follow-On Proceed document.