Change Request

| Title: | CR_TS_Multicast_C_1_2_Flow_Updates | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA COM PCPS |  |
| Doc to Change: | OMA-PCPS-TS-Multicast-V1.0-20140922-D |  |
| Submission Date: | 06 November 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Bob Fredericks – Motorola Solutions; |  |
| Replaces: | OMA-COM-PCPS-2014-0300R01R01-CR_TS_Multicast_C_1_2_Flow_Updates.doc |  |

Reason for Change

This change request updates the Appendix C.1.2 “Starting a Multicast PoC Channel” flow diagram and associated steps to be illustrative for LTE networks as well as GPRS networks. This CR addresses CONR issue D022:

| D022 | 2014.10.08 | T | C.1.2 | Source: Motorola Solutions Inc D001-MSI1 Form: OMA-PCPS-TS-Multicast-V1_0-20141008-D Comment: The flow and accompanying text is GPRS specific and needs to be updated to accommodate both EPS and GPRS similarly to the changes made for C.1.1. Proposed Change: Incorporate the updates in CR OMA-COM-PCPS-2014-0XXX-CR_TS_Multicast_C_1_2_Flow_Updates. | Status: OPEN |
| --- | --- | --- | --- | --- | --- |

Additionally, this CR merges in the RFC6665 updates for this flow from CR 298R01 which addresses CONRR comment D027 for flow C.1.2 as well as other flows addressed in separate CRs.

| D027 | 2014.10.08 | T | C | Source: BlackBerry Form: <INP doc, mtg, confcall> Comment: Flows neeed updating to RFC 6665 Proposed Change: Update flows to RFC 6665 | Status: OPEN |
| --- | --- | --- | --- | --- | --- |

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The PCPS group is recommended to agree to the changes and incorporate it into the corresponding document.

Detailed Change Proposal

- Updates the Multicast TS Appendix C.1.2 “Connecting to a Multicast PoC Channel” flow diagram and associated steps to be illustrative for LTE networks as well as GPRS networks. Additionally, this CR merges in the RFC6665 updates for this flow only from CR 298R01 which addresses CONRR comment D027.

.

.

.

- From C.1.2 “Connecting to a Multicast PoC Channel”

This subclause shows how a PoC Client connects to a Multicast PoC Channel during an ongoing PoC Session.

The PoC Server is performing the role of a Participating PoC Function.

The PoC Client is either performing the role of a PoC Client or the role of a MBMS Client.

Figure 3 "Connecting to a Multicast PoC Channel" shows the message flow for the scenario.

Figure 3: Connecting to a Multicast PoC Channel.

The steps of the flow are as follows:

1. SIP INFO request (from PoC Server A to SIP/IP Core A)

The PoC Server A (participating) sends a SIP INFO request to SIP/IP Core A.

| Request-URI | sip:PoC-UserA@networkA.net; gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 |
| --- | --- |
| SIP HEADERS |  |
| User-Agent: | PoC-serv/OMA2.1 |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerA.networkA.net;gr;session= prearranged >;+g.poc.talkburst; +g.poc.multicast;isfocus |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER, |
| SDP PARAMETERS |  |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:premium |
| m= | audio 2642 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| i= | speech |
| a= | label:aa |
| a | recvonly |
| m= | application 2643 udp TBCP |
| a= | fmtp:TBCP multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | audio 1234 RTP/AVP 98 |
| a= | rtpmap:98 AMR |
| c= | IN IP6 0:0:0:1F |
| a= | label bb |
| a | sendonly |
| a= | mbms-mode:broadcast-mbsfn 24 |

NOTE: For GPRS Systems, the mbms-mode parameter will be set to “broadcast 24 1”

2. SIP INFO request (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A routes the SIP INFO request to PoC Client A.

| Request-URI | sip:PoC-ClientA@PoCClientA..networkA.net |
| --- | --- |
| SIP HEADERS |  |
| User-Agent: | PoC-serv/OMA2.1 |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerA.networkA.net;gr;session= prearranged >;+g.poc.talkburst; +g.poc.multicast;isfocus |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER, |
| SDP PARAMETERS |  |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:premium |
| m= | audio 2642 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| i= | speech |
| a= | label:aa |
| a | recvonly |
| m= | application 2643 udp TBCP |
| a= | fmtp:TBCP multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | audio 1234 RTP/AVP 98 |
| a= | rtpmap:98 AMR |
| c= | IN IP6 0:0:0:1F |
| a= | label bb |
| a | sendonly |
| a= | mbms-mode:broadcast-mbsfn 24 |

NOTE: For GPRS Systems, the mbms-mode parameter will be set to “broadcast 24 1”

3. SIP 200 "OK" response (from PoC Client A to SIP/IP Core A)

The PoC Client A sends a SIP 200 OK response message toSIP/IP Core A.

| SIP HEADERS |  |
| --- | --- |
| Require: | timer |
| Server: | PoC-client/OMA2.1 Acme-Talk5000/v1.01 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128 >;+sip.instance="< urn:gsma:imei:90420156-025763-0>">;+g.poc.talkburst; +g.poc.multicast |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER, |

4. SIP 200 "OK" response (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A routes the SIP 200 "OK" response to PoC Server A.

Contents contained in the SIP 200 "OK" response is the same as in step 3.

5. [RRC] MBSFN AREA CONFIGURATION  (from Radio Access/Core Network A to PoC Client)

Radio Access/Core Network A sends a [RRC] MBSFN AREA CONFIGURATION message over the MCCH logical channel which is received by the PoC Client (MBMS Client).

NOTE: For GPRS systems, Radio Access/Core Network A sends a [RRC] MBMS MODIFIED SERVICES INFO message instead of the MBSFN AREA CONFIGURATION message over the MCCH or DCCH logical channel which is received by PoC Client A.

6. SIP UPDATE request (from PoC Client A to SIP/IP Core A)

The PoC Client A sends a SIP UPDATE request to SIP/IP Core A.

| Request-URI | sip:TokenA%PoC-Session-Identity1@ PoC-ServerX.networkX.net%@PoC-ServerA.networkA.net |
| --- | --- |
| SIP HEADERS |  |
| User-Agent: | PoC-client/OMA2.1 Acme-Talk5000/v1.01 |
| Contact: | <sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>">;+g.poc.talkburst; +g.poc.multicast |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER, |
| SDP PARAMETERS |  |
| c= | IN IP6 5555::bbb:ccc:ddd:eee |
| a= | poc-qoe:premium |
| m= | audio 3456 RTP/AVP 95 |
| a= | rtpmap: 95 AMR |
| i= | speech |
| a= | label:aa |
| a= | sendonly |
| m= | application 3457 udp TBCP |
| a= | fmtp:TBCP multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | audio 1234 RTP/AVP 98 |
| a= | rtpmap:98 AMR |
| c= | IN IP6 0:0:0:1F |
| a= | label bb |
| a | recvonly |
| a= | mbms-mode:broadcast-mbsfn 24 |

NOTE: For GPRS Systems, the mbms-mode parameter will be set to “broadcast 24 1”

7. SIP UPDATE request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A routes the SIP UPDATE request to PoC Server A.

Contents contained in the SIP UPDATE request is the same as in step 6.

8. SIP 200 "OK" response (from PoC Server A to SIP/IP Core A)

The PoC Server A (participating) sends a SIP 200 "OK" response message to SIP/IP Core A.

| SIP HEADERS |  |
| --- | --- |
| Require: | timer |
| Server: | PoC-serv/OMA2.1 |
| Session-Expires: | 1800;refresher=uac |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerA.networkA.net;session= prearranged;gr >;+g.poc.talkburst;+g.poc.multicast;isfocus |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER, |
| SDP PARAMETERS |  |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:premium |
| m= | audio 2642 RTP/AVP 95 |
| a= | rtpmap: 95 AMR |
| i= | speech |
| a= | label:aa |
| a= | recvonly |
| m= | application 2643 udp TBCP |
| a= | fmtp:TBCP multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | audio 1234 RTP/AVP 98 |
| a= | rtpmap:98 AMR |
| c= | IN IP6 0:0:0:1F |
| a= | label bb |
| a | sendonly |
| a= | mbms-mode:broadcast-mbsfn 24 |

NOTE: For GPRS systems, the mbms-mode parameter will be set to “broadcast 24 1”

9. SIP 200 "OK" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A routes the SIP 200 "OK" response to PoC Client A.

Contents contained in the SIP 200 "OK" response is the same as in step 8.