Change Request

| Title: | Solving Kodiak CONR Comments against OMA-PCPS-TS-Control_Plane-V1_0-20141008-D | Public OMA Confidential |
| --- | --- | --- |
| To: | PCPS WG |  |
| Doc to Change: | OMA-PCPS-TS-Control_Plane-V1_0-20141008-D |  |
| Submission Date: | 05 Nov 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Tom Hiller, Kodiak Networks, thiller@kodiaknetworks.com |  |
| Replaces: | n/a |  |

- Reason for Change

Solve Kodiak Network comments contained in “OMA-COM-PCPS-2014-0301_INP_-CONR_Input_Kodiak” as applies to OMA-PCPS-TS-Control_Plane-V1_0-20141008-D.

R01 resulted in some comments below being solved in other CRs and/or in general on-line editing during the Shanghai meeting.  See the entire CONR report for details.

| I060 | 2014.11.03 | T | General | Source: Kodiak Networks I001-KOD Form: OMA-PCPS-TS-Control_Plane-V1_0-20141008-D Comment: Normative references must point to PCPS 1.0, which means that PCPS 1.0 references are the default. PoC 2.1 references become informative. Inside table of references: PoC 2.1 references = informative [OMA-PoC-CP] → informative [OMA-PoC-UP] → informative PCPS references = normative Create PCPS references ... [OMA-PCPS-CP] ↔ OMA-PCPS-TS-Control_Plane-V1_0 [OMA-PCPS-UP] ↔ OMA-PCPS-TS-User_Plane-V1_0 [OMA-PCPS-V1.0-Document-Mgmt] ↔ OMA-TS-PoC_Document_Management-V2_1 and so forth PoC 2.1 XML schemas → PCPS 1.0 XML schemas. Outside table of references: Default PoC 2.1 references change [OMA-PoC-CP] → [OMA-PCPS-CP] [OMA-PoC-UP] → [OMA-PCPS-UP] and so forth. See next comment regards edits to XML schemas Proposed Change: See associated OMC-PCPS-CR-TS-Control_Plane-Kodiak with edits to CP | Status: OPEN |
| --- | --- | --- | --- | --- | --- |
| I061 | 2014.11.03 | T | 3 | Source: Kodiak Networks I002-KOD Form: OMA-PCPS-TS-Control_Plane-V1_0-20141008-D Comment: Add PCPS to abbreviations. Proposed Change: See associated OMC-PCPS-CR-TS-Control_Plane-Kodiak with edits to CP | Status: OPEN |
| I062 | 2014.11.03 | T | 4 | Source: Kodiak Networks I003-KOD Form: OMA-PCPS-TS-Control_Plane-V1_0-20141008-D Comment: Identify that the PCPS 1.0, which is based on PoC 2.1, is a distinct enabler with distinct XML schemas. Add explanation that PCPS version 1.0 is an update of PoC version 2.1. Outline that terms like PoC Session is retained. Version history for 1.0, 2.0, and 2.1 still exist. Add a new section to introduce PCPS version 1.0. Proposed Change: See associated OMC-PCPS-CR-TS-Control_Plane-Kodiak with edits to CP | Status: OPEN |
| I063 | 2014.11.03 | T | 6.1.a | Source: Kodiak Networks I004-KOD Form: OMA-PCPS-TS-Control_Plane-V1_0-20141008-D Comment: “6.1a Backward compatibility” Edit to reflect this specification is PCPS version 1.0 and to use the procedures of PCPS, in particular, contained in this (i.e., PCPS Control Plane) specification. Proposed Change: See associated OMC-PCPS-CR-TS-Control_Plane-Kodiak with edits to CP | Status: OPEN |
| I064 | 2014.11.03 | T | Appendix XXXXX | Source: Kodiak Networks I005-KOD Form: OMA-PCPS-TS-Control_Plane-V1_0-20141008-D Comment: User-agent and Server headers in annex titled “XXXX Release version in User-agent and Server headers need to reflect this is PCPS 1.0, not PoC 2.1 or previous version of PoC. poc-product-version can have value OMA_PCPS_1.0 value. Next update the example in this annex. Proposed Change: See associated OMC-PCPS-CR-TS-Control_Plane-Kodiak with edits to CP | Status: OPEN |
| I065 | 2014.11.03 | T | EEEEEE 1.1.2.2 | Source: Kodiak Networks I006-KOD Form: OMA-PCPS-TS-Control_Plane-V1_0-20141008-D Comment: Create new section for “XML Schema extension for PCPS 1.0”. This amounts to a third section, i.e., following “E.1.1.2.1 XML Schema extension for PoC V2.0” and “E.1.1.2.2 XML Schema extension for PoC V2.1”. Proposed Change: See associated OMC-PCPS-CR-TS-Control_Plane-Kodiak with edits to CP | Status: OPEN |
| I066 | 2014.11.03 | T | EEEEEE.1.1.4 | Source: Kodiak Networks I007-KOD Form: OMA-PCPS-TS-Control_Plane-V1_0-20141008-D Comment: The annex mentions PoC 2.0 and PoC 2.1. Add mention of PCPS 1.0, too. Proposed Change: See associated OMC-PCPS-CR-TS-Control_Plane-Kodiak with edits to CP | Status: OPEN |

- Impact on Backward Compatibility

None identified

- Impact on Other Specifications

None identified

- Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

- Recommendation

Accept edits below

- Detailed Change Proposal

R01:  Changes below that are struck through are solved in other contributions or during on-line editing of the Control Plane itself. The remaining comments herein addressed XML schema related revisions. In those remaining revisions below there were minor wording changes to harmonize with general wording being used throughout.

The following edits are also outlined in the CONR file from Kodiak (“OMA-COM-PCPS-2014-0301_INP_-CONR_Input_Kodiak”) comments I001 through I007, applicable to “OMA-PCPS-TS-Control_Plane-V1_0-20141008-D”.

The following summarizes the changes below.

Change 5: Revise “Extensions to PoC-Settings” annex

Change 6: Revise “Release Version in User-agent and Server headers” to reflect PCP 1.0

Extensions to PoC-Settings

The text shows the annex as starting at “EEEEEE”, but for ease of formatting in CR, the author uses a single 'E'.

Appendix DDDDDD.   XML Schema Extensions

Appendix EEEEEE.    Extensions to PoC-Settings

      - Structure

The general PoC Service Settings structure is described in [RFC4354] with the PoC and PCPS  specific extensions described in this subclause.

The following elements and attributes extending the <entity> child element of the <poc-settings> element are specified by PoC 2.0 Enabler:

- a <ipii-settings> element containing the Invited Parties Identity Information Mode setting;

- a <inc-media-settings> element containing the Included Media Content in a Request Support setting;

- a <ref-media-settings> element containing the Referenced Media Content in a Request Support setting;

- a <text-content-settings> element containing the Text Content in a Request Support setting;

- a <pocbox-settings> element containing the PoC Box use setting;

- a <privacy-settings> element containing the Privacy setting;

The following elements and attributes extending the <entity> child element of the <poc-settings> element are specified by PCPC 1.0 Enabler:

- a <icsb-user-settings> element containing the Incoming Condition Based PoC Session Barring User setting;

- a <ocsb-user-settings> element containing the Outgoing Condition Based PoC Session Barring User setting;

- a <imcb--settings> element containing the Incoming Media Content Barring setting; and,

- a <imsb--settings> element containing the Incoming Media Stream Barring setting.

The following elements and attributes of the <ipii-settings> element are used by PoC Enabler:

- the <invited-party-identity-information> element.

The following elements and attributes of the <inc-media-settings> element are used by PoC Enabler:

- the <included-media-support> element.

The following elements and attributes of the <ref-media-settings> element are used by PoC Enabler:

- the <referenced-media-support> element.

The following elements and attributes of the <text-content-settings> element are used by PoC Enabler:

- the <text-content-support> element.

The following elements and attributes of the <pocbox-settings> element are used by PoC Enabler:

- the <pocbox-use> element.

The following elements and attributes of the <privacy-settings> element are used by PoC Enabler:

- the <privacy> element.

The following elements and attributes of the <icsb-user-settings> element are used by PoC Enabler:

- the <conditions-profile> element.

The following elements and attributes of the <ocsb-user-settings> element are used by PoC Enabler:

- the <conditions-profile> element.

The following elements and attributes of the <invited-party-identity-information> element are used by PoC Enabler:

- the "active" attribute.

The following elements and attributes of the <included-media-support> element are used by PoC Enabler:

- the "active" attribute.

The following elements and attributes of the <referenced-media-support> element are used by PoC Enabler:

- the "active" attribute.

The following elements and attributes of the <text-content-support> element are used by PoC Enabler:

- the "active" attribute.

The following elements and attributes of the <icsb-user-settings> element are used by PoC Enabler:

- the <conditions-profile> element.

The following elements and attributes of the <ocsb-user-settings> element are used by PoC Enabler:

- the <conditions-profile> element.

The following elements and attributes of the <incoming-media-content-barring> element are used by PoC Enabler:

- the "active" attribute.

The following elements and attributes of the <incoming-media-stream-barring> element are used by PoC Enabler:

- the "active" attribute.

      - XML Schema

##### XML Schema extension for PoC V2.0

This XML Schema uses the extendable space for other PoC Service Settings in [RFC4354] to define the PoC V2.0 specific Service Settings.

The following XML namespace prefix SHALL be used, when generating PoC Service Settings document:

xmlns:PoC2Set="urn:oma:xml:pcps:poc2.0-settings"

The Extended Service Settings SHALL conform to the XML schema described in [OMA-PCPS-POC2.0-SETTINGS].

##### XML Schema extension for PoC V2.1

This XML Schema uses the extendable space for other PoC Service Settings in [RFC4354] to define the PoC V2.1 specific Service Settings.

The following XML namespace prefix SHALL be used, when generating PoC Service Settings document:

xmlns:PoC2_1Set="urn:oma:xml:pcps:poc2.1-settings"

The Extended Service Settings SHALL conform to the XML schema described in[OMA-PCPS-POC2.1-SETTINGS].

##### XML Schema extension for PCPS 1.0

This XML Schema uses the extendable space for other PoC Service Settings in [RFC4354] to define the PCPS 1.0 specific Service Settings.

The following XML namespace prefix SHALL be used, when generating PoC Service Settings document:

xmlns:PCPS1_0Set ="urn:oma:xml:pcps:poc2.1-settings"

The Extended Service Settings SHALL conform to the XML schema described in [OMA-PCPS-POC2.1-SETTINGS].

      - Validation Constraints

None.

      - Data Semantics

The data semantics of the PoC and PCPS specific extensions are specified in this subclause.

The <invited-party-identity-information> element is used to indicate whether the PoC Client supports the Invited Parties Identity Information Mode. The value for the "active" attribute is of Boolean type. The possible values are:

"false"	instructs the Home PoC Server to not deliver the invited parties identity information to the PoC Client for the Served PoC User. This is the default value in absence of the element.

"true"	instructs the Home PoC Server to deliver the invited parties identity information to the PoC Client for the Served PoC User.

The <included-media-support> element is used to indicate whether the PoC Client supports Media Content included in a request. The value for the "active" attribute is of Boolean type. The possible values are:

"false"	instructs the Home PoC Server to remove any Media Content included in the request before forwarding the request to the PoC Client for the Served PoC User. This is the default value in absence of the element.

"true"	instructs the Home PoC Server to forward any Media Content included in the request to the PoC Client for the Served PoC User.

The <referenced-media-support> element is used to indicate whether the PoC Client supports Referenced Media Content included in a request. The value for the "active" attribute is of Boolean type. The possible values are:

"false"	instructs the Home PoC Server to remove any reference to Media Content included in the request before forwarding the request to the PoC Client for the Served PoC User. This is the default value in absence of the element.

"true"	instructs the Home PoC Server to forward any reference to Media Content included in the request to the PoC Client for the Served PoC User.

The <text-content-support> element is used to indicate whether the PoC Client supports Text Content included in a request. The value for the "active" attribute is of Boolean type. The possible values are:

"false"	instructs the Home PoC Server to remove any Text Content included in the request before forwarding the request to the PoC Client for the Served PoC User. This is the default value in absence of the element.

"true"	instructs the Home PoC Server to forward any Text Content included in the request to the PoC Client for the Served PoC User.

The <pocbox-use> element is used to indicate whether the PoC Client wants to route the request to a PoC Box or whether the routing to a PoC Box is subject to conditions. The values are of enumerated string type. The possible values are:

"unwilling"	instructs the Home PoC Server to not route the request to the UE PoC Box or NW PoC Box of the Served PoC User. This is the default value in absence of the element.

"unconditional"	instructs the Home PoC Server to route the request to the UE PoC Box or NW PoC Box of the Served PoC User based on the PoC Box access rules for the default PoC Box, i.e. the  <pocbox-type> element, as specified in [OMA-PCPS-Document-Mgmt] unconditionally.

"conditional"	instructs the Home PoC Server to route the request to the UE PoC Box or NW PoC Box of the Served PoC User based on the PoC Box access rules for the default PoC Box , i.e. the  <pocbox-type> element, as specified in [OMA-PCPS-Document-Mgmt] only conditionally.

The <privacy> element is used to indicate whether the PoC Client wants to be anonymous or not. The values are of enumerated string type. The possible values are:

"none"	instructs the Home PoC Server to allow the identity of the PoC Client for the Served PoC User in responses to requests by including the value in the Privacy header. This is the default value in absence of the element.

"id"	instructs the Home PoC Server to not reveal the identity of the PoC Client for the Served PoC User in responses to requests by including the value in the Privacy header.

The <conditions-profile> element is used to indicate the conditions profile to be activated for ICSB or OCSB. The values are of enumerated string type. The possible values are:

"none"	instructs the Home PoC Server not to conditionally bar incoming or outgoing invitations based on conditions defined by the PoC User.

"private"	instructs the Home PoC Server to conditionally bar incoming or outgoing invitations based on  ICSB or OCSB  conditions defined for  the PoC User and having the <sphere> element value “private”.

"business"	instructs the Home PoC Server to conditionally bar incoming or outgoing invitations based on  ICSB or OCSB  conditions defined for the PoC User and having the <sphere> element value  “business”.

The <incoming-media-content-barring> element is used to indicate whether the Incoming Media Content Barring is active for Media Content included in a request. The value for the "active" attribute is of Boolean type. The possible values are:

"false"	instructs the Home PoC Server to not remove Media Content included in the request according to Served PoC User’s access rules before forwarding the request to the PoC Client for the Served PoC User. This is the default value in absence of the element.

"true"	instructs the Home PoC Server to remove Media Content included in the request according to Served PoC User’s access rules before forwarding the request to the PoC Client for the Served PoC User..

The <incoming-media-stream-barring> element is used to indicate whether the Incoming Media Stream Barring is active for Media Streams included in a request. The value for the "active" attribute is of Boolean type. The possible values are:

"false"	instructs the Home PoC Server to not remove Media Streams included in the request according to Served PoC User’s access rules before forwarding the request to the PoC Client for the Served PoC User. This is the default value in absence of the element.

"true"	instructs the Home PoC Server to remove Media Streams included in the request according to Served PoC User’s access rules before forwarding the request to the PoC Client for the Served PoC User.

      - XML example of PoC Service Settings

The following is an example of a PoC-settings document involving PoC 2.0 and PCPS 1.0

<?xml version="1.0" encoding="UTF-8"?>

<poc-settings xmlns="urn:oma:params:xml:ns:pcps:poc-settings" 	xmlns:PoC2Set="urn:oma:xml:pcps:poc2.0-settings">

xmlns:PCPS1_0Set="urn:oma:xml:pcps:poc2.1-settings">

<entity id="do39s8zksn2d98x">

<isb-settings>

<incoming-session-barring active="true"/>

</isb-settings>

<am-settings>

<answer-mode>automatic</answer-mode>

</am-settings>

<ipab-settings>

<incoming-personal-alert-barring active="false"/>

</ipab-settings>

<sss-settings>

<simultaneous-sessions-support active="true"/>

</sss-settings>

<PoC2Set:ipii-settings>

<PoC2Set:invited-party-identity-information active="true"/>

</PoC2Set:ipii-settings>

<PoC2Set:inc-media-settings>

<PoC2Set:included-media-support active="true"/>

</PoC2Set:inc-media-settings>

<PoC2Set:ref-media-settings>

<referenced-media-support active="true"/>

</PoC2Set:ref-media-settings>

<PoC2Set:text-content-settings>

<PoC2Set:text-content-support active="true"/>

</PoC2Set:text-content-settings>

<PoC2Set:pocbox-settings>

<PoC2Set:pocbox-use>conditional</PoC2Set:pocbox-use>

</PoC2Set:pocbox-settings>

<PoC2Set:privacy-settings>

<PoC2Set:privacy>none</PoC2Set:privacy>

</PoC2Set:privacy-settings>

<PCPS1_0Set:icsb-user-settings>

<PCPS1_0Set:conditions-profile>private</PCPS1_0Set:conditions-profile>

</PCPS1_0Set:icsb-user-settings>

<PCPS1_0Set:ocsb-user-settings>

<PCPS1_0Set:conditions-profile>business</PCPS1_0Set:conditions-profile>

</PCPS1_0Set:ocsb-user-settings>

<PCPS1_0Set:imcb-settings>

<PCPS1_0Set:incoming-media-content-barring active="true"/>

</PCPS1_0Set:imcb-settings>

<PCPS1_0Set:imsb-settings>

<PCPS1_0Set:incoming-media-stream-barring active="true"/>

</PCPS1_0Set:imcb-settings>

</entity>

</poc-settings>

Revision for PCPS 1.0 of User-agent and Server headers

Appendix WWWWW. General

Appendix XXXXX. Release version in User-agent and Server headers

User-Agent and Server headers are used to indicate the release version and product information of the PoC Clients, PoC Servers, PoC Boxes and PoC Crisis Event Handling Entities.

The PoC Client, the PoC Server, the PoC Box and the PoC Crisis Event Handling Entity shall implement the User-Agent and Server headers, according to rules and procedures of [RFC3261] with the clarifications in this subclause specific for PoC.

The User-Agent and Server headers ABNF are specified in [RFC3261] and extended as follows:

Server = "Server" HCOLON server-val *(LWS server-val)

User-Agent =  "User-Agent" HCOLON server-val *(LWS server-val)

server-val =  product / comment

product  =  poc-product / token [SLASH product-version]

product-version = token

This specification allows having several server-val tags. The first of those server-val tags shall be encoding according to the following ABNF:

poc-product = "PoC-" poc-device-token (SLASH poc-product-version)

poc-device-token = "client" | "serv" | "nwbox" | "uebox" |  "crisisentity" token

poc-product-version = "OMA2.1" | “OMA_PCPS_1.0”

Where

client = PoC Client

serv = PoC Server

nwbox = NW PoC Box

uebox = UE PoC Box

crisisentity = PoC Crisis Event Handling Entity

poc-product-version = OMA PoC release version

Example 1:

In this example PoC Client acting as UAC and the PoC Server acting as UAS are OMA PCPS release version 1.0 products. The PoC Client has inserted its own company and product name and version "Acme-Talk5000/v1.01".

User-Agent: PoC-client/OMA_PCPS_1.0  Acme-Talk5000/v1.01

Server: PoC-serv/OMA_PCPS_1.0

Example 2:

In this example both the PoC Server acting as UAC and the PoC Client acting as UAS are OMA PCPS release version 1.0 products.

User-Agent: PoC-serv/OMA_PCPS_1.0

Server: PoC-client/OMA_PCPS_1.0

Example 3:

In this example both the PoC Server acting as a UAC and the UE PoC Box acting as UAS are OMA PCPS release version 1.0 products.

User-Agent: PoC-serv/OMA_PCPS_1.0

Server: PoC-uebox/OMA_PCPS_1.0

Example 4:

In this example the PoC Server acting as a UAC and the PoC Crisis Event Handling Entity is acting as UAS are OMA PCPS release version 1.0  products.

User-Agent: PoC-serv/OMA_PCPS_1.0

Server: PoC-crisisentity/OMA_PCPS_1.0