Change Request

| Title: | Lightweight M2M Use Cases | Public OMA Confidential |
| --- | --- | --- |
| To: | DM-WG |  |
| Doc to Change: | OMA-RD-LightweightM2M-V1_0_20110818-D.doc |  |
| Submission Date: | 10 Sept 2011 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Kenji Hori, KDDI, hori@kddilabs,jp; Ming Lai, Telcordia, mlai@telcordia.com; Friedhelm Rodermund, Vodafone, Friedhelm.Rodermund@vodafone.com; Yuichi Ishikawa, KDDI, yi-ishikawa@kddi.com |  |
| Replaces: | n/a |  |

Reason for Change

Lightweight M2M should have use cases on automatic provisioning and registration.

This CR proposes the automatic provisioning and registration use cases with derived requirements.

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG member should agree on this CR.

Detailed Change Proposal

6.1  High-Level Functional Requirements  (new text)

# Requirements	(Normative)

## High-Level Functional Requirements

6.1 High-Level Functional Requirements

| Label | Description | Release |
| --- | --- | --- |
| LightweightM2M-HLF-00X1 | The lightweight M2M enabler SHALL support initial provisioning of M2M devices. | 1.0 |
| LightweightM2M-HLF-00X2 | The initial provisioning function for M2M devices SHALL support automatic configuration of the M2M devices required for the management by the M2M Server and possibly M2M Gateway. | 1.0 |
| LightweightM2M-HLF-00X3 | The initial provisioning function for M2M devices SHALL support the M2M devices with Wide Area Network (WAN) interfaces as well as those without WAN interfaces. | 1.0 |
| LightweightM2M-HLF-00X4 | The lightweight M2M enabler SHALL support automatic registration of M2M devices. | 1.0 |

Appendix B. Use Cases (new text)

Use Cases	(Informative)

Basic use cases are introduced here to clarify the functional requirements for Lightweight M2M.

Initial Provisioning of M2M Devices (new section)

Initial Provisioning of M2M Devices

Short Description

A subscriber wants to have his M2M device (IP or non-IP) managed by a communication service operator. The subscriber creates an account with a selected operator for the M2M device, and registers the M2M device with the operator’s M2M Server and possibly M2M Gateway. The M2M device is then configured for management by the M2M Server and possibly M2M Gateway.

An IP M2M device has the IP communication layer and an IP address for addressing while a non-IP M2M device uses a non-IP communication mechanism (e.g., SMS, Circuit Switched Data).  Examples of  IP M2M devices include smart meters, surveillance cameras, cleaning robots, smart appliances connected with a IP-based wide area network (WAN, e.g., 3G/4G cellular) or local area network (LAN, e.g., WiFi).  Examples of non-IP devices include bio-medical sensors, telematics sensors, utility meters connected with a non-IP-based WAN (e.g., 2G/2.5G cellular) or LAN (e.g., Zigbee, ANT+). When an M2M device has only the LAN interface, an M2M Gateway is needed to connect the M2M device with the M2M Server in operator’s WAN.

Market Benefits

The subscriber user may purchase the M2M device from open retail channel, or an M2M service operator, or a communication service provider. The user may choose one communication service operator as the device’s Management Authority after the purchase of the device.

Automatic  Registration of M2M Devices

Automatic Registration of M2M Devices

Short Description

Once the subscriber initiates the registration of a M2M device (fixed, nomadic, or mobile), the M2M Server, with possible involvement of the M2M Gateway, will complete the registration process automatically with the service provider without manual steps by the subscriber or the operator staff.

Market Benefits

Given the possible large number of M2M devices, which may have no or limited manual input mechanism, automatic registration of M2M devices fulfil the initial device configuration and association with network correctly and reduces the customer service costs.