# Change Request

| Title: | Message Structure | Public OMA Confidential |
| --- | --- | --- |
| To: | DM WG |  |
| Doc to Change: | OMA-DM-LightweightM2M-TS-V1_0-20120904-D.doc |  |
| Submission Date: | 10 Sept. 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Yawen Niu, CMCC, niuyawen@chinamobile.com |  |
| Replaces: | n/a |  |

Reason for Change

Message structure is the basic and important part for lightweightM2M protocol. The uniform message structure can decrease the complexity for terminal and platform data processing.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and agree this CR

Detailed Change Proposal

- 5.4 Message frame format

Message should be composed of message head and message body. Message head is the Mandatory public part for each message. It contains the basic message information, includes the message Total Length, Command Code, Sequence Number, Resend Number, Version, Reserved word, and Terminal Serial Number.

| Total Length | Command Code | Sequence_Number | Resend Number | Version | Reserved | Terminal Serial Number |
| --- | --- | --- | --- | --- | --- | --- |

The detailed explaination for each word of message head is below:

Total Length: The total data length (unit: bytes) for each message.

Command Code: What action the message is to execute.

Sequence Number. Sequence number of the message. It is used to identify each message, and match the require-response message pair. The rule of the generation of Sequence Number is:

- The LWM2M Device and the M2M Service Provider maintain the Sequence Number separately. The response-request pair should use the same Sequence Number for matching.

- During the same connection, Sequence Number should be increased for each message. When the Sequence Number reach the max, it should return to the start.

- For the resend message, the Sequence Number must NOT be increased.

Resend Number. The number to identify how many times the message resends. It should be set to 0 for the first time to send, and be increased when resend. When Resend Number exceed the max number, the message transmit for this time failed.

Version: Version of the lightweightM2M protocol.

Reserved: Reserved word for extension.

Terminal Serial Number: The unique identification for each terminal.

Message body is composed of the content part and the abstract part. The content part contains the interactive data, and the abstract part is to validate the message integrity and legitimacy.