# Change Request

| Title: | Transfer Protocol | Public OMA Confidential |
| --- | --- | --- |
| To: | DM WG |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0_0-20121030-D.doc |  |
| Submission Date: | 9 Nov. 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Zach Shelby, Sensinode, zach@sensinode.com Seongyoon Kim, LGE, seongyoon.kim@lge.com Yawen Niu, CMCC, niuyawen@chinamobile.com |  |
| Replaces: | n/a |  |

Reason for Change

This contribution adds a simple transfer protocol binding to IETF CoAP for binary messages and to IETF DTLS for security.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to discuss and accept this CR

Detailed Change Proposal

## Normative References

| [CoAP] | Shelby, Z., Hartke, K., Bormann, C., and B. Frank, "Constrained Application Protocol (CoAP)", draft-ietf-core-coap-12 (work in progress), Sept 2012. |
| --- | --- |
| [OBSERVE] | # Hartke, K. “Observing Resources in CoAP”, draft-ietf-core-observe-07 (work in progress), Sept 2012. |
| [RFC6690] | Shelby, Z. “Constrained RESTful Environments (CoRE) Link Format”, RFC6690, Aug 2012. |
| [RFC6347] | Rescorla, E. and N. Modadugu, "Datagram Transport Layer Security Version 1.2", RFC 6347, January 2012. |
| [RFC6655] | McGrew, D. and D. Bailey, "AES-CCM Cipher Suites for TLS", RFC6655, July 2012. |

# Transport Layer Binding and Encoding

The LWM2M interfaces use the IETF Constrained Application Protocol [CoAP] as an underlying transfer protocol across IP and SMS bearers. This protocol defines the message header, request/response codes, message options and retransmission mechanisms. This section defines the subset of features from the IETF CoAP specification to be used by LWM2M interfaces.

## Required Features

For realization of the LWM2M interfaces, only the basic binary CoAP message header, and a small subset of options are required. This section explicitly defines the features of the CoAP standard that are required for LWM2M.

- The 4-byte binary CoAP message header is defined in Section 3 of [CoAP]. This same base message is used for Request and Response interactions.

- Confirmable, Acknowledgement and Reset messages MUST be supported. The Reset message is used as a message layer error message in response to a malformed Confirmable message.

- GET, PUT, POST and DELETE methods MUST be supported. LWM2M Operations map to these methods.

- A subset of Response Codes MUST be supported needed for LWM2M response message mapping.

- The Uri-Path Option MUST be supported to indicate the identifier of the interface, Object and Resource being requested.

- The Location-Path Option MUST be supported to indicate the handle of a registration for future update and delete operations.

- The Uri-Query Option MUST be supported.

- The Content-Type Option MAY be used to indicate the media type of the payload. A default value of plain/text is assumed, allowing this option to be elided for most payloads.

- The Token Option MAY be used to enable multiple requests in parallel with an endpoint, and MUST be supported for the Information Reporting interface.

## URI Identifier & Operation Mapping

Although CoAP supports a URI in requests, it is not used in the same way as in HTTP. The URI in CoAP is broken down into binary parts, minimizing overhead and complexity. In LWM2M only path segment and query string URI components are needed. The URI path is used to simply identify the interface, Object or Resource that the request is for, and is encoded in Uri-Path options. The LWM2M Registration interface also makes use of query string parameters to pass on meta-data with the request separately from the payload. Each query parameter is encoded in a Uri-Query Option. Likewise, the LWM2M Operations for each interface are mapped to CoAP Methods.

### Registration Interface

The registration interface is used by a LWM2M Client to register with a LWM2M Server, identified by the LWM2M Server URI. Registration is performed by sending a CoAP POST to the LWM2M Server URI, with registration parameters passed as query string parameters as per Table 1 and supported Objects included in the payload as per [RFC6690]. The response includes Location-Path Options, which indicate the path to use for updating or deleting the registration.

Registration update is performed by sending a CoAP PUT to the Location path returned to the LWM2M Client as a result of a successful registration.

De-registration is performed by sending a CoAP DELETE to the Location path returned to the LWM2M Client as a result of a successful registration.

| Logical Operation | CoAP Method | URI | Success | Failure |
| --- | --- | --- | --- | --- |
| Write Uplink | POST | /rd?ep={Endpoint Client Name}&lt={Lifetime}&sms={MSISDN} | 2.01 Created | 4.00 Bad Request |
| Update Uplink | PUT | /{location} | 2.04 Changed | 4.00 Bad Request |
| Delete Uplink | DELETE | /{location} | 2.02 Deleted | 4.00 Bad Request |

Table 2: Operation to Method and URI Mapping

Figure 1: Example registration, update and de-registration exchanges (shorthand in [CoAP] example style, actual messages using CoAP binary headers)

### Bootstrap Interface

The bootstrap interface is used to optionally configure a LWM2M Client so that it can successfully register with a LWM2M Server. The client bootstrap operation is performed by sending a CoAP GET request to the LWM2M Server at the /bs path including the Endpoint Client Name as a query string parameter. The response will include the needed resources of the Bootstrap Object for that Client in TLV format.

Editor’s Note: The re-bootstrap interface needs definition.

| Logical Operation | CoAP Method | URI | Success | Failure |
| --- | --- | --- | --- | --- |
| Read Uplink | GET | /bs?ep={Endpoint Client Name} | 2.05 Content | 4.00 Bad Request |

Table 4: Operation to Method and URI Mapping

Figure 2: Example of a client bootstrap exchange.

### Information & Management Interface

The Information & Management Interface is used to access an Object or an individual Resource of an Object. An Object is identified by the path /{Object ID} and a resource by the path /{Object ID}/{Resource ID}.

An Object or Resource is Read by sending a CoAP GET to the corresponding path. The response includes the value in the corresponding Plain Text, TLV or JSON format.

An Object or Resource is Written to by sending a CoAP PUT to the corresponding path. The request includes the value to be written in the corresponding Plain Text, TLV or JSON format.

An Object or Resource is Executed by sending a CoAP POST to the corresponding path. The request MAY include a payload if so specified by the Resource.

| Logical Operation | CoAP Method | Path | Success | Failure |
| --- | --- | --- | --- | --- |
| Read Downlink | GET | /{Object ID}/{Resource ID} | 2.05 Content | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Write Downlink | PUT | /{Object ID}/{Resource ID} | 2.04 Changed | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Execute Downlink | POST | /{Object ID}/{Resource ID} | 2.04 Changed | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |

Table 3: Registration Interface URI

Figure 3: Example of Information & Management interface exchanges.

### Information Reporting Interface

Periodic and event-triggered reporting about resource values from the LWM2M Client to the LWM2M Server is achieved through CoAP Observation [OBSERVE]. This simple mechanism allows the LWM2M Server to send an Observe GET request for an object, which results in asynchronous notifications whenever that object changes (periodically or as a result of an event). The minimum and maximum period of notifications can be controlled by including the minimum (pmin) and/or maximum (pmax) period for notifications to be sent in seconds.

| Interface | CoAP Method | Path | Success | Failure |
| --- | --- | --- | --- | --- |
| Read Downlink | GET with Observe option | /{Object ID}/{Resource ID}?pmin={minimum period}&pmax={maximum period} | 2.05 Content with Observe option | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Write Uplink | Asynchronous Response |  | 2.04 Changed |  |

Table 7: Operation to Method Mapping

Figure 4: Example of an Information Reporting exchange.

## Transport Bindings

### UDP Binding

The CoAP binding for UDP is defined in [CoAP]. The protocol has a IANA registered scheme of coap:// and a default port of 5683. The UDP binding is used in NoSec (no security) mode. Reliability over the UDP transport is provided by the built-in retransmission mechanism of CoAP.

### DTLS Binding

The DTLS binding for CoAP is defined in Section 9 of [CoAP]. Datagram Transport Layer Security (DTLS) [RFC6347] is a long-lived session based security solution for UDP. It provides a secure handshake with session key generation, mutual authentication, data integrity and confidentiality.  DTLS is also used for authorization on individual CoAP resources. A LWM2M implementation MUST support the Pre-Shared Key mode of DTLS with Cipher TLS_PSK_WITH_AES_128_CCM_8 [RFC6655] as defined in Section 9.1.3.1 of [CoAP], and in addition MAY support Raw Public Key or Certificate modes as defined in Section 9.1.3.2 and 9.1.3.3 of [CoAP], respectively.

An LWM2M Client and Server SHOULD keep a DTLS session in use as long as possible (even across sleep cycles).

Editor’s Note: Decide if Raw Public Key would be a more suitable MUST IMPLEMENT security mode for LWM2M.

### SMS Binding

CoAP is used over SMS in this transport binding by placing a CoAP message in the SMS payload using 8-bit encoding. SMS concatenation MAY be used for messages larger than 140 characters. CoAP retransmission is disabled for this binding. An LWM2M Client indicates the use of this binding by including a parameter (sms) in its registration to the LWM2M Server including the node’s MSISDN number. The LWM2M Client MAY interact with the server using both UDP and SMS bindings.