| Lightweight Machine to Machine Technical Specification |  |  |  |
| --- | --- | --- | --- |
| Draft Version 1.0 – 30 Oct 2012 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-TS-LightweightM2M-V1_0-20121030-D |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2012 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

Contents

1.	Scope	5

2.	References	6

2.1	Normative References	6

2.2	Informative References	7

3.	Terminology and Conventions	8

3.1	Conventions	8

3.2	Definitions	8

3.3	Abbreviations	8

4.	Introduction	9

5.	Protocol Overview	10

6.	Interfaces	11

6.1	Device Discovery & Registration Interface	12

6.1.1	Registration	12

6.1.2	Update	12

6.1.3	De-registration	12

6.1.4	Registration Payload	12

6.1.5	Flows	12

6.2	Bootstrap Interface	13

6.2.1	Flows	13

6.3	Management and Service Interface	13

6.3.1	Read	13

6.3.2	Write	13

6.3.3	Execute	13

6.3.4	Flows	13

6.4	Information Reporting Interface	13

6.4.1	Flows	13

7.	Identifiers and Resources	14

7.1	Resource Model	14

7.2	Identifiers	15

7.3	Data Formats for Transferring Resource Information	15

7.3.1	Plain Text	16

7.3.2	Opaque	16

7.3.3	TLV	16

7.3.4	JSON	16

8.	Security Consideration	18

8.1	Channel Security	18

8.2	Access Control	18

8.2.1	Access Control List (ACL)	18

8.2.2	Access Type	18

8.2.3	Authorization of Operations	19

8.2.4	Quering ACL/Access Type	19

9.	Transport Layer Binding and Encoding	20

Appendix A.	Example Objects and Resources	21

Appendix B.	Change History (Informative)	23

B.1	Approved Version History	23

B.2	Draft/Candidate Version <current version> History	23

Appendix C.	Static Conformance Requirements (Normative)	24

C.1	SCR for XYZ Client	24

C.2	SCR for XYZ Server	24

Appendix D.	<Additional Information>	25

D.1	App Headers	25

D.1.1	More Headers	25

Figures

Figure 1: Example Figure	9

Tables

Table 1: Example Table	9

Scope

<< Define as it relates to Open Mobile Alliance Activity.  If it adds clarity, define what is not in the scope.  DELETE THIS COMMENT >>

References

The policy for reference lists is:

1.	OMA documents listed should have at least one approved version – draft-only docs should not be referenced.  Exception exists for documents that will be approved with or after the referenced doc is approved (may be part of same enabler package).  In short – approved docs should not reference unapproved docs.

2.	When a reference is made to an OMA specification, then Open Mobile Alliance with the TM symbol (™) should be used in the description.

3.	The name + version (no date) for OMA specifications are generally sufficient – dates should be used only if there is a specific reason to limit the usage.

4.	For references to WAP Forum docs, dates should not be included as DID's for the old WAP Forum specifications are enough and the reference description should refer to WAP Forum™.

5.	References to other affiliate docs should similarly provide sufficient information to uniquely determine the needed document and should provide the appropriate source information.

6.	The URL for OMA material (new OMA and affiliate) should always be http://www.openmobilealliance.org (an exception is OMNA that is reached through http://www.openmobilealliance.org/tech/omna)

Models to use

[REFLABEL]	<General Model> “Ref Title”, Ref information (source, date, id),
URL:http//<ref-source>/

[OMADOC]	<OMA Model> “OMA Document Title”, Open Mobile Alliance™, OMA-<docname>{-<version>}, URL:http//www.openmobilealliance.org/

If there are no entries in the table – enter ‘none’ to be clear.

DELETE THIS COMMENT

Normative References

| [IOPPROC] | “OMA Interoperability Policy and Process”, Version 1.1, Open Mobile Alliance™, OMA-IOP-Process-V1_1, URL:http://www.openmobilealliance.org/ |
| --- | --- |
| [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |
| [RFC2234] | “Augmented BNF for Syntax Specifications: ABNF”. D. Crocker, Ed., P. Overell. November 1997, URL:http://www.ietf.org/rfc/rfc2234.txt |
| [LWM2M-AD] | “Lightweight Machine to Machine Architecture”, Open Mobile Alliance™, OMA-AD-LightweightM2M-V1_0, URL:http://www.openmobilealliance.org/ |
|  | << Add/Remove reference rows as needed! >> |

Informative References

|  | << Add/Remove reference rows as needed! >> |
| --- | --- |

Terminology and Conventions

Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

.

Definitions

| Term 1 | Definition |
| --- | --- |
| Term 2 | Definition |

Abbreviations

| OMA | Open Mobile Alliance |
| --- | --- |
| xxx | xxx |

Introduction

<< From a market perspective...

What can you do with this specification?

What problem does this solve?

How can this specification be applied?

Consider the target audience and provide deployment examples as possible.

DELETE THIS COMMENT >>

Protocol Overview

Interfaces

According to the architecture diagram [LWM2M-AD], there are four interfaces: 1) Device Discovery and Registration 2) Bootstrap 3) Management and Service 4) Information Reporting. The logical operations for the four interfaces can be classified as uplink operations and downlink operations. There are three types of logical operations in the downlink, including “read”, “write”, “execute”. There is one type of logical operation in the uplink “write”.

Figure 1 shows the logical operation model for interface “Device Discovery and Registration”. For this interface, the operation is uplink “write”. The write operation on this interface consists of three different variations, registration, update, and de-registration.

Figure 1 Device Discovery and Registration

Figure 2 shows the logical operation model for interface “Bootstrap”. For this interface, the operation is downlink “write”. This operation allows the LWM2M Server to set Resources of the bootstrap Object on the LWM2M Client for the purpose of bootstrap configuration.

Figure 2 Bootstrap

Figure 3 shows the logical operation model for interface “Management and Service”. For this interface, the operations are downlink “read”, “write” and “execute”. These operations are used to interact with the Resources and Objects of the LWM2M Client. Read is used to read the current value of one or more Resources, write is used to update the value of one or more Resources, and execute is used to initiate an action defined by a Resource.

Figure 3 Management and Service

Figure 4 shows the logical operation model for interface “Information Reporting”. For this interface, the operation is uplink “write”. This operation sends the LWM2M Server a new value related to a Resource on the LWM2M Client.

Figure 4 Information Reporting

The relationship between logical operations and interfaces is listed in the following table 1.

Table 1 the relationship of logical operations and interfaces

| Interfaces | Direction | Logical Operation |
| --- | --- | --- |
| Device Discovery and Registration | Uplink | Write (Register, Update, De-register) |
| Bootstrap | Downlink | Write |
| Management and Service | Downlink | Read, Write, Execute |
| Information Reporting | Uplink | Write |

Device Discovery & Registration Interface

The Discovery & Registration Interface is used by a LWM2M Client to register with one or more LWM2M Servers, maintain each registration and de-register from a Server. The registration is based on the Resource Model and Identifiers defined in Section 7. When registering, the LWM2M Client indicates its Endpoint Name, registration lifetime and the list of Objects the Client supports. The registration is soft state, with a lifetime indicated by the registration lifetime. The LWM2M Client periodically performs an update and allows the Client to update its registration information. Finally, when shutting down or discontinuing use of a Server, the Client performs a de-registration.

EDITOR NOTE: Remove the heart-beat

EDITOR NOTE: Lifetime should have a default value in the specification, value TBD. Client can set a custom lifetime.

EDITOR NOTE: Explain that the server removes the registration after the lifetime expires (unless there is an update).

Registration

Update

De-registration

Registration Payload

TODO: This section defines the payload format of the registration message, which describes the Objects made available by the Client. This can be done in a simple way using RFC6690 (low overhead, no complex formats like XML).

Flows

TODO: This section includes a message exchange diagram for each flow.

Bootstrap Interface

TODO: Define that use of the Bootstrap interface is optional, as a device may be sufficiently pre-configured.

TODO: Define that this interface makes use of the LWM2M Bootstrap Object, which will be defined in an appendix of this specification.

Flows

EDITOR NOTE: Need to discuss pre-provisioning of the Bootstrap Object or provisioning from a SIM card in this section.

Management and Service Interface

TODO: Define how the operations interact with the Objects and Resources of the Resource Model.

TODO: Define how Objects or Resources with multiple instances can be accessed in one request.

Read

Write

Execute

Flows

Information Reporting Interface

TODO: Define what meta-data needs to be configured on a resouce using the Management and Service Interface, in order for information reporting to succeed.

Flows

Identifiers and Resources

This section defines the identifiers and resource model for the LWM2M Enabler.

Resource Model

The LWM2M Enabler defines a simple resource model where each piece of information made available by the LWM2M Client is a Resource, and Resources are logically organized into Objects. Figure 1 illustrates this structure, and the relationship between Resources, Objects and the LWM2M Client. The LWM2M Client may have any number of Resources, each of which belongs to an Object.

Figure 1 Relationship between LWM2M Client, Object and Resources

Resources are defined per Object, and each resource is given a unique identifier within that Object. Resources are accessed directly by the LWM2M Server. Each Resource is defined to have one or more Operations that it supports. A Resource MAY contain multiple instances of that Resource. Objects and Resources SHOULD have associated Access Control Lists (ACLs) that control what the LWM2M Server can access using what operations. Figure 2 shows which operations the resources support, and how Objects and Resources are associated with ACLs. In the example, Resource 1 supports read, write and execute, while Resource 2 supports only read operations.

Figure 2 Supported operations and access control lists

An Object defines a grouping of Resources, for example the Firmware Update Object would contain all the Resources used for firmware update purposes. Each Object is assigned a unique OMA Management Object ID and corresponding ID Index, which may be used to perform Group Operations on the set of Resources associated with that Object. The LWM2M enabler defines standard Objects and Resources and vendor specific Objects may be added for their own uses.

EDITOR NOTE: Add more clarification about the vendor specific Objects.

Identifiers

Seven identifiers are defined by the LWM2M Enabler: Endpoint Client Name, LWM2M Server URI, Short Server ID, Human Readable Object URN, Object ID, Resource ID and Resource Instance ID. These identifiers are defined in Table 1.

Table 1 Definition of LWM2M terms

| Identifier | Semantics | Description |
| --- | --- | --- |
| Endpoint Client Name | String (max 63 bytes) | Uniquely identifies the LWM2M Client on one LWM2M Server. Provided to the Server during Registration For example the IMEI, serial number or a logical name of the device. |
| LWM2M Server URI | URI | Uniquely identifies the LWM2M Server. Provided to the Client during Bootstrap procedure. |
| Short Server ID | 16-bit unsigned integer | Short integer ID, assigned by the LWM2M Client. This identifier uniquely identifies each LWM2M Server configured for the LWM2M Client. |
| Human Readable Object URN | URN for the OMA Management Object | Assigned by the object specification |
| Object ID | 16-bit unsigned integer | Uniquely identifies the Object in the LWM2M Client. Short integer ID, assigned by OMA |
| Resource ID | 16-bit unsigned integer | Uniquely identifies the Resource in the Object. Short integer ID, assigned by the object specification |
| Resource Instance ID | 16-bit unsigned integer | Uniquely identifies the Resource Instance in the Resource. Short integer ID, assigned by the LWM2M Server/Client |

Editor’s Note: where to store these identifiers is TBD

Data Formats for Transferring Resource Information

Four data formats are defined by the LWM2M Enabler in this section. The LWM2M Server MUST support all data formats. The plain text and opaque formats MUST be supported by both the LWM2M Server and the LWM2M Client. The LWM2M Client MUST support the TLV data format for Object or multiple-instance Resource requests.

The Object specification defines the data format that a Resource supports, either plain text or opque for singular Resources or TLV for multiple instance Resources.

In addition to the data formats defined in the Object specification, a LWM2M Client MAY choose to support the JSON format for Object or multiple instance Resource requests.

Plain Text

The plain text format is used for write and read operations on singular Resources where the value of the resource is simply represented as as a UTF-8 encoded string (similar to the Media Type text/plain). This string can contain a character sequence, integer number, decimal number or any other sequence of valid UTF-8 characters.

For example a request to Resource 0 from Object 3 would return the following plain text payload:

Open Mobile Alliance

This data format has a Media Type of application/lwm2m-plain (TBD).

Opaque

The opque format is used for write and read operations on singular Resources where the value of the resource is an opaque sequence of binary octets (similar to the Media Type application/octet-stream). This data format is used for binary resources such as firmware images or application specific binary formats.

This data format has a Media Type of application/lwm2m-opaque (TBD).

TLV

For requests to Objects or Resources with multiple instances, the binary TLV (type-length-value) format is used to represent an array of values using a company binary representation, which is easy to process on simple embedded devices.

The format is an array of the following byte sequence, where each array entry represents a Resource or Resource Instance:

| Field | Format and Length | Description |
| --- | --- | --- |
| ID | 16-bit unsigned integer | The Resource ID or Resource Instance ID of the array entry |
| Length | 16-bit unsigned integer | The Length of the following field in bytes |
| Value | Sequence of bytes, length indicated by the Length field | Either a plain text or opque value depending on the Resource’s data format |

EDITORS NOTE: Mention that if there is a singular value in the array, that is OK.

This data format has a Media Type of application/lwm2m-tlv.

JSON

For requests to Objects or Resources with multiple instances, a simple JSON format may be used where a set of values is represented. Each entry of the JSON format is a key:value pair, where the key is the Resource ID for requests to an Object or the instance number for requests to a Resource with multiple instances.

This data format has a Media Type of application/lwm2m-json (TBD).

The format when an Object is requested follows the following syntax:

{  "<Resource ID>":"<Value>",

"<Resource ID>":"<Value>",

"<Resource ID>":"<Value>"

}

For example a request to Example Object 3 in the example would return the following JSON payload:

{ "0":"Open Mobile Alliance",

"1":"LWM2M v1.0",

"2":"9347112"

}

The format when a Resource with multiple instances is requested follows the following syntax:

{

"<Resource Instance ID>":"<Value>",

"<Resource Instance ID>":"<Value>",

"<Resource Instance ID>":"<Value>"

}

Security Consideration

Channel Security

Editor’s note: if we decide to use CoAP, we can rely on underlying layer security mechanism so we don’t need to define any mechanisms here. However, the related DTLS binding information and mandatory DTLS modes could be defined here instead of in the protocol binding section.

Access Control

Access Control List (ACL)

To authorize the logical operation sent from the LWM2M Server, the LWM2M Client uses Access Control List (ACL) to determine which the LWM2M Server can do which logical operations on the resources in the LWM2M Client. ACL is assigned per Object and the ACL value is applied to all the Resources and Instances of the Resources which belong to the Object.

ACL is represented in the list of Short Server ID and access right of the corresponding LWM2M Server. Using the Short Server ID instead of the LWM2M Server URI can reduce the space overhead and increase the processing efficiencies. The access right is 1 byte and each bit of the access right represents whether the respective logical operation is authorized or not. For example, if 1st bit of access right is 1, then the LWM2M Server is authorized to read the Object and Resources or Instances of Resources in the Object. Detail information of ACL is described in the table X.

Editor’s Note: Special Short Server ID needs to be defined for wild card

| Field | Format and Length | Description |
| --- | --- | --- |
| ACL | Variable bytes depending on the number of ACL entiries | List of ACL entries |
| ACL entry | 3 bytes | The first 2 bytes: Short Server ID The last 1 byte: access right 1st lsb: Read 2nd lsb: Write 3rd lsb: Execute Other bits are reserved for future use |

Access Type

Access type defines which logical operation the Resource supports. Therefore access type is assigned per the Resource and all the Instances of the Resource inherit the access type. The access type is 1 byte and each bit of the access type represents whether the respective logical operation is supported by the Resource or not. For example, if 1st bit of access type is 1, it means that the Resource supports read logical operation.

Editor’s Note: Access Type is a constant value, normally assigned when the resource is created. How the LWM2M Server learns the access type is TBD (e.g., through the resource discovery).

| Field | Format and Length | Description |
| --- | --- | --- |
| Access Type | 1 byte | 1st lsb: Read 2nd lsb: Write 3rd lsb: Execute Other bits are reserved for future use |

Authorization of Operations

Operations on Resource or Resource Instance

If the LWM2M Server accesses a Resource or Resource Instance(s), the LWM2M Client authorizes the logical operation based on the conjunction of the access right and the access type. The LWM2M Server can execute the logical operation that is permitted by both the access right and the access type. Practically, the permitted logical operations can be computed by the bitwise AND operation of the access right and the access type.

Operations on Object

If the LWM2M Server accesses an Object, the LWM2M Client authorizes the logical operation by performing authorization mechanism to each Resource in the Object according to 8.2.3.1.

Quering ACL/Access Type

Editor’s Note: ACL is exposed to the LWM2M Server through an interface which will be defined later. The LWM2M Server can modify the ACL using this interface.

Editor’s Note: how to query ACL and access type is TBD

Transport Layer Binding and Encoding

Example Objects and Resources

This section lists example Objects and Resources that LWM2M could define. The actual objects will be defined in an object specification.

| Example Object | Example Object ID | Example Resource | Example Resource ID | Logical Operations | Type | Constrain |
| --- | --- | --- | --- | --- | --- | --- |
| Bootstrapping | 1 |  |  |  |  |  |
|  |  | Server Location | 0 | R, W | String |  |
|  |  | Endpoint Name | 1 | R | String |  |
|  |  | Heard Beat | 2 | R, W | Unsigned Integer | 4 bytes. Unit: second. Default value is 30 seconds. |
| Firmware Update | 2 |  |  |  |  |  |
|  |  | Firmware | 0 | R, W | String |  |
|  |  | Firmware Name | 1 | R | String |  |
|  |  | Last Updated | 2 | R | TIME |  |
|  |  | Version | 3 | R, W | String |  |
|  |  | Size | 4 | R, W | Unsigned Integer |  |
|  |  | Do Upgrade | 5 | E | BOOL |  |
| Device Info | 3 |  |  |  |  |  |
|  |  | Manufacturer | 0 |  | String |  |
|  |  | Model Number | 1 |  | Unsigned Integer |  |
|  |  | Serial Number | 2 |  | String |  |
|  |  | Device ID | 3 |  | String |  |
| Location | 4 |  |  |  |  |  |
|  |  | GPS Location | 0 |  | String |  |
|  |  | GPS Fix | 1 |  | String |  |
|  |  | APN Location | 2 |  | String |  |
|  |  | X,Y Location | 3 |  | String |  |
|  |  | Update Period | 4 |  | TIME |  |
| Cellular Network Info | 5 |  |  |  |  |  |
|  |  | Celluar ID | 0 | R | Unsigned Integer | 4 bytes. First2 bytes for LAC, last 2 bytes for CI. |
|  |  | Signal Strength | 1 | R | Integer |  |
| Time | 6 |  |  |  |  |  |
|  |  | time | 0 | R,W | TIME |  |
| Sensor Info | 7 |  |  |  |  |  |
|  |  | type | 0 | R,W | Unsigned Integer | 0: temperature 1: Humidity 2: pressure 3: CO2 4: … |
|  |  | interface | 1 | R,W | Unsigned Integer | 0: RS232 1: RS485 2:… |
| Bearer Info | … |  |  |  |  |  |

Editor’s Note: Add an example about how to access multiple instances of resource.

Editor’s Note: Reserve range for OMA, other SDO, vendors for object ID registration.

Editor’s Note: How to reuse the existing management objects need further study.

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |

Draft/Candidate Version <current version> History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-TS-LightweightM2M-V1_0-20120904-D | 04 Sep 2012 | all | TS baseline agreed as in OMA-DM-LightweightM2M-2012-0078-INP_TS_kick_off |
| Draft Version OMA-TS-LightweightM2M-V1_0-20120918-D | 18 Sep 2003 | 6, 7 | Incorporates input to committee: OMA-DM-LightweightM2M-2012-0083R01-CR_Skeleton_Base_Line OMA-DM-LightweightM2M-2012-0090R02-CR_TS_Resource_Model OMA-DM-LightweightM2M-2012-0061R04-CR_Interfaces |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121024-D | 24 Oct 2012 | 6, 7, Appendix A | OMA-DM-LightweightM2M-2012-0095R01-CR_TS_Interface_and_Resource_Additions |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121030-D | 30 Oct 2012 | 7, 8 | OMA-DM-LightweightM2M-2012-0097R01-CR_Identifiers_and_Security_Considerations |

Static Conformance Requirements	(Normative)

The notation used in this appendix is specified in [IOPPROC].

The following is a model of a set of SCR tables.  DELETE THIS COMMENT

SCR for XYZ Client

| Item | Function | Reference | Status | Requirement |
| --- | --- | --- | --- | --- |
| XYZ-C-001 | Something mandatory | Section x.y | M | (XYZ-C-001 OR XYZ-C-003) AND XYZ-C-002 |
| XYZ-C-002 | Something optional | Section x.y | O |  |
| XYZ-C-003 | Dependencies on ZYX | Section x.y | M | ZYX:MCF |
| XYZ-C-004 | Dependencies on ZYX | Section x.y | O | ZYX:OCF |

SCR for XYZ Server

| Item | Function | Reference | Status | Requirement |
| --- | --- | --- | --- | --- |
| XYZ-S-001 | Something mandatory | Section x.y | M | XYZ-S-001 OR XYZ-S-002 OR XYZ-S-003 |
| XYZ-S-002 | Something optional | Section x.y | O |  |
| XYZ-S-003 | Dependencies on ZYX | Section x.y | M | ZYX:MSF |
| XYZ-S-004 | Dependencies on ZYX | Section x.y | O | ZYX:OSF |

<Additional Information>

If needed, add annex to provide additional information to support the document.  In general, this information should be informative, as normative material should be contained in the primary body of the document.

Note that the styles for the headers in the appendix (App1, App2, App3) are different than the main body.  The use below is intended to validate the styles to be used.  Remove if not needed.

DELETE THIS COMMENT

App Headers

<More text>

More Headers

<More text>

Even More Headers

<More text>