# Change Request

| Title: | Information Management and Reporting Interfaces | Public OMA Confidential |
| --- | --- | --- |
| To: | DM WG |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0_0-20121030-D.doc |  |
| Submission Date: | 9 Nov. 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Zach Shelby, Sensinode, zach@sensinode.com |  |
| Replaces: | n/a |  |

Reason for Change

This contribution completes the Information Management and Reporting Interface section.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to discuss and accept this CR

Detailed Change Proposal

## Management & Service Interface

This interface is used by the LWM2M Server to access Resources available from a LWM2M Client using Read, Write or Execute operations. The operations that a Resource supports are defined in the definition of its Object.

### Read

The Read operation is used to access the value of a Resource, an array of Resource Instances or an entire Object at once. The Read operation has the following parameters:

Table 1 Read parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object to read, if no Resource ID is indicated, then the whole Object is returned. |
| Resource ID | No | - | Indicates the Resource to read. |

### Write

The Write operation is used to change the value of a Resource, an array of Resources Instances or multiple Resources from an Object at once. The Write operation has the following parameters:

Table 2 Write parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object to write, if no Resource ID is indicated, then the included payload is an Object containing the Resource values to be written. |
| Resource ID | No | - | Indicates the Resource to write. The payload is the new Resource or Resource Instances. |
| New Value | Yes | - | The new value included in the payload to update the Object or Resource. |

### Execute

The Execute operation is used to initiate some action, and an associated value MAY be included depending on the Resource definition. Such a value can be used to pass possible parameters needed for the execution task to the Client, e.g. the number of seconds to wait before rebooting on a reboot resource. The presence and format of this Execute Value is defined by the Object specification. The Execute operation can only be performed on individual resources. A LWM2M Client MUST return an error on receiving an operation to execute an Object.

Table 3 Execute parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Resource ID | Yes | - | Indicates the Resource to execute. |
| Execute Value | No | - | The execute value included in the payload to update the Object or Resource (if the Resource supports a value for execution). |

## Information Reporting Interface

This interface is used by a LWM2M Server to observe any changes in a Resource on a LWM2M Client, receiving notifications when new values are available. This observation relationship is initiated by sending an Observe operation to the L2M2M Client for an Object or Resource. This operation MAY contain optional Maximum and/or Minimum Period parameters to control how often these notifications are sent. The Minimum Period has a default value of 1 second, and to limit congestion notifications for an Observation SHOULD NOT be sent faster than this. An observation ends when a Cancel Observation operation is performed or the LWM2M Server is no longer reachable.

### Observe

The Observe operation includes the following parameters:

Table 4 Observe parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object to observe, if no Resource ID is indicated, then the whole Object is observed. |
| Resource ID | No | - | Indicates the Resource to observe. |
| Minimum Period | No | 1 | When present, the minimum period indicates the minimum time in seconds the server SHOULD wait between sending a new notification. In the absence of this parameter, the Minimum Period is defined by the Default Minimum Period set in the LWM2M Server Object related to that Server. |
| Maximum Period | No | - | When present, the maximum period indicated the maximum time in seconds the server SHOULD wait between sending the next notification (regardless if the value has changed). In the absence of this parameter, the maximum period is up to the server. The maximum period MUST be greater than the minimum period parameter. In the absence of this parameter, the Maximum Period is defined by the Default Maximum Period set in the LWM2M Server Object related to that Server. |

The following example shows how the Minimum and Maximum period parameters work as shown in Figure 1. A Server makes an Observation for a temperature resource that is updated inside the Client at irregular periods (based on change). The Server makes an Observation with Minimum Period = 10 Seconds and Maximum Period = 60 Seconds. The Client will wait at least 10 Seconds before sending a Notification to the Server (even if the resource has changed before that), and no longer than 60 Seconds before sending a Notification (even if the resource has not changed yet). The Notification is sent anywhere between 10-60 seconds upon change.

Figure 1 Example of Minimum and Maximum periods in an Observation.

### Notify

The Notify operation is sent to the LWM2M Server during a valid observation on an Object or Resource. This operation includes the new value of the Object or Resource.

Table 5 Notify parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Updated Value | Yes | - | The new value included in the payload about the Object or Resource. |

### Cancel Observation

The Cancel Observation operation is sent to the LWM2M Client to end an observation relationship. The operation includes the following parameters:

Table 5 Cancel Observation parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object to stop observing, if no Resource ID is indicated, then the whole Object is indicated. |
| Resource ID | No | - | Indicates the Resource to stop observing. |