# Change Request

| Title: | Location for Device Object | Public OMA Confidential |
| --- | --- | --- |
| To: | DM WG |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0_0-20130206-D.doc |  |
| Submission Date: | 13 Feb. 2013 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Zach Shelby, Sensinode, zach@sensinode.com |  |
| Replaces: | n/a |  |

Reason for Change

This CR adds support for GPS location to the Device Object

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to discuss and accept this CR

Detailed Change Proposal

LWM2M Object: Device

Description: This LWM2M objects provide a range of device related information which can be queried by the LWM2M server, and a device reboot and factory reset function.

Object info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Device | 3 |  | No |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Manufacturer | 0 | R | No | String |  | - | Manufacturer name |
| Model Number | 1 | R | No | String |  | - | A model identifier (manufacturer specified string) |
| Serial Number | 2 | R | No | String |  | - | Serial Number |
| LWM2M client version Editor’s note: Where to put this Resource is TBD | 5 | R | No | String |  | - | Client version identifier |
| Firmware version | 7 | R | No | String |  | - | Current firmware version |
| Reboot | 8 | E | No | - |  | - | Reboot the device to restore the Device from unexpected firmware failure. |
| Factory reset | 9 | E | No | - |  | - | Perform factory reset of the device to make the Device have the same configuration as at the initial deployment. |
| Power source status | 10 | R | No | Integer |  |  | 0=power connected 1=running on battery |
| Battery level | 11 | R | Yes | Integer |  | - | Contains the current battery level as a percentage (with a range from 0 to 100). This value is only valid when the value of Power source status is 1. |
| Memory free | 16 | R | No | Integer |  | - | Estimated current available amount of storage space which can store data and software in the device (expressed in kilobytes). |
| Device state | 17 | R | No | Integer |  |  | 0=Idle 1=Active 2=Power Saving Mode |
| Error condidtion | 18 | R | No | Integer |  |  | 0=no error 1=hardware error 2=software error 3= … <more error codes to be specified> |
| Location | 19 | R | No | String |  | Lat, Lon | The global location of the device as a string in the format {Latitude, Longitude}. |
| Location Fix | 20 | R | No | Boolean |  |  | This resource indicates if there is currently a valid fix on the location as a Boolean value. |