# Change Request

| Title: | Error Code | Public OMA Confidential |
| --- | --- | --- |
| To: | DM LightweightM2M |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0_0-20130314-D |  |
| Submission Date: | 02 Apr 2013 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | zhangyongyj@chinamobile.com |  |
| Replaces: | n/a |  |

Reason for Change

- This CR updates the list of error code to record malfunctions or risks of the LWM2M client.

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and agree this CR.

Detailed Change Proposal

Update the error code list

- B.3 LWM2M Object: Device

Description: This LWM2M objects provide a range of device related information which can be queried by the LWM2M server, and a device reboot and factory reset function.

Object info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Device | 3 |  | No |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Manufacturer | 0 | R | No | String |  | - | Manufacturer name |
| Model Number | 1 | R | No | String |  | - | A model identifier (manufacturer specified string) |
| Serial Number | 2 | R | No | String |  | - | Serial Number |
| LWM2M client version Editor’s note: Where to put this Resource is TBD | 5 | R | No | String |  | - | Client version identifier |
| Firmware version | 7 | R | No | String |  | - | Current firmware version |
| Reboot | 8 | E | No | - |  | - | Reboot the device to restore the Device from unexpected firmware failure. |
| Factory reset | 9 | E | No | - |  | - | Perform factory reset of the device to make the Device have the same configuration as at the initial deployment. |
| Power source status | 10 | R | No | Integer |  |  | 0=power connected 1=running on battery |
| Battery level | 11 | R | Yes | Integer |  | - | Contains the current battery level as a percentage (with a range from 0 to 100). This value is only valid when the value of Power source status is 1. |
| Memory free | 16 | R | No | Integer |  | - | Estimated current available amount of storage space which can store data and software in the device (expressed in kilobytes). |
| Device state | 17 | R | No | Integer |  |  | 0=Idle 1=Active 2=Power Saving Mode |
| Error code | 18 | R | Yes | Integer |  |  | 0=No error 1=Low battery power 2=External power supply off 3=GPS module fail 4=Low received signal strength 5=Out of memory 6=SMS connection fail 7=Cellular connection fail 9=WiFi Connection fail 10=Wireline connection fail 11=Unknown peripheral malfunction 12=Printer malfunction 13=Scanner malfunction 14=IC card reader malfunction 15=POS reader malfunction 16=Key board malfunction 17=Screen malfunction <more error codes to be specified> When the single device object instance is initiated, there is only one error code resource instance whose value equal to 0 that means no error. When the first error happens, LWM2M Client changes error code resource instance to any non-zero value to indicate the error type. When any other error happens, a new error code resource instance is created. This error code resource MAY be observed by a LWM2M server. If any new error code instance is created, a Notify message should be send to LWM2M Server. How to deal with client’s error report depends on the policy of server. |
| Reset Error Code | 19 | E | No | - | - | - | Delete all error code resource instances and create only one zero-value error code that implies no error. |
| Current Time | 20 | R,W | No | TBD | 64bits |  | Current UNIX time of the client. The LWM2M Client should be responsible to increase this time value as every second elapses. LWM2M server is able to write this resource to make the client synchronized with a server. |
| Time Zone | 21 | R | No | Integer |  |  | GMT+X, where X is the value this resource. |