# Change Request

| Title: | Update Connectivity Object | Public OMA Confidential |
| --- | --- | --- |
| To: | DM LightweightM2M |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0_0-20130314-D |  |
| Submission Date: | 02 Apr 2013 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | zhangyongyj@chinamobile.com |  |
| Replaces: | n/a |  |

Reason for Change

- To make the connectivity object more comprehensive.

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and agree this CR.

Detailed Change Proposal

Update Connectivity Object

- B.4 LWM2M Object: Connectivity

Description: This LWM2M objects enables management of parameters related to network connectivity.

Object info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Connectivity | 4 |  | No |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Network Bearer | 0 | R | No | Integer | 8-bit | - | Indicates the current underlying network bearer among the below network bearer list. The LWM2M Server can change this value only if the value is listed in Available Network Bearer Resource. 0: GSM cellular network 1: TD-SCDMA cellular network 2: WCDMA cellular network 3: CDMA2000 cellular network 4: WiMAX cellular network 5: LTE-TDD cellular network 6: LTE-FDD cellular network 7~20: Reserved for other type cellular network 20: WLAN network 21: Bluetooth network 22: Zigbee network 23~40: Reserved for other type local wireless network 41: Ethernet 42: DSL 43~50: reserved for others type wire line network. |
| Available Network Bearer | 1 | R | Yes | Integer | 8-bit | - | Indicates list of current available network bearer. Each Resource Instance has the value among the network bearer list. |
| Radio signal strength | 2 | R | No | Integer |  | dBm | This node contains the average value of the received signal strength indication used in current network bearer in case Network Bearer Resource has 0~6(cellular network) or 20~22(Wireless). |
| LQ Uplink | 3 | R | No | Integer |  |  | This contains received link quality for uplink (e.g., LQI for IEEE 802.15.4, RxQual Uplink for GSM) used in current network bearer. Editor’s note: may need reference |
| LQ Downlink | 4 | R | No | Integer |  |  | This contains received link quality for downlink (e.g., LQI for IEEE 802.15.4, RxQual Downlink for GSM) used in current network bearer. Editor’s note: may need reference |
| Cell ID | 5 | R | No | Integer |  | - | Cell ID in case Network Bearer Resource has 0~6(cellular network). |
| MNC | 6 | R | No | Integer |  |  | Home network’s Mobile Network Code used in current network bearer in case Network Bearer Resource has 0~6(cellular network). Editor’s note: may need reference |
| MCC | 7 | R | No | Integer |  |  | Home Network’s Mobile Country Code used in current network bearer in case Network Bearer Resource has 0~6(cellular network). |
| Connection log | 8 | R | No | String |  | - | Contains a log of recent connection attempts to the servers, and whether the connection was successful or not. Editor’s Note: need to define log format. |
| IP Addresses | 9 | R | Yes | String |  | - | The IP addresses assigned to the connectivity interface. (e.g., IPv4, IPv6, etc.) |
| Parent IP Addresses | 10 | R | Yes | String |  | - | The IP address of the next-hop IP router in case Network Bearer Resource has 1(Wireless). (e.g., IPv4, IPv6, etc.) Note: these IP Addresses doesn’t indicate the Server IP address |
| Link Utilization | 11 | R | No | Integer | 0-100 | % | The average utilization of the link to the next-hop IP router in % in case Network Bearer Resource has 1(Wireless). |
| APN | 12 | W, R | Yes | String |  | - | Access Point Name in case Network Bearer Resource has 0~6(cellular network). |
| GPRS Dialing Number | 13 | W,R | No | String |  | - | GPRS Dialing Number is used by the client to initiate a GPRS connection in case Network Bearer Resource has 0 (GSM). |
| GPRS User Name | 14 | W,R | No | String |  | - | GPRS User Name is used by the client to initiate a GPRS connection in case Network Bearer Resource has 0 (GSM). |
| GPRS Password | 15 | W,R | No | String |  | - | GPRS Password is used by the client to initiate a GPRS connection in case Network Bearer Resource has 0 (GSM). |
| SMSC | 16 | W,R | No | String |  | - | Short Message Service Centre number for short message service. This is mandatory when SMS binding is adopted. |