Change Request

| Title: | Data Encoding | Public OMA Confidential |
| --- | --- | --- |
| To: | DM WG |  |
| Doc to Change: | OMA-RD-LightweightM2M-V1_2-20181010-D |  |
| Submission Date: | 2018-10-11 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | David Navarro, IoTerop, david.navarro@ioterop.com |  |
| Replaces: | n/a |  |

Reason for Change

Reduction of the bandwidth usage of LwM2M.

To transmit a single resource value, LwM2M allows four encodings:

- Plain text (3 bytes “123” or 5 bytes “12.34”) overhead varies.

- SenML (24 bytes “[{"n":"/1/0/1","v":123}]” or 26 bytes “[{"n":"/6/0/0","v":12.34}]”) overhead is at least 21 bytes.

- CBOR (7 bytes “0xA1 0x00 0xA2 0x2F31 0x18 0x7B” or 14 bytes “0xA1 0x00 0xA2 0x2F30 0xFA 0x414570A4”) overhead is 6 bytes.

- TLV (3 bytes “0xC1 0x01 0x7B” or 6 bytes “0xC4 0x00 0x414570A4”) overhead is 2 bytes

With the deprecation of the TLV format, a low overhead representation of single resource values is missing.

A possible technical solution is to transmit the value as a binary blob. There would be no overhead (“0x7B” or “0x414570A4”).

In the TS, this would be implemented in Appendix C (Data Types) by adding a new column for binary that would duplicates what is stated for TLV encoding.

E.g.:

| Data Type | Text Format | TLV Format | Binary |
| --- | --- | --- | --- |
| Unsigned Integer | Represented as an ASCII unsigned integer. | Represented as a binary unsigned integer in network byte order. The value may be 1 (8-bit), 2 (16-bit), 4 (32-bit) or 8 (64-bit) bytes long as indicated by the Length field. | Represented as a binary unsigned integer in network byte order. The value may be 1 (8-bit), 2 (16-bit), 4 (32-bit) or 8 (64-bit) bytes long as indicated by the Payload Length. |
| Float | Represented as an ASCII signed numeric representation. | Represented as a binary floating point value [IEEE 7542008] [FLOAT]. The value may use the binary32 (4 byte length) or binary64 (8 byte length) format as indicated by the Length field. | Represented as a binary floating point value [IEEE 7542008] [FLOAT]. The value may use the binary32 (4 byte length) or binary64 (8 byte length) format as indicated by the Payload Length. |
| Boolean | Represented as the ASCII value 0 or 1. | Represented as an 8 bit unsigned Integer with value 0, or 1. The Length of a Boolean value MUST always be 1 byte. | Represented as an 8 bit unsigned Integer with value 0, or 1. The Payload Length of a Boolean value MUST always be 1 byte. |

The may issue is the content-format to use when transmitting such a payload.

One possibility is to reuse “application/octet-stream”. Both the Server and the Client would know how to interpret this value because of the LwM2M Data Model which defines the data type of the targeted resource. This is similar to the usage of “text/plain”.

Another possibility is to register new media types for the various data types defined in LwM2M e.g. “application/signed-integer”, “application/unsigned-integer”, “application/float” etc.

Impact on Backward Compatibility

<statement describing the scope and nature of impacts of the change on the compatibility with previous versions of the document>

Impact on Other Specifications

<statement describing impacts on other specifications, this may relate to dependencies (either way), or on related requirements or technology material covered in other documents

if changes are required in other documents describe the plan to handle these (e.g. companion CRs being submitted for those docs, liaisons with owning groups are proposed, etc.)>

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

<provide a description of the intended actions to be taken by the group>

Detailed Change Proposal

(optional)Brief description of specific change

### Encoding and Standardized Data Models

| Label | Description | Release |
| --- | --- | --- |
| LightweightM2M-ENC-1 | Lightweight M2M SHOULD provide further optimization of encoding method of single resource value. |  |
| LightweightM2M-ENC-2 | Lightweight M2M SHOULD provide further optimization of encoding method of Object or Object Instance values. |  |

Table 4: High-Level Functional Requirements – Encoding