Change Request

| Title: | Scheduling Periodic Messages | Public OMA Confidential |
| --- | --- | --- |
| To: | DM WG |  |
| Doc to Change: | OMA-RD-LightweightM2M-V1_2-20181023-D.docx |  |
| Submission Date: | 2018-11-12 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Sarah Chang, Huawei, changhongna@huawei.com Echo Xu, Huawei, echo.xubei@huawei.com |  |
| Replaces: | n/a |  |

Reason for Change

With the intelligent acceleration of smart cities, more and more terminals access the LwM2M Server through 3GPP network. The number of connections in each communication cell is limited. To avoid network congestion caused by simultaneous reporting of data at the same time, it is recommended that the LwM2M Server can schedule the data reporting time of terminals in the same communication cell to achieve load balancing and improve network utilization.

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

WG to approve this CR.

Detailed Change Proposal

Reasoning

- 5.1.12 Scheduling Periodic Report Time

In a communication cell, different NB-IoT devices, such as gas meters and water meters, report data to the LwM2M Server at 12:00 in the morning as shown in figure 1. Due to the limited number of network connections, some devices fail to connect to the network. In this case, the failed devices try to access to the network again which cases more power consumption. However, maybe no data is reported at 03:00 in the morning. As a result, the load on the entire network is unbalanced.

In this case, the LwM2M Server can schedule the reporting time of devices according to the load of the entire cell, so as to disperse the reporting time of various equipment, as shown in figure 2.

Figure 1 Periodic Report without Scheduled by LwM2M Server

Figure 2 Periodic Report with Scheduled by LwM2M Server

- 6.12 Device Management and Service Enablement Interface

| Label | Description | Release |
| --- | --- | --- |
| LightweightM2 M-DMSE-001 | The LWM2M enabler SHALL allow the LWM2M Server to schedule the report time of a LwM2M Client. | 1.2 |