| ## APN connection profile |  |
| --- | --- |
| ### Description |  |
| This object specifies resources to enable a device to connect to an APN. |  |
| ### Object definition |  |
| Name; Object ID; Object Version; LWM2M Version. APN connection profile; 11; 1.1; 1.0. Object URN; Instances; Mandatory. urn:oma:lwm2m:oma:11.1.1; Multiple; Optional. |  |
| ### Resource Definitions |  |
| ID; Name; Operations; Instances; Mandatory; Type; Range or Enumeration; Units; Description. 0; Profile name; RW; Single; Mandatory; String; Human-readable identifier. Multiple connection profiles can share the same APN value but e.g. have different credentials.. 1; APN; RW; Single; Optional; String; Presented to network during connection to PDN e.g. ‘internet.15.234’. This resource is not included in case ‘Auto select APN by device’ resource has the value TRUE. If the APN resource is present but contains an empty string, then the device shall not provide an APN in the connection request (invoking default APN procedures in the network).. 2; Auto select APN by device; RW; Single; Optional; Boolean; If this resource is present for a connection profile, it enables the device to choose an APN according to a device specific algorithm. It provides a fall-back mechanism e.g. for some MVNO SIMs the configured APN may not work. Resource not included in case the ‘APN’ resource is specified.. 3; Enable status; RW; Single; Optional; Boolean; Allows the profile to be remotely activated or deactivated. True: connection is activated False: connection is de-activated.. 4; Authentication Type; RW; Single; Mandatory; Integer; Enumerated type: 0: PAP 1: CHAP 2: PAP or CHAP 3: None. 5; User Name; RW; Single; Optional; String; Used with e.g. PAP.. 6; Secret; RW; Single; Optional; String; Used with e.g. PAP.. 7; Reconnect Schedule; RW; Single; Optional; String; Comma separated list of retry delay values in seconds to be used in case of unsuccessful connection establishment attempts. e.g. “10,60,600,3600,86400”. 8; Validity (MCC, MNC); RW; Multiple; Optional; String; Coma separated mobile country code, then mobile network code – for which this APN is valid.. 9; Connection establishment time (1); R; Multiple; Optional; Time; UTC time of connection request. See note (1). 10; Connection establishment result (1); R; Multiple; Optional; Integer; 0 = accepted 1 = rejected. 11; Connection establishment reject cause (1); R; Multiple; Optional; Integer; 0-111; Reject cause, see [3GPP-TS_24.008, 3GPP-TS_24.301]. 12; Connection end time (1); R; Multiple; Optional; Time; UTC time of connection end.. 13; TotalBytesSent; R; Single; Optional; Integer; Rolling counter for total number of bytes sent via this interface since last device reset.. 14; TotalBytesReceived; R; Single; Optional; Integer; Rolling counter for total number of bytes received via this interface since last device reset.. 15; IP address (2); RW; Multiple; Optional; String; May be IPv4 or IPv6 address.. 16; Prefix length(2); RW; Multiple; Optional; String; Associated with IPv6 address.. 17; Subnet mask (2); RW; Multiple; Optional; String; Subnet mask.. 18; Gateway (2); RW; Multiple; Optional; String; Gateway.. 19; Primary DNS address (2); RW; Multiple; Optional; String; Primary DNS address.. 20; Secondary DNS address (2); RW; Multiple; Optional; String; Secondary DNS address.. 21; QCI (3); RW; Single; Optional; Integer; 1-9; For LTE and NB-IoT only QCI=Quality of service Class Identifier This resource enables the LWM2M server to signal the LWM2M client which QCI it shall request from the network. See [3GPP-TS_23.203, and 3GPP-TS_24.301] for a description of QCI values. See note (3).. 22; Vendor specific extensions; R; Single; Optional; Objlnk; Links to a vendor specific object.. 23; TotalPacketsSent; R; Single; Optional; Integer; Rolling counter for total number of packets sent via this interface since last device reset.. 24; PDN Type; RW; Single; Optional; Integer; 0=Non-IP 1=IPv4 2=IPv6 3=IPv4v6. 25; APN Rate Control; R; Single; Optional; Integer; Determines the number of allowed uplink PDU transmissions per time interval per APN. Rate Control information is part of the Protocol Configuration Options (PCO) according to [3GPP-TS_24.301], [3GPP-TS_24.008 and 3GPP-TS_27.060]. 26; Additional exception reports allowed; R; Single; Optional; Boolean; Indicates if additional exception reports allowed when maximum APN rate reached; see 3GPP-TS_24.008 section 10.5.6.3.2 ]. |  |
| Notes: (1) For each activated PDP context request, the device may store at least one value of ‘Connection establishment time’, ‘connection establishment result’, ‘Connection end time’ and if activation is unsuccessful then a ‘connection establishment reject cause’. It is a device decision how many instances to keep. (2) These resources are used in case IP related parameters are defined statically, and are also set with the IP related parameters in case of dynamic IP address assignment. The normal use case would be to have one IPv4 and one IPv6 address which have each associated a prefix length (IPv6 only), a subnet mask, a gateway, and a primary and secondary DNS address. (3) For LTE a higher QoS may be established by setting up an additional bearer (“dedicated bearer”) in addition to the default bearer which is established to the default APN. A dedicated bearer which is set up by the network on request by the device containing a requested QCI value can either be established to the same APN as the default bearer or to another APN. The QoS of a dedicated bearer may be modified on request by the device. The association of QoS values and APNs for a subscriber is stored in the network and checked during the establishment of a bearer. |  |