| ## Cellular connectivity |  |
| --- | --- |
| ### Description |  |
| This object specifies resources to enable a device to connect to a 3GPP or 3GPP2 bearers, including GPRS/EDGE, UMTS, LTE, NB-IoT, SMS. For cellular connectivity, this object focuses on Packet Switched (PS) connectivity and doesn’t aim to provide comprehensive Circuit Switched (CS) connectivity management. |  |
| ### Object definition |  |
| Name; Object ID; Object Version; LWM2M Version. Cellular connectivity; 10; 1.1; 1.0. Object URN; Instances; Mandatory. urn:oma:lwm2m:oma:10:1.1; Single; Optional. |  |
| ### Resource Definitions |  |
| ID; Name; Operations; Instances; Mandatory; Type; Range or Enumeration; Units; Description. 0; SMSC address; RW; Single; Optional; String; E.164 address of SMSC. Applicable for 3GPP2 networks where SMSC is not available from a smart card, or for 3GPP/3GPP2 networks to provide the application with a customer specific SMSC. The application decides how to use this parameter, e.g. precedence over UICC based SMSC address.. 1; Disable radio period; RW; Single; Optional; Integer; 0-65535; minutes; Time period for which the device shall disconnect from cellular radio (PS detach, CS detach if applicable). Can be used to handle network overload situations. The value is a positive integer (0 to 65535), duration can be from 1 minute to 65535 minutes (approximately 45 days). As soon the server writes a value which is >0 the device SHALL disconnect. When the period has elapsed the device MAY reconnect.. 2; Module activation code; RW; Single; Optional; String; Configurable in case the application needs to issue a code (e.g. via AT command) to activate the module. e.g. “*98”.. 3; Vendor specific extensions; R; Single; Optional; Objlnk; Links to a vendor specific object.. 4; PSM Timer (1); RW; Single; Optional; Integer; 1min-992 days; s; Power saving mode (PSM) timer as defined in [3GPP-TS_23.682]. PSM Timer = Extended T3412. Max interval between periodic TAU if there is no other transmission from the device. During most of this time the device is considered as unreachable and can therefore go into a deep sleep mode while keeping the PDN connection(s) active.. 5; Active Timer (1); RW; Single; Optional; Integer; 2sec-31 min; s; Active timer = T3324 as defined in [3GPP-TS_24.008]. The time the UE has to remain reachable after transitioning to idle state in case there is pending data from the NW to send out. At the end of T3324 UE can go into a deep sleep mode while keeping the PDN connection(s) active.. 6; Serving PLMN Rate control; R; Single; Optional; Integer; Only when Signalling Radio Bearers are used to transfer data, Data over NAS (DoNAS) , it indicates the maximum number of allowed uplink PDU transmissions per 6 minute interval aggregated across all PDN connections. See [3GPP-TS_23.401], octet 3 to 4 of the Serving PLMN rate control IE. (The same value is applicable on the downlink direction as well). 7; eDRX parameters for Iu mode (1); RW; Single; Optional; Opaque; 8 bits; Extended DRX parameters (Paging Time Window and eDRX value) for Iu mode which the UE can request from the network. This resource is encoded as octet 3 in [3GPP-TS_24.008, clause 10.5.5.32].. 8; eDRX parameters for WB-S1 mode (1); RW; Single; Optional; Opaque; 8 bits; Extended DRX parameters (Paging Time Window and eDRX value) for WB-S1 mode which the UE can request from the network. This resource is encoded as octet 3 in [3GPP-TS_24.008, clause 10.5.5.32].. 9; eDRX parameters for NB-S1 mode (1); RW; Single; Optional; Opaque; 8 bits; Extended DRX parameters (Paging Time Window and eDRX value) for NB-S1 mode which the UE can request from the network. This resource is encoded as octet 3 in [3GPP-TS_24.008, clause 10.5.5.32].. 10; eDRX parameters for A/Gb mode (1); RW; Single; Optional; Opaque; 8 bits; Extended DRX parameters (Paging Time Window and eDRX value) for A/Gb mode which the UE can request from the network. This resource is encoded as octet 3 in [3GPP-TS_24.008, clause 10.5.5.32].. 11; Activated Profile Names; R; Multiple; Mandatory; Objlnk; Links to instances of the “APN connection profile” object representing every APN connection profile that has an activated PDN connection.. 12; Coverage Enhancement Level; R; Single; Optional; Integer; 0-4; Indicates the Coverage Enhancement Level of the UE in the serving cell. The Coverage Enhancement levels are defined and specified in 3GPP TS 36.331 and 36.213. 0 No Coverage Enhancement in the serving cell 1 Coverage Enhancement level 0 2 Coverage Enhancement level 1 3 Coverage Enhancement level 2 4 Coverage Enhancement level 3. 13; Power Saving Modes; R; Single; Optional; Integer; 16-bits; This Resource is used to indicate the Power Saving Modes supported by the LwM2M Client. Each bit set in the Resource Instance value corresponds to a supported mode. Several bit set, means several modes are supported. 1st LSB 0: PSM 2nd LSB 1: eDRX Other bits are reserved for future use. 14; Active Power Saving Modes(1); RW; Single; Optional; Integer; 16-bits; This Resource is used to indicate which Power Saving Mode(s) is (are) currently active in the LwM2M Client. Each bit set in the Resource Instance value corresponds to an active mode. 1st LSB 0: PSM 2nd LSB 1: eDRX Other bits are reserved for future use. 15; Release Assistance Indication Usage; RW; Single; Optional; Integer; 8-bits; This resource is used to indicate whether client is using RAI to indicate to the NW when it can release the connection and if so for which types of messages, i.e. confirmable, non-confirmable, both confirmable and non-confirmable. (Coding as specified in 3GPP-TS_24.310, clause 9.9.4.25) 0 – Not used 1 – Only when no further uplink or downlink data transmission subsequent to the uplink data transmission is expected 2 - Only when a single downlink data transmission and no further uplink data transmission subsequent to the uplink data transmission is expected 3 – Used for both confirmable and non-confirmable messages. |  |
| Notes: (1) This parameter is controlled by the 3GPP network. A LWM2M server may write a suggested value to this resource. As soon a LWM2M server changes the current value the device then suggests the new value to the network upon the next practical opportunity e.g. by inclusion of the requested value in an Attach or TAU request. In case the network confirms the suggested value then the device shall apply that value. In case the network provides a different value then the value provided by the network shall be applied by the device and the resource value shall be set to that value. The LWM2M server may use the observe function to be notified upon any changes of PSM or Active Timer. By using the observe function the LWM2M server will get notified in case the value suggested by the LWM2M server is not accepted by the network, or, if the value is changed by the network for any reason (e.g. if the device moves into new tracking areas there is no guarantee that the above values remain the same and are not altered by the network). Editor’s Note: Additional resources for eDRX might be useful e.g. to reflect the eDRX value in milliseconds which would then also make such Resources observable, thus, making the server aware if the network assigns different values than requested by the device. |  |