| ### Communication Characteristics |  |
| --- | --- |
| Description |  |
| This object enables configuration of various generic communications parameters. |  |
| ### Object definition |  |
| Name; Object ID; Object Version; LWM2M Version. Communication Characteristics; 17; 1.0; 1.0. Object URN; Instances; Mandatory. urn:oma:lwm2m:oma:17; Single; Optional. |  |
| ### Resource Definitions |  |
| ID; Name; Operations; Instances; Mandatory; Type; Range or Enumeration; Units; Description. 0; Maximum uplink packet size; R; Single; Optional; Integer; octets; Maximum uplink packet size supported by the communications channel. In case of 3GPP NB-IoT this applies to the maximum uplink packet size supported via the Non Access Stratum (NAS). This information can be retrieved via the Protocol Configuration Options as defined in 3GPP TS 24.008.. 1; Maximum downlink packet size; R; Single; Optional; Integer; octets; Maximum downlink packet size supported by the communications channel, and supported by the device buffer.. 2; CoAP ACK_TIMEOUT; RW; Single; Optional; Integer; See [RFC 7252 CoAP]. 3; CoAP ACK_RANDOM_FACTOR; RW; Single; Optional; Float; See [RFC 7252 CoAP]. 4; CoAP MAX_RETRANSMIT; RW; Single; Optional; Integer; See [RFC 7252 CoAP]. 5; CoAP NSTART; RW; Single; Optional; Integer; See [RFC 7252 CoAP]. 6; CoAP DEFAULT_LEISURE; RW; Single; Optional; Integer; See [RFC 7252 CoAP]. 7; CoAP PROBING_RATE; RW; Single; Optional; Integer; See [RFC 7252 CoAP]. |  |