| Lightweight Machine to Machine Technical Specification |  |  |  |
| --- | --- | --- | --- |
| Draft Version 1.0 – 12 Apr 2013 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-TS-LightweightM2M-V1_0-20130412-D |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2013 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

Contents

1.	Scope	6

2.	References	7

2.1	Normative References	7

2.2	Informative References	8

3.	Terminology and Conventions	9

3.1	Conventions	9

3.2	Definitions	9

3.3	Abbreviations	9

4.	Introduction	10

4.1	Version 1.0	10

5.	Interfaces	11

5.1	Bootstrap Interface	13

5.1.1	Bootstrap Information	13

5.1.2	Bootstrap Modes	14

5.1.2.4	Server Initiated Bootstrap	15

5.1.3	Bootstrap Sequence	16

5.1.4	Bootstrap Security	16

5.1.5	LWM2M Object: LWM2M Bootstrap Server	16

5.2	Device Discovery & Registration Interface	17

5.2.1	Registration	18

5.2.2	Update	20

5.2.3	De-registration	20

5.3	Device Management & Service Enablement Interface	20

5.3.1	Read	22

5.3.2	Write	22

5.3.3	Execute	23

5.3.4	Create	23

5.3.5	Delete	23

5.4	Information Reporting Interface	24

5.4.1	Observe	24

5.4.2	Notify	27

5.4.3	Cancel Observation	28

6.	Identifiers and Resources	29

6.1	Resource Model	29

6.2	Identifiers	30

6.3	Data Formats for Transferring Resource Information	31

6.3.1	Plain Text	31

6.3.2	Opaque	31

6.3.3	TLV	31

6.3.4	JSON	32

7.	Security	34

7.1	Channel Security	34

8.1.1 Pre-Shared Keys	34

8.1.2 Raw Public Key Certificates	34

8.1.3 X.509 Certificates	34

7.2	Access Control	35

7.2.1	Access Control List (ACL)	35

7.2.2	Access Type	35

8.1.1	Authorization	36

9.	Transport Layer Binding and Encoding	38

9.1	Required Features	38

9.2	URI Identifier & Operation Mapping	38

9.3	Firewall/NAT	38

9.3.1	Registration Interface	39

9.3.2	Bootstrap Interface	40

9.3.3	Device Management & Service Enablement Interface	41

9.3.4	Information Reporting Interface	44

9.4	Queue Mode Operation	44

9.5	Response Codes	46

9.6	Transport Bindings	48

9.6.1	UDP Binding	48

9.6.2	SMS Binding	48

Appendix A.	Data Types	49

Appendix B.	LWM2M Object Template and Guidelines   (Informative)	51

B.1	Object Template	51

B.2	Guidelines	51

Appendix C.	LWM2M Objects defined by OMA   (Normative)	52

C.1	LWM2M Object: LWM2M Server	52

B.1.1 Unbootstrapping	54

C.2	LWM2M Object: Access Control	55

C.2.1	Object Instance Consideration	56

C.3	LWM2M Object: Device	57

C.4	LWM2M Object: Connectivity Monitoring	58

C.5	LWM2M Object: Firmware	61

B.5.1 Firmware Update Consideration	63

C.6	LWM2M Object: Location	63

Appendix D.	Storage of LWM2M Bootstrap Message on the Smartcard (Normative)	66

D.1	File structure	66

D.2	Bootstrap Message on UICC (Activated in 3G Mode)	66

D.2.1	Access to the file structure	66

D.2.2	Files Overview	67

D.2.3	Access Method	67

D.2.4	Access Conditions	67

D.2.5	Requirements on the 3G UICC	67

D.3	Files Description	68

D.3.1	Object Directory File, EF ODF	68

D.3.2	Bootstrap Data Object Directory File, EF DODF-bootstrap	68

D.3.3	EF  LWM2M_Bootstrap	69

Appendix E.	Change History (Informative)	70

E.1	Approved Version History	70

E.2	Draft/Candidate Version <current version> History	70

Appendix F.	Static Conformance Requirements (Normative)	72

F.1	SCR for XYZ Client	72

F.2	SCR for XYZ Server	72

Appendix G.	<Additional Information>	73

G.1	App Headers	73

G.1.1	More Headers	73

Figures

Figure 1 Device Discovery and Registration	12

Figure 2 Bootstrap	12

Figure 4 Information Reporting	13

Figure 5 Registration Interface example flows.	14

Figure 6. Procedure of Client Initiated Bootstrap	16

Figure 7. Procedure of Server Initiated Bootstrap	17

Figure 8. Example flow of Device Management & Service Enablement Interface	18

Figure 11 Relationship between LWM2M Client, Object and Resources	23

Figure 12 Supported operations and access control lists	24

Figure 4: File structure for Bootstrap Message on 3G UICC	51

Tables

Table 1 Relationship of logical operations and interfaces	13

Table 4 Read parameters	18

Table 4 Write parameters	18

Table 5 Execute parameters	19

Table 6 Create parameters	19

Table 7 Delete parameters	19

Table 8 Observe parameters	20

Table 11 Definition of LWM2M terms	24

Table 11 TLV format and description	25

Table 11 JSON fields	26

Table 11 Access Type operations	29

Table 2: Operation to Method and URI Mapping	31

Table 3:  Operation to Method Mapping	33

Table 4: Response Codes	36

# Scope

<< Define as it relates to Open Mobile Alliance Activity.  If it adds clarity, define what is not in the scope.  DELETE THIS COMMENT >>

# References

The policy for reference lists is:

1.	OMA documents listed should have at least one approved version – draft-only docs should not be referenced.  Exception exists for documents that will be approved with or after the referenced doc is approved (may be part of same enabler package).  In short – approved docs should not reference unapproved docs.

2.	When a reference is made to an OMA specification, then Open Mobile Alliance with the TM symbol (™) should be used in the description.

3.	The name + version (no date) for OMA specifications are generally sufficient – dates should be used only if there is a specific reason to limit the usage.

4.	For references to WAP Forum docs, dates should not be included as DID's for the old WAP Forum specifications are enough and the reference description should refer to WAP Forum™.

5.	References to other affiliate docs should similarly provide sufficient information to uniquely determine the needed document and should provide the appropriate source information.

6.	The URL for OMA material (new OMA and affiliate) should always be http://www.openmobilealliance.org (an exception is OMNA that is reached through http://www.openmobilealliance.org/tech/omna)

Models to use

[REFLABEL]	<General Model> “Ref Title”, Ref information (source, date, id),
URL:http//<ref-source>/

[OMADOC]	<OMA Model> “OMA Document Title”, Open Mobile Alliance™, OMA-<docname>{-<version>}, URL:http//www.openmobilealliance.org/

If there are no entries in the table – enter ‘none’ to be clear.

DELETE THIS COMMENT

## Normative References

| [IOPPROC] |  | “OMA Interoperability Policy and Process”, Version 1.1, Open Mobile Alliance™, OMA-IOP-Process-V1_1, URL:http://www.openmobilealliance.org/ |
| --- | --- | --- |
| [RFC2119] |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |
| [RFC2234] |  | “Augmented BNF for Syntax Specifications: ABNF”. D. Crocker, Ed., P. Overell. November 1997, URL:http://www.ietf.org/rfc/rfc2234.txt |
| [LWM2M-AD] |  | “Lightweight Machine to Machine Architecture”, Open Mobile Alliance™, OMA-AD-LightweightM2M-V1_0, URL:http://www.openmobilealliance.org/ |
| [CoAP] | Shelby, Z., Hartke, K., Bormann, C., and B. Frank, "Constrained Application Protocol (CoAP)", draft-ietf-core-coap-14 (work in progress), Mar 2013. |  |
| [OBSERVE] | Hartke, K. “Observing Resources in CoAP”, draft-ietf-core-observe-08 (work in progress), Mar 2013. |  |
| [RFC6690] | Shelby, Z. “Constrained RESTful Environments (CoRE) Link Format”, RFC6690, Aug 2012. |  |
| [RFC6347] | Rescorla, E. and N. Modadugu, "Datagram Transport Layer Security Version 1.2", RFC 6347, January 2012. |  |
| [RFC6655] | McGrew, D. and D. Bailey, "AES-CCM Cipher Suites for TLS", RFC6655, July 2012. |  |
| [RFC5487] | Pre-Shared Key Cipher Suites for TLS with SHA-256/384 and AES Galois Counter Mode |  |
| [RFC5246] | The Transport Layer Security (TLS) Protocol Version 1.2 |  |
| [RFC5289] | TLS Elliptic Curve Cipher Suites with SHA-256/384 and AES Galois Counter Mode (GCM) |  |
| [PKCS#15] | PKCS #15 v1.1: Cryptographic Token Information Syntax Standard”, RSA Laboratories, June 6, 2000. URL: ftp://ftp.rsasecurity.com/pub/pkcs/pkcs-15/pkcs-15v1_1.pdf |  |
| [ETSI TS 102.221] | “Smart Cards; UICC-Terminal interface; Physical and logical characteristics”, (ETSI TS 102 221 release 11), URL:http://www.etsi.org/ |  |

## Informative References

|  | << Add/Remove reference rows as needed! >> |
| --- | --- |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

.

## Definitions

| Queue Mode | The interaction model between an LWM2M Client and LWM2M Server is based on that LWM2M Server queues the requests. |
| --- | --- |

## Abbreviations

| OMA | Open Mobile Alliance |
| --- | --- |
| xxx | xxx |

# Introduction

## Version 1.0

This enabler defines the application layer communication protocol between the LWM2M Server and the LWM2M Client which is placed in the LWM2M Device. The OMA Lightweight M2M enabler includes device management and service enablement for LWM2M Devices. The target LWM2M Devices for this enabler are mainly resource constrained devices. Therefore, this enabler provides a light and compact protocol as well as an efficient resource data model.

Client-Server architecture is introduced for LWM2M enabler. The LWM2M enabler has two components, LWM2M Server and LWM2M Client. Four interfaces are designed between these two components as shown below:

- Device Discovery and Registration

- Bootstrap

- Device management and service enablement

- Information Reporting

# Interfaces

According to the architecture diagram [LWM2M-AD], there are four interfaces: 1) Bootstrap  2) Device Discovery and Registration  3) Device Management and Service Enablement 4) Information Reporting. The logical operations for the four interfaces can be classified as uplink operations and downlink operations. The logical operations of each interface are defined in this section, and then mapped to protocol mechanisms in Section 9.

Figure 1 shows the logical operation model for interface “Bootstrap”. For this interface, the operations are uplink Client Initiated Bootstrap or downlink Server Initiated Bootstrap. These operations are used to initialize the needed Object for the Client to register with one or more LWM2M Servers. Bootstrapping is also defined using Manufacturer Pre-configuration (e.g. storage in Flash) or SmartCard Provisioning (storage in a SmartCard).

Figure 1 Bootstrap

Figure 2 shows the logical operation model for the interface “Device Discovery and Registration”. For this interface, the operations are uplink and consists of Registration, Update, and De-registration.

Figure 2 Device Discovery and Registration

Figure 3 shows the logical operation model for interface “Device Management and Service Enablement”. For this interface, the operations are downlink and consist of “Read”, “Create”, “Delete”, “Write” and “Execute”. These operations are used to interact with the Resources, Objects and Object Instances of the LWM2M Client. Read is used to read the current value of one or more Resources, Write is used to update the value of one or more Resources, and Execute is used to initiate an action defined by a Resource. Create and Delete are use to create or delete Object Instances.

Figure 3 Device Management and Service Enablement

Figure 4 shows the logical operation model for interface “Information Reporting”. For this interface, the operation is downlink Observe or Cancel Observation, and uplink Notify. This interface is used to send the LWM2M Server a new value related to a Resource on the LWM2M Client.

Figure 4 Information Reporting

The relationship between logical operations and interfaces is listed in the following table 1.

Table 1 Relationship of logical operations and interfaces

| Interface | Direction | Logical Operation |
| --- | --- | --- |
| Device Discovery and Registration | Uplink | Register, Update, De-register |
| Bootstrap | Uplink | Request Bootstrap |
| Bootstrap | Downlink | Write Bootstrap |
| Device Management and Service Enablement | Downlink | Read, Create, Delete, Write, Execute |
| Information Reporting | Downlink | Observe, Cancel Observation |
| Information Reporting | Uplink | Notify |

## Bootstrap Interface

Bootstrap process is used to provision essential information into the LWM2M Client to enable the LWM2M Client  to register to a certain LWM2M Server.

This chapter describes what information is conveyed in the bootstrap message, where the LWM2M Client puts that information and how to provision the bootstrap information. Please note the LWM2M Client SHALL support at least one bootstrap mode (i.e. Manufacturer Pre-configuration, SmartCard Provisioning, LWM2M Server-initiated Bootstrap, LWM2M Client-initiated Bootstrap) whereas the LWM2M Server MUST support this interface.

### Bootstrap Information

This section specifies what information is conveyed in the bootstrap message to the LWM2M Client and where the LWM2M Client stores the information.

The information is listed in Table 3. A LWM2M Client MAY be configured to use one or more LWM2M Servers, with a set of bootstrap information for each LWM2M Server. The LWM2M Client SHOULD be pre-configured with LWM2M Bootstrap Server Object Instance (refer to Bootstrap Object instance: 5.2.4)

Table 3. Bootstrap Information List

| Entity | Semantics | Description | Required |
| --- | --- | --- | --- |
| LWM2M Bootstrap Server | Object Instance | Stores account of the LWM2M Bootstrap Server according to 5.2.1(i.e Bootstrap Server URI and bootstrap keying material ..) | No* |
| LWM2M Server | Object Instance | Stores account of the LWM2M Server according to Appendix B.1 | No* |
| Additional Objects (e.g.: Connectivity Object) | Object Instances | Stores the specific Object information | No |

* If LWM2M Bootstrap Server Object Instance is not configured, the LWM2M server MUST be configured

### Bootstrap Modes

The LWM2M enabler defines several bootstrap modes: Manufacturer Pre-configuration bootstrap, SmartCard Provisioning, Server initiated Bootstrap and Client initiated Bootstrap.

#### Manufacturer Pre-configuration

In this mode, the LWM2M Client is already provisioned at manufacturer stage. It means that necessary Object Instances have been installed before deployment. If LWM2M Server has been configured then the LWM2M Client doesn’t need any communication with a  Bootstrap Server before the registration.

#### SmartCard Provisioning

When the Device supports a SmartCard, the LWM2M Client SHALL support retrieval, and processing of Bootstrap Message contained in the SmartCard as described in Appendix C. When Bootstrap Message retrieval is successful, the LWM2M Client SHALL process the Bootstrap Message from the SmartCard and SHALL apply it to the corresponding Object Instances.

SmartCard can be provisionned with regular LWM2M  Servers URI/DTLS keying material but also with  LWM2M Bootstrap  Server URI/DTLS keying material.

In this mode, the LWM2M Client SHALL  also check that the bootstrap data previously bootstrapped from the SmartCard are unchanged in the SmartCard.If changed, the previous bootstrap information SHALL be disabled in the LWM2M Client, and the LWM2M Client SHALL apply the new bootstrap data from SmartCard to the Device configuration.

#### Client initiated Bootstrap

This bootstrap mode can be activated in 2 cases:

- no LWM2M server has been preconfigured by SmartCard Provisioning or Manufacturer Pre-configuration mode

- attempts of registration on all LWM2M Servers have failed

In this mode, the LWM2M Client retrieves the bootstrap message from a LWM2M Bootstrap Server, Prior to the client initiated bootstrap, the LWM2M Client needs to have LWM2M Bootstrap Server Object Instance. The below figure shows the client initiated bootstrap flows.

Figure 6. Procedure of Client Initiated Bootstrap

Step #1: Request bootstrap to bootstrap URI

The LWM2M Client requests bootstrap to LWM2M Bootstrap Server URI which has been pre-provisioned. When requesting the bootstrap, the LWM2M Client sends its Endpoint Client Name.

Step #2: Provision bootstrap information

The LWM2M Server provisions bootstrap information, which is specified in 5.1.1.

### Server Initiated Bootstrap

In this mode, the LWM2M Bootstrap Server provisions the bootstrap message to the LWM2M Client. Prior to provisioning, the LWM2M Bootstrap Server needs to know a LWM2M Device is ready for bootstrapping; For instance Network Provider can inform the LWM2M Bootstrap Server of the LWM2M Device connecting to the Network Provider.

The LWM2M Bootstrap Server is now in a position where it can send out a Bootstrap Message based on LWM2M Bootstrap Server Object Instance. This message contains enough information for the LWM2M Client to be able to register to the LWM2M Servers, the Server Object Instances which are  contained in bootstrap message.

The below figure shows the server initiated bootstrap flows.

Figure 7. Procedure of Server Initiated Bootstrap

Step #1: Provision bootstrap information

The LWM2M Bootstrap Server provisions bootstrap information which is specified in 6.2.1.

Note: even though the connection is alive, Client initiated bootstrap and Server initiated bootstrap MAY be used for updating some read-only resources in the Client.

### Bootstrap Sequence

The LWM2M Client MUST follow the procedure specified as below.

- If the LWM2M Device has SmartCard, the LWM2M Client tries to obtain bootstrap information from the SmartCard.

- If the LWM2M Client is not provioned by the SmartCard, the Client tries to obtain bootstrap information by using Manufacturer pre-configuration

- If LWM2M Client has any LWM2M Server Object Instances, the Client tries to register to the Server(s) of the LWM2M Server Object Instance.

- If LWM2M Client fails to register to all the LWM2M Servers or the Client doesn’t have any LWM2M Server Object Instance, the Client performs Client initiated bootstrap.

### Bootstrap Security

Since the messages conveyed in Bootstrap interface are sensitive, to separate communication channel, security mechanism and/or key for Bootstrap interface MUST be different from the ones that are used in the other interfaces.

If the LWM2M Client or the LWM2M Server needs bootstrapping for some reason, the LWM2M Client or the LWM2M Server MUST proceed to a complete new bootstrap session.

Editor’s note: “some reason” should be clarified

If the LWM2M Server wants to change security materials (i.e., LWM2M Server URI, Security Mode, and Security Key),, the Client MUST disconnect DTLS session first  and change MUST be handled  through a new bootstrap session.

### LWM2M Object: LWM2M Bootstrap Server

Description: This LWM2M object SHOULD be pre-configured and MUST NOT be accessed by LWM2M Server

Object Info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| LWM2M Bootstrap Server |  |  | No |

Resource Info:

| Resource Name | Type | Range or Enumeration | Descriptions |
| --- | --- | --- | --- |
| LWM2M Bootstrap Server URI | String | 0 – 255 bytes | Uniquely identifies the LWM2M Bootstrap Server, and is in the form: “coaps://host:port”, where host is an IP address or FQDN, and port is the UDP port of the Server. |
| Security Mode | Integer | 8-bit | Determines which security mode of DTLS is used 0: Pre-Shared Key mode 1: Raw Public Key mode 2: Certificate mode |
| Security Key | Binary | Variable | Stores security key of security mode. The format of the keying material is defined by the security mode. |

## Device Discovery & Registration Interface

The Discovery & Registration Interface is used by a LWM2M Client to register with one or more LWM2M Servers, maintain each registration and de-register from a Server. The registration is based on the Resource Model and Identifiers defined in Section 7. When registering, the LWM2M Client indicates its Endpoint Name, registration lifetime and the list of Objects the Client supports and created Object Instances. The registration is soft state, with a lifetime indicated by the registration lifetime. The LWM2M Client periodically performs an update of its registration information to the registered Server(s). If the lifetime of a registration expires without receiving an update from the Client, the Server removes the registration. Finally, when shutting down or discontinuing use of a Server, the Client performs a de-registration.

Figure 5 Registration Interface example flows.

### Registration

Registration is performed when a Client sends a regiser operation to the Server. After the LWM2M Device is turned on and bootstrap procedure has completed, The Client MUST perform registration to the Server of which the Client has a Server Object Instance. The registration includes the Endpoint Client Name (which MUST be unique on that Server) along with optional Lifetime,  SMS Binding Support parameters, LWM2M Verrsion parameters, and the list of Objects that the Client supports and available Object Instances. Upon receiving a registration messge from the client the server records the IP address and port from the registration message for all interactions with that client. If the IP address or the port changes the client must send a registration update to the server. Table 3 describes the behaviour of a Client and Server when different modes are enabled (SMS or Queue Mode). The Client SHOULD register with the modes indicated by the Binding Preference resource of the LWM2M Server Object.

Table 2 Registration parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Endpoint Client Name | Yes |  | See section 7.2 |
| Lifetime | No | 86400 | See section B.1. If Lifetime Resource does not exist in a Server Object Instance, the Client MUST NOT send this parameter and the Server MUST regard lifetime of the Client as 86400 seconds The registration is removed by the Server if a new registration or update is not received within this lifetime. |
| SMS Binding Support | No |  | Inclusion of this parameter indicates the Client supports the SMS binding. The value of this parameter is the MSISDN of the Client can be reached at. |
| Objects and Object Instances | Yes |  | The list of Objects supported and Object Instances available on the Client. |
| LWM2M Version | No | 1.0 | Indicates the version of the LWM2M enabler that the LWM2M Client supports. This parameter is required only for LWM2M versions > 1.0. |
| Queue Mode | No |  | Indicates that the LWM2M Client requests the LWM2M Server to work in Queue Mode. |

The list of Objects and Object Instances is included in the payload of the registration message. Each Object is described as a Link in the CoRE Link Format [RFC6690]. The Target component of the link is required, and consists of the Object path. Any other parameters included in the link MUST be silently ignored, unless specified for use by this specification. The Media Type of this payload is application/link-format.

The payload for a Client supporting LWM2M Server, Access Control, Device, Connectivity and Firmware Objects from Appendix B would simply be:

</1>,</2>,</3>,</4>,</5>

If Objects Instances are also available on the client, then the format would be (this example assumes existing Instances for everything except the Firmware Object):

</1/0>, </2/0>, </2/1>, </3/0>,</4/0>,</5>

By default, the RFC6690 links of Objects are located under the root path as in the examples above. However, devices might be hosting other resources on an endpoint, and there may be the need to place Objects under an alternative path. This is achieved by including an OMA LWM2M link in addition to the Object links as follows, e.g. to place Objects under the “/lwm2m” path:

</lwm2m>;rt="oma.lwm2m", </lwm2m/1/0>, </lwm2m/2/0>, </lwm2m/2/1>, </lwm2m/3/0>,</lwm2m/4/0>,</lwm2m/5>

The RFC6690 Resource Type value “oma.lwm2m” will be registered with the appropriate IANA registry for this purpose.

If the Client supports the JSON data format for all the Objects it SHOULD inform the server by including the content type in the root path link using the ct= link attribute. An example is as follows (note that the content type value 100 is an example, the actual value will be assigned by IANA for the LWM2M JSON format).

</>;rt="oma.lwm2m";ct=100, </1/0>, </2/0>, </2/1>, </3/0>,</4/0>,</5>

Note: if the Client sends registration to the Server even though the Server has registration information of the Client, the Server removes the existing registration and performs the new registration. This situation happens when the Client forgets the state of the Server (e.g., factory reset).

Table 3 Relationship of different modes

| Queue Mode Enabled | SMS Enabled | Behaviour |
| --- | --- | --- |
| No | No | The Server expects that the Client is reachable via the UDP binding at any time. This is the normal default mode of operation. |
| No | Yes | The Server MAYsend requests to a Client using the SMS binding. The Client MUST send the immediate response to such a request over the SMS binding. The Client sends notifications using either the SMS or UDP bindings depending on the Binding Preference resource of the LWM2M Server Object. |
| Yes | No | The Server MUST Queue all requests to the Client, sending requests when the Client is on-line (immediately after it sends a Registration Update message) using the UDP binding. |
| Yes | Yes | The Server MUST Queue all requests to the Client, sending requests when the Client is on-line (immediately after it sends a Registration Update message). Requests SHOULD be sent to the Client using the SMS binding. The Client MUST send the immediate response to such a request over the SMS binding. The Client sends notifications using either the SMS or UDP bindings depending on the Binding Preference resource of the LWM2M Server Object. |

### Update

Periodically, the Client updates its registration with a Server by sending an update Operation to the Server. This update message MUST contain only changed parameters to update Client status compared to the last registration parameters sent to the Server. Different from registration message, for update message, all the registration parameters specified in Table 2 are OPTIONAL. When a Client’s IP address or port changes for any reason, the Client MUST send a new update Operation to the Server only if UDP binding is used.

### De-registration

When a Client will no longer be available, the Client SHOULD send a de-registrater  Operation to the Server. Upon receiving this message, the Server will remove the registration information from the Server.

## Device Management & Service Enablement Interface

This interface is used by the LWM2M Server to access Resources available from a LWM2M Client using Read, Write,  Execute, Create, or Delete operations. The operations that a Resource supports are defined in the definition of its Object.

Figure 8. Example flows of Device Management & Service Enablement Interface

### Read

The Read operation is used to access the value of a Resource, an array of Resource Instances, or an entire Object Instance at once. The Read operation has the following parameters:

Table 4 Read parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | No | - | Indicates the Object Instance to read. If no Object Instance ID is indicated, then the Object Instances of Objects, which the Server is authorized to, are returned. |
| Resource ID | No | - | Indicates the Resource to read. If no Resource ID is indicated, then the whole Object Instance is returned. |

### Write

The Write operation is used to change the value of a Resource, an array of Resources Instances or multiple Resources from an Object Instance at once. The Write operation has the following parameters:

Table 5 Write parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | Yes | - | Indicates the Object Instance to write. |
| Resource ID | No | - | Indicates the Resource to write. The payload is the new value for the Resource. If no Resource ID is indicated, then the included payload is an Object Instance containing the Resource values. |
| New Value | Yes | - | The new value included in the payload to update the Object Instance or Resource. |
| Minimum Period | No | 1 | When present, the minimum period indicates the minimum time in seconds the Client SHOULD wait from the time when sending the last notification to the time when sending a new notification. In the absence of this parameter, the Minimum Period is defined by the Default Minimum Period set in the LWM2M Server Object Instance related to that Server. |
| Maximum Period | No | - | When present, the maximum period indicated the maximum time in seconds the Client SHOULD wait from the time when sending the last notification to the time sending the next notification (regardless if the value has changed). In the absence of this parameter, the maximum period is up to the server. The maximum period MUST be greater than the minimum period parameter. In the absence of this parameter, the Maximum Period is defined by the Default Maximum Period set in the LWM2M Server Object Instance related to that Server. |
| Greater Than | No | - | When present, the Client SHOULD notify its value when the value is above the number specified in parameter |
| Less Than | No | - | When present, the Client SHOULD notify its value when the value is below the number specified in the parameter |
| Step | No | - | When present, the Client SHOULD notify its value when the value is changed more than the number specified in the parameter from the Resource value when the client receives the Observe operation. |

Minimum Period, Maximum Period, Greater Than, Less Than, and Step are only to do with Observe operation.

Greater Than, Less Than, and Step MUST be specified only when Resource ID is indicated.

Greater Than, Less Than, and Step parameters MUST be supported only when the Resource type is numeral (e.g., integer, decimal).

### Execute

The Execute operation is used to initiate some action, and can only be performed on individual resources. A LWM2M Client MUST return an error (see subclause 9) on receiving an operation to execute an Object Instance.

Table 6 Execute parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | Yes | - | Indicates the Object Instance. |
| Resource ID | Yes | - | Indicates the Resource to execute. |

### Create

The Create operation is used for the Server to create an Object Instance. This operation MUST target either the Object Instance which has not yet been instantiated or the Object. The Object ID MUST be listed in Objects and Object Instances parameter in the previous Registration or Update message. The Create operation has the following parameters:

Table 7 Create parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | No | - | Indicates the Object Instance to create. If this Resource is not specified, the Client assigns the ID of the Object Instance. |
| New Value | Yes | - | The new value included in the payload to create the Object Instance. |

### Delete

The Delete operation is used for the Server to delete an Object Instance. This operation MUST target the Object Instance. The Object ID MUST be listed in Objects and Object Instances parameter in the previous Registration or Update message. The Delete operation has the following parameters:

Table 8 Delete parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | Yes | - | Indicates the Object Instance to delete. |

## Information Reporting Interface

This interface is used by a LWM2M Server to observe any changes in a Resource on a LWM2M Client, receiving notifications when new values are available. This observation relationship is initiated by sending an Observe operation to the L2M2M Client for an Object Instance or Resource. This operation MAY contain optional Maximum and/or Minimum Period parameters to control how often these notifications are sent from the client. The Minimum Period is to limit congestion notifications, an Observation SHOULD NOT be sent faster than this. An observation ends when a Cancel Observation operation is performed or the LWM2M Server is no longer reachable.

Figure 9. Example flow for Information Reporting Interface

### Observe

The Observe operation includes the following parameters:

Table 9 Observe parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | Yes | - | Indicates the Object Instance to observe. If no Resource ID is indicated, then the whole Object Instance is observed. |
| Resource ID | No | - | Indicates the Resource to observe. |

If the minimum period and maximum period value are the same, then the Client sends notification in every that period.

The following example shows how the Minimum and Maximum period parameters work as shown in Figure 10. A Server makes an Observation for a temperature resource that is updated inside the Client at irregular periods (based on change). The Server makes an Observation when the Minimum Period = 10 Seconds and Maximum Period = 60 Seconds have been set for that resource a priori. The Client will wait at least 10 Seconds before sending a Notification to the Server (even if the resource has changed before that), and no longer than 60 Seconds before sending a Notification (even if the resource has not changed yet). The Notification is sent anywhere between 10-60 seconds upon change.

Figure 10 Example of Minimum and Maximum periods in an Observation. This example assumes the the Minimum Period has been set to 10 and the Maximum Period set to 60 for the resource /21/4/3 before making the Observation.

### Notify

The Notify operation is sent to the LWM2M Server during a valid observation on an Object, Object Instance, or Resource. This operation includes the new value of the Object, Object Instance or Resource.

Table 10 Notify parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Updated Value | Yes | - | The new value included in the payload about the Object Instance or Resource. |

### Cancel Observation

The Cancel Observation operation is sent to the LWM2M Client to end an observation relationship. The operation includes the following parameters:

Table 11 Cancel Observation parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | Yes | - | Indicates the Object Instance to stop observing. If no Resource ID is indicated, then the whole Object Instance is indicated. |
| Resource ID | No | - | Indicates the Resource to stop observing. |

# Identifiers and Resources

This section defines the identifiers and resource model for the LWM2M Enabler.

## Resource Model

The LWM2M Enabler defines a simple resource model where each piece of information made available by the LWM2M Client is a Resource, and Resources are logically organized into Objects. Figure  illustrates this structure, and the relationship between Resources, Objects and the LWM2M Client. The LWM2M Client may have any number of Resources, each of which belongs to an Object.

Figure 11 Relationship between LWM2M Client, Object and Resources

Resources are defined per Object, and each resource is given a unique identifier within that Object. Each Resource is defined to have one or more Operations that it supports. A Resource MAY consist of multiple instances as defined in the Object specification.

An Object defines a grouping of Resources, for example the Firmware Object contains all the Resources used for firmware update purposes. Each Object is assigned a unique OMA Management Object identifier and corresponding index which identifies an Object defined in this specification. The LWM2M enabler defines standard Objects and Resources. Further Objects may be added by OMA or other organizations to enable additional M2M Services.

An Object MUST be instantiated either by the LWM2M Server or the LWM2M Client, which is called Object Instance before using the functionality of an Object. After an Object Instance is created, the LWM2M Server can access that Object Instance and Resources which belong to that Object Instance.

The LWM2M Server can perform Operations which are defined in each interface chapter. How to convey the Operation data is defined in 7.3.

The LWM2M enabler defines access control mechanism per Object Instance. Object Instances SHOULD have associatedan Access Control Object Instance which contains Access Control Lists (ACLs) that control with which operations  the LWM2M Server can access the Resources of an Object Instance. The access control mechanism  is defined in 8.2. Figure 122 shows which operations the resources support, and how Object Instances and Resources are associated with ACLs. In the example, Resource 1 supports read, write and execute, while Resource 2 supports only read operations.

Figure 12 Supported operations and access control lists

## Identifiers

Height identifiers are defined by the LWM2M Enabler: Endpoint Client Name, LWM2M Bootstrap Server URI ,LWM2M Server URI, Short Server ID, Human Readable Object URN, Object ID, Object Instance ID, Resource ID and Resource Instance ID. These identifiers are defined in Table .

Table 12 Definition of LWM2M terms

| Identifier | Semantics | Description |
| --- | --- | --- |
| Endpoint Client Name | String (max 63 bytes) | Uniquely identifies the LWM2M Client on one LWM2M Server (including LMWM2M Bootstrap Server). Provided to the LWM2M Server during Registration, also provided to LMWM2M Bootstrap Server during Bootstrap session). For example the IMEI, serial number or a logical name of the device. |
| LWM2M Bootstrap Server URI | URI | Uniquely identifies the LWM2M Bootstrap Server. Provided to the Client during Bootstrap procedure |
| LWM2M Server URI | URI | Uniquely identifies the LWM2M Server. Provided to the Client during Bootstrap procedure. |
| Short Server ID | 16-bit unsigned integer | Short integer ID, assigned by the LWM2M Client. This identifier uniquely identifies each LWM2M Server configured for the LWM2M Client. Default Short Server ID is 0 and default Short Server ID MUST not be used for identifying the LWM2M Server |
| Human Readable Object URN | URN for the OMA Management Object | Assigned by the Object specification |
| Object ID | 16-bit unsigned integer | Uniquely identifies the Object in the LWM2M Client. Short integer ID, assigned by OMA |
| Object Instance ID | 8-bit unsigned integer | Uniquely identifies the Object Instance in the Object. Short integer ID, assigned by the LWM2M Server/Client |
| Resource ID | 16-bit unsigned integer | Uniquely identifies the Resource in the Object. Short integer ID, assigned by the object specification |
| Resource Instance ID | 8-bit unsigned integer | Uniquely identifies the Resource Instance in the Resource. Short integer ID, assigned by the LWM2M Server/Client Example: multiple IP address resources |

Editor’s Note: possibly a section with multiple examples would be useful

## Data Formats for Transferring Resource Information

Four data formats are defined by the LWM2M Enabler in this section. The LWM2M Server MUST support all data formats. The plain text and opaque formats MUST be supported by the LWM2M Client. The LWM2M Client MUST support the TLV data format for Object Instance or multiple-instance Resource requests.

The Object specification defines the data format that a Resource supports, either plain text or opaque for singular Resources or TLV for multiple instance Resources.

In addition to the data formats defined in the Object specification, a LWM2M Client MAY choose to support the JSON format for Object Instance or multiple instance Resource requests.

### Plain Text

The plain text format is used for write and read operations on singular Resources where the value of the resource is simply represented as as an UTF-8 encoded string. This string can contain a character sequence, integer number, decimal number or any other sequence of valid UTF-8 characters.

For example a request to Resource 0 from Object Instance of Object 3 would return the following plain text payload:

Open Mobile Alliance

This data format has a Media Type of text/plain.

### Opaque

The opaque format is used for write and read operations on singular Resources where the value of the resource is an opaque sequence of binary octets. This data format is used for binary resources such as firmware images or application specific binary formats.

This data format has a Media Type of octet-stream.

### TLV

For requests to Object Instance or Resource with multiple instances, the binary TLV (Tag-Length-Value) format is used to represent an array of values or a singular value using a company binary representation, which is easy to process on simple embedded devices. The 32-bit TLV tags have the capability to represent any Resource Instance of any Object Instance

The format is an array of the following byte sequence, where each array entry represents a Resource or Resource Instance:

Table 13 TLV format and description

| Field | Format and Length | Description |
| --- | --- | --- |
| Tag | 32-bit unsigned integer | The first 8-bit represent Object Instance ID, the following 16-bit represent the Resource ID, the last 8-bit represent the Resource Instance ID. |
| Length | 16-bit unsigned integer | The Length of the following field in bytes |
| Value | Sequence of bytes, length indicated by the Length field | Value of the tag. Type of the value depends on the Resource’s data type. |

For example a request to Example Resource 1 of Object 3 Instance 1 (the Battery level of Device Object) is:

Tag=0x01000B00, Length=0x0000.

And the example return is:

Tag=0x01000B00, Length=0x0010, Value= 0x0064 (value 100 in decimal).

This data format has a Media Type of application/lwm2m-tlv.

Editor’s Note: Align this media type with DM style, and register with IANA

### JSON

For requests to Object Instance or Resource with multiple instances, a JSON format may be used where a set of values and metadata is represented. Each entry of the JSON format is a resource, where the name is the URI path relative to the request URI.

The JSON is useful for returning multi-level resources from the resource tree, for example requesting an entire instance (all Objects Instances, Resources and Resource Instances at once). The JSON format also includes optional time fields, which allows for multiple versions of representations to be sent in the same payload. The time fields are only used when sending notifications.

This data format has a Media Type of application/senml+json.

Editor’s Note: Align this media type with DM style, and register with IANA ?

The format when an Object or Object Instance is requested follows the following syntax:

{"e":[

{"n": "URI Path", "t": "Time", "v": Float Value, "bv": Boolean Value, "sv":"String Value"},

{"n": "URI Path", "t": "Time", "v": Float Value, "bv": Boolean Value, "sv":"String Value"}],

"bt":"Base Time"

}

Table 14 JSON fields

| Field | JSON Variable | Mandatory? | Description |
| --- | --- | --- | --- |
| Object Root | e | Yes | The root of the value array. |
| URI Path | n | Yes | The path of the resource relative to the request URI (/Object/Object Instance/Resource/Resource Instance) |
| Time | t | No | The time of the representation relative to the Base Current Time in seconds (a negative integer) for a notification. Required only for historical representations. |
| Base Time | bt | No | The base current time which the Time values are relative to. |
| Float Value | v | One value field is mandatory | Value as a JSON float if the resource data type is integer or decimal. |
| Boolean Value | bv |  | Value as a JSON Boolean if the resource data type is boolean. |
| String Value | sv |  | Value as a JSON string for all other resource data types. |

Editorial note: the format for UNIX time to be defined

Editorial note: the time format (integer) to be defined for the base time.

For example a request to Example Object 2 Instance 1 (Read /2/1) in the example would return the following JSON payload:

{“e”:[

{"n":"0","sv":"Open Mobile Alliance"},

{"n":"1","sv":"LWM2M v1.0"},

{"n":"2","sv":"9347112"}]

}

For example a notification about a Resource containing multiple historical representations of a temperature resource in the example could result in the following JSON payload:

{“e”:[

{"n":"1/2","v":"22.4","t":"-5"},

{"n":"1/2","v":"22.9","t":"-30"},

{"n":"1/2","v":"24.1","t":"-50"}],

"bt":"25462634"

}

# Security

## Channel Security

The LWM2M protocol is based on [CoAP] principles. [CoAP] runs over UDP. The UDP channel security is defined by the Datagram Transport Layer Security (DTLS) [RFC6347] which is the equivalent of TLS v1.2 [RFC5246] for HTTP.

DTLS supports most of the Cipher Suites defined in TLS. (Refers to TLS Cipher Suite registry http://www.iana.org/assignments/tls-parameters/tls-parameters.xml)

Considering that every M2M device can be managed by any LWM2M server the choice of Cipher Suites is not limited to the list defined in Section 9 of [CoAP].

The DTLS binding for [CoAP] is defined in Section 9 of [CoAP]. DTLS is a long-lived session based security solution for UDP. It provides a secure handshake with session key generation, mutual authentication, data integrity and confidentiality. DTLS is also used for authorization on individual [CoAP] resources.

An LWM2M Client and Server SHOULD keep a DTLS session in use as long as possible (even across sleep cycles).

The keying material for all of the modes below is obtained using one of the bootstrap mechanisms defined in section x.

### 8.1.1 Pre-Shared Keys

An LWM2M server MUST support the Pre-Shared Key mode of DTLS with the Cipher Suites below:

  - TLS_PSK_WITH_AES_128_CCM_8 [RFC6655] as defined in Section 9.1.3.1 of [CoAP]

  - TLS_PSK_WITH_AES_128_CBC_SHA256 as defined in [RFC5487]

An LWM2M client MUST support the Pre-Shared Key mode of DTLS with at least one of the Cipher Suites specified above.

The use of other Cipher Suites is OPTIONAL for LWM2M.

For all Cipher Suites using AES in an LWM2M implementation the hashing functions SHALL NOT be SHA-1. The use of SHA256 is recommended.

An LWM2M client will negotiate with the LWM2M server the best method during the DTLS handshake for establishing the DTLS session.

### 8.1.2 Raw Public Key Certificates

If an LWM2M server supports Raw Public Key Certificates it MUST support the Cipher Suites below:

- TLS_ECDHE_ECDSA_WITH_AES_128_CCM_8 as defined in Section 9.1.3.2 of [CoAP]

- TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 as defined in [RFC5289]

If an LWM2M client supports Raw Public Key Certificates it MUST support at least one of the above Cipher Suites.

For ECDHE and ECDSA, SHA-1 SHALL NOT be used and the minimum key length SHALL be 256 bits.

The use of other Cipher Suites is OPTIONAL for LWM2M.

### 8.1.3 X.509 Certificates

If an LWM2M server supports X.509 Certificate mode it MUST support the Cipher Suites below:

- TLS_ECDHE_ECDSA_WITH_AES_128_CCM_8 as defined in Section 9.1.3.3 of [CoAP].

- TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 as defined in [RFC5289]

If an LWM2M client supports X.509 Certificate mode it MUST support at least one of the above Cipher Suites.

For ECDHE and ECDSA, SHA-1 SHALL NOT be used and the minimum key length SHALL be 256 bits.

The use of other Cipher Suites is OPTIONAL for LWM2M.

This mode requires the use of X.509v3 Certificates [RFC5280]. Certificates used in LWM2M SHOULD be signed by a root certificate, either by a public root CA or a private root.

A Client Certificate MUST include the Endpoint Name used to register the device in the Subject Common Name (CN) field of the Certificate. Upon registration, the Server MUST check that this CN field matches the Endpoint Name parameter of the registration message during authentication and MUST reject the handshake if these fields do not match. The Server SHOULD also verify that the Certificate is signed by the indicated Issuer.

A Server Certificate SHOULD include Subject and/or SubjectAltName fields listing its known DNS names and IP addresses which are included in the LWM2M Server URI resources of endpoints. A Client SHOULD check that these fields of the Certificate match the URI used to register with the Server. The Client SHOULD also verify that the Certificate is signed by the indicated Issuer.

## Access Control

### Access Control List (ACL)

To authorize the logical operation sent from the LWM2M Server, the LWM2M Client uses Access Control List (ACL) to determine which logical operations a LWM2M server can perform on the resources in the LWM2M Client. ACL is assigned per Object Instance. The ACL value is applied to all the Resources which belong to the Object Instance. ACL is stored in an Access Control Object Instance.

ACL Resource has list of Resource Instances called ACL entry consisting of Short Server ID and access right of the corresponding LWM2M Server. Using the Short Server ID instead of the LWM2M Server URI can reduce the space overhead and increase the processing efficiencyDetailed information of ACL is described in Appendix B.2.

Default ACL entry consisting of default Short Server ID (i.e. 0) and its access right MAY be used to grant access rights to LWM2M Servers which are not specified in the ACL Resource. It means that the LWM2M Servers which own their ACL entry in ACL Resource have the access right of that ACL entry and the other LWM2M Servers which do not own their ACL entry in the ACL Resource have the access right of the default ACL entry if default ACL entry exists in ACL Resource. If ACL Resource has default ACL entry only all the LWM2M Servers have the same access right of the default ACL entry.

Each Access Control Object Instance MUST be managed by an LWM2M Server called Access Control Owner. The other Servers except Access Control Owner MUST not manage the Access Control Object Instance.

### Access Type

Access type defines which logical operation the Resource supports. Therefore access type is assigned per Resource and all the Resource Instances inherit the access type. The access type is 1 byte and each bit of the access type represents whether the respective logical operation is supported by the Resource or not. For example, if 1st bit of access type is 1, it means that the Resource supports read logical operation.

Table 15 Access Type operations

| Field | Format and Length | Description |
| --- | --- | --- |
| Access Type | 1 byte | 1st lsb: Read 2nd lsb: Write 3rd lsb: Execute 4th lsb: Create - 5th lsb: Delete Other bits are reserved for future use |

### Authorization

For authorizing logical operation sent from the LWM2M Server, the LWM2M Client verifies the access based on the conjunction of the access right and the access type. This chapter specifies how the LWM2M Client obtains access rights of the LWM2M Server and authorizes logical operation on Resource and on Object Instance separately.

The LWM2M Server and the LWM2M Client MUST support the authorization procedure described in this section.

#### Obtaining Access Right

For obtaining access right of Object Instance for a certain LWM2M Server, the LWM2M Client MUST perform the following procedure:

- If the LWM2M Client has only one LWM2M Server Object Instance, the LWM2M Server has full access right (i.e. create, write, read, execute, delete) without checking Access Control Object Instance.

- If the LWM2M Client has more than one LWM2M Server Object Instance, the LWM2M Client finds an Access Control Object Instance which contains ACL of Object Instance which the LWM2M Server accesses.

  - If the ACL Resource has ACL entry which contains Short Server ID of the LWM2M Server, the LWM2M Server has an access right as per the ACL entry.

  - If ACL doesn’t have ACL entry of the LWM2M Server, the LWM2M Server has an access right as per the  default ACL entry if the default ACL entry exists.

  - If ACL doesn’t have ACL entry of the LWM2M Server and the default ACL entry doesn’t exist, the LWM2M Server has no access right.

#### Operation on Resource

If the LWM2M Server accesses a Resource, the LWM2M Client obtains an access right of the LWM2M Server for the Object Instance that Resource belongs to according to 8.2.3.1 and checks whether the access right is granted to perform the logical operation. If it is not granted, the LWM2M Client MUST send “ACL Permission Denied” error code to the LWM2M Server. If the access right is granted, the LWM2M Client verifies whether the Resource supports the logical operation. If the logical operation is not supported by the Resource, the LWM2M Client MUST send “Access Type Permission Denied” error code to the LWM2M Server. If the Resource supports the logical operation, the LWM2M Client performs the logical operation and sends response if needed.

#### Operation on Object Instance

If the LWM2M Server accesses an Object Instance, the LWM2M Client obtains an access right of the LWM2M Server for Object Instance according to 8.2.3.1 and checks whether the access right is granted to perform the logical operation. If the logical operation is not granted, the LWM2M Client MUST send “ACL Permission Denied” to the LWM2M Server. If the access right is granted, the LWM2M Client checks whether each Resource supports the logical operation. If the logical operation is “write”, the LWM2M Client MUST perform the logical operation on the Object Instance and sends response only if all the Resources in the operation are allowed to perform the “write” logical operation. If it is not allowed, the LWM2M Client MUST inform the LWM2M Server of the Resources which don’t support the logical operation by sending “Access Type Permission Denied” error code for the Resources. If the logical operation is “read”, the LWM2M Client MUST retrieve all the Resources except the Resource(s) which doesn’t support “read” operation and sends the retrieved Resource(s) information to the LWM2M Server. If the logical operation is “execute”, the LWM2M Client MUST not perform the logical operation.

#### 8.2.3.3 Observe/Notify Operation Consideration

If the LWM2M Server sends an Observe operation on an Object Instance or a Resource, the Client MUST check whether the LWM2M Server is authorized to read operation on the Object Instance or the Resource according to 8.2.3.1. If the LWM2M Server is not authorized, the Client MUST reject the Observe operation and send error response.

If the Client needs to send a Notify operation containing an Object Instance or a Resource to the Server, the Client MUST check whether the Server is authorized to read operation on the Object Instance or the Resource according to 8.2.3.1. If the LWM2M Server is not authorized, the Client MUST NOT send the Notify operation and MUST perfom Cancel Observation operation.

# Transport Layer Binding and Encoding

The LWM2M interfaces use the IETF Constrained Application Protocol [CoAP] as an underlying transfer protocol across IP and SMS bearers. This protocol defines the message header, request/response codes, message options and retransmission mechanisms. This section defines the subset of features from the IETF CoAP specification to be used by LWM2M interfaces.

## Required Features

For realization of the LWM2M interfaces, only the basic binary CoAP message header, and a small subset of options are required. This section explicitly defines the features of the CoAP standard that are required for LWM2M.

- The 4-byte binary CoAP message header is defined in Section 3 of [CoAP]. This same base message is used for Request and Response interactions.

- Confirmable, Acknowledgement and Reset messages MUST be supported. The Reset message is used as a message layer error message in response to a malformed Confirmable message. Non-Confirmable messages MAY be used by a Client for sending Information Reporting notifications as per [Observe].

- GET, PUT, POST and DELETE methods MUST be supported. LWM2M Operations map to these methods.

- A subset of Response Codes MUST be supported for LWM2M response message mapping.

- The Uri-Path Option MUST be supported to indicate the identifier of the interface, Object Instance and Resource being requested.

- The Location-Path Option MUST be supported to indicate the handle of a registration for future update and delete operations.

- The Uri-Query Option MUST be supported.

- The Content-Type Option MAY be used to indicate the media type of the payload. A default value of plain/text is assumed, allowing this option to be elided for most payloads.

- The Token Option MAY be used to enable multiple requests in parallel with an endpoint, and MUST be supported for the Information Reporting interface.

## URI Identifier & Operation Mapping

Although CoAP supports a URI in requests, it is not used in the same way as in HTTP. The URI in CoAP is broken down into binary parts, minimizing overhead and complexity. In LWM2M only path segment and query string URI components are needed. The URI path is used to simply identify the interface, Object Instance or Resource that the request is for, and is encoded in Uri-Path options. The LWM2M Registration interface also makes use of query string parameters to pass on meta-data with the request separately from the payload. Each query parameter is encoded in a Uri-Query Option. Likewise, the LWM2M Operations for each interface are mapped to CoAP Methods.

## Firewall/NAT

For a firewall to support LWM2M, it should be configured to allow outgoing UDP packets to destination port 5683 (other ports can be configured), and allow incoming UDP packets back to the source address/port of the outgoing UDP packet for a period of at least 240 seconds.  These UDP packets may contain DTLS or CoAP payloads. When a firewall is configured as such any Clients behind it should use Queue Mode.

For a firewall to support LWM2M it can be configured to allow both outgoing and incoming UDP packets to destination port 5683 (other ports can be configured). These UDP packets may contain DTLS or CoAP payloads. When a firewall is configured as such any Clients behind it are not required to use Queued Mode, but may use it for other reasons (e.g. a battery powered sleeping device).

Any Clients behind a NAT can use Queued Mode. There are other mechanisms to transverse a NAT, however they are out of scope for this specification.

### Registration Interface

The registration interface is used by a LWM2M Client to register with a LWM2M Server, identified by the LWM2M Server URI. Registration is performed by sending a CoAP POST to the LWM2M Server URI, with registration parameters passed as query string parameters as per Table 16 and Object and Object Instances included in the payload as per [RFC6690]. The Client MAY add “</>;ct=50” in the payload to inform the Server of the Client supporting JSON data format for all the Objects. The response includes Location-Path Options, which indicate the path to use for updating or deleting the registration. The server MUST return a location under the /rd path segment.

Registration update is performed by sending a CoAP PUT to the Location path returned to the LWM2M Client as a result of a successful registration.

De-registration is performed by sending a CoAP DELETE to the Location path returned to the LWM2M Client as a result of a successful registration.

Table 16: Operation to Method and URI Mapping

| Logical Operation | CoAP Method | URI | Success | Failure |
| --- | --- | --- | --- | --- |
| Write Uplink | POST | /rd?ep={Endpoint Client Name}&lt={Lifetime}&sms={MSISDN} &lwm2m={version}&q | 2.01 Created | 4.00 Bad Request |
| Update Uplink | PUT | /{location}?ep={Endpoint Client Name}&lt={Lifetime}&sms={MSISDN} &lwm2m={version} | 2.04 Changed | 4.00 Bad Request |
| Delete Uplink | DELETE | /{location} | 2.02 Deleted | 4.00 Bad Request |

Figure 13: Example registration, update and de-registration exchanges (shorthand in [CoAP] example style, actual messages using CoAP binary headers)

### Bootstrap Interface

The bootstrap interface is used to optionally configure a LWM2M Client so that it can successfully register with a LWM2M Server. The client bootstrap operation is performed by sending a CoAP POST request to the LWM2M Boostrap Server at the /bs path including the Endpoint Client Name as a query string parameter.

In client initiated bootstrap, when the Bootstrap Server receives Request Bootstrap logical operation, the Bootstrap Server performs Write logical operation. In server initiated bootstrap, the Bootstrap Server performs Write logical operation. The Write logical operation targets to an Object Instance or a Resource. The Write logical operation can be sent multiple times. Different from Write operation in Device Management and Service Enablement interface, the Client MUST write the payload regardless of an existence of the targeting Object Instance or Resource.

Table 17: Operation to Method and URI Mapping

| Logical Operation | CoAP Method | URI | Success | Failure |
| --- | --- | --- | --- | --- |
| Request Bootstrap | POST | /bs?ep={Endpoint Client Name} | 2.04 Changed | 4.00 Bad Request |
| Write | PUT | /{Object ID}/{Object Instance ID}/ {Resource ID} | 2.04 Changed | 4.00 Bad Request |

Figure 14: Example of Client initiated Bootstrap exchange.

### Device Management & Service Enablement Interface

The Device Management & Service Enablement Interface is used to access an Object Instance or an individual Resource of an Object Instance. An Object Instance is identified by the path /{Object ID}/{Object Instance ID}. If Object doesn’t support multiple Object Instances, the Object Instance is identified by the path /{Object ID}/0. A Resource is identified by the path /{Object ID}/{Object Instance ID}/{Resource ID}.

An Object Instance or Resource is Read by sending a CoAP GET to the corresponding path. The response includes the value in the corresponding Plain Text, TLV or JSON format.

An Object Instance or Resource is Written to by sending a CoAP PUT to the corresponding path. The request includes the value to be written in the corresponding Plain Text, TLV or JSON format.

A Resource is Executed by sending a CoAP POST to the corresponding path.

An Object Instance is created by sending a CoAP POST to the corresponding path. The request includes the value to be written in the corresponding TLV or JSON format.

An Object Instance is deleted by sending a CoAP DELETE to the corresponding path.

Table 18:  Operation to Method Mapping

| Logical Operation | CoAP Method | Path | Success | Failure |
| --- | --- | --- | --- | --- |
| Read | GET | /{Object ID}/{Object Instnace ID}/{Resource ID} | 2.05 Content | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Write | PUT | /{Object ID}/{Object Instnace ID}/{Resource ID} ?pmin={minimum period}&pmax={maximum period}&gt={greater than}&lt={less than}&st={step} | 2.04 Changed | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Execute | POST | /{Object ID}/{Object Instnace ID}/{Resource ID} | 2.04 Changed | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Create | POST | /{Object ID}/{Object Instance ID} | 2.01 Created | 4.00 Bad Request, 4.05 Method Not Allowed, XXX ID Not Allowed |
| Delete | DELETE | /{Object ID}/{Object Instance ID} | 2.02 Deleted | 4.00 Bad Request, 4.05 Method Not Allowed, XXX instance ID Not Existing |

pmin, pmax, gt, lt, and/or st MAY be specified for using them in observe operation.

Figure 16: Example of Device Management & Service Enablement interface exchanges.

### Information Reporting Interface

Periodic and event-triggered reporting about resource values from the LWM2M Client to the LWM2M Server is achieved through CoAP Observation [OBSERVE]. This simple mechanism allows the LWM2M Server to send an Observe GET request for an Object Instance, which results in asynchronous notifications whenever that Object Instance changes (periodically or as a result of an event). The minimum and maximum period of notifications can be controlled by including the minimum (pmin) and/or maximum (pmax) period for notifications to be sent in seconds.

Table 19: Operation to Method Mapping

| Logical Operation | CoAP Method | Path | Success | Failure |
| --- | --- | --- | --- | --- |
| Read Downlink | GET with Observe option | /{Object ID}/{Object Instnace ID}/{Resource ID} | 2.05 Content with Observe option | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Write Uplink | Asynchronous Response |  | 2.04 Changed |  |

Figure 17: Example of an Information Reporting exchange.

## Queue Mode Operation

When the Client has registered with low communication access availability Queue Mode (it has included the “q” query string parameter when registering), The Server does not immediately send downlink requests on this interface, but instead waits until the Client is online.

A Client lets the Server know it is awake by sending a registration update message as a Confirmable message. The Server then makes any queued requests to the Client in a serial fasion. The Client SHOULD wait at least ACK_TIMEOUT [COAP] seconds from the last CoAP message it sent to the Server before intentionally going offline. If the Server is not successful in sending a request, then it stops emptying the queue and keeps the request for the next time the Client is online.

A typical Queue Mode sequence follows the following steps:

- The device (i.e. LWM2M Client) registers to the LWM2M Server and requests the LWM2M Server to run in Queue mode by including the “q” query string parameter.

- The device uses the CoAP ACK_TIMEOUT parameter to set a timer for how long it shall stay awake since last sent message to the LWM2M Server.

- When LWM2M Server receives a message from the Client (e.g. a notification or a registration update), it checks its request queue for the device and performs the needed CoAP operation(s) (e.g. GET, PUT, and POST). Note: There could be several requests in the queue). Each request is sent serially to the Client, waiting for request to be Acknowledged before sending the next request. If a request is unsuccessful then it is returned to the queue.  The device may have pending Observer notifications.

Below is an example flow for Queue Mode in relation to Device Management & Service Enablement Interface

Figure 3: Example of Device Management & Service Enablement interface exchanges for Queue Mode.

Below is an example flow for Queue Mode in relation to Information Reporting Interface

Figure 4: Example of an Information Reporting exchange for Queue Mode.

## Response Codes

This Chapter lists available response codes of each logical operation. The codes are divided into each interface. These are the only valid response codes defined in this specification.

Table 20: Response Codes

| Logical Operations | Available CoAP Response Codes | Reason Phrase |
| --- | --- | --- |
| Device Discovery and Registration Interface |  |  |
| Register | 2.01 Created | Register operation is completed successfully |
|  | 4.00 Bad Request | The mandatory parameter is not specified or unknown parameter is specified Unknown Endpoint Client Name |
| Update | 2.04 Changed | Update operation is completed successfully |
|  | 4.00 Bad Request | The mandatory parameter is not specified or unknown parameter is specified or URI is not found |
| De-register | 2.02 Deleted | De-register operation is completed successfully |
|  | 4.00 Bad Request | URI is not found |
| Bootstrap Interface |  |  |
| Request Bootstrap | 2.04 Changed | Request Bootstrap is completed successfully |
|  | 4.00 Bad Request | Unknown Endpoint Client Name |
| Write | 2.04 Changed | Write operation is completed successfully |
|  | 4.00 Bad Request | The format of data to be written is different Target (i.e., Object) is not allowed for Write operation |
| Device Management and Service Enablement Interface |  |  |
| Create | 2.01 Created | Create operation is completed successfully |
|  | 4.00 Bad Request | Target (i.e., Resource) is not allowed for Create operation Target (i.e., Object) already exists |
|  | 4.01 Unauthorized | Access Right Permission Denied |
| Read | 2.05 Content | Read operation is completed successfully |
|  | 4.00 Bad Request |  |
|  | 4.04 Not Found | URI of Read operation is not found |
|  | 4.01 Unauthorized | Access Right Permission Denied |
|  | 4.05 Method Not Allowed | Operation is not supported |
| Write | 2.04 Changed | Write operation is completed successfully |
|  | 4.00 Bad Request | The format of data to be written is different Target (i.e., Object) is not allowed for Write operation |
|  | 4.04 Not Found | URI of Write operation is not found |
|  | 4.01 Unauthorized | Access Right Permission Denied |
|  | 4.05 Method Not Allowed | Access Type Permission Denied |
| Delete | 2.02 Deleted | Delete operation is completed successfully |
|  | 4.00 Bad Request | Target (i.e., Object) is not allowed for Delete operation |
|  | 4.01 Unauthorized | Access Right Permission Denied |
| Execute | 2.04 Changed | Execute operation is completed successfully |
|  | 4.00 Bad Request | Target (i.e., Object, Object Instance) is not allowed for Execute operation |
|  | 4.04 Not Found | URI of Execute operation is not found |
|  | 4.01 Unauthorized | Access Right Permission Denied |
|  | 4.05 Method Not Allowed | Operation is not supported |
| Information Reporting Interface |  |  |
| Observe | 2.05 Content | Observe operation is completed successfully |
|  | 4.00 Bad Request | Target (i.e., Object) is not allowed for Observe operation |
|  | 4.04 Not Found | URI of Observe operation is not found |
|  | 4.05 Method Not Allowed | Operation is not supported |
| Cancel Observation | 2.05 Content | Cancel Observation operation is completed successfully |
|  | 4.00 Bad Request | URI is not currently observed |
|  | 4.04 Not Found | URI of Cancel Observation operation is not found |
| Notify | 2.04 Changed | Notify operation is completed successfully |

## Transport Bindings

### UDP Binding

The CoAP binding for UDP is defined in [CoAP]. The protocol has a IANA registered scheme of coap:// and a default port of 5683. The UDP binding is used in NoSec (no security) mode. Reliability over the UDP transport is provided by the built-in retransmission mechanism of CoAP.

### SMS Binding

CoAP is used over SMS in this transport binding by placing a CoAP message in the SMS payload using 8-bit encoding. SMS concatenation MAY be used for messages larger than 140 characters. CoAP retransmission is disabled for this binding. An LWM2M Client indicates the use of this binding by including a parameter (sms) in its registration to the LWM2M Server including the node’s MSISDN number. The LWM2M Client MAY interact with the server using both UDP and SMS bindings.

Editor’s Note: Specification of security needed (DTLS over SMS?), and need to define concatenation, possibly limit the size of a COAP message.

Editor:s Note: RD requirement to configure the reporting channel: notifications preferred over UDP binding or SMS binding if enables (under LWM2M server object).

Data Types

This section defines the data types that a resource can be defined to be.

| Data Type | Description |
| --- | --- |
| String | A UTF-8 string, the minimum and/or maximum length of the String MAY be defined. |
| Integer | An integer value. It SHOULD be defined if the Integer is signed and the maximum range of the value. This data type is also used for the purpose of enumeration. |
| Decimal | A decimal value. It SHOULD be defined if the Decimal is signed and the maximum range of the value. |
| Boolean | A boolean with the value 0 for False and the value 1 for True. |
| Binary | A sequence of binary octets, the minimum and/or maximum length of the String MAY be defined. |
| Time | Unix Time. A 64-bit signed Integer representing the number of seconds since Jan 1st, 1970 in the UTC time zone. |

LWM2M Object Template and Guidelines   (Informative)

This Appendix provides the template to be used for the specification of LWM2M objects. Furthermore, guidelines for the creation of LWM2M objects are provided.

Object Template

Appendix C.x	 LWM2M Object: <LWM2M object name>

Decription:

Object  info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Object Name | 16-bit Unsigned Integer | urn:oma:lwm2m:{oma,ext,x}:{Object ID} | Yes/No |

Resource info:

| Resource Name | Resource ID | Operations | Multiple Instances? | Mandatory? | Type | Range or Enumeration | Units | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Resource Name | 0 | R (Read), W (Write), E (Execute) | Yes/No | Yes/No | String, Integer, Decimal, Boolean, Binary, Time | If any | If any | Description |

Guidelines

LWM2M objects must be registered with with the OMNA Lightweight Object registry. There are three classes of Objects in which an Object can be registered:

- OMA Objects (oma label) – Objects defined by the Open Mobile Alliance.

- 3rd Party SDO Objects (ext label) – Objects defined by a 3rd party SDO.

- Vendor Specific Objects (x label) – Objects defined by a vendor or individual, such an object may be either private (no DDF or Specification made available) or public.

Each one of these classes is assigned a range of IDs by OMNA.

The URN format for an Object is automatically built from the class of Object and the Object ID as follows:

urn:oma:lwm2m:{oma,ext,x}:{Object ID}

LWM2M Objects defined by OMA   (Normative)

This Appendix provides LWM2M Objects defined by OMA. Other organizations and companies may define additional LWM2M according to the guidelines and template provided in Annex B

The following LWM2M objects have been defined by OMA

- LWM2M Server

- Access Control

- Device

- Connectivity

- Firmware

- Location

LWM2M Object: LWM2M Server

Description: This LWM2M objects provides the data related to a LWM2M server, the initial access rights, and security related data.

Object Info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| LWM2M Server | 1 |  | Yes |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Mandatory? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LWM2M Server URI | 0 | R, W | No |  | String | 0 – 255 bytes | - | Uniquely identifies the LWM2M Server, and is in the form: “coaps://host:port”, where host is an IP address or FQDN, and port is the UDP port of the Server. |
| Lifetime | 1 | R, W | No |  | Integer | 32-bit | s | Specify the lifetime of the registration in seconds. |
| Security Mode | 2 | R | No |  | Integer | 8-bit | - | Determines which security mode of DTLS is used 0: Pre-Shared Key mode 1: Raw Public Key mode 2: Certificate mode |
| Security Key | 3 | R | No |  | Binary | Variable | - | Stores security key of security mode. The format of the keying material is defined by the security mode. |
| Short Server ID | 4 | R | No |  | Unsigned Integer | 8-bit | - | Short integer ID, assigned by the LWM2M Client. This identifier uniquely identifies each LWM2M Server configured for the LWM2M Client. default Short Server ID (i.e. 0) MUST not be used for identifying the LWM2M Server |
| Default Minimum Period | 5 | R, W | No |  | Unsigned Integer | 16-bit | s | The default value the Client should use for the Minimum Period of an Observation in the absence of this parameter being included in an Observation. If this Resource doesn’t exist, the default value is 1. |
| Default Maximum Period | 6 | R, W | No |  | Unsigned Integer | 16-bit | s | The default value the Client should use for the Maximum Period of an Observation in the absence of this parameter being included in an Observation. |
| Disable | 7 | E | No |  |  |  |  | If this Resource is executed, this LWM2M Server Object is disabled for a certain period defined in the Disabled Timeout Resource. In this period, the LWM2M Client MUST NOT send any message to the Server and ignore all the messages from the Server. When disabled, Client MUST perform de-registration process and underlying network connection between the Client and Server MUST be disconnected. |
| DisableTimeout | 8 | R, W | No |  | Unsigned Integer | 32-bit | s | A period to disable the Server. After this period, the LWM2M Client MUST perform registration process to the Server. If this Resource is not set, a default timeout value is 86400 (1 day). |
| Notification Storing When Disabled or Offline | 9 | R, W | NO |  | Boolean | 1-bit |  | If true, the LWM2M Client stores Observe Notifications to the LWM2M Server while the LWM2M Server account is disabled or the Client is offline. After the LWM2M Server account is enabled or the Client is online, the LWM2M Client reports the stored Notifications to the Server. If false, the LWM2M Client discards all the Observe Notifications or temporally disables the Observe function while the LWM2M Server is disabled or the Client is offline. The default value is true. The maximum number of storing Notification per the Server is up to the implementation. |
| Binding Preference | 10 | R, W | No |  | Integer | 0-3 |  | This resource defines the transport binding and mode preferred by the Client for this Server. 0: UDP 1: UDP with Queue Mode 2: SMS 3: SMS with Queue Mode |

### B.1.1 Unbootstrapping

If a Server Object Instance is to be deleted, some related resources and configurations need to be deleted or modified. Therefore when Delete operation is sent to a Server Object Instance and Delete operation is authorized, the Client MUST proceed following procedure.

- If there is an Object Instance that can be accessed only by a Server of the Server Object Instance (i.e., the Server is Access Control Owner and there is only one ACL entry for the Server in an Access Control Object Instance), the Object Instance and the corresponding the Access Control Object Instance MUST be deleted

- If an Object Instance can be accessed by multiple Servers including the Server, then

  - An ACL entry for the Server in Access Control Object Instance for the Object Instance MUST be deleted

  - If the Server is Access Control Owner of the Access Control Object Instance, then the Access Control Owner MUST be changed to another Server according to below rules:
The Client MUST choose the Server who has highest sum of each number assigned to an access right ( Write: 1, Delete: 1) for the Access Control Owner. If some Servers get the same with the highest, the Client MUST choose one of them for the Access Control Owner

- Observation from the Server MUST be deleted

- the Server Object Instance MUST be deleted

- Client MUST send de-registration message to the Server

Editor’s Note: Where to put this section is TBD

Editor’s Note: Adding “de-registration message MAY be sent to the Servers” in the description of Factory Reset Resource in Device Object specification

Editor’s Note: whether to inform the Server of new Access Control Owner is TBD

LWM2M Object: Access Control

Description: Access Control Object is used to check whether the LWM2M Server has access right for performing an operation. Each Access Control Object Instance contains ACL for a certain Object Instance.

Object Info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Access Control | 2 |  | Yes |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Mandatory? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Object ID | 0 | R, W | NO |  | Unsigned Integer | 16-bit | - | See Table 3. |
| Object Instance ID | 1 | R, W | NO |  | Unsigned Integer | 8-bit | - | See Table 3. |
| ACL | 2 | R, W | YES |  | Binary | 8-bit | - | Resource Instance ID MUST be the same with Short Server ID of a certain LWM2M Server which has an access right. Resource Instance ID 0 is for default ACL entry. Value corresponding to the Resource Instance ID is 1 byte access right value specified as below. 1st lsb: Read 2nd lsb: Write 3rd lsb: Execute 4th lsb: Create 5th lsb: Delete Other bits are reserved for future use |
| Access Control Owner | 3 | R, W | NO |  | Unsigned Integer | 8-bit | - | Short Server ID of a certain LWM2M Server. This LWM2M Server only can manage these Resources of the Object Instance. |

Object Instance Consideration

This Object MUST be instantiated in two cases:

| Case 1: Object Instance for authorizing Read, Write, Execute and Delete operation | Case 2: Object Instance for authorizing Create operation |
| --- | --- |
| The Object Instance MUST be used for authorizing Read, Write, Execute and Delete operation. In this case, Object ID, Object Instance ID, ACL, and Access Control Owner Resource MUST be specified. Access right for Create operation in the Object Instance MUST be ignored. This Object Instance MUST be managed by Access Control Owner. | The Object Instance MUST be used for authorizing Create operation. In this case, Only Object ID and ACL MUST be specified and the other Resources MUST NOT be specified. Access right except for Create operation MUST be ignored. This Object Instance MUST be managed by Bootstrap interface. The LWM2M Server can only perform Create operation on Object Instances, Object IDs of which are listed in the Object Instance. |

Editor’s note: wording should be improved.

Editors’note: Add Example of Object Instance

LWM2M Object: Device

Description: This LWM2M Object provides a range of device related information which can be queried by the LWM2M Server, and a device reboot and factory reset function.

Object info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Device | 3 |  | No |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Mandatory? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Manufacturer | 0 | R | No |  | String |  | - | Human readable manufacturer name |
| Model Number | 1 | R | No |  | String |  | - | A model identifier (manufacturer specified string) |
| Serial Number | 2 | R | No |  | String |  | - | Serial Number |
| Firmware version | 7 | R | No |  | String |  | - | Current firmware version |
| Reboot | 8 | E | No |  | - |  | - | Reboot the device to restore the Device from unexpected firmware failure. |
| Factory reset | 9 | E | No |  | - |  | - | Perform factory reset of the device to make the Device have the same configuration as at the initial deployment. |
| Power source status | 10 | R | No |  | Integer |  |  | 0=power connected 1=running on battery |
| Battery level | 11 | R | Yes |  | Integer |  | - | Contains the current battery level as a percentage (with a range from 0 to 100). This value is only valid when the value of Power source status is 1. |
| Memory free | 16 | R | No |  | Integer |  | - | Estimated current available amount of storage space which can store data and software in the device (expressed in kilobytes). |
| Error code | 18 | R | Yes |  | Integer |  |  | 0=No error 1=Low battery power 2=External power supply off 3=GPS module fail 4=Low received signal strength 5=Out of memory 6=Peripheral malfunction <more error codes to be specified> When the single device object instance is initiated, there is only one error code resource instance whose value equal to 0 that means no error. When the first error happens, LWM2M Client changes error code resource instance to any non-zero value to indicate the error type. When any other error happens, a new error code resource instance is created. This error code resource MAY be observed by a LWM2M server. If any new error code instance is created, a Notify message should be send to LWM2M Server. How to deal with client’s error report depends on the policy of server. |
| Reset Error Code | 19 | E | No |  | - | - | - | Delete all error code resource instances except the only one that implies no error. |
| Current Time | 20 | R,W | No |  | Integer | 64bits unsigned integer |  | Current UNIX time of the client. The LWM2M Client should be responsible to increase this time value as every second elapses. LWM2M server is able to write this resource to make the client synchronized with a server. |
| Time Zone | 21 | R, W | No |  | Decimal |  |  | Indicates which time zone the LWM2M Device located. GMT+X, where X is the value this resource in hours. |

LWM2M Object: Connectivity Monitoring

Description: This LWM2M objects enables monitoring of parameters related to network connectivity.

In this general Connectivity Object, the resources are limited to the most general cases common to most network bearers. It is recommended to read the description, which refers to relevant standard development organizations (e.g. 3GPP, IEEE).

The goal of the connectivity object is to carry information reflecting the more up to date values of the current connection for monitoring purposes. Resources such as signal quality, signal level; cell id are retrieved during connected mode at least for cellular networks.

Object info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Connectivity Monitoring | 4 |  | No |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Mandatory? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network Bearer | 0 | R | No |  | Integer | 8-bit | - | Indicates the current underlying network bearer among the below network bearer list. 0~20 are Cellular Bearers 0: GSM cellular network 1: TD-SCDMA cellular network 2: WCDMA cellular network 3: CDMA2000 cellular network 4: WiMAX cellular network 5: LTE-TDD cellular network 6: LTE-FDD cellular network 7~20: Reserved for other type cellular network 21~40 are Wireless Bearers 21: WLAN network 22: Bluetooth network 23: IEEE 802.15.4 network 24~40: Reserved for other type local wireless network 41~50 are Wireline Bearers 41: Ethernet 42: DSL 43: PLC 44~50: reserved for others type wireline networks. |
| Available Network Bearer | 1 | R | Yes |  | Integer | 8-bit | - | Indicates list of current available network bearer. Each Resource Instance has the value among the network bearer list. |
| Radio signal strength | 2 | R | No |  | Integer |  | dBm | This node contains the average value of the received signal strength indication used in current network bearer in case Network Bearer Resource has Cellular Networks (RXLEV range 0…64) 0 is < 110dBm, 64 is >-48 dBm) Refers to [3GPP.44018] for more details on Network Measurement Report encoding and [3GPP 45.008] or Wireless Network refers to the appropriate wireless standard. |
| Link Quality | 3 | R | No |  | Integer |  |  | This contains received link quality (e.g., LQI for IEEE 802.15.4, (Range (0..255)), RxQual Downlink (for GSM range is 0…7) Refers to [3GPP.44018] for more details on Network Measurement Report encoding. |
| IP Addresses | 4 | R | Yes |  | String |  | - | The IP addresses assigned to the connectivity interface. (e.g., IPv4, IPv6, etc.) |
| Parent IP Addresses | 5 | R | Yes |  | String |  | - | The IP address of the next-hop IP router in case Network Bearer Resource has 1(Wireless). (e.g., IPv4, IPv6, etc.) Note: these IP Addresses doesn’t indicate the Server IP address |
| Link Utilization | 6 | R | No |  | Integer | 0-100 | % | The average utilization of the link to the next-hop IP router in % in case Network Bearer Resource has 1(Wireless). |
| APN | 7 | R | Yes |  | String |  | - | Access Point Name in case Network Bearer Resource Resource is a Cellular Network. |
| Cell ID | 8 | R | No |  | Integer |  | - | Serving Cell ID in case Network Bearer Resource is a Cellular Network. As specified in TS [3GPP 23.003] and in [3GPP. 24.008]. Range (0…65535) in GSM/EDGE UTRAN Cell ID is 28 bits length Cell Identity in WCDMA/TD-SCDMA. Range: (0..268435455). LTE Cell ID. Length is 28 bits Parameter definitions in [3GPP 25.331]. |
| SMNC | 9 | R | No |  | Integer |  | - | Serving Mobile Network Code. In case Network Bearer Resource has 0(cellular network). Range (0…999) As specified in TS [3GPP 23.003] |
| SMCC | 10 | R | No |  | Integer |  | - | Serving Mobile Country Code. . In case Network Bearer Resource has 0(cellular network). Range (0…999) As specified in TS [3GPP 23.003] |

LWM2M Object: Firmware

Description: This LWM2M Object enables FW management of firmware which is to be updated. This Object includes installing firmware package, updating firmware, and performing actions after updating firmware

Object Info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Firmware | 5 |  | No |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Mandatory? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Package | 1 | W | No |  | Binary | - | - | Firmware package |
| Update | 2 | E | No |  | - | - | - | Updates firmware by using the firmware package stored in Package This Resource is only executable when the value of the State Resource is Downloaded. |
| State | 3 | R | No |  | Unsigned Integer |  |  | Indicates current state with respect to this firmware update. This value is set by the LWM2M Client. 1: Idle (before downloading or after updating) 2: Downloading (The data sequence is on the way) 3: Downloaded If writing the firmware package to Package Resource is done, the state changes to Downloaded. If writing an empty string to Package Resource is done, the state changes to Idle. If performing the Update Resource failed, the state remains at Downloaded. If performing the Update Resource was successful, the state changes from Downloaded to Idle. |
| UpdateSupportedObjects | 4 | R, W | No |  | Boolean | 1-bit |  | If this value is true, the LWM2M Client MUST inform the registered LWM2M Servers of Objects and Object Instances parameter by sending an Update or Registration message after the firmware update operation at the next practical opportunity if supported Objects in the Client have changed, in order for the Servers to promptly manage newly installed Objects. If false, Objects and Object Instances parameter MUST be reported at the next periodic Update message. The default value is false. |
| Update Result | 5 | R | No |  | Unsigned Integer |  | - | 0: Default value. Once the updating process is initiated, this Resource SHOULD be reset to default value. 1: Firmware updated successfully, 2: Not enough storage for the new firmware package. 3. Out of memory during downloading process. 4: Connection lost during downloading process. 5: CRC check failure for new downloaded package. 6: Unsupported package type. This Resource MAY be reported by sending Observe operation. |

- B.5.1 Firmware Update Consideration

If some Objects are not supported after firmware update, the Client MUST delete all the Object Instances of the Objects that are not supported.

LWM2M Object: Location

Description: This LWM2M objects provide a range of device related information which can be queried by the LWM2M server, and a device reboot and factory reset function.

Object info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Location | 5 |  | No |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Mandatory? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitude | 0 | R | No |  | Decimal |  | Deg | The decimal notation of latitude, e.g. -43.5723 [World Geodetic System 1984] |
| Longitude | 1 | R | No |  | Decimal |  | Deg | The decimal notation of longitude, e.g. 153.21760 [World Geodetic System 1984] |
| Altitude | 2 | R | No |  | Decimal |  | m | The decimal notation of altitude in meters above sea level. |
| Uncertainty | 3 | R | No |  | Decimal |  | m | The accuracy of the position in meters. |
| Velocity | 4 | R | No |  | Refers to 3GPP GAD specs |  | Refers to 3GPP GAD specs | The velocity of the device as defined in 3GPP 23.032 GAD specification. This set of values may not be available if the device is static. |
| Timestamp | 5 | R | No |  | Time |  |  | The timestamp of when the location measurement was performed. |

Storage of LWM2M Bootstrap Message on the Smartcard (Normative)

This section aims at specifying the storage mechanism of Bootstrap Message on UICC SmartCard platform type [ETSI TS 102.221] activated in 3G mode.

Editor Note : There is no rational to equip LWM2M device with 2G-only Smart Card.

File structure

The information format is based on [PKCS#15] specification. The Bootstrap Message is located under the PKCS#15 directory allowing the card issuer to decide the identifiers and the file locations. The smartcard operations that are relevant include:

- Application selection

- Cardholder verification

File access (select file, read, write)The [PKCS#15] specification defines a set of files. Within the PKCS#15 application, the starting point to access these files is the Object Directory File (ODF). The EF(ODF) contains pointers to other directory files. These directory files contain information on different types of objects (authentication objects, data objects, etc).  For the purpose of Bootstrap Message, EF (ODF) SHALL contain the EF Record describing the DODF-bootstrap. The EF(ODF) is described in section C.3.1 and [PKCS#15].

EF(ODF) contains pointers to one or more Data Object Directory Files (DODF) in priority order (i.e. the first DODF has the highest priority). Each DODF is regarded as the directory of data objects known to the PKCS#15 application. For the purposes of LWM2M bootstrapping, EF(DODF-bootstrap) contains pointer to the Bootstrap Message, namely LWM2M_Bootstrap File. The EF(DODF-bootstrap) is described in section C.3.2 and [PKCS#15].

The provisioning files are stored as PKCS#15 opaque data objects.

The support of smartcard Bootstrap Message will be indicated  by the presence in the EF DIR (see [ETSI TS 102.221]) of an application template as defined here after.

The RECOMMENDED format of EF(DIR) is a linear fixed record in order to be in line with  [ETSI TS 102.221].

EF (DIR) SHALL contain the application template used for a PKCS#15 application as defined in [PKCS#15]. Application template SHALL consist of Application identifier (tag 0x4F) and Path (tag 0x51) information.

The EF(ODF) and EF(DODF-bootstrap) SHALL be used by the Device to determine the path of the LWM2M_Bootstrap file.

UICC SmartCard platforms can support two modes of activation :  2G and 3G. In the context of LWM2M, for Device simplification, UICC SHALL be activated  in 3G Mode

UICC smartcard platform activated in a 3G mode has the physical and logical characteristics according to [ETSI TS 102.221]. In that case, smartcard operations for accessing the Bootstrap Message are specified in section C2.

Bootstrap Message on UICC (Activated in 3G Mode)

Access to the file structure

To select the PKCS#15 application, the Device:

- SHALL  evaluate the PKCS#15 application template – i.e. PKCS#15 AID - present in the EF (DIR),

- SHALL  open a logical channel using UICC Command MANAGE CHANNEL as specified in [ETSI TS 102.221],

- SHALL  select the PKCS#15 ADF using the PKCS#15 AID as parameter of the UICC Command SELECT, using direct application selection as defined in [ETSI TS 102.221].

LWM2M_Bootstrap file will be located under the PKCS#15 ADF.

Files Overview

Figure 4: File structure for Bootstrap Message on 3G UICC

Access Method

UICC Commands Read Binary and Update Binary, as defined in [ETSI TS 102.221], are used to access bootstrap data.

Access Conditions

The Device is informed of the access conditions of provisioning files by evaluating the “private” and “modifiable” flags in the corresponding DODF-bootstrap files structure.

In the case where one of the above mentioned flag is set, cardholder verification is required. The Device must evaluate the PIN references that must be verified as defined in [ETSI TS 102.221] (ie evaluate FCP)

Requirements on the 3G UICC

To retrieve the Bootstrap Message from the 3G UICC, the Device SHALL perform the following steps:

- Select PKCS#15 file structure as specified in C.2.1.

- Read ODF to locate the DODF-bootstrap,

- Read DODF-bootstrap to locate the LWM2M_Bootstrap file,

- Read the LWM2M_Bootstrap file

Files Description

All files defined are binary files as defined in [ETSI TS 102.221]. These files are read and updated using 3G UICC Commands related to the application they belong to.

Object Directory File, EF ODF

The mandatory Object Directory File (ODF) ([PKCS#15], section 5.5.1) contains pointers to other EFs, each one containing a directory of PKCS#15 objects of a particular class (e.g. DODF-bootstrap). The File ID is specified in [PKCS#15]. The card issuer decides the file size. The EF (ODF) can be read but it SHALL  NOT be modifiable by the user.

The EF (ODF) is described below:

| Identifier: default 0x5031, see [PKCS#15] | Structure: Binary |  | Mandatory |
| --- | --- | --- | --- |
| File size: decided by the card issuer |  | Update activity: low |  |
| Access Conditions: READ ALW UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |
| Description |  |  |  |
| See [PKCS#15] |  |  |  |

Bootstrap Data Object Directory File, EF DODF-bootstrap

This Data Object Directory File provisioning contains directories of provisioning data objects ([PKCS#15], section 6.7) known to the PKCS#15 application.

The File ID is described in the EF (ODF). The file size depends on the number of provisioning objects stored in the smartcard. Thus, the card issuer decides the file size.

| Identifier: 0x6430, See ODF | Structure: Binary |  | Mandatory |
| --- | --- | --- | --- |
| File size: decided by the card issuer |  | Update activity: low |  |
| Access Conditions: READ ALW or Universal / application / Local PIN (UICC, See section C.2) UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |
| Description |  |  |  |
| See hereafter and [PKCS#15] |  |  |  |

The EF (DODF-bootstrap) SHALL  contain information on provisioning objects:

- Readable label describing the provisioning document (CommonObjectAttributes.label). The ME could display this label to the user.

- Flags indicating whether the provisioning document is private (i.e., is protected with a PIN) and/or modifiable (CommonObjectAttributes.flags). The card issuer decides whether or not a file is private (it does not need to be if it does not contain any sensitive information)

- Object identifier indicating a LWM2M boostrap object and the type of the provisioning object (CommonDataObjectAttributes.applicationOID)

- Pointer to the contents of the provisioning document (Path.path)

EF  LWM2M_Bootstrap

Only the card issuer can modify EF  LWM2M_Bootstrap

.

| Identifier: See DODF | Structure: Binary |  | Optional |
| --- | --- | --- | --- |
| File size: decided by the card issuer |  | Update activity: low |  |
| Access Conditions: READ ALW or Universal / application / Local PIN (UICC, See section C.2) UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |
| Description |  |  |  |
| Contains a Bootstrap Message |  |  |  |

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |

Draft/Candidate Version <current version> History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-TS-LightweightM2M-V1_0-20120904-D | 04 Sep 2012 | all | TS baseline agreed as in OMA-DM-LightweightM2M-2012-0078-INP_TS_kick_off |
| Draft Version OMA-TS-LightweightM2M-V1_0-20120918-D | 18 Sep 2003 | 6, 7 | Incorporates input to committee: OMA-DM-LightweightM2M-2012-0083R01-CR_Skeleton_Base_Line OMA-DM-LightweightM2M-2012-0090R02-CR_TS_Resource_Model OMA-DM-LightweightM2M-2012-0061R04-CR_Interfaces |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121024-D | 24 Oct 2012 | 6, 7, Appendix A | OMA-DM-LightweightM2M-2012-0095R01-CR_TS_Interface_and_Resource_Additions |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121030-D | 30 Oct 2012 | 7, 8 | OMA-DM-LightweightM2M-2012-0097R01-CR_Identifiers_and_Security_Considerations |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121117-D | 17 Nov 2012 | 2, 6, 7, 8, 9, 10 | OMA-DM-LightweightM2M-2012-0088R04-CR_Transfer_Protocol OMA-DM-LightweightM2M-2012-0098R02-CR_Bootstrap_Information_and_Modes OMA-DM-LightweightM2M-2012-0099R01-CR_Default_ACL_Entry OMA-DM-LightweightM2M-2012-0100R02-CR_Authorization_Procedure_and_Error_Code OMA-DM-LightweightM2M-2012-0104R01-CR_Registration_Interface |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121130-D | 30 Nov 2012 |  | OMA-DM-LightweightM2M-2012-0107R01-CR_Appendix_for_LWM2M_Objects. OMA-DM-LightweightM2M-2012-0106R02-CR_Information_Interfaces. OMA-DM-LightweightM2M-2012-0108R01-CR_LWM2M_Server_Account_Object. OMA-DM-LightweightM2M-2012-0109R01-CR_Authorization_Update |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121206-D | 06 Dec 2012 | 6 | OMA-DM-LightweightM2M-2012-0110R01-CR_Interfaces_Intro_Update |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121219-D | 19 Dec 2012 | 6,7,8,9, Annex | OMA-DM-LightweightM2M-2012-0111R01-CR_Object_Instance_Introduction OMA-DM-LightweightM2M-2012-0112-CR_Object_Template_Update OMA-DM-LightweightM2M-2012-0113R02-CR_Access_Control OMA-DM-LightweightM2M-2012-0114-CR_Update_Operation_Modification OMA-DM-LightweightM2M-2012-0115-CR_Connection_Control |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130123-D | 22 Jan 2013 | 2, 7, 8, 9, Annex | OMA-DM-LightweightM2M-2012-0101R03-CR_change_of_the_TLV_data_format OMA-DM-LightweightM2M-2012-0117-CR_remove_example_objects_and_resources OMA-DM-LightweightM2M-2013-0001R04-CR_Firmware_Object OMA-DM-LightweightM2M-2013-0003R01-CR_LwM2M_Client_and_Server_Security_Considerations OMA-DM-LightweightM2M-2013-0006-CR_Security_Mode_in_RessourceInfo_Table |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130206-D | 6 Feb 2013 |  | OMA-DM-LightweightM2M-2013-0004R03-CR_SmartCard_Bootstrap OMA-DM-LightweightM2M-2013-0005R01-CR_device_object OMA-DM-LightweightM2M-2013-0007-CR_Object_Instance_Modification |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130226-D | 26 Feb 2013 |  | OMA-DM-LightweightM2M-2013-0002R04-CR_Adding_Creatable_Object OMA-DM-LightweightM2M-2013-0008R02-CR_Improvement_to_the_JSON_format_for_IETF_alignment OMA-DM-LightweightM2M-2013-0013R01-CR_LWM2M_Version_CoAP_Option OMA-DM-LightweightM2M-2013-0014R01-CR_Data_Format_Negotiation OMA-DM-LightweightM2M-2013-0015R02-CR_Notification_Aggregation_and_Reporting OMA-DM-LightweightM2M-2013-0016R03-CR_Connectivity OMA-DM-LightweightM2M-2013-0019R01-CR_SmartCard_Bootstrap_Appendix OMA-DM-LightweightM2M-2013-0020-CR_Response_Code |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130301-D | 01 Mar 2013 |  | OMA-DM-LightweightM2M-2013-0011R03-CR_Failure_indication_for_firmware_object OMA-DM-LightweightM2M-2013-0022R03-CR_TLV_Tags OMA-DM-LightweightM2M-2013-0023R01-CR_location_object |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130314-D | 14 Mar 2013 |  | OMA-DM-LightweightM2M-2012-0116R03-CR_Bootstrap_Interface_Chapter_Modification OMA-DM-LightweightM2M-2013-0018R01-CR_Bootstrap_Interface_Transport_Binding OMA-DM-LightweightM2M-2013-0024R04-CR_Time_Resource OMA-DM-LightweightM2M-2013-0026R05-CR_Erro_Code OMA-DM-LightweightM2M-2013-0027R01-CR_Delete_Object_Instance OMA-DM-LightweightM2M-2013-0028R01-CR_Location_objet_speed_direction |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130409-D | 09 Apr 2013 |  | OMA-DM-LightweightM2M-2013-0047R02-CR_major_TS_cleanup OMA-DM-LightweightM2M-2013-0029R01-CR_Server_Object_Instance_Deletion OMA-DM-LightweightM2M-2013-0030R01-CR_Registration_Update OMA-DM-LightweightM2M-2013-0032-CR_Read_Operation_Update OMA-DM-LightweightM2M-2013-0044R01-CR_Response_Code_Update OMA-DM-LightweightM2M-2013-0034R01-CR_Device_Object_Update OMA-DM-LightweightM2M-2013-0035R01-CR_Bootstrap_Interface_Update OMA-DM-LightweightM2M-2013-0037R01-CR_Access_Control_Update OMA-DM-LightweightM2M-2013-0038R02-CR_Firmware_Object_Update OMA-DM-LightweightM2M-2013-0039-CR_Moving_Response_Code_Chapter |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130412-D | 12 Apr 2013 |  | OMA-DM-LightweightM2M-2013-0054R02-CR_Bootstrap_Process_Update OMA-DM-LightweightM2M-2013-0051-CR_certificate_definition OMA-DM-LightweightM2M-2013-0052-CR_root_resource OMA-DM-LightweightM2M-2013-0053R01-CR_connectivity_object_update OMA-DM-LightweightM2M-2013-0050-CR_object_template_and_datatypes OMA-DM-LightweightM2M-2013-0036R02-CR_Information_Reporting_Update OMA-DM-LightweightM2M-2013-0049R01-CR_queue_mode |

Static Conformance Requirements	(Normative)

The notation used in this appendix is specified in [IOPPROC].

The following is a model of a set of SCR tables.  DELETE THIS COMMENT

SCR for XYZ Client

| Item | Function | Reference | Status | Requirement |
| --- | --- | --- | --- | --- |
| XYZ-C-001 | Something mandatory | Section x.y | M | (XYZ-C-001 OR XYZ-C-003) AND XYZ-C-002 |
| XYZ-C-002 | Something optional | Section x.y | O |  |
| XYZ-C-003 | Dependencies on ZYX | Section x.y | M | ZYX:MCF |
| XYZ-C-004 | Dependencies on ZYX | Section x.y | O | ZYX:OCF |

SCR for XYZ Server

| Item | Function | Reference | Status | Requirement |
| --- | --- | --- | --- | --- |
| XYZ-S-001 | Something mandatory | Section x.y | M | XYZ-S-001 OR XYZ-S-002 OR XYZ-S-003 |
| XYZ-S-002 | Something optional | Section x.y | O |  |
| XYZ-S-003 | Dependencies on ZYX | Section x.y | M | ZYX:MSF |
| XYZ-S-004 | Dependencies on ZYX | Section x.y | O | ZYX:OSF |

<Additional Information>

If needed, add annex to provide additional information to support the document.  In general, this information should be informative, as normative material should be contained in the primary body of the document.

Note that the styles for the headers in the appendix (App1, App2, App3) are different than the main body.  The use below is intended to validate the styles to be used.  Remove if not needed.

DELETE THIS COMMENT

App Headers

<More text>

More Headers

<More text>

Even More Headers

<More text>