| # Lightweight Machine to Machine # Technical Specification |  |  |  |
| --- | --- | --- | --- |
| Draft Version 1.0 – 02 August 2013 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-TS-LightweightM2M-V1_0-20130802-D |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2013 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

# Contents

1.	Scope	7

2.	References	8

2.1	Normative References	8

2.2	Informative References	8

3.	Terminology and Conventions	10

3.1	Conventions	10

3.2	Definitions	10

3.3	Abbreviations	10

4.	Introduction	11

4.1	Version 1.0	11

5.	Interfaces	13

5.1	Bootstrap Interface	14

5.1.1	Bootstrap Information	15

5.1.2	Bootstrap Modes	16

5.1.3	Bootstrap Sequence	18

5.1.4	Bootstrap Security	19

5.2	Device Registration Interface	20

5.2.1	Register	20

5.2.2	Update	23

5.2.3	De-register	23

5.3	Device Management & Service Enablement Interface	23

5.3.1	Read	25

5.3.2	Discover Resources	25

5.3.3	Write	26

5.3.4	Write Attributes	26

5.3.5	Execute	27

5.3.6	Create	27

5.3.7	Delete	28

5.4	Information Reporting Interface	28

5.4.1	Observe	29

5.4.2	Notify	29

5.4.3	Cancel Observation	30

6.	Identifiers and Resources	32

6.1	Resource Model	32

6.2	Identifiers	33

6.2.1	Endpoint Client Name	34

6.2.2	Reusable Resources	35

6.3	Data Formats for Transferring Resource Information	35

6.3.1	Plain Text	35

6.3.2	Opaque	35

6.3.3	TLV	35

6.3.4	JSON	39

7.	Security	41

7.1	UDP Channel Security	41

7.1.1	Pre-Shared Keys	42

7.1.2	Raw Public Key Certificates	42

7.1.3	X.509 Certificates	43

7.1.4	“NoSec” mode	43

7.2	Access Control	44

7.2.1	Access Control Object	44

7.2.2	Authorization	44

8.	Transport Layer Binding and Encodings	46

8.1	Required Features	46

8.2	URI Identifier & Operation Mapping	46

8.2.1	Firewall/NAT	46

8.2.2	Registration Interface	47

8.2.3	Bootstrap Interface	48

8.2.4	Device Management & Service Enablement Interface	50

8.2.5	Information Reporting Interface	53

8.3	Queue Mode Operation	54

8.4	Response Codes	57

8.5	Transport Bindings	59

8.5.1	UDP Binding	59

8.5.2	SMS Binding	59

Appendix A.	Change History (Informative)	60

A.1	Approved Version History	60

A.2	Draft/Candidate Version <current version> History	60

Appendix B.	Data Types	63

Appendix C.	LWM2M Object Template and Guidelines   (Informative)	64

C.1	Object Template	64

C.2	OMNA Guidelines	65

C.2.1	Object Registry	65

C.2.2	Resource Registry	65

Appendix D.	LWM2M Objects defined by OMA   (Normative)	66

D.1	LWM2M Object: LWM2M Server Access Security	67

D.1.1 Security Key Resource Format	68

D.2	LWM2M Object: LWM2M Server	69

D.2.1	Unbootstrapping	70

D.3	LWM2M Object: Access Control	72

D.3.1	Object Instance Consideration	73

D.4	LWM2M Object: Device	74

D.5	LWM2M Object: Connectivity Monitoring	77

D.6	LWM2M Object: Firmware	79

D.6.1	Firmware Update Consideration	80

D.7	LWM2M Object: Location	81

D.8	LWM2M Object: Connectivity Statistics	82

Appendix E.	Example LWM2M Client (Informative)	83

Appendix F.	Storage of LWM2M Bootstrap Information on the Smartcard (Normative)	88

F.1	File structure	88

F.2	Bootstrap Information on UICC (Activated in 3G Mode)	88

F.2.1	Access to the file structure	88

F.2.2	Files Overview	89

F.2.3	Access Method	89

F.2.4	Access Conditions	89

F.2.5	Requirements on the 3G UICC	89

F.3	Files Description	90

F.3.1	Object Directory File, EF ODF	90

F.3.2	Bootstrap Data Object Directory File, EF DODF-bootstrap	90

F.3.3	EF  LWM2M_Bootstrap	91

Appendix G.	Secure channel between Smartcard and LWM2M Device Storage for secure Bootstrap Data provisioning (Normative)	93

Appendix H.	Static Conformance Requirements (Normative)	95

H.1	SCR for LWM2M Client	95

H.2	SCR for LWM2M Server	95

# Figures

Figure 1  The overall architecture of the LWM2M Enabler.	10

Figure 2  The protocol stack of the LWM2M Enabler.	11

Figure 3: Bootstrap	12

Figure 4: Device Discovery and Registration	12

Figure 5: Device Management and Service Enablement	12

Figure 6: Information Reporting	13

Figure 7: Procedure of Client Initiated Bootstrap	15

Figure 8: Procedure of Server Initiated Bootstrap	16

Figure 9: Device Discovery & Registration Interface example flows.	18

Figure 10: Example flows of Device Management & Service Enablement Interface	22

Figure 11: Example flow for Information Reporting Interface for the RSSI resource of the Connectivity Monitoring Object of the example client (Appendix E).	27

Figure 12: Example of Minimum and Maximum periods in an Observation.	28

Figure 13: Relationship between LWM2M Client, Object and Resources	30

Figure 14: Supported operations and access control lists	31

Figure 15: Example registration, update and de-registration exchanges (shorthand in [CoAP] example style, actual messages using CoAP binary headers)	45

Figure 16: Example of Client initiated Bootstrap exchange.	46

Figure 17: Example of Server initiated Bootstrap exchange.	47

Figure 18: Example of Device Management & Service Enablement interface exchanges.	48

Figure 19:  Example of Object Creation and Deletion.	49

Figure 20: Example of an Information Reporting exchange.	50

Figure 21: Example of Device Management & Service Enablement interface exchanges for Queue Mode.	51

Figure 22: Example of an Information Reporting exchange for Queue Mode.	52

Figure 23: Example of Device Management & Service Enablement interface exchanges for Queue Mode with SMS Registration Update Trigger.	53

Figure 24: File structure for Bootstrap Message on 3G UICC	83

Figure 25: Bootstrap Infromation transfer from Smartcard to LWM2M Device using Secure channel according to [GLOBALPLATFORM] [GP SCP03] [GP AMD_A]	88

# Tables

Table 1: Relationship of logical operations and interfaces	13

Table 2: Bootstrap Information List	14

Table 3: Registration parameters	19

Table 4: Read parameters	23

Table 5: Read parameters	23

Table 6: Write parameters	24

Table 7: Write parameters	24

Table 8: Execute parameters	25

Table 9: Create parameters	25

Table 10: Delete parameters	26

Table 11: Observe parameters	27

Table 12: Notify parameters	27

Table 13: Cancel Observation parameters	28

Table 14: LWM2M Identifiers	31

Table 15: TLV format and description	34

Table 17: Operation to Method and URI Mapping	45

Table 18: Operation to Method and URI Mapping	46

Table 19:  Operation to Method Mapping	47

Table 20: Operation to Method Mapping	49

Table 21: Response Codes	53

Table 23: LWM2M Server Access Security Object [0]	78

Table 24: LWM2M Server Access Security Object [1]	78

Table 25: LWM2M Server Access Security Object [2]	79

Table 26: LWM2M Server Object [1]	79

Table 27: LWM2M Server Object [2]	79

Table 28: Access Control Object [0] (for the Device Object)	80

Table 29: Access Control Object [1] (for the Connectivity Monitoring Object)	80

Table 30: Access Control Object [2] (for the Firmware Object)	80

Table 31: Device Object	81

Table 32: Connectivity Monitoring Object	81

# Scope

The present document defines the LWM2M protocol and the initial set of LWM2M objects.

# References

## Normative References

| [3GPP-TS_23.003] |  | 3GPP TS 23.003 “Numbering, addressing and identification” |
| --- | --- | --- |
| [IOPPROC] |  | “OMA Interoperability Policy and Process”, Version 1.1, Open Mobile Alliance™, OMA-IOP-Process-V1_1, URL:http://www.openmobilealliance.org/ |
| [RFC2119] |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |
| [RFC2234] |  | “Augmented BNF for Syntax Specifications: ABNF”. D. Crocker, Ed., P. Overell. November 1997, URL:http://www.ietf.org/rfc/rfc2234.txt |
| [LWM2M-AD] |  | “Lightweight Machine to Machine Architecture”, Open Mobile Alliance™, OMA-AD-LightweightM2M-V1_0, URL:http://www.openmobilealliance.org/ |
| [CoAP] | Shelby, Z., Hartke, K., Bormann, C., and B. Frank, "Constrained Application Protocol (CoAP)", draft-ietf-core-coap-14 (work in progress), Mar 2013. |  |
| [OBSERVE] | Hartke, K. “Observing Resources in CoAP”, draft-ietf-core-observe-08 (work in progress), Mar 2013. |  |
| [RFC4122] | “A Universally Unique Identifier (UUID) URN Namespace”, P. Leach, et al. July 2005, URL:http://www.ietf.org/rfc/rfc4122.txt |  |
| [RFC6690] | Shelby, Z. “Constrained RESTful Environments (CoRE) Link Format”, RFC6690, Aug 2012. |  |
| [RFC6347] | Rescorla, E. and N. Modadugu, "Datagram Transport Layer Security Version 1.2", RFC 6347, January 2012. |  |
| [RFC6655] | McGrew, D. and D. Bailey, "AES-CCM Cipher Suites for TLS", RFC6655, July 2012. |  |
| [RFC5487] | Pre-Shared Key Cipher Suites for TLS with SHA-256/384 and AES Galois Counter Mode |  |
| [RFC5246] | The Transport Layer Security (TLS) Protocol Version 1.2 |  |
| [RFC5289] | TLS Elliptic Curve Cipher Suites with SHA-256/384 and AES Galois Counter Mode (GCM) |  |
| [PKCS#15] | PKCS #15 v1.1: Cryptographic Token Information Syntax Standard”, RSA Laboratories, June 6, 2000. URL: ftp://ftp.rsasecurity.com/pub/pkcs/pkcs-15/pkcs-15v1_1.pdf |  |
| [ETSI TS 102.221] | “Smart Cards; UICC-Terminal interface; Physical and logical characteristics”, (ETSI TS 102 221 release 11), URL:http://www.etsi.org/ |  |
| [GLOBALPLATFORM] | GlobalPlatform v2.2.1 - January 2011 - |  |
| [GP SCP03] | GlobalPlatform Secure Channel Protocol 03 (SCP 03) Amendment D v1.1 Sept 2009 |  |

## Informative References

| [OMADICT] | “Dictionary for OMA Specifications”, Open Mobile Alliance™, OMA-ORG-Dictionary-Vx_y, URL:http://www.openmobilealliance.org/ |
| --- | --- |
| [DMREPPRO] | “OMA Device Management Representation Protocol, Version 1.3”. Open Mobile Alliance. OMA-TS-DM_RepPro-V1_3. URL:http://www.openmobilealliance.org |
| [ETSI TS 102.225] | ETSI TS 102 225 (V11.0.0): "Smart Cards; Secured packet structure for UICC based applications (Release 11)" |
| [ETSI TS 102 226] | ETSI TS 102 226 (V11.0.0): "Smart cards; Remote APDU structure for UICC based applications (Release 11)" |
| [3GPP TS 31.115] | 3GPP TS 31.115 (V10.1.0): "Remote APDU Structure for (U)SIM Toolkit applications (Release 10)" |
| [3GPP TS 31.116] | 3GPP TS 31.116 (V10.2.0): "Remote APDU Structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications (Release 10)" |
| [3GPP2 C.S0078-0] | 3GPP2 C.S0078-0 (V1.0):: "Secured packet structure for CDMA Card Application Toolkit (CCAT) applications" |
| [3GPP2 C.S0079-0] | 3GPP2 C.S0079-0 (V1.0)"Remote APDU Structure for CDMA Card Application Toolkit (CCAT) applications " |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

.

## Definitions

| Queue Mode | The interaction model between an LWM2M Client and LWM2M Server is based on that LWM2M Server queues the requests. |
| --- | --- |
| Kindly consult [OMADICT] for more definitions used in this document. |  |

## Abbreviations

| LWM2M | Lightweight Machine to Machine (refers to this OMA enabler) |
| --- | --- |

Kindly consult (OMADICT] for more abbreviations used in this document.

# Introduction

## Version 1.0

This enabler defines the application layer communication protocol between a LWM2M Server and a LWM2M Client, which is located in a LWM2M Device. The OMA Lightweight M2M enabler includes device management and service enablement for LWM2M Devices. The target LWM2M Devices for this enabler are mainly resource constrained devices. Therefore, this enabler makes use of a light and compact protocol as well as an efficient resource data model.

A Client-Server architecture is introduced for the LWM2M Enabler, where the LWM2M Device acts as a LWNM2M Client and the M2M service, platform or application acts as the LWM2M Server. The LWM2M Enabler has two components, LWM2M Server and LWM2M Client. Four interfaces are designed between these two components as shown below:

- Device Discovery and Registration

- Bootstrap

- Device management and service enablement

- Information Reporting

This architecture is shown in Figure 1. The LWM2M Enabler uses the Constained Application Protocol (CoAP) with UDP and SMS bindings. Datagram Transport Layer Security (DTLS) provides security for the enabler. The LWM2M Enabler protocol stack is shown in Figure 2.

Figure 1  The overall architecture of the LWM2M Enabler.

Figure 2  The protocol stack of the LWM2M Enabler.

# Interfaces

According to the architecture diagram [LWM2M-AD], there are four interfaces: 1) Bootstrap 2) Device Discovery and Registration  3) Device Management and Service Enablement 4) Information Reporting. The logical operations for the four interfaces can be classified as uplink operations and downlink operations. The logical operations of each interface are defined in this section, and then mapped to protocol mechanisms in Section 8 Transport Layer Bindings and Encodings.

Figure 1 shows the logical operation model for interface “Bootstrap”. For this interface, the operations are uplink operation named “Request Bootstrap” and a downlink operation named “Write” and “Delete””. These operations are used to initialize the needed object(s) for the LWM2M Client to register with one or more LWM2M Servers. Bootstrapping is also defined using Manufacturer Pre-configuration (e.g. storage in Flash) or SmartCard Provisioning (storage in a SmartCard).

# Figure 3: Bootstrap

# Editor’s Note: Update above figure (Client Initiated Bootstrap  Request Bootstrap, Server Initiated Bootstrap  Write, Delete)

Figure 2 shows the logical operation model for the interface “Device Discovery and Registration”. For this interface, the operations are uplink operations named “Registration”, “Update” and “De-register”.

# Figure 4: Device Discovery and Registration

Figure 3 shows the logical operation model for interface “Device Management and Service Enablement”. For this interface, the operations are downlink operations named “Read”, “Create”, “Delete”, “Write” and “Execute”. These operations are used to interact with the Resources, Resource Instances, Objects and Object Instances of the LWM2M Client. The “Read” operation is used to read the current value of one or more Resources; the “Write” operation is used to update the value of one or more Resources and the “Execute” operation is used to initiate an action defined by a Resource. The “Create” and “Delete” operations are use to create or delete Object Instances.

# Figure 5: Device Management and Service Enablement

Figure 4 shows the logical operation model for interface “Information Reporting”. For this interface, the operation are downlink operations “Observe” or “Cancel Observation” and an uplink operation “Notify”. This interface is used to send the LWM2M Server a new value related to a Resource on the LWM2M Client.

# Figure 6: Information Reporting

The relationship between logical operations and interfaces is listed in the following table 1.

# Table 1: Relationship of logical operations and interfaces

| Interface | Direction | Logical Operation |
| --- | --- | --- |
| Device Discovery and Registration | Uplink | Register, Update, De-register |
| Bootstrap | Uplink | Request Bootstrap |
| Bootstrap | Downlink | Write, Delete |
| Device Management and Service Enablement | Downlink | Read, Create, Delete, Write, Execute |
| Information Reporting | Downlink | Observe, Cancel Observation |
| Information Reporting | Uplink | Notify |

## Bootstrap Interface

The Bootstrap Interface is used to provision essential information into the LWM2M Client to enable the LWM2M Client to perform the logical operation “Register” with one or more LWM2M Servers.

There are four bootstrap modes supported by the LWM2M Enabler:

- Manufacturer Pre-configuration

- SmartCard Provisioning

- Client Initiated Bootstrap

- Server Initiated Bootstrap

The LWM2M Client SHALL support at least one bootstrap mode specified in the Bootstrap Interface.

The LWM2M Server MUST support all the bootstrap modes specified in the Bootstrap Interface.

This chapter describes what information is conveyed across the Bootstrap Interface, where the LWM2M Client puts that information and how to provision the Bootstrap Information for each of these bootstrap modes.

### Bootstrap Information

This section specifies the information that needs to be configured in LWM2M Client for accessing LWM2M Server(s).
This Information can be available before performing the Bootstrap Sequence described in Section 5.1.3 or obtained as a result of the Bootstrap Sequence.

A LWM2M Server Access Security Object Instance MAY provide the necessary information for the LWM2M Client to contact a LWM2M Bootstrap Server which, in return will  configure the LWM2M Client with the necessary Bootstrap information for registering with a proper  LWM2M Server.

This Bootstrap Information is namely composed of LWM2M Server Object Instance, Server Access Security Object Instance, and ACL Object Instance.

In addition, the LWM2M Client may also require additional LWM2M Objects (e.g., Connectivity Object) that may be required for application purposes. These additional objects may be pre-configured (factory), provisioned (SmartCard) or configured by the LWM2M Bootstrap Server, in the LWM2M Client. The identification of which objects, beyond the ones mandatory for registration is implementation specific and beyond the scope of the LWM2M Enabler.

The LWM2M Client MAY be configured to use one or more LWM2M Servers with a set of Bootstrap Information for each LWM2M Server.

The LWM2M Client SHALL be pre-configured for being able to register with at least one LWM2M Server.

The LWM2M Client SHOULD be pre-configured with a proper LWM2M Access Security Object Instance allowing to register with the LWM2M Bootstrap Server.

The LWM2M Server Access Security Object Instances are only manageable by LWM2M Bootstrap Sever and Smart Card provisioning.

# Table 2: Bootstrap Information List

| Entity | Semantics | Description | Required |
| --- | --- | --- | --- |
| LWM2M Server Access Security | Object Instance | Stores keying material for accessing a given LWM2M Server according to Appendix D.1 | Yes |
| LWM2M Server | Object Instance | Stores account of a LWM2M Server according to Appendix D.2 | No |
| LWM2M ACL | Object Instance | Stores a LWM2M Server ACL (default) according to Appendix D.3 | No |
| Additional Objects (e.g.: Connectivity Object) | Object Instances | Stores the specific Object information | No |

### Bootstrap Modes

This section of the specification provides description and further information for each of the following Bootstrap Modes:

- Manufacturer Pre-configuration

- SmartCard Provisioning

- Client Initiated Bootstrap

- Server Initiated Bootstrap

#### Manufacturer Pre-configuration

In this mode, the LWM2M Client has been configured with the necessary Bootstrap Information prior to deployment of the device.

#### Smartcard Provisioning

When the Device supports a Smartcard, the LWM2M Client SHALL retrieve  and process the bootstrap data contained in the SmartCard as described in Appendix F. When the bootstrap data retrieval is successful, the LWM2M Client SHALL process the bootstrap data from the Smartcard and SHALL apply the Bootstrap Information to its configuration.

Due to the sensible nature of the Bootstrap information, a secure channel SHOULD be established between the Smartcard and the LWM2M Device.

When such a secure channel is established between the Smartcard and the LWM2M Device, this secure channel SHALL be based on [GLOBALPLATFORM]   procedure, mainly described in Appendix G.

In this mode, the LWM2M Client SHALL also ensure that the Bootstrap Information previously retrieved from the SmartCard is unchanged within the SmartCard. If Bootstrap Information is changed, the previous Bootstrap Information SHALL be disabled in the LWM2M Client and the LWM2M Client SHALL apply the new Bootstrap Information from SmartCard to its configuration.

Disabling the bootstrap data (e.g., removing the SmartCard) within the LWM2M Client requires the Bootstrap Information created from the bootstrap data of the previous SmartCard SHALL be deleted.

Checking for SmartCard Change and Disabling, SHALL be performed by LWM2M Client, each time a “registration” or  “registration update” operation take place, with a LWM2M Server provisioned from SmartCard. As usual, the Bootstrap security rules (5.1.4) then apply.

Informative Note: Bootstrap Information in Smartcard can be updated by using Smartcard OTA protocol as specified in ETSI TS 102 225 [ETSI TS 102.225] / TS 102 226 [ETSI TS 102 226] and extensions such as 3GPP TS 31.115 [3GPP TS 31.115] / TS 31.116 [3GPP TS 31.116] and 3GPP2 C.S0078-0 [3GPP2 C.S0078-0] / C.S0079-0 [3GPP2 C.S0079-0].

#### Client Initiated Bootstrap

As defined in section 5.1.3 Bootstrap Sequence, scenarios exist when the LWM2M Server is not configured within the LWM2M Client or attempts to perform the logical operation “Register” with LWM2M Servers have failed.

When these conditions occur, the Client Initiated Bootstrap mode provides a mechanism for the LWM2M Client to retrieve the Bootstrap Information from a LWM2M Bootstrap Server.

The Client Initiated Bootstrap mode requires of having configured in the LWM2M Client,  a LWM2M Access Security Server Object Instance referencing the Bootstrap Server  .

The figure below depicts the Client Initiated Bootstrap flow.

# Figure 7: Procedure of Client Initiated Bootstrap

# Step #1: Request bootstrap to bootstrap URI

The LWM2M Client sends a “Request Bootstrap” logical operation to LWM2M Bootstrap Server URI which has been pre-provisioned. When requesting the bootstrap, the LWM2M Client sends the LWM2M Client’s “Endpoint Client Name” as a parameter in order to allow the LWM2M Bootstrap Server to provision the proper Bootstrap Information for the LWM2M Client.

Step #2: Configure Bootstrap Information

The LWM2M Bootstrap Server configures the LWM2M Client with the Bootstrap Information using the “Write” and/or “Delete”  logical operation.

#### Server Initiated Bootstrap

In this mode, the LWM2M Bootstrap Server configures the Bootstrap Information in the LWM2M Client without the LWM2M Client sending a bootstrap request to the LWM2M Bootstrap Server.

As the LWM2M Client does not initiate the “Request Bootstrap” logical operation to the LWM2M Bootstrap Server, the LWM2M Bootstrap Server needs to know if a LWM2M Device is ready for bootstrapping before the LWM2M Client can be configured by the LWM2M Bootstrap Server. The mechanism that a LWM2M Bootstrap Server gains this knowledge is implementation specific. A common scenario is that elements in the Network Provider’s network informs the LWM2M Bootstrap Server of the LWM2M Device when the LWM2M Device connects to the Network Provider’s network.

Once the LWM2M Bootstrap Server has been notified that the LWM2M Device is ready to receive the Bootstrap Information, the LWM2M Bootstrap Server configures the LWM2M Client with the Bootstrap Information using the “Write” and/or “Delete” logical operation.

The figure below depicts the Server Initiated Bootstrap flow.

# Figure 8: Procedure of Server Initiated Bootstrap

# Step #1: Configure Bootstrap Information

The LWM2M Bootstrap Server configures the Bootstrap Information in the LWM2M Client using the “Write” and/or “Delete” logical operation.

### Bootstrap Sequence

The LWM2M Client SHALL follow the procedure specified as below when attempting to bootstrap a LWM2M Device:

- If the LWM2M Device has SmartCard, the LWM2M Client tries to obtain Bootstrap Information from the SmartCard using the SmartCard Provisioning bootstrap mode.

- If the LWM2M Client is not configured using the SmartCard Provisioning bootstrap mode, the LWM2M Client tries to obtain the Bootstrap Information by using Manufacturer Pre-configuration bootstrap mode.

- If the LWM2M Client has any LWM2M Server Object Instances from the previous steps, the LWM2M Client tries to register to the LWM2M Server(s) configured in the LWM2M Server Object Instance(s).

- If LWM2M Client fails to register to all the LWM2M Servers or the Client doesn’t have any LWM2M Server Object Instances, and the LWM2M Client hasn’t received a Server Initiated Bootstrap within the ClientHoldOffTime, the LWM2M Client performs the Client Initiated Bootstrap.

### Bootstrap Security

The information conveyed through the Bootstrap Interface is sensitive and requires that communication session, security mechanisms and/or keys MUST be different instances from the one that is used for the other LWM2M Interfaces.

If the LWM2M Client or the LWM2M Server needs to convey Bootstrap Information across the Bootstrap Interface, the LWM2M Client or the LWM2M Server MUST establish a new secure communication session.

If security materials (e.g. LWM2M Server URI, Security Mode, and Security Key), are changed in the LWM2M Client, the LWM2M Client MUST disconnect the existing communication session between the LWM2M Server and LWM2M Client and establish a new secure communication session between the LWM2M Server and LWM2M Client using the security mechanism and/or keys have been configured by Bootstrap Interface.

## Device Registration Interface

The Device Registration Interface is used by a LWM2M Client to register with one or more LWM2M Servers, maintain each registration and de-register from a LWM2M Server. The registration is based on the Resource Model and Identifiers defined in Section 6 Identifiers and Resources. When registering, the LWM2M Client performs the “Register” logical operation and provides the properties the LWM2M Server requires to contact the LWM2M Client (e.g., End Point Name); maintain the registration and session (e.g., Lifetime, Queue Mode) between the LWM2M Client and LWM2M Server as well as knowledge of the Objects the LWM2M Client supports and existing Object Instances in the LWM2M Client. The registration is soft state, with a lifetime indicated by the Lifetime Resource of that LWM2M Server Object Instance. The LWM2M Client periodically performs an update of its registration information to the registered LWM2M Server(s) by performing the “Update” logical operation. If the lifetime of a registration expires without receiving an update from the LWM2M Client, the LWM2M Server removes the registration. Finally, when shutting down or discontinuing use of a LWM2M Server, the LWM2M Client performs a “De-register” logical operation.

The Binding Resource of the LWM2M Server Object informs the LWM2M Client of the transport protocol preferences of the LWM2M Server for the communication session between the LWM2M Client and LWM2M Server. The LWM2M Client SHOULD perform the logical operations with the modes indicated by the Binding Resource of the LWM2M Server Object Instance.

# Figure 9: Device Registration Interface example flows.

### Register

Registration is performed when a LWM2M Client sends a “Register” logical operation to the LWM2M Server. After the LWM2M Device is turned on and the bootstrap procedure has been completed, the LWM2M Client MUST perform a “Register” logical operation to each LWM2M Server that the LWM2M Client has a Server Object Instance.  Table 3 describes the parameters used for the “Register” logical operation.

The “Register” logical operation includes the Endpoint Client Name parameter along with other parameters listed in Table 3. The “Register” logical operation MUST include a value for the Endpoint Client Name parameter that is unique on that LWM2M Server.

Upon receiving a ”Register” logical operation from the LWM2M Client, the LWM2M Server records the IP address and port from the IP packet of the registration message and uses this information for all future interactions with that LWM2M Client.

If the LWM2M Client sends a “Register” logical operation to the LWM2M Server even though the LWM2M Server has registration information of the LWM2M Client, the LWM2M Server removes the existing registration information and performs the new ”Register” logical operation. This situation happens when the LWM2M Client forgets the state of the LWM2M Server (e.g., factory reset).

# Table 3: Registration parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Endpoint Client Name | Yes |  | See Section 6.2 |
| Lifetime | No | 86400 | If Lifetime Resource does not exist in a LWM2M Server Object Instance (see section D.1), the Client MUST NOT send this parameter and the Server MUST regard lifetime of the Client as 86400 seconds The registration SHOULD be removed by the Server if a new registration or update is not received within this lifetime. |
| LWM2M Version | No | 1.0 | Indicates the version of the LWM2M Enabler that the LWM2M Client supports. This parameter is required only for LWM2M versions > 1.0. |
| Binding Mode | No | U | Indicates current binding and Queue mode of the LWM2M Client. “U” means UDP binding, and "S" means SMS binding. The "Q" can be appended to represent the binding works in the Queue mode. For example, "UQS" means the Client uses both the UDP binding with Queue Mode enabled and the SMS binding with Queue Mode disabled. The valid values of the parameter are listed in the section 5.2.1.1. |
| SMS Number | No |  | The value of this parameter is the MSISDN where the Client can be reached for use with the SMS binding. |
| Objects and Object Instances | Yes |  | The list of Objects supported and Object Instances available on the Client. |

The list of Objects and Object Instances is included in the payload of the registration message. Each Object is described as a Link in the CoRE Link Format [RFC6690]. The Target component of the link is required, and consists of the Object path. Any other parameters included in the link MUST be silently ignored, unless specified for use by the LWM2M Enabler. The Media Type of this payload is application/link-format.

The payload for a LWM2M Client supporting LWM2M Server, Access Control, Device, Connectivity Monitoring and Firmware Objects from Appendix D would simply be:

</1>, </2>, </3>, </4>, </5>

If Objects Instances are already available on the LWM2M Client at the time of registration, then the format would be (for the example client of Appendix E):

</1/101>, </1/102>, </2/0>, </2/1>, </2/2>, </3/0>, </4/0>, </5>

By default, the RFC6690 links of Objects are located under the root path as in the examples above. However, devices might be hosting other resources on an endpoint, and there may be the need to place Objects under an alternative path. This is achieved by including an OMA LWM2M link in addition to the Object links as follows, e.g. to place Objects under the “/lwm2m” path:

</lwm2m>;rt="oma.lwm2m", </lwm2m/1/101>, </lwm2m/1/102>, </lwm2m/2/0>, </lwm2m/2/1>, </lwm2m/2/2>, </lwm2m/3/0>,</lwm2m/4/0>,</lwm2m/5>

The RFC6690 Resource Type parameter (i.e., rt="oma.lwm2m") MAY be used to provide the information that the path in front of the Resource Type parameter is used for the LWM2M enabler.

The Resource Type value “oma.lwm2m” is registered with the appropriate IANA registry for this purpose.

If the LWM2M Client supports the JSON data format for all the Objects it SHOULD inform the LWM2M Server by including the content type in the root path link using the ct= link attribute. An example is as follows (note that the content type value 100 is an example, the actual value will be assigned by IANA for the LWM2M JSON format).

</>;rt="oma.lwm2m";ct=100, </1/101>, </1/102>, </2/0>, </2/1>, </2/2>, </3/0>, </4/0>, </5>

#### Behavior with Current Transport Binding and Mode

Behavior of the LWM2M Server and the LWM2M Client is differentiated by Current Transport Binding and Mode. Current Transport Binding and Mode is decided by “Binding” Resource set by the LWM2M Server and whether SMS and/or Queue Mode are supported by the LWM2M Client. Queue Mode is useful when the LWM2M Device is not reachable by the LWM2M Server at all the times and it could help the LWM2M Client sleep longer. Table 4 describes the behaviour of the LWM2M Server and the LWM2M Client for each Current Transport Binding and Mode.

# Table 4: Behaviour with Current Transport Binding and Mode

| Current Transport Binding and Mode | Behaviour |
| --- | --- |
| U (UDP) | The LWM2M Server expects that the LWM2M Client is reachable via the UDP binding at any time. The LWM2M Server MUST send requests to a LWM2M Client using the UDP binding. The LWM2M Client MUST send the response to such a request over the UDP binding. This is the normal default mode of operation. |
| UQ (UDP with Queue Mode) | The Server MUST queue all requests to the LWM2M Client, sending requests via UDP when the Client is on-line as described in Section 8.4 Queue Mode Operation. The LWM2M Server MUST send requests to a LWM2M Client using the UDP binding. The LWM2M Client MUST send the response to such a request over the UDP binding. |
| S (SMS) | The LWM2M Server expects that the LWM2M Client is reachable via the SMS binding at any time. The LWM2M Server MUST send requests to a LWM2M Client using the SMS binding. The LWM2M Client MUST send the response to such a request over the SMS binding. |
| SQ (SMS with Queue Mode) | The Server MUST queue all requests to the LWM2M Client, sending requests via SMS when the Client is on-line as described in Section 8.4 Queue Mode Operation. Requests MUST be sent to the LWM2M Client using the SMS binding. The LWM2M Client MUST send the response to such a request over the SMS binding. |
| US (UDP and SMS) | The LWM2M Server expects that the LWM2M Client is reachable via the UDP binding at any time. The LWM2M Server expects that the LWM2M Client is reachable via the SMS binding at any time. If the LWM2M Server sends requests to a LWM2M Client using the UDP binding, The LWM2M Client MUST send the response to such a request over the UDP binding. If the LWM2M Server sends requests to a LWM2M Client using the SMS binding, The LWM2M Client MUST send the immediate response to such a request over the SMS binding. |
| UQS (UDP with Queue Mode and SMS) | The Server MUST queue all requests to the LWM2M Client, sending requests via UDP when the Client is on-line as described in Section 8.4 Queue Mode Operation. The LWM2M Server expects that the LWM2M Client is reachable via the SMS binding at any time. If the LWM2M Server sends requests to a LWM2M Client using the UDP binding, The LWM2M Client MUST send the response to such a request over the UDP binding. If the LWM2M Server sends requests to a LWM2M Client using the SMS binding, The LWM2M Client MUST send the immediate response to such a request over the SMS binding. The LWM2M Server MAY request the LWM2M Client to perform “Update” operation via UDP by sending “execute” logical operation on “Registration Update Trigger” Resource via SMS. |

UQSQ and USQ are not supported.

### Update

Periodically or based on certain events within the LWM2M Client, the LWM2M Client updates its registration information with a LWM2M Server by sending an “Update” logical operation to the LWM2M Server. This ”Update” logical operation MUST contain only changed parameters to update the LWM2M Client status compared to the last registration parameters sent to the LWM2M Server.

The “Update” logical operation is different from ”Register” logical operation in that all the registration parameters specified in Table 3 are OPTIONAL for the “Update” logical operation.

If the LWM2M Client is using the UDP binding to communicate with a LWM2M Server and LWM2M Client’s IP address or the port changes, the LWM2M Client MUST send an “Update” logical operation to the LWM2M Server.

### De-register

When a LWM2M Client determines that it no longer requires to be available to a LWM2M Server (e.g., LWM2M Device shutdown), the Client SHOULD send a “De-register” logical operation to the LWM2M Server. Upon receiving this message, the LWM2M Server removes the registration information from the LWM2M Server.

## Device Management & Service Enablement Interface

The Device Management and Service Enable Interface is used by the LWM2M Server to access Object Instances and Resources available from the LWM2M Client. The interface provides this access though the use of “Create”, “Read”, “Write”, “Delete” or “Execute” logical operations. The logical operations that an Object or Resource supports is defined in the Object definition using the Object Template. The Object Template is described in Appendix B.1 Object Template. . The Normative Objects defined by the LWM2M Enabler are described in Appendix C.

# Figure 10: Example flows of Device Management & Service Enablement Interface

### Read

The “Read” logical operation is used to access the value of a Resource, an array of Resource Instances, an Object Instance or all the Object Instances of an Object. The “Read” logical operation has the following parameters:

# Table 4: Read parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | No | - | Indicates the Object Instance to read. If no Object Instance ID is indicated, then the Object Instances of Objects, which the Server is authorized to, are returned. |
| Resource ID | No | - | Indicates the Resource to read. If no Resource ID is indicated, then the whole Object Instance is returned. |

### Discover Resources

The “Discover” logical operation is used to discover attributes of an individual Resource, or all the Resources of an Object Instance or all the Object Instances of an Object. The returned payload is a list of application/link-format CoRE Links [RFC6690] for each resource including attributes of the resource. The “Discover” logical operation has the following parameters:

# Table 5: Read parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | No | - | Indicates the Object Instance. |
| Resource ID | No | - | Indicates the Resource. |

The response to this request includes the list of resources described using RFC6690 links, along with any attributes configured for targeting Object, Object Instance, or Resource.

If Object ID is only specified, the LWM2M Client responses which Resources are implemented at the LWM2M Client among Resources specified in Object specification. The attributes can be in the response if attributes are configured. For example: if Object ID is 6 without any other parameters,

</6//1>, </6//2>, </6//3>, </6//4>

,which means that Client supports 1,2,3, and 4 Resources among Resources of Connectivity Statistics Object with attributes of that Object.The above example shows the case for an Object with no instances. For cases with object instances, resourse links are returned as below.

If Object ID and Object Instance ID are only specified, all the resource and their attributes of that Object Instance are returned. For example: if Object ID is 3 and Object Instance ID is 2, then

</3/2/1>;pmin=10;pmax=60;lt=42.2
</3/2/1>;pmin=10;pmax=60;lt=42.2
</3/2/1>;pmin=10;pmax=60;lt=42.2

If Object ID, Object Instance ID and Resource ID are specified, the attributes of that Resource are returned. For example: if Object ID is 3, Object Instance ID is 2, and Resource ID is 1, then

</3/2/1>;pmin=10;pmax=60;lt=42.2

If a Resource, an Object Instance, or an Object has attributes for multiple LWM2M Servers, then one link is returned for each and the ep= attribute is used to indicate the Short Server ID of the LWM2M Server. For example: if Object ID is 3 and Object Instance ID is 2, and Resource ID is 1 with two Observe operations from two Servers, then

</3/2/1>;ep=1;pmin=10;pmax=60;lt=42.2,
</3/2/1>;ep=2;pmax=300

### Write

The “Write” logical operation is used to change the value of a Resource, an array of Resources Instances or multiple Resources from an Object Instance. The logical operation permits multiple Resources to be modified within the same instance of the logical operation. The “Write” logical operation has the following parameters:

# Table 6: Write parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | Yes | - | Indicates the Object Instance to write. |
| Resource ID | No | - | Indicates the Resource to write. The payload is the new value for the Resource. If no Resource ID is indicated, then the included payload is an Object Instance containing the Resource values. |
| New Value | Yes | - | The new value included in the payload to update the Object Instance or Resource. |

### Write Attributes

The “Write Attributes” logical operation is used to change the attributes of a Resource or multiple Resources from an Object Instance. The logical operation permits multiple Resources to be modified within the same instance of the logical operation. The “Write Attributes” logical operation has the following parameters:

# Table 7: Write parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | Yes | - | Indicates the Object Instance to write. |
| Resource ID | No | - | Indicates the Resource to write. The payload is the new value for the Resource. If no Resource ID is indicated, then the included payload is an Object Instance containing the Resource values. |
| Minimum Period | No | 1 | When present, the minimum period indicates the minimum time in seconds the Client SHOULD wait from the time when sending the last notification to the time when sending a new notification. In the absence of this parameter, the Minimum Period is defined by the Default Minimum Period set in the LWM2M Server Object Instance related to that Server. |
| Maximum Period | No | - | When present, the maximum period indicated the maximum time in seconds the Client SHOULD wait from the time when sending the last notification to the time sending the next notification (regardless if the value has changed). In the absence of this parameter, the maximum period is up to the server. The maximum period MUST be greater than the minimum period parameter. In the absence of this parameter, the Maximum Period is defined by the Default Maximum Period set in the LWM2M Server Object Instance related to that Server. |
| Greater Than | No | - | When present, the Client SHOULD notify its value when the value is above the number specified in parameter |
| Less Than | No | - | When present, the Client SHOULD notify its value when the value is below the number specified in the parameter |
| Step | No | - | When present, the Client SHOULD notify its value when the value is changed more than the number specified in the parameter from the Resource value when the client receives the Observe operation. |

Minimum Period, Maximum Period, Greater Than, Less Than, and Step are only to do with Observe operation.

Maximum and/or Minimum Period parameters are used to control how often the “Notify” logical operation is sent by the LWM2M Client for the observed Object Instance or Resource.

Greater Than, Less Than, and Step MUST be specified only when Resource ID is indicated.

Greater Than, Less Than, and Step parameters MUST be supported only when the Resource type is numeral (e.g., integer, decimal).

### Execute

The “Execute” logical operation is used to initiate some action, and can only be performed on individual Resources. A LWM2M Client MUST return an error when the “Execute” logical operation is received for anObject Instance(s) or Resource Instance(s).

# Table 8: Execute parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | Yes | - | Indicates the Object Instance. |
| Resource ID | Yes | - | Indicates the Resource to execute. |

### Create

The “Create” logical operation is used for LWM2M Server to create an Object Instance within the LWM2M Client. The “Create” logical operation MUST target either an Object Instance which has not yet been instantiated or the Object.

The Object Instance that is created in the LWM2M Client by the LWM2M Server MUST be an Object type supported by the LWM2M Client and announced to the LWM2M Server using the “Register” and “Update” logical operations of the Device Discovery and Registration Interface.

The Create operation has the following parameters:

# Table 9: Create parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | No | - | Indicates the Object Instance to create. If this Resource is not specified, the Client assigns the ID of the Object Instance. |
| New Value | Yes | - | The new value included in the payload to create the Object Instance. |

### Delete

The “Delete” logical operation is used for LWM2M Server to delete an Object Instance within the LWM2M Client.

The Object Instance that is deleted in the LWM2M Client by the LWM2M Server MUST be an Object Instance that is announced by the LWM2M Client to the LWM2M Server using the “Register” and “Update” logical operations of the Device Discovery and Registration Interface.

The Delete operation has the following parameters:

# Table 10: Delete parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | Yes | - | Indicates the Object Instance to delete. |

## Information Reporting Interface

The Information Reporting Interface is used by a LWM2M Server to observe any changes in a Resource on a LWM2M Client, receiving notifications when new values are available. This observation relationship is initiated by sending an “Observe” logical operation to the L2M2M Client for an Object, an Instance or a Resource. An observation ends when a “Cancel Observation” logical operation is performed.

# Figure 11: Example flow for Information Reporting Interface for the RSSI resource of the Connectivity Monitoring Object of the example client (Appendix E).

### Observe

The LWM2M Server initiates an observation request for changes of a specific Resource, Resources within an Object Instance or for all the Object Instances of an Object within the LWM2M Client.

Related parameters for “Observe” logical operation are described in 5.3.2 Write and those parameters are configured by “Write” logical operation. This separation enables cache mechanism at intermediate network node (e.g., proxy server), the LWM2M Server, and/or LWM2M Client.

The Observe operation includes the following parameters:

# Table 11: Observe parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | No | - | Indicates the Object Instance to observe. If no Object Instance ID is indicated, then all the Object Instances of Objects are observed and Resource ID MUST NOT be specified. |
| Resource ID | No | - | Indicates the Resource to observe. If no Resource ID is indicated, then the whole Object Instance is observed. |

### Notify

The “Notify” logical operation is sent from the LWM2M Client to the LWM2M Server during a valid observation on Object Instance or Resource. This operation includes the new value of the Object Instance or Resource. The “Notify” logical operation SHOULD be sent when conditions (i.e., Minimum Period, Maximum Period, Greater Than, Less Than, Step) for “Observe” logical operation are met.

The LWM2M Client SHOULD NOT send the “Notify” logical operation for the changed Resource or Object Instance before the Minimum Period expires.

The LWM2M Client SHOULD send the “Notify” logical operation for the changed Resource or Object Instance before the Maximum Period expires.

# Table 12: Notify parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Updated Value | Yes | - | The new value included in the payload about the Object Instance or Resource. |

If the minimum period and maximum period value are the same, then the LWM2M Client sends notification in every that period.

The following example shows how the Minimum and Maximum period parameters work as shown in section 5.3.4. A LWM2M Server makes an observation for a Temperature Resource that is updated inside the LWM2M Client at irregular periods (based on change). The LWM2M Server makes an observation when the Minimum Period = 10 Seconds and Maximum Period = 60 Seconds have been set for that Resource a priori. The LWM2M Client will wait at least 10 Seconds before sending a “Notify” logical operation to the LWM2M Server (even if the Resource has changed before that), and no longer than 60 Seconds before sending a “Notify” logical operation (even if the Resource has not changed yet). The “Notify” logical operation is sent anywhere between 10-60 seconds upon change.

# Figure 12: Example of Minimum and Maximum periods in an Observation.

This example assumes the Minimum Period has been set to 10 and the Maximum Period set to 60 for the Resource /4/0/2 before making the observation.

### Cancel Observation

The “Cancel Observation” logical operation is sent from the LWM2M Server to the LWM2M Client to end an observation relationship for Object Instance or Resource. The operation includes the following parameters:

# Table 13: Cancel Observation parameters

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | No | - | Indicates the Object Instance to stop observing. If no Object Instance ID is indicated, then Object ID is indicated and Resource ID MUST NOT be specified. |
| Resource ID | No | - | Indicates the Resource to stop observing. If no Resource ID is indicated, then the whole Object Instance is indicated. |

# Identifiers and Resources

This section defines the identifiers and resource model for the LWM2M Enabler.

## Resource Model

The LWM2M Enabler defines a simple resource model where each piece of information made available by the LWM2M Client is a Resource.  Resources are logically organized into Objects.  Figure 9 illustrates this structure, and the relationship between Resources, Objects and the LWM2M Client. The LWM2M Client may have any number of Resources, each of which belongs to an Object. Resources and Objects have the capability to have multiple instances of the Resource or Object.

# Figure 13: Relationship between LWM2M Client, Object and Resources

Resources are defined per Object, and each Resource is given a unique identifier within that Object. Each Object and Resource is defined to have one or more logical operations that it supports. A Resource MAY consist of multiple instances as defined in the Object specification.

An Object defines a grouping of Resources, for example the Firmware Object contains all the Resources used for firmware update purposes. Each Object is assigned a unique OMA Management Object identifier and corresponding index which identifies an Object defined for the LWM2M Enabler. The LWM2M Enabler defines standard Objects and Resources. Further Objects may be added by OMA or other organizations to enable additional M2M Services.

An Object MUST be instantiated either by the LWM2M Server or the LWM2M Client, which is called Object Instance before using the functionality of an Object. After an Object Instance is created, the LWM2M Server can access that Object Instance and Resources which belong to that Object Instance.

The LWM2M Client and/or LWM2M Server perform logical operations on Objects, Resources, Object Instance and Resources Instances as described in Section 5 Interfaces. These logical operations are conveyed as described in Section 8 Transport Layer Binding and Encoding and how to convey the Operation data is defined in 7.3.

The LWM2M Enabler defines an access control mechanism per Object Instance. Object Instances SHOULD have an associated Access Control Object Instance. An Access Control Object Instances contains Access Control Lists (ACLs) that define which logical operations on the contained Resource are allowed for which LWM2M Server(s). The access control mechanism is defined in Section 7.2 Access Control. Figure 14 shows and example of the logical operations the Resources support and how Object Instances and Resources are associated with ACLs. In the example, Object Instance 0 for Object 0 has 2 Resources. Resource 1 supports the ”Read”, “Write” and ”Execute” logical operations, while Resource 2 supports only the “Read” logical operation.

# Figure 14: Supported operations and access control lists

## Identifiers

The LWM2M Enabler defines specific identifiers for entities used within the LWM2M Protocol. These identifiers are defined in Table 12.

# Table 14: LWM2M Identifiers

| Identifier | Semantics | Description |
| --- | --- | --- |
| Endpoint Client Name | URN | Identifies the LWM2M Client on one LWM2M Server (including LWM2M Bootstrap Server). Provided to the LWM2M Server during Registration, also provided to LWM2M Bootstrap Server when executing the Bootstrap procedure. Recommended URN formats are documented in Section 6.2.1 Endpoint Client Name. |
| LWM2M Bootstrap Server URI | URI | Uniquely identifies the LWM2M Bootstrap Server. Provided to the LWM2M Client during the Bootstrap procedure |
| LWM2M Server URI | URI | Uniquely identifies the LWM2M Server. Provided to the Client during Bootstrap procedure. |
| Short Server ID | 16-bit unsigned integer | Uniquely identifies each LWM2M Server configured for the LWM2M Client. The identifier is assigned during the Bootstrap procedure. Default Short Server ID is 0 and default Short Server ID MUST NOT be used for identifying the LWM2M Server. |
| Human Readable Object URN | URN for the OMA Management Object | Assigned by the Object specification. |
| Object ID | 16-bit unsigned integer | Uniquely identifies the Object in the LWM2M Client. This identifier is assigned by OMA. |
| Object Instance ID | 16-bit unsigned integer | Uniquely identifies the Object Instance of the Object within the LWM2M Client. This identifier is assigned by LWM2M Client or LWM2M Server. |
| Resource ID | 16-bit unsigned integer | Uniquely identifies the Resource within the Object. Short integer ID, with a range assigned by the Object specification and unique to that Object, and a Reusable Resource ID range assigned by OMA and re-usable between Objects. |
| Resource Instance ID | 16-bit unsigned integer | Uniquely identifies the Resource Instance in the Resource. This identifier is assigned by LWM2M Client or LWM2M Server. |

### Endpoint Client Name

Following formats are RECOMMENDED for this identifier to guarantee uniqueness:

Format

UUID URN: Identify a device using a Universally Unique IDentifier (UUID). The UUID specifies a valid, hex digit character string as defined in [RFC4122]. The format of the URN is urn:uuid:########-####-####-############ OPS URN: Identify a device using the format <OUI> "-" <ProductClass> "-" <SerialNumber> as defined in Section 3.4.4 of [TR-069]. The format of the URN is urn:dev:ops:<OUI> "-" <ProductClass> "-" <SerialNumber>. OS URN: Identify a device using the format <OUI> "-"<SerialNumber> as defined in Section 3.4.4 of [TR-069]. The format of the URN is urn:dev:os:<OUI> "-"<SerialNumber>. IMEI URN: Identify a device using an International Mobile Equipment Identifiers [3GPP-TS_23.003]. The IMEI URN specifies a valid, 15 digit IMEI. The format of the URN is urn:imei:############### ESN URN: Identify a device using an Electronic Serial Number. The ESN specifies a valid, 8 digit ESN. The format of the URN is urn:esn:######## MEID URN: Identify a device using a Mobile Equipment Identifier. The MEID URN specifies a valid, 14 digit MEID. The format of the URN is urn:meid:##############

Other URN formats MAY be used. In particular, URN formats defined in [DMREPPRO] chapter 5.5 can be used.

### Reusable Resources

When Objects are designed for a similar purpose, for example Objects for use in network management, or Objects for use in embedded device automation, similar resources are useful in more than one Object. For example in embedded device automation, Objects for different purposes may contain common resource types such as digital input, digital output, analogue input, analogue output, dimmer value, unit, min measurement, max measurement, value range etc.

If a resource can feasibly be re-used with the same meaning in multiple Object definitions, it can be defined as a Reusable Resource ID and registered with OMNA. Other Objects may then make use of this Reusable Resource ID in another Object definition. The definition of the Resource MUST be the same with the exception of the Multiple Resource, Mandatory and Description fields.

## Data Formats for Transferring Resource Information

Four data formats are defined by the LWM2M Enabler in this section. The LWM2M Server MUST support all data formats. The plain text and opaque formats MUST be supported by the LWM2M Client. The LWM2M Client MUST support the TLV data format for Object Instance or multiple-instance Resource requests.

The Object specification defines the data format that a Resource supports, either plain text or opaque for singular Resources or TLV for multiple instance Resources.

In addition to the data formats defined in the Object specification, a LWM2M Client MAY choose to support the JSON format for Object Instance or multiple instance Resource requests.

### Plain Text

The plain text format is used for ”Read” and “Write” logical operations on singular Resources where the value of the Resource is simply represented as an UTF-8 encoded string. This string can contain a character sequence, integer number, decimal number or any other sequence of valid UTF-8 characters as per Appendix A.

For example a request to the example client’s Device Object, Manufacturer Resource would return the following plain text payload:

# Req: GET /3//0

# Res: 2.04 Content

Open Mobile Alliance

# This data format has a Media Type of application/vnd.oma.lwm2m+text

### Opaque

The opaque format is used for ”Read” and “Write” logical operations on singular Resources where the value of the Resource is an opaque sequence of binary octets. This data format is used for binary Resources such as firmware images or application specific binary formats.

This data format has a Media Type of application/vnd.oma.lwm2m+opaque

### TLV

For requests to Object Instance or Resource which supports multiple instances (Resource Instance), the binary TLV (Type-Length-Value) format is used to represent an array of values or a singular value using a compact binary representation, which is easy to process on simple embedded devices. The format has a minimum overhead per value of just 2 bytes and a maximum overhead of 5 bytes depending on the type of Identifier and length of the value. The maximum size of an Object Instance or Resource in this format is 16.7 MB. The format is self-describing, thus a parser can skip TLVs for which the resource is not known.

This data format has a Media Type of application/vnd.oma.lwm2m+tlv

The format is an array of the following byte sequence, where each array entry represents an Object Instance, Resource or Resource Instance:

# Table 15: TLV format and description

| Field | Format and Length | Description |
| --- | --- | --- |
| Type | 8-bits masked field: 0bxxxxxxxx (MSB is the bit following 0b) Bit numbering is 0 for the LSB to 7 for the MSB | Bits 7-6: Indicates the type of Identifier 00= Object Instance in which case the Value contains one or more Resource TLVs 01= Resource Instance with Value for use within a Multiple Resource TLV 10= Multiple Resource, in which case the Value contains one or more Resource Instance TLVs 11= Resource with Value |
|  |  | Bit 5: Indicates the Length of the Identifier 0=The Identifier field of this TLV is 8 bits long 1=The Identifier field of this TLV is 16 bits long |
|  |  | Bit 4-3: Indicates the type of Length. 00=No length field, the value immediately follows the Identifier field in is of the length indicated by Bits 6-8 of this field 01 = The Length field is 8-bits and Bits 6-8 MUST be ignored 10 = The Length field is 16-bits and Bits 6-8 MUST be ignored 11 = The Length field is 24-bits and Bits 6-8 MUST be ignored |
|  |  | Bits 2-0: A 3-bit unsigned integer indicating the Length of the Value. |
| Identifier | 8-bit or 16-bit unsigned integer as indicated by the Type field. | The Object Instance, Resource or Resource Instance ID as indicted by the Type field. |
| Length | 0-24-bit unsigned integer as indicated by the Type field. | The Length of the following field in bytes |
| Value | Sequence of bytes of Length | Value of the tag. The format of the value depends on the Resource’s data type (See Appendix A). |

Each TLV entry starts with a Type byte that indicates if the TLV contains an Object Instance, a Resource, Multiple Resources, or a Resource Instance. Object Instance and Resource with Resource Instance TLVs contains other TLVs in their value. The hierarchy is as follows and may be up to 3 levels deep. The Object Instance TLV is only required if multiple Object Instances are returned in a request.

- Object Instance TLV, which contains

  - Resource TLVs or

  - Multiple Resource TLVs, which contains

    - Resource Instance TLVs

#### Single Object Instance Request Example

In this example, a request for the Device Object of the LWM2M example client is made (GET /3//). The client responds with a TLV payload including all of the readable resources. This TLV payload would have the following format. Since the Device Object has no Instances, no Object Instance TLV entry is needed. The total payload size with the TLV encoding is 86 bytes.

| TLV | Type Byte | ID Byte(s) | Length Byte(s) | Value | Total Bytes |
| --- | --- | --- | --- | --- | --- |
| Manufacturer Resource | 0b11 0 01 000 | 0x00 | 0x14 (20 bytes) | Open Mobile Alliance [String] | 23 |
| Model Number | 0b11 0 01 000 | 0x01 | 0x16 (22 bytes) | “Lightweight M2M Client” [String] | 25 |
| Serial Number | 0b11 0 01 000 | 0x02 | 0x09 (9 bytes) | “345000123” [String] | 12 |
| Firmware Version | 0b11 0 00 011 | 0x03 | (3 bytes) | “1.0” [String] | 5 |
| Available Power Sources | 0b10 0 00 110 | 0x06 | (6 byte) | The next two rows | 2 |
| Available Power Sources[0] | 0b01 0 00 001 | 0x00 | (1 byte) | 0X01 [8-bit Integer] | 3 |
| Available Power Sources[1] | 0b01 0 00 001 | 0x01 | (1 byte) | 0X05 [8-bit Integer] | 3 |
| Power Source Voltage | 0b10 0 01 000 | 0x07 | 0x08 (8 bytes) | The next two rows | 3 |
| Power Source Voltage[0] | 0b01 0 00 010 | 0x00 | (2 bytes) | 0X0ED8 [16-bit Integer] | 4 |
| Power Source Voltage[1] | 0b01 0 00 010 | 0x01 | (2 bytes) | 0X1388 [16-bit Integer] | 4 |
| Power Source Current | 0b10 0 00 111 | 0x08 | (7 bytes) | The next two rows | 2 |
| Power Source Current[0] | 0b01 0 00 001 | 0x00 | (1 byte) | 0X7D [8-bit Integer] | 3 |
| Power Source Current[1] | 0b01 0 00 010 | 0x01 | (2 bytes) | 0X0384 [16-bit Integer] | 4 |
| Battery Level | 0b11 0 00 001 | 0x09 | (1 byte) | 0x64 [8-bit Integer] | 3 |
| Memory Free | 0b11 0 00 001 | 0x0A | (1 byte) | 0x0F [8-bit Integer] | 3 |
| Error Code | 0b10 0 00 011 | 0x0B | (3 byte) | The next row | 2 |
| Error Code[0] | 0b01 0 00 001 | 0x00 | (1 byte) | 0x00 [8-bit Integer] | 3 |
| Current Time | 0b11 0 00 100 | 0x0D | (4 byte) | 0x5182428F [32-bit Integer] | 6 |
| Time Zone | 0b11 0 00 110 | 0x0E | (6 byte) | “+02:00” [String] | 8 |
| Supported Binding and Modes | 0b11 0 00 001 | 0x0F | (1byte) | “U” [String] | 3 |
| Total |  |  |  |  | 121 |

#### Multiple Object Instance Request Example

In this example, a request for both the ACL Objects of the LWM2M example client is made (GET /2). The client responds with a TLV payload including both Object Instances (0 and 1) and their resources. This TLV payload would have the following format. The total payload size with the TLV encoding is 32 bytes.

| TLV | Type Byte | ID Byte(s) | Length Byte(s) | Value | Total Bytes |
| --- | --- | --- | --- | --- | --- |
| ACL Object Instance 0 | 0b00 0 01 000 | 0x00 | (17 bytes) | The next 5 rows | 2 |
| Object ID | 0b11 0 00 001 | 0x00 | (1 byte) | 0x03 [8-bit Integer] | 3 |
| ACL | 0b10 0 00 110 | 0x02 | (6 bytes) | The next 2 rows | 2 |
| ACL [1] | 0b01 0 00 001 | 0x01 | (1 byte) | 0b11 10 0000 | 3 |
| ACL [2] | 0b01 0 00 001 | 0x02 | (1 byte) | 0b10 00 0000 | 3 |
| Access Control Owner | 0b11 0 00 001 | 0x03 | (1 byte) | 0x01 [8-bit Integer] | 3 |
| ACL Object Instance 1 | 0b00 0 01 000 | 0x01 | (17 bytes) | The next 5 rows | 2 |
| Object ID | 0b11 0 00 001 | 0x00 | (1 byte) | 0x04 [8-bit Integer] | 3 |
| ACL | 0b10 0 00 001 | 0x02 | (6 bytes) | The next 2 rows | 2 |
| ACL [1] | 0b01 0 00 001 | 0x01 | (1 byte) | 0b10 00 0000 | 3 |
| ACL [2] | 0b01 0 00 001 | 0x02 | (1 byte) | 0b10 00 0000 | 3 |
| Access Control Owner | 0b11 0 00 001 | 0x03 | (1 byte) | 0x01 [8-bit Integer] | 3 |
| Total |  |  |  |  | 32 |

### JSON

For requests to Object Instance or Resource which supports multiple instances (Resource Instance), a JSON format may be used where a set of values and metadata is represented. Each entry of the JSON format is a Resource, where the name is the URI path relative to the request URI.

The JSON is useful for returning multi-level Resources from the resource tree, for example requesting all Instances of an Object, Resources and Resource Instances within a LWM2M Client within the same response. The JSON format also includes optional time fields, which allows for multiple versions of representations to be sent in the same payload. The time fields are only used when sending notifications.

This data format has a Media Type of application/vnd.oma.lwm2m+json

The format when an Object or Object Instance is requested follows the following syntax:

{"e":[

{"n": "URI Path", "t": "Time", "v": Float Value, "bv": Boolean Value, "sv":"String Value"},

{"n": "URI Path", "t": "Time", "v": Float Value, "bv": Boolean Value, "sv":"String Value"}],

# "bt":"Base Time"

}

# Table 16: JSON format and description

| Field | JSON Variable | Mandatory? | Description |
| --- | --- | --- | --- |
| Object Root | e | Yes | The root of the value array. |
| URI Path | n | Yes | The path of the resource relative to the request URI (/Object/Object Instance/Resource/Resource Instance) |
| Time | t | No | The time of the representation relative to the Base Current Time in seconds (a negative integer) for a notification. Required only for historical representations. |
| Base Time | bt | No | The base current time which the Time values are relative to as a Time data type (See Appendix B) |
| Float Value | v | One value field is mandatory | Value as a JSON float if the resource data type is integer or decimal. |
| Boolean Value | bv |  | Value as a JSON Boolean if the resource data type is boolean. |
| String Value | sv |  | Value as a JSON string for all other resource data types. Opaque data types must be Base64 encoded in the String Value field. |

For example a request to Device Object of the LWM2M example client (Get /3//) would return the following JSON payload. This example has a size a 397 bytes.

{“e”:[

{"n":"0","sv":"Open Mobile Alliance"},

{"n":"1","sv":"Lightweight M2M Client"},

{"n":"2","sv":"345000123"},

{"n":"3","sv":"1.0"},

{"n":"6/0","v":"1"},

{"n":"6/1","v":"5"},

{"n":"7/0","v":"3800"},

{"n":"7/1","v":"5000"},

{"n":"8/0","v":"125"},

{"n":"8/1","v":"900"},

{"n":"9","v":"100"},

{"n":"10","v":"15"},

{"n":"11/0","v":"0"},

{"n":"13","v":"1367491215"},

{"n":"14","sv":"+02:00"},

{"n":"15","sv":"U"}]

}

For example a notification about a Resource containing multiple historical representations of a Temperature Resource in the example could result in the following JSON payload:

{“e”:[

{"n":"1/2","v":"22.4","t":"-5"},

{"n":"1/2","v":"22.9","t":"-30"},

{"n":"1/2","v":"24.1","t":"-50"}],

"bt":"25462634"

}

# Security

The LWM2M protocol is based on [CoAP] principles and utilizes the UDP and SMS transport channel bindings of the protocol. The LWM2M protocol utilizes the security mechanisms of these channel bindings to implement authentication, confidentiality, and data integrity features of the protocol between communicating LWM2M entities.

For authentication of communicating LWM2M entities, the LWM2M protocol requires that all communication between LWM2M Clients and LWM2M Servers as well as LWM2M Clients and LWM2M Bootstrap Servers are authenticated using mutual authentication. This means that a:

- LWM2M Client MUST authenticate a LWM2M Server prior to exchange of any information.

- LWM2M Server MUST authenticate a LWM2M Client prior to exchange of any information.

- LWM2M Client MUST authenticate a LWM2M Bootstrap Server prior to exchange of any information.

- LWM2M Bootstrap Server MUST authenticate a LWM2M Client prior to exchange of any information.

For confidentiality and data integrity of information between communicating LWM2M entities, the LWM2M protocol requires that all communication between LWM2M Clients and LWM2M Servers as well as LWM2M Clients and LWM2M Bootstrap Servers are encrypted and integrity protected.  This means that a:

- LWM2M Client MUST encrypt and integrity protect data communicated to a LWM2M Server.

- LWM2M Server MUST encrypt and integrity protect data communicated to a LWM2M Client.

- LWM2M Client MUST encrypt and integrity protect data communicated to a LWM2M Bootstrap Server.

- LWM2M Bootstrap Server MUST encrypt and integrity protect data communicated to a LWM2M Client.

The LWM2M protocol specifies that authorization of LWM2M Servers to access Object Instances and Resources within the LWM2M Client is provided through Access Control Object Instances within the LWM2M Client.

## UDP Channel Security

The UDP channel security for [COAP] is defined by the Datagram Transport Layer Security (DTLS) [RFC6347], which is the equivalent of TLS v1.2 [RFC5246] for HTTP and utilizes a subset  of the Cipher Suites defined in TLS. (Refers to TLS Cipher Suite registry http://www.iana.org/assignments/tls-parameters/tls-parameters.xml)

The DTLS binding for CoAP is defined in Section 9 of [CoAP]. DTLS is a long-lived session based security solution for UDP. It provides a secure handshake with session key generation, mutual authentication, data integrity and confidentiality.

Since the LWM2M protocol utilizes DTLS for authentication, data integrity and confidentiality purposes, the LWM2M Client and LWM2M Server SHOULD keep a DTLS session in use as long as possible (even across sleep cycles).

Note that the Client-Server relationship of DTLS (i.e., who initiated the handshake) is separate from the Client-Server relationship of LWM2M.

Considering that any device with a LWM2M Client can be managed by any LWM2M Server and LWM2M Bootstrap Server the choice of Cipher Suites is not limited to the list defined in Section 9 of [CoAP].Due the sensitive nature of Bootstrap Information, a particular care has to be taken to ensure protection of that data inducing constraints and dependencies within LWM2M Client/ Bootstrap Server relationship according to the adopted security mode.

Concerning Smartcard provisioning, the same care has to be taken and a secure channel between the SmartCard and the LWM2M Device has to be established  as described in Appendix G in reference to  [GLOBALPLATFORM 3], [GP SCP03].

The keying material used to secure the exchange of information using DTLS session is obtained using one of the bootstrap modes defined in Section 5.1.2 Bootstrap Modes. The formats of the keying material carried in the LWM2M Server Access Security Object Instances are defined in Appendix D.2.1.

The Resources (i.e., “Security Mode”, “Public Key or Identity” and “Secret Key”)  in the LWM2M Server Object that are associated with the keying material are used for providing UDP channel security in “Device Registration”, “Device Management & Service Enablement”, and “Information Reporting” Interfaces.

The Resources (i.e., “Security Mode”, “Public Key or Identity” and “Secret Key”) in the LWM2M Bootstrap Server Object that are associated with the keying material are used for providing channel security in Bootstrap Interface.

### Pre-Shared Keys

A LWM2M server MUST support the Pre-Shared Key mode of DTLS with the Cipher Suites below:

- TLS_PSK_WITH_AES_128_CCM_8 [RFC6655] as defined in Section 9.1.3.1 of [CoAP]

- TLS_PSK_WITH_AES_128_CBC_SHA256 as defined in [RFC5487]

A LWM2M Client MUST support the Pre-Shared Key mode of DTLS with at least one of the Cipher Suites specified for the LWM2M Server. The LWM2M Client MUST use the value of the "Public Key or Identity" Resource for "PSK identity" in [RFC4279] and the value of "Secret Key" Resource for "PSK" in [RFC4279] as defined in Appendix D.2.1.

The LWM2M Client and LWM2M Server MAY support the use of other Cipher Suites.

For all Cipher Suites using AES in an LWM2M implementation, the hashing functions SHOULD be SHA256.

For all Cipher Suites using AES in an LWM2M implementation, the hashing functions SHALL NOT be SHA-1, and SHALL NOT be MD5, and SHALL NOT be any other hashing function that is weaker than SHA-1 and MD5 or otherwise deprecated..

A LWM2M Client negotiates with the LWM2M Server the best method during the DTLS handshake for establishing the DTLS session.

This security mode is appropriate for LWM2M deployments where there is an existing trust relationship between the LWM2M Server and Client. The same PSKs and PSK IDs need to be generated, and installed on the Client and Server. When using a Bootstrap Server, this security mode requires a 3-way trust relationship between the Bootstrap Server, LWM2M Server(s) and LWM2M Client(s): namely  Bootstrap Server got the secret key (PSK) from Server(s), and should also share a pre-provisioned secret with Client(s)  for ensuring secure DTLS Bootstrap communication

Using SmartCard  PSK provisioning needs no pre-existing trust  relationship between   LWM2M Server(s) and LWM2M Client(s).

### Raw Public Key Certificates

If a LWM2M Server supports Raw Public Key Certificates it MUST support the Cipher Suites below:

- TLS_ECDHE_ECDSA_WITH_AES_128_CCM_8 as defined in Section 9.1.3.2 of [CoAP]

- TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 as defined in [RFC5289]

If a LWM2M Client supports Raw Public Key Certificates it MUST support at least one of the Cipher Suites supported by the LWM2M Server.

The LWM2M Client MUST use the value of the "Public Key or Identity" Resource for its Raw Public Key certificate and the value of "Secret Key" Resource for its Private Key as defined in Appendix D.2.1.

If the LWM2M Client and LWM2M Server supports Raw Public Key Certificates, they MAY support the use of other Cipher Suites.

If the LWM2M Client or LWM2M Server supports ECDHE and ECDSA for Raw Public Key Certificates, SHA-1 SHALL NOT be used, and MD5 SHALL NOT be used, and any other hashing function that is weaker than SHA-1 and MD5 or otherwise deprecated SHALL NOT be used. The minimum key length SHALL be at least 256 bits.

This security mode is appropriate for LWM2M deployments where there is an existing trust relationship between the LWM2M Server and Client. The same Public Keys (or hashed IDs of these keys) need to be generated, and installed on the Client and Server. When using a Bootstrap Server, this security mode requires a 3-way trust relationship between the Bootstrap Server, LWM2M Server(s) and LWM2M Client(s): namely Bootstrap Server got the Client private key  from Server(s), and should also share a pre-provisioned secret with Client(s)  for ensuring  secure DTLS Bootstrap communication.

Using SmartCard  RPK certificates provisioning needs no pre-existing  trust  relationship between   LWM2M Server(s) and LWM2M Client(s).

### X.509 Certificates

The X.509 Certificate mode requires the use of X.509v3 Certificates [RFC5280].

Certificates used in LWM2M SHOULD be signed by a root certificate, either by a public root CA or a private root.

If a LWM2M server supports X.509 Certificate mode it MUST support the Cipher Suites below:

- TLS_ECDHE_ECDSA_WITH_AES_128_CCM_8 as defined in Section 9.1.3.3 of [CoAP].

- TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 as defined in [RFC5289]

If a LWM2M Client supports X.509 Certificate mode it MUST support at least one of the Cipher Suites supported by the LWM2M Server. The LWM2M Client MUST use the value of the "Public Key or Identity" Resource for its X.509 certificate and the value of "Secret Key" Resource for its Private Key as defined in Appendix D.2.1.

If the LWM2M Client and LWM2M Server supports X.509 Certificate mode, they MAY support the use of other Cipher Suites.

If the LWM2M Client or LWM2M Server supports ECDHE and ECDSA for X.509 Certificate mode, SHA-1 SHALL NOT be used, and MD5 SHALL NOT be used, and any other hashing function that is weaker than SHA-1 and MD5 or otherwise deprecated SHALL NOT be used. The minimum key length SHALL be at least 256 bits.

A LWM2M Client Certificate MUST include the Endpoint Name parameter used to register the device in the Subject Common Name (CN) field of the Certificate. Upon registration, the LWM2M Server MUST check that this CN field matches the Endpoint Name parameter of the registration message during authentication and MUST reject the handshake if these fields do not match. The LWM2M Server SHOULD also verify that the Certificate is signed by the indicated Issuer.

A LWM2M Server Certificate SHOULD include Subject and/or SubjectAltName fields listing its known DNS names and IP addresses which are included in the LWM2M Server URI resource of the LWM2M Sever Object Instance. The LWM2M Server may use a wild card certificate for the DNS with the host represented as an * and the rest of the domain fully qualified (e.g., *.acme.com). A wildcard with only a top level domain is not permitted (e.g., *.com). The LWM2M Client SHOULD check that these fields of the Certificate match the URI used to register with the LWM2M Server. The LWM2M Client SHOULD also verify that the Certificate is signed by the indicated Issuer.

This security mode does not require a pre-existing trust relationship (if all entities used X.509 certificate security mode) between the LWM2M Client and LWM2M Server, nor between a LWM2M Bootstrap Server and a LWM2M Client. However, all entities need a trust relationship with the CA(s) that issued the certificates used in LWM2M Servers and Clients.

Using SmartCard  with certificates provisioning  needs no pre-existing  trust  relationship between   LWM2M Server(s) and LWM2M Client(s).

A LWM2M Client SHOULD wait until it has accurate absolute time before contacting the LWM2M Server or LWM2M Bootstrap Server.  If a LWM2M Client chooses to contact the LWM2M Server or LWM2M Bootstrap Server before it has accurate absolute time (or if it does not support absolute time), it MUST ignore those components of the LWM2M Server or LWM2M Bootstrap Server certificate that involve absolute time, e.g. not-valid-before and not-valid-after certificate restrictions.

### “NoSec” mode

It is highly recommended to always use LWM2M with one of the security mechanisms described above. However, there are few scenarios and use cases where security is provided by lower layers. For example LWM2M devices in a controlled environment behind a gateway, or, tests focussing first on other functions before performing end-to-end tests including security.

## Access Control

### Access Control Object

To authorize the logical operation sent from the LWM2M Server, the LWM2M Client checks Instances of Access Control Object. Each Access Control Object Instance is assigned per Object Instance that requires authorization. Within the Access Control Object Instance is an ACL Resource that determines which logical operations a LWM2M Server can perform on the Object Instance within the LWM2M Client.

The ACL Resource can have multiple Resource Instances where each Resource Instance has a Resource Instance ID that is represented by the Short Server ID of the LWM2M Server and its access rights.

An ACL Resource Instance consisting of the default Short Server ID (i.e. 0) and its access right MAY be used to grant access rights to LWM2M Servers which are not specified in the ACL Resource Instances.

Each ACL Object Instance MUST be managed by a LWM2M Server called Access Control Owner. The other LWM2M Servers except Access Control Owner MUST NOT manage the Access Control Object Instance.

The ACL Object Instance is further defined and described in Appendix D.2 and Examples of Access Control Object Instance is specified in Appendix B.

### Authorization

The LWM2M Client authorizes logical operations requested by a LWM2M Server by verifying the access to the requested Object Instance based on the conjunction of the access right to the Object Instance for the LWM2M Server and the supported logical operationsof the Resource. This chapter specifies how the LWM2M Client obtains the access rights of the LWM2M Server to the Object Instance and authorizes the logical operation on the Resource or Object Instance.

The LWM2M Object specification defines the logical operations that are allowed to be performed for each Resource within an Object Instance.  If a Resource has Resource Instances then the logical operations allowed by the LWM2M Client for the Resource are applied to each Resources Instance.

The LWM2M Server and the LWM2M Client MUST support the authorization procedure described in Section 7.2.3 Authorization.

#### Obtaining Access Right

For obtaining the access right of an Object Instance for a LWM2M Server, the LWM2M Client MUST perform the following procedure:

- If the LWM2M Client has only one LWM2M Server Object Instance, the LWM2M Server has full access rights without checking Access Control Object Instance.

- If the LWM2M Client has more than one LWM2M Server Object Instance, the LWM2M Client finds an Access Control Object Instance associated with the Object Instance that the LWM2M Server has requested to access with the following procedure.

  - If the Client has an ACL Resource Instance for the LWM2M Server, the LWM2M Server has an access right of that Resource Instance.

  - If the Client doesn’t have ACL Resource Instance for the Server, the LWM2M Server has an access right of default Short Server ID in ACL Resource Instance if it exists.

  - If the Client doesn’t have ACL Resource Instance for the default Short Server ID, then, the LWM2M Server has no access right.

#### Operation on Resource

If the LWM2M Server accesses a Resource, the LWM2M Client obtains an access right of the LWM2M Server for the Object Instance that Resource belongs to according to Section 7.2.3.1 Obtaining Access Right and checks whether the access right is granted to perform the logical operation.

If the logical operation is not permitted, the LWM2M Client MUST send an “Access Right Permission Denied” error code to the LWM2M Server.

If the logical operation is permitted, the LWM2M Client verifies whether the Resource supports the logical operation.

If the logical operation is not supported by the Resource, the LWM2M Client MUST send an “Operation is not supported” error code to the LWM2M Server.

If the Resource supports the logical operation, the LWM2M Client performs the logical operation.

#### Operation on Object Instance

If the LWM2M Server accesses an Object Instance, the LWM2M Client obtains an access right of the LWM2M Server for Object Instance according to Section 7.2.3.1 Obtaining Access Right and checks whether the access right is granted to perform the logical operation.

If the logical operation is not permitted, the LWM2M Client MUST send an “Access Right Permission Denied” error code to the LWM2M Server.

If the logical operation is permitted, the LWM2M Client performs the following cases based on the logical operation.

- For the  “Write” logical operation, the LWM2M Client MUST perform the logical operation on the Object Instance only if all the Resources conveyed in the logical operation are allowed to perform the “Write” logical operation. If any Resource does not support the “Write” logical operation, the LWM2M Client MUST inform the LWM2M Server of the Resources which don’t support the logical operation by sending “Operation is not supported” error code for the affected Resources.

- For the “Read” logical operation, the LWM2M Client MUST retrieve all the Resources except the Resource(s) which doesn’t support the “Read” logical operation and sends the retrieved Resource(s) information to the LWM2M Server.

- For the “Execute” logical operation, the LWM2M Client MUST NOT perform the logical operation.

- For the “Create” logical operation, the LWM2M Client MUST perform the logical operation on the Object Instance only if all the Resources conveyed in the logical operation are allowed to perform the “Write” logical operation and all the mandatory Resources are specified. If any Resource does not support the “Write” logical operation, the LWM2M Client MUST inform the LWM2M Server of the Resources which don’t support the logical operation by sending “Operation is not supported” error code for the affected Resources. If all the mandatory Resources are not specified, the LWM2M Client MUST send an “Bad Request” error code to the LWM2M Server.

- For the “Delete” logical operation, the LWM2M Client MUST perform the logical operation.

#### Observe/Notify Operation Consideration

If the LWM2M Server sends an “Observe” logical operation on an Object Instance or a Resource, the LWM2M Client MUST check whether the LWM2M Server is authorized for the “Read” logical operation. If the LWM2M Server is not authorized to perform the “Read” logical operation, the LWM2M Client MUST reject the “Observe” logical operation and send error response as defined in the respective sections.

If the LWM2M Client needs to send a “Notify” logical operation containing an Object Instance or a Resource to the Server, the LWM2M Client MUST check whether the LWM2M Server is authorized for the “Read” logical operation. If the LWM2M Server is not authorized, the Client MUST NOT send the “Notify” logical operation and MUST perform the “Cancel Observation” logical operation.

# Transport Layer Binding and Encodings

The LWM2M interfaces use the IETF Constrained Application Protocol [CoAP] as an underlying transfer protocol across IP and SMS bearers. This protocol defines the message header, request/response codes, message options and retransmission mechanisms. This section defines the subset of features from the IETF CoAP specification to be used by LWM2M interfaces.

## Required Features

For realization of the LWM2M interfaces, only the basic binary CoAP message header, and a small subset of options are required. This section explicitly defines the features of the CoAP standard that are required for LWM2M.

- The 4-byte binary CoAP message header is defined in Section 3 of [CoAP]. This same base message is used for Request and Response interactions.

- Confirmable, Acknowledgement and Reset messages MUST be supported. The Reset message is used as a message layer error message in response to a malformed Confirmable message. Non-Confirmable messages MAY be used by a Client for sending Information Reporting notifications as per [Observe].

- GET, PUT, POST and DELETE methods MUST be supported. LWM2M Operations map to these methods.

- A subset of Response Codes MUST be supported for LWM2M response message mapping.

- The Uri-Path Option MUST be supported to indicate the identifier of the interface, Object Instance and Resource being requested.

- The Location-Path Option MUST be supported to indicate the handle of a registration for future update and delete operations.

- The Uri-Query Option MUST be supported.

- The Content-Type Option MAY be used to indicate the media type of the payload. A default value of plain/text is assumed, allowing this option to be elided for most payloads.

- The Token Option MAY be used to enable multiple requests in parallel with an endpoint, and MUST be supported for the Information Reporting interface.

## URI Identifier & Operation Mapping

Although CoAP supports a URI in requests, it is not used in the same way as in HTTP. The URI in CoAP is broken down into binary parts, minimizing overhead and complexity. In LWM2M only path segment and query string URI components are needed. The URI path is used to simply identify the interface, Object Instance or Resource that the request is for, and is encoded in Uri-Path options. The LWM2M Registration interface also makes use of query string parameters to pass on meta-data with the request separately from the payload. Each query parameter is encoded in a Uri-Query Option. Likewise, the LWM2M Operations for each interface are mapped to CoAP Methods.

### Firewall/NAT

For a firewall to support LWM2M, it should be configured to allow outgoing UDP packets to destination port 5683 (other ports can be configured), and allow incoming UDP packets back to the source address/port of the outgoing UDP packet for a period of at least 240 seconds.  These UDP packets may contain DTLS or CoAP payloads. When a firewall is configured as such any Clients behind it should use Queue Mode.

For a firewall to support LWM2M it can be configured to allow both outgoing and incoming UDP packets to destination port 5683 (other ports can be configured). These UDP packets may contain DTLS or CoAP payloads. When a firewall is configured as such any Clients behind it are not required to use Queued Mode, but may use it for other reasons (e.g. a battery powered sleeping device).

Any Clients behind a NAT can use Queued Mode. There are other mechanisms to transverse a NAT, however they are out of scope for the LWM2M Enabler.

### Registration Interface

The registration interface is used by a LWM2M Client to register with a LWM2M Server, identified by the LWM2M Server URI. Registration is performed by sending a CoAP POST to the LWM2M Server URI, with registration parameters passed as query string parameters as per Table 17 and Object and Object Instances included in the payload as per [RFC6690]. The Client MAY add “</>;ct=TBD” in the payload to inform the Server of the Client supporting JSON data format for all the Objects. The response includes Location-Path Options, which indicate the path to use for updating or deleting the registration. The server MUST return a location under the /rd path segment.

Registration update is performed by sending a CoAP PUT to the Location path returned to the LWM2M Client as a result of a successful registration.

De-registration is performed by sending a CoAP DELETE to the Location path returned to the LWM2M Client as a result of a successful registration.

# Table 17: Operation to Method and URI Mapping

| Logical Operation | CoAP Method | URI | Success | Failure |
| --- | --- | --- | --- | --- |
| Register | POST | /rd?ep={Endpoint Client Name}&lt={Lifetime}&sms={MSISDN} &lwm2m={version}&b={binding} | 2.01 Created | 4.00 Bad Request |
| Update | PUT | /{location}?lt={Lifetime} | 2.04 Changed | 4.00 Bad Request |
| De-register | DELETE | /{location} | 2.02 Deleted | 4.00 Bad Request |

Figure 15: Example register, update and de-register logical operation exchanges (shorthand in [CoAP] example style, actual messages using CoAP binary headers)

### Bootstrap Interface

The bootstrap interface is used to optionally configure a LWM2M Client so that it can successfully register with a LWM2M Server. The client bootstrap operation is performed by sending a CoAP POST request to the LWM2M Bootstrap Server at the /bs path including the Endpoint Client Name as a query string parameter.

In client initiated bootstrap, when the Bootstrap Server receives Request Bootstrap logical operation, the Bootstrap Server performs Write and/or Delete logical operation. In server initiated bootstrap, the Bootstrap Server performs Write logical operation. The Write or Delete logical operation targets to an Object Instance or a Resource. The Write and Delete logical operation can be sent multiple times. Only in Bootstrap Interface, Delete logical operation MAY target to “/” URI to delete all the existing Object Instances in the LWM2M Client for initializing before LWM2M Bootstrap Server sends Write logical operation(s) to the LWM2M Client. Different from Write operation in Device Management and Service Enablement interface, the Client MUST write the payload regardless of an existence of the targeting Object Instance or Resource.

# Table 18: Operation to Method and URI Mapping

| Logical Operation | CoAP Method | URI | Success | Failure |
| --- | --- | --- | --- | --- |
| Request Bootstrap | POST | /bs?ep={Endpoint Client Name} | 2.04 Changed | 4.00 Bad Request |
| Write | PUT | /{Object ID}/{Object Instance ID}/ {Resource ID} | 2.04 Changed | 4.00 Bad Request |
| Delete | DELETE | /{Object ID}/{Object Instance ID} | 2.02 Deleted | 4.00 Bad Request |

# Figure 16: Example of Client initiated Bootstrap exchange.

# Figure 17: Example of Server initiated Bootstrap exchange.

### Device Management & Service Enablement Interface

The Device Management & Service Enablement Interface is used to access an Object Instance or an individual Resource of an Object Instance. An Object Instance is identified by the path /{Object ID}/{Object Instance ID}. If Object doesn’t support multiple Object Instances, the Object Instance is identified by the path /{Object ID}/0. A Resource is identified by the path /{Object ID}/{Object Instance ID}/{Resource ID}.

An Object Instance or Resource is Read by sending a CoAP GET to the corresponding path. The response includes the value in the corresponding Plain Text, TLV or JSON format.

An Object Instance or Resource is Written to by sending a CoAP PUT to the corresponding path. The request includes the value to be written in the corresponding Plain Text, TLV or JSON format.

A Resource is Executed by sending a CoAP POST to the corresponding path.

An Object Instance is created by sending a CoAP POST to the corresponding path. The request includes the value to be written in the corresponding TLV or JSON format.

An Object Instance is deleted by sending a CoAP DELETE to the corresponding path.

Resource attributes MAY be set by a LWM2M Server using the Write Operation on a Resource ID, and can be accessed using the Read Operation. One or more attributes can be written at a time, whereas only one attribute can be read at a time.  The values of these attributes are used by the Information Reporting interface to determine how often Notifications are sent regarding that Resource. A LWM2M Client MAY support a set of these attributes for each LWM2M Server it is configured for.

# Table 19:  Operation to Method Mapping

| Logical Operation | CoAP Method | Path | Success | Failure |
| --- | --- | --- | --- | --- |
| Read | GET | /{Object ID}/{Object Instance ID}/{Resource ID} | 2.05 Content | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Discover Resources | GET Accept: application/link-format | /{Object ID}/{Object Instance ID}/{Resource ID} | 2.05 Content | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Write | PUT | /{Object ID}/{Object Instance ID}/{Resource ID} ?pmin={minimum period}&pmax={maximum period}&gt={greater than}&lt={less than}&st={step} | 2.04 Changed | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Write Attributes | PUT | /{Object ID}/{Object Instance ID}/{Resource ID} ?pmin={minimum period}&pmax={maximum period}&gt={greater than}&lt={less than}&st={step} | 2.04 Changed | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Execute | POST | /{Object ID}/{Object Instance ID}/{Resource ID} | 2.04 Changed | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Create | POST | /{Object ID}/{Object Instance ID} | 2.01 Created | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Delete | DELETE | /{Object ID}/{Object Instance ID} | 2.02 Deleted | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |

# Figure 18: Example of Device Management & Service Enablement interface exchanges.

Figure 19:  Example of Object Creation and Deletion.

### Information Reporting Interface

Periodic and event-triggered reporting about Resource values from the LWM2M Client to the LWM2M Server is achieved through CoAP Observation [OBSERVE]. This simple mechanism allows the LWM2M Server to send an Observe GET request for an Object Instance, which results in asynchronous notifications whenever that Object Instance changes (periodically or as a result of an event). The LWM2M Server can cancel Observe operation by sending CoAP GET without Observe option. Cancel Observation works as the same as Read operation in CoAP transport layer binding. The LWM2M Server may set the Observe attributes of a Resource to affect the behavior its notifications using the Write Attributes Operation (see Section 5.3.4).

# Table 20: Operation to Method Mapping

| Logical Operation | CoAP Method | Path | Success | Failure |
| --- | --- | --- | --- | --- |
| Observe | GET with Observe option | /{Object ID}/{Object Instance ID}/{Resource ID} | 2.05 Content with Observe option | 4.00 Bad Request, 4.04 Not Found, 4.05 Method Not Allowed |
| Cancel Observation | GET | /{Object ID}/{Object Instance ID}/{Resource ID} | 2.05 Content | 4.00 Bad Request, 4.04 Not Found 4.01 Unauthorized, 4.05 Method Not Allowed |
| Notify | Asynchronous Response |  | 2.04 Changed |  |

# Figure 20: Example of an Information Reporting exchange.

## Queue Mode Operation

When the Client has registered with low communication access availability Queue Mode (it has included the “q” query string parameter when registering), The Server does not immediately send downlink requests on this interface, but instead waits until the Client is online.

A Client lets the Server know it is awake by sending a registration update message as a Confirmable message. The Server then makes any queued requests to the Client in a serial fashion. The Client MUST wait at least ACK_TIMEOUT [COAP] seconds from the last CoAP message it sent to the Server before intentionally going offline. If the Server is not successful in sending a request, then it stops emptying the queue and keeps the request for the next time the Client is online.

A typical Queue Mode sequence follows the following steps:

- The LWM2M Client registers to the LWM2M Server and requests the LWM2M Server to run in Queue mode by using the correct Binding value in the registration.

- The LWM2M Client uses the CoAP ACK_TIMEOUT parameter to set a timer for how long it shall stay awake since last sent message to the LWM2M Server. After ACK_TIMEOUT without any messages from the LWM2M Server, the LWM2M Client SHOULD sleep until sending next periodic “Update” operation.

- When LWM2M Server receives a message from the Client (e.g. a notification or a registration update), it checks its request queue for the LWM2M Client and performs the needed CoAP operation(s) (e.g. GET, PUT, and POST). Note: There could be several requests in the queue). Each request is sent serially to the Client, waiting for request to be Acknowledged before sending the next request. If a request is unsuccessful then it is returned to the queue.  The LWM2M Client may have pending Observer notifications.

# Below is an example flow for Queue Mode in relation to Device Management & Service Enablement Interface.

# Figure 21: Example of Device Management & Service Enablement interface exchanges for Queue Mode.

# Below is an example flow for Queue Mode in relation to Information Reporting Interface

# Figure 22: Example of an Information Reporting exchange for Queue Mode.

Queue Mode may also be used to reach LWM2M Clients behind a NAT using an SMS based trigger mechanism. Typically, a LWM2M Client registers to the LWM2M Server with binding preference set to UDP with queue mode and SMS Below is an example flow for Queue Mode in relation to Information Reporting Interface using SMS Registration Update Trigger:

# Figure 23: Example of Device Management & Service Enablement interface exchanges for Queue Mode with SMS Registration Update Trigger.

## Response Codes

This Chapter lists available response codes of each logical operation. The codes are divided into each interface. These are the only valid response codes defined in for the LWM2M Enabler.

# Table 21: Response Codes

| Logical Operations | Available CoAP Response Codes | Reason Phrase |
| --- | --- | --- |
| Device Discovery and Registration Interface |  |  |
| Register | 2.01 Created | Register operation is completed successfully |
|  | 4.00 Bad Request | The mandatory parameter is not specified or unknown parameter is specified Unknown Endpoint Client Name |
|  | 4.09 Conflict | The Endpoint Client Name results in a duplicate entry on the LWM2M Server. |
| Update | 2.04 Changed | Update operation is completed successfully |
|  | 4.00 Bad Request | The mandatory parameter is not specified or unknown parameter is specified or URI is not found |
| De-register | 2.02 Deleted | De-register operation is completed successfully |
|  | 4.00 Bad Request | URI is not found |
| Bootstrap Interface |  |  |
| Request Bootstrap | 2.04 Changed | Request Bootstrap is completed successfully |
|  | 4.00 Bad Request | Unknown Endpoint Client Name |
| Write | 2.04 Changed | Write operation is completed successfully |
|  | 4.00 Bad Request | The format of data to be written is different Target (i.e., Object) is not allowed for Write operation |
| Device Management and Service Enablement Interface |  |  |
| Create | 2.01 Created | Create operation is completed successfully |
|  | 4.00 Bad Request | Target (i.e., Resource) is not allowed for Create operation Target (i.e., Object) already exists |
|  | 4.01 Unauthorized | Access Right Permission Denied |
| Read | 2.05 Content | Read operation is completed successfully |
|  | 4.00 Bad Request |  |
|  | 4.04 Not Found | URI of Read operation is not found |
|  | 4.01 Unauthorized | Access Right Permission Denied |
|  | 4.05 Method Not Allowed | Operation is not supported |
| Write | 2.04 Changed | Write operation is completed successfully |
|  | 4.00 Bad Request | The format of data to be written is different Target (i.e., Object) is not allowed for Write operation |
|  | 4.04 Not Found | URI of Write operation is not found |
|  | 4.01 Unauthorized | Access Right Permission Denied |
|  | 4.05 Method Not Allowed | Access Type Permission Denied |
| Delete | 2.02 Deleted | Delete operation is completed successfully |
|  | 4.00 Bad Request | Target (i.e., Object) is not allowed for Delete operation |
|  | 4.01 Unauthorized | Access Right Permission Denied |
| Execute | 2.04 Changed | Execute operation is completed successfully |
|  | 4.00 Bad Request | Target (i.e., Object, Object Instance) is not allowed for Execute operation |
|  | 4.04 Not Found | URI of Execute operation is not found |
|  | 4.01 Unauthorized | Access Right Permission Denied |
|  | 4.05 Method Not Allowed | Operation is not supported |
| Information Reporting Interface |  |  |
| Observe | 2.05 Content | Observe operation is completed successfully |
|  | 4.00 Bad Request | Target (i.e., Object) is not allowed for Observe operation |
|  | 4.04 Not Found | URI of Observe operation is not found |
|  | 4.05 Method Not Allowed | Operation is not supported |
| Cancel Observation | Response codes for Cancel Observation are the same as the codes for Read operation |  |
| Notify | 2.04 Changed | Notify operation is completed successfully |

## Transport Bindings

### UDP Binding

The CoAP binding for UDP is defined in [CoAP]. The protocol has a IANA registered scheme of coap:// and a default port of 5683. The UDP binding is used in NoSec (no security) mode. Reliability over the UDP transport is provided by the built-in retransmission mechanism of CoAP.

### SMS Binding

CoAP is used over SMS in this transport binding by placing a CoAP message in the SMS payload using 8-bit encoding. SMS concatenation MAY be used for messages larger than 140 characters. CoAP retransmission is disabled for this binding. An LWM2M Client indicates the use of this binding by including a parameter (sms) in its registration to the LWM2M Server including the node’s MSISDN number. The LWM2M Client MAY interact with the server using both UDP and SMS bindings.

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |

Draft/Candidate Version <current version> History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-TS-LightweightM2M-V1_0-20120904-D | 04 Sep 2012 | All | TS baseline agreed as in OMA-DM-LightweightM2M-2012-0078-INP_TS_kick_off |
| Draft Version OMA-TS-LightweightM2M-V1_0-20120918-D | 18 Sep 2003 | 6, 7 | Incorporates input to committee: OMA-DM-LightweightM2M-2012-0083R01-CR_Skeleton_Base_Line OMA-DM-LightweightM2M-2012-0090R02-CR_TS_Resource_Model OMA-DM-LightweightM2M-2012-0061R04-CR_Interfaces |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121024-D | 24 Oct 2012 | 6, 7, Appendix A | OMA-DM-LightweightM2M-2012-0095R01-CR_TS_Interface_and_Resource_Additions |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121030-D | 30 Oct 2012 | 7, 8 | OMA-DM-LightweightM2M-2012-0097R01-CR_Identifiers_and_Security_Considerations |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121117-D | 17 Nov 2012 | 2, 6, 7, 8, 9, 10 | OMA-DM-LightweightM2M-2012-0088R04-CR_Transfer_Protocol OMA-DM-LightweightM2M-2012-0098R02-CR_Bootstrap_Information_and_Modes OMA-DM-LightweightM2M-2012-0099R01-CR_Default_ACL_Entry OMA-DM-LightweightM2M-2012-0100R02-CR_Authorization_Procedure_and_Error_Code OMA-DM-LightweightM2M-2012-0104R01-CR_Registration_Interface |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121130-D | 30 Nov 2012 |  | OMA-DM-LightweightM2M-2012-0107R01-CR_Appendix_for_LWM2M_Objects. OMA-DM-LightweightM2M-2012-0106R02-CR_Information_Interfaces. OMA-DM-LightweightM2M-2012-0108R01-CR_LWM2M_Server_Account_Object. OMA-DM-LightweightM2M-2012-0109R01-CR_Authorization_Update |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121206-D | 06 Dec 2012 | 6 | OMA-DM-LightweightM2M-2012-0110R01-CR_Interfaces_Intro_Update |
| Draft Version OMA-TS-LightweightM2M-V1_0-20121219-D | 19 Dec 2012 | 6,7,8,9, Annex | OMA-DM-LightweightM2M-2012-0111R01-CR_Object_Instance_Introduction OMA-DM-LightweightM2M-2012-0112-CR_Object_Template_Update OMA-DM-LightweightM2M-2012-0113R02-CR_Access_Control OMA-DM-LightweightM2M-2012-0114-CR_Update_Operation_Modification OMA-DM-LightweightM2M-2012-0115-CR_Connection_Control |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130123-D | 22 Jan 2013 | 2, 7, 8, 9, Annex | OMA-DM-LightweightM2M-2012-0101R03-CR_change_of_the_TLV_data_format OMA-DM-LightweightM2M-2012-0117-CR_remove_example_objects_and_resources OMA-DM-LightweightM2M-2013-0001R04-CR_Firmware_Object OMA-DM-LightweightM2M-2013-0003R01-CR_LwM2M_Client_and_Server_Security_Considerations OMA-DM-LightweightM2M-2013-0006-CR_Security_Mode_in_RessourceInfo_Table |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130206-D | 6 Feb 2013 |  | OMA-DM-LightweightM2M-2013-0004R03-CR_SmartCard_Bootstrap OMA-DM-LightweightM2M-2013-0005R01-CR_device_object OMA-DM-LightweightM2M-2013-0007-CR_Object_Instance_Modification |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130226-D | 26 Feb 2013 | All | OMA-DM-LightweightM2M-2013-0002R04-CR_Adding_Creatable_Object OMA-DM-LightweightM2M-2013-0008R02-CR_Improvement_to_the_JSON_format_for_IETF_alignment OMA-DM-LightweightM2M-2013-0013R01-CR_LWM2M_Version_CoAP_Option OMA-DM-LightweightM2M-2013-0014R01-CR_Data_Format_Negotiation OMA-DM-LightweightM2M-2013-0015R02-CR_Notification_Aggregation_and_Reporting OMA-DM-LightweightM2M-2013-0016R03-CR_Connectivity OMA-DM-LightweightM2M-2013-0019R01-CR_SmartCard_Bootstrap_Appendix OMA-DM-LightweightM2M-2013-0020-CR_Response_Code |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130301-D | 01 Mar 2013 | All | OMA-DM-LightweightM2M-2013-0011R03-CR_Failure_indication_for_firmware_object OMA-DM-LightweightM2M-2013-0022R03-CR_TLV_Tags OMA-DM-LightweightM2M-2013-0023R01-CR_location_object |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130314-D | 14 Mar 2013 | All | OMA-DM-LightweightM2M-2012-0116R03-CR_Bootstrap_Interface_Chapter_Modification OMA-DM-LightweightM2M-2013-0018R01-CR_Bootstrap_Interface_Transport_Binding OMA-DM-LightweightM2M-2013-0024R04-CR_Time_Resource OMA-DM-LightweightM2M-2013-0026R05-CR_Erro_Code OMA-DM-LightweightM2M-2013-0027R01-CR_Delete_Object_Instance OMA-DM-LightweightM2M-2013-0028R01-CR_Location_objet_speed_direction |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130409-D | 09 Apr 2013 | All | OMA-DM-LightweightM2M-2013-0047R02-CR_major_TS_cleanup OMA-DM-LightweightM2M-2013-0029R01-CR_Server_Object_Instance_Deletion OMA-DM-LightweightM2M-2013-0030R01-CR_Registration_Update OMA-DM-LightweightM2M-2013-0032-CR_Read_Operation_Update OMA-DM-LightweightM2M-2013-0044R01-CR_Response_Code_Update OMA-DM-LightweightM2M-2013-0034R01-CR_Device_Object_Update OMA-DM-LightweightM2M-2013-0035R01-CR_Bootstrap_Interface_Update OMA-DM-LightweightM2M-2013-0037R01-CR_Access_Control_Update OMA-DM-LightweightM2M-2013-0038R02-CR_Firmware_Object_Update OMA-DM-LightweightM2M-2013-0039-CR_Moving_Response_Code_Chapter |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130412-D | 12 Apr 2013 | All | OMA-DM-LightweightM2M-2013-0054R02-CR_Bootstrap_Process_Update OMA-DM-LightweightM2M-2013-0051-CR_certificate_definition OMA-DM-LightweightM2M-2013-0052-CR_root_resource OMA-DM-LightweightM2M-2013-0053R01-CR_connectivity_object_update OMA-DM-LightweightM2M-2013-0050-CR_object_template_and_datatypes OMA-DM-LightweightM2M-2013-0036R02-CR_Information_Reporting_Update OMA-DM-LightweightM2M-2013-0049R01-CR_queue_mode |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130522-D | 22 May 2013 | All | OMA-DM-LightweightM2M-2013-0062R01-CR_TS_Editorial_streamlining OMA-DM-LightweightM2M-2013-0041R02-CR_Update_Error_Code OMA-DM-LightweightM2M-2013-0048R03-CR_Statistician_Object OMA-DM-LightweightM2M-2013-0055R01-CR_Cancel_Observation OMA-DM-LightweightM2M-2013-0056R01-CR_TLV_update OMA-DM-LightweightM2M-2013-0057-CR_SMS_trigger OMA-DM-LightweightM2M-2013-0058R06-CR_Security_key_formats OMA-DM-LightweightM2M-2013-0059-CR_Adding_Create_Operation_Example OMA-DM-LightweightM2M-2013-0061R01-CR_Adding_Access_Control_Example OMA-DM-LightweightM2M-2013-0063R01-CR_Reserved_Resource_ID_Space OMA-DM-LightweightM2M-2013-0064-CR_Update_Server_Deletion Plus editorial changes done by the editor |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130610-D | 10 June 2013 | All | OMA-DM-LightweightM2M-2013-0070-CR_examples_update OMA-DM-LightweightM2M-2013-0066R01-CR_16bit_instance_IDs OMA-DM-LightweightM2M-2013-0067-CR_Observe_Operation_Range OMA-DM-LightweightM2M-2013-0068-CR_Server_Initiated_Bootstrap_Procedure OMA-DM-LightweightM2M-2013-0069R03-CR_observe_read_parameters OMA-DM-LightweightM2M-2013-0071R03-CR_Endpoint_Client_Name_–_Type_Attribute OMA-DM-LightweightM2M-2013-0072-CR_TS_X509_Validation_Rules OMA-DM-LightweightM2M-2013-0073-CR_Security_Section_Update OMA-DM-LightweightM2M-2013-0075-CR_Mandatory_Fields_and_Object_Template_Update OMA-DM-LightweightM2M-2013-0076R01-CR_Queue_Mode_Clarification OMA-DM-LightweightM2M-2013-0078R02-CR_BootstrapInformation___Objects OMA-DM-LightweightM2M-2013-0079R01-CR_Appendix_F_upgrade OMA-DM-LightweightM2M-2013-0081R02-CR_LWM2M_Security_Implications OMA-DM-LightweightM2M-2013-0082-CR_Data_Format_for_Resource_Supporting_Multiple_Instances OMA-DM-LightweightM2M-2013-0083R01-CR_no_sec_mode OMA-DM-LightweightM2M-2013-0084-CR_firmware_URI OMA-DM-LightweightM2M-2013-0085R01-CR_power_info_improvements OMA-DM-LightweightM2M-2013-0087R01-CR_Data_Type_Usage_Cleaning Plus editorial changes done by the editor |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130717-D | 17 July 2013 | All | It incorporates: OMA-DM-LightweightM2M-2013-0103-CR_A103_TLV_bit_ordering OMA-DM-LightweightM2M-2013-0098-CR_A132 OMA-DM-LightweightM2M-2013-0097-CR_A046_A099_A100_A101_A104_A118_A119_139_140 OMA-DM-LightweightM2M-2013-0096R01-CR_A047_SC_Secure_Channel OMA-DM-LightweightM2M-2013-0095-CR_A180_Appendix_F OMA-DM-LightweightM2M-2013-0094R01-CR_Addressing_Comment_A187 OMA-DM-LightweightM2M-2013-0092R03-CR_Appendix_D_Fixes OMA-DM-LightweightM2M-2013-0089R01-CR_TLV_and_Device_Object_Examples_Fixes Plus editorial changes done by Seongyoon on behalf of the editor |
| Draft Version OMA-TS-LightweightM2M-V1_0-20130802-D | 02 Aug 2013 | All | OMA-DM-LightweightM2M-2013-0100R02-CR_121 OMA-DM-LightweightM2M-2013-0101R01-CR_Client_and_Server_Initiated_Bootstrap_Update OMA-DM-LightweightM2M-2013-0102R01-CR_Resolving_Comments_on_Section_5.2 OMA-DM-LightweightM2M-2013-0108-CR_D.4_Clarification OMA-DM-LightweightM2M-2013-0110-CR_Secure_Channel_Fix |

Data Types

This section defines the data types that a Resource can be defined to be.

| Data Type | Description | Text Format | TLV Format |
| --- | --- | --- | --- |
| String | A UTF-8 string, the minimum and/or maximum length of the String MAY be defined. | Represented as a UTF-8 string. | Represented as a UTF-8 string of Length bytes. |
| Integer | An 8, 16, 32 or 64-bit signed integer. The valid range of the value for a resource SHOULD be defined. This data type is also used for the purpose of enumeration. | Represented as an ASCII signed integer. | Represented as a binary signed integer in network byte order, where the first (most significant) bit is 0 for a positive integer and 1 for a negative integer. The value may be 1 (8-bit), 2 (16-bit), 4 (32-bit) or 8 (64-bit) bytes long as indicated by the Length field. |
| Float | A 32 or 64-bit floating point value. The valid range of the value for a resource SHOULD be defined. | Represented as an ASCII signed decimal. | Represented as an IEEE 754-2008 [REF] binary floating point value. The value may use the binary32 (4 byte Length) or binary64 (8 byte Length) format as indicated by the Length field. |
| Boolean | An integer with the value 0 for False and the value 1 for True. | Represented as the ASCII value 0 or 1. | Represented as an Integer with value 0 , or 1. The Length of a Boolean value MUST always be 1. |
| Opaque | A sequence of binary octets, the minimum and/or maximum length of the String MAY be defined. |  | Represented as a sequence of binary data of Length bytes. |
| Time | Unix Time. A signed integer representing the number of seconds since Jan 1st, 1970 in the UTC time zone. | Represented as an ASCII integer. | Same representation as Integer. |

REF = IEEE Computer Society (August 29, 2008). IEEE Standard for Floating-Point Arithmetic. IEEE. doi:10.1109/IEEESTD.2008.4610935. ISBN 978-0-7381-5753-5. IEEE Std 754-2008

LWM2M Object Template and Guidelines   (Informative)

This Appendix provides the template to be used for the specification of LWM2M objects. Furthermore, guidelines for the creation of LWM2M objects are provided.

Object Template

Appendix D.x	 LWM2M Object: <LWM2M object name>

Description:

Object info:

| Object | Object ID | Object URN | Multiple Instances? | Mandatory? |
| --- | --- | --- | --- | --- |
| Object Name | 16-bit Unsigned Integer | urn:oma:lwm2m:{oma,ext,x}:{Object ID} | Yes/No | Yes/No |

R

- Object: specifies the Object name.

- Object ID: specifies the Object ID.

- Object URN: specifies the Object URN. The format of the Object URN is “urn:oma:lwm2m:{oma,ext,x}:{Object ID}” and {} part means that those values are variable and filled with real value. For example, Object URN of LWM2M Server Object is “urn:oma:lwm2m:oma:1”.

- Multiple Instances: indicates whether this Object supports multiple Object Instances. If this field is “Yes” then the number of Object Instance can be from 0 to many. If this field is “No” then the number of Object Instance can be from 0 to 1. If Mandatory of Object is “Yes” and Multiple Instances of the Object is “No” then, the number of Object Instance MUST be 1.

- Mandatory: if this field is “Yes”, then the LWM2M Client MUST support Object. If this field is “No”, then the LWM2M Client can support Object if Object is needed for deployment.

Resource info:

| Resource Name | Resource ID | Supported Operations | Multiple Instances? | Mandatory? | Data Type | Range or Enumeration | Units | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Resource Name | 0 | R (Read), W (Write), E (Execute) | Yes/No | Yes/No | String, Integer, Float, Boolean, Opaque, Time | If any | If any | Description |

- Resource Name: specifies the Resource name.

- Resource ID: specifies the Resource ID which is unique within Object.

- Supported Operations: indicates which operations the Resource supports in “Device Management & Service Enablement” Interface. This field can have a combination of R (Read), W (Write), and E (Execute). This field also have empty value, which means that this field is not allowed to access via “Device Management & Service Enablement” Interface.

- Multiple Instances: indicates whether this Resource supports multiple Resource Instances. If this field is “Yes” then the number of Resource Instance can be from 0 to many. If this field is “No” then the number of Resource Instance can be from 0 to 1. If Mandatory of Resource is “Yes” and Multiple Instances of the Resource is “No” then, the number of Resource Instance MUST be 1.

- Mandatory: if this field is “Yes”, then the LWM2M Client MUST support Resource. If this field is “No”, then the LWM2M Client can support the Resource if the Resource is needed for deployment.

- Data Type: Data Type indicates the type of Resource value. Data Types used in this enabler are described in Appendix B Data Types.

- Range or Enumeration: this field limits the value of Resource.

- Units: specifies the unit of the Resource value.

- Description: specifies the Resource description.

OMNA Guidelines

This section defines guidelines for OMNA regarding registries and protocol ID ranges to be maintained.

Object Registry

LWM2M objects must be registered with the OMNA Lightweight Object registry. There are three classes of Objects in which an Object can be registered:

- OMA Objects (oma label) – Objects defined by the Open Mobile Alliance.

- 3rd Party SDO Objects (ext label) – Objects defined by a 3rd party SDO.

- Vendor Specific Objects (x label) – Objects defined by a vendor or individual, such an object may be either private (no DDF or Specification made available) or public.

Each one of these classes is assigned a range of IDs by OMNA.

The URN format for an Object is automatically built from the class of Object and the Object ID as follows:

urn:oma:lwm2m:{oma,ext,x}:{Object ID}

Resource Registry

LWM2M Objects are specified as being composed of Resources, each identified by a Resource ID. Resources can either be specific to each Object with meaning only when used in that Object, or Reusable Resources can be registered, assigned an ID from the OMNA range and re-used in any Object. The following Resource ID ranges are defined:

- Object specific Resource ID range – Defined by the Object specification.

- Reusable Resource ID range – Registered by an Object Specification, with the Resource ID assigned by OMNA. Defined in any Object specification. Resources from this Resource ID range can be re-used in any Object.

- Reserved range – Range or Resource IDs reserved for future use.

A Reusable Resource ID registration entry MUST define the Resource Name, Resource ID (assigned by OMNA), Supported Operations, Data Type, Range or Enumeration, Units and Description of the Resource.

LWM2M Objects defined by OMA   (Normative)

This Appendix provides LWM2M Objects defined by OMA. Other organizations and companies may define additional LWM2M according to the guidelines and template provided in Annex C.

# The following LWM2M objects have been defined by OMA

- LWM2M Server Access Security

- LWM2M Server

- Access Control

- Device

- Connectivity

- Firmware

- Location

- Connectivity Statistics

LWM2M Object: LWM2M Server Access Security

Description: This LWM2M object provides the keying material of a LWM2M Client appropriate to access a specified LWM2M Server. One Object Instance SHOULD address a LWM2M Bootstrap Server

These LWM2M object resources MUST only be changed by a LWM2M Bootstrap Server or SmartCard provisioning and MUST NOT be accessible by any other Server.

Object Info:

| Object | Object ID | Object URN | Multiple Instances? | Mandatory? |
| --- | --- | --- | --- | --- |
| LWM2M Server Access Security | 0 |  | Yes | Yes |

Resource Info:

| Resource Name | Resource ID | Supported Operations | Multiple Instances? | Mandatory? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LWM2M Bootstrap Server URI | 0 |  | No | Yes | String | 0 – 255 bytes | - | Relevant information for Bootstrap Server only. Uniquely identifies the LWM2M Bootstrap Server, and is in the form: “coaps://host:port”, where host is an IP address or FQDN, and port is the UDP port of the Server. |
| BootStrapServer | 1 |  | No | Yes | Boolean | 1-bit | - | Determins if the current instance concerns a LWM2M Bootstrap Server (true) or a standard LWM2M Server(false) |
| Security Mode | 2 |  | No | Yes | Integer | 0-3 | - | Determines which security mode of DTLS is used 0: Pre-Shared Key mode 1: Raw Public Key mode 2: Certificate mode 3: NoSec mode |
| Public Key or Identity | 3 |  | No | Yes | Opaque |  | - | Stores the Certificate (Certificate mode), public key (RPK mode) or PSK Identity (PSK mode). The format is defined in Section D.2.1. |
| Secret Key | 4 |  | No | Yes | Opaque |  | - | Stores the secret key or private key of the security mode. The format of the keying material is defined by the security mode in Section D.2.1. This resource MUST only be changed by a bootstrap server and MUST NOT be readable by any server. |
| Short Server ID | 5 |  | No | Yes | Integer | 1-65535 | - | Relevant information for non Bootstrap Server only. Used as link to associate server object instance. |
| ClientHoldOffTime | 6 |  | No | Yes | Integer |  | s | Relevant information for a Bootstrap Server only. The number of seconds to wait before initiating a Client Initiated Bootstrap once the LWM2M Client has determined it should initiate this bootstrap mode |

- D.1.1 Security Key Resource Format

This section defines the format of the Secret Key and Public Key and Identity resources of the LWM2M Server and LWM2M Bootstrap Objects. These resources are used to configure the security mode and keying material that a Client uses with a particular Server. The Objects are configured on the Client using one of the Bootstrap mechanisms described in Section 5.1. The use of this keying material for each security mode is defined in Section 7.1.

- D.1.1.1 Pre-Shared Key (PSK) Mode

The PSK is a binary shared secret key between the Client and Server of the appropriate length for the Cipher Suite used [RFC4279]. This key is composed of a suqeuence of binary bytes in the Secret Key resource. The default PSK Cipher Suites defined in this specification use a 128-bit AES key. Thus this key would be represented in 16 bytes in the Secret Key Resource.

The corresponding PSK Identity for this PSK is stored in the Public Key or Identity resource. The PSK Identity is simply stored as a UTF-8 String as per [RFC4279]. Clients and Servers MUST support a PSK Identity of at least 128 bytes in length as required by [RFC4279].

- D.1.1.2 Raw-Public Key (RPK) Mode

The raw-public key mode requires a public key and a private key of the appropriate type and length for the Cipher Suite used. These keys are carried as a sequence of binary bytes with the public key stored in the Public Key or Identity Resource, and the private key stored in the Secret Key Resource. The default RPK Cipher Suites defines in this specification use a 256-bit ECC key. Thus the Certificate Resource would contain a 32 byte public key and the Secret Key Resource a 32 byte private key.

- D.1.1.3 Certificate Mode

The Certificate mode requires an X.509v3 Certificate along with a matching private key. The private key is stored in the Secret Key Resource as in RPK mode. The Certificate is simply represented as binary X.509v3 in the value of the Public Key or Identity Resource.

LWM2M Object: LWM2M Server

Description: This LWM2M objects provides the data related to a LWM2M Server. A Bootstrap Server has no such an object instance associated to it.

Object Info:

| Object | Object ID | Object URN | Multiple Instances? | Mandatory? |
| --- | --- | --- | --- | --- |
| LWM2M Server | 1 |  | Yes | Yes |

Resource Info:

| Resource Name | Resource ID | Supported Operations | Multiple Instances? | Mandatory? | Data Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LWM2M Server URI | 0 | R, W | No | Yes | String | 0 – 255 bytes | - | Uniquely identifies the LWM2M Server, and is in the form: “coaps://host:port”, where host is an IP address or FQDN, and port is the UDP port of the Server. |
| Lifetime | 1 | R, W | No | Yes | Integer |  | s | Specify the lifetime of the registration in seconds. |
| Short Server ID | 2 | R | No | Yes | Integer | 1-65535 | - | This identifier uniquely identifies each LWM2M Server configured for the LWM2M Client. default Short Server ID (i.e. 0) MUST not be used for identifying the LWM2M Server |
| Default Minimum Period | 3 | R, W | No | No | Integer |  | s | The default value the Client should use for the Minimum Period of an Observation in the absence of this parameter being included in an Observation. If this Resource doesn’t exist, the default value is 1. |
| Default Maximum Period | 4 | R, W | No | No | Integer |  | s | The default value the Client should use for the Maximum Period of an Observation in the absence of this parameter being included in an Observation. |
| Disable | 5 | E | No | No |  |  |  | If this Resource is executed, this LWM2M Server Object is disabled for a certain period defined in the Disabled Timeout Resource. In this period, the LWM2M Client MUST NOT send any message to the Server and ignore all the messages from the Server. When disabled, Client MUST perform de-registration process and underlying network connection between the Client and Server MUST be disconnected. |
| Disable Timeout | 6 | R, W | No | No | Integer |  | s | A period to disable the Server. After this period, the LWM2M Client MUST perform registration process to the Server. If this Resource is not set, a default timeout value is 86400 (1 day). |
| Notification Storing When Disabled or Offline | 7 | R, W | No | Yes | Boolean |  |  | If true, the LWM2M Client stores Observe Notifications to the LWM2M Server while the LWM2M Server account is disabled or the Client is offline. After the LWM2M Server account is enabled or the Client is online, the LWM2M Client reports the stored Notifications to the Server. If false, the LWM2M Client discards all the Observe Notifications or temporally disables the Observe function while the LWM2M Server is disabled or the Client is offline. The default value is true. The maximum number of storing Notification per the Server is up to the implementation. |
| Binding | 8 | R, W | No | Yes | String | The possible values of Resource are listed in 5.2.1.1 |  | This Resource defines the transport binding configured for the Client. If the Client supports the binding specified in this Resource, the Client MUST use that for current binding and mode. |
| Registration Update Trigger | 9 | E | No | Yes |  |  |  | If this resource is executed the LWM2M Client SHALL perform an “Update” logical operation with this LWM2M Server using the Current Transport Binding and Mode. |

Unbootstrapping

If a Server Object Instance is to be deleted, some related resources and configurations need to be deleted or modified. Therefore when Delete operation is sent to a Server Object Instance and Delete operation is authorized, the Client MUST proceed following procedure.

- If there is an Object Instance that can be accessed only by a Server of the Server Object Instance (i.e., the Server is Access Control Owner and there is only one ACL Resource Instance for the Server in an Access Control Object Instance), the Object Instance and the corresponding the Access Control Object Instance MUST be deleted

- If an Object Instance can be accessed by multiple Servers including the Server, then

  - An ACL Resource Instance for the Server in Access Control Object Instance for the Object Instance MUST be deleted

  - If the Server is Access Control Owner of the Access Control Object Instance, then the Access Control Owner MUST be changed to another Server according to below rules:
The Client MUST choose the Server who has highest sum of each number assigned to an access right ( Write: 1, Delete: 1) for the Access Control Owner. If some Servers get the same with the highest, the Client MUST choose one of them for the Access Control Owner

- Observation from the Server MUST be deleted

- the Server Object Instance MUST be deleted

- Client MUST send de-registration message to the Server

Note: To monitor the change of Access Control Owner, the Server MAY observe Access Control Owner Resource.

LWM2M Object: Access Control

Description: Access Control Object is used to check whether the LWM2M Server has access right for performing an operation. Each Access Control Object Instance contains ACL for a certain Object Instance.

Object Info:

| Object | Object ID | Object URN | Multiple Instances? | Mandatory? |
| --- | --- | --- | --- | --- |
| Access Control | 2 |  | Yes | No |

Resource Info:

| Resource Name | Resource ID | Supported Operations | Multiple Instances? | Mandatory? | Data Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Object ID | 0 | R, W | NO | Yes | Integer | 1-65535 | - | See Table 3. The Object ID and The Object Instance ID are applied for |
| Object Instance ID | 1 | R, W | NO | Yes | Integer | 1-65535 | - | See Table 14: LWM2M Identifiers. |
| ACL | 2 | R, W | YES | Yes | Integer | 8-bit | - | Resource Instance ID MUST be the same with Short Server ID of a certain LWM2M Server which has an access right. Resource Instance ID 0 is for default Short Server ID. Value corresponding to the Resource Instance ID is 1 byte access right value specified as below. 1st lsb: Read 2nd lsb: Write 3rd lsb: Execute 4th lsb: Create 5th lsb: Delete 6th lsb: Discover Attributes 7th lsb: Write Attributes Other bits are reserved for future use |
| Access Control Owner | 3 | R, W | NO | Yes | Integer | 0-65535 | - | Short Server ID of a certain LWM2M Server. This LWM2M Server only can manage these Resources of the Object Instance. |

Object Instance Consideration

This Object MUST be instantiated in two cases:

| Case 1: Object Instance for authorizing Read, Write, Execute and Delete operation | Case 2: Object Instance for authorizing Create operation |
| --- | --- |
| The Object Instance MUST be used for authorizing Read, Write, Execute and Delete operation. In this case, Object ID, Object Instance ID, ACL, and Access Control Owner Resource MUST be specified. Access right for Create operation in the Object Instance MUST be ignored. This Object Instance MUST be managed by Access Control Owner. | The Object Instance MUST be used for authorizing Create operation. In this case, Only Object ID and ACL MUST be specified and the other Resources MUST NOT be specified. Access right except for Create operation MUST be ignored. This Object Instance MUST be managed by Bootstrap interface. |

LWM2M Object: Device

Description: This LWM2M Object provides a range of device related information which can be queried by the LWM2M Server, and a device reboot and factory reset function.

Object info:

| Object | Object ID | Object URN | Multiple Instances? | Mandatory? |
| --- | --- | --- | --- | --- |
| Device | 3 |  | No | Yes |

Resource Info:

| Resource Name | Resource ID | Supported Operations | Multiple Instances? | Mandatory? | Data Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Manufacturer | 0 | R | No | No | String |  | - | Human readable manufacturer name |
| Model Number | 1 | R | No | No | String |  | - | A model identifier (manufacturer specified string) |
| Serial Number | 2 | R | No | No | String |  | - | Serial Number |
| Firmware Version | 3 | R | No | No | String |  | - | Current firmware version |
| Reboot | 4 | E | No | Yes | - |  | - | Reboot the LWM2M Device to restore the Device from unexpected firmware failure. |
| Factory Reset | 5 | E | No | No | - |  | - | Perform factory reset of the LWM2M Device to make the LWM2M Device have the same configuration as at the initial deployment. When this Resource is executed, “De-register” logical operation MAY be sent to the LWM2M Server(s) before factory reset of the LWM2M Device |
| Available Power Sources | 6 | R | Yes | No | Integer | 0-7 |  | 0 – DC power 1 – Internal Battery 2 – External Battery 4 – Power over Ethernet 5 – USB 6 – AC (Mains) power 7 – Solar |
| Power Source Voltage | 7 | R | Yes | No | Integer |  | mV | Present voltage for each Available Power Sources Resource Instance. Each Resource Instance ID MUST map to the value of Available Power Sources Resource |
| Power Source Current | 8 | R | Yes | No | Integer |  | mA | Present current for each Available Power Source |
| Battery Level | 9 | R | No | No | Integer | 0-100 | % | Contains the current battery level as a percentage (with a range from 0 to 100). This value is only valid when the value of Available Power Sources Resource is 1. |
| Memory Free | 10 | R | No | No | Integer |  | KB | Estimated current available amount of storage space which can store data and software in the LWM2M Device (expressed in kilobytes). |
| Error Code | 11 | R | Yes | Yes | Integer |  |  | 0=No error 1=Low battery power 2=External power supply off 3=GPS module failure 4=Low received signal strength 5=Out of memory 6=SMS failure 7=IP connectivity failure 8=Peripheral malfunction When the single Device Object Instance is initiated, there is only one error code Resource Instance whose value is equal to 0 that means no error. When the first error happens, the LWM2M Client changes error code Resource instance to any non-zero value to indicate the error type. When any other error happens, a new error code Resource instance is created. This error code Resource MAY be observed by the LWM2M server. How to deal with LWM2M Client’s error report depends on the policy of the LWM2M Server. |
| Reset Error Code | 12 | E | No | No | - | - | - | Delete all error code Resource instances and create only one zero-value error code that implies no error. |
| Current Time | 13 | R,W | No | No | Time |  |  | Current UNIX time of the LWM2M Client. The LWM2M Client should be responsible to increase this time value as every second elapses. The LWM2M Server is able to write this Resource to make the LWM2M Client synchronized with the LWM2M Server. |
| UTC Offset | 14 | R, W | No | No | String |  |  | Indicates in which time zone the LWM2M Device is located. UTS+X [ISO 8601], where X is the Resource value in hours. |
| Supported Binding and Modes | 15 | R | No | Yes | String |  |  | Indicates which bindings and modes are supported in the LWM2M Client. The possible values of Resource are combination of “U” or “UQ” and “S” or “SQ”. |

LWM2M Object: Connectivity Monitoring

Description: This LWM2M objects enables monitoring of parameters related to network connectivity.

In this general Connectivity Object, the Resources are limited to the most general cases common to most network bearers. It is recommended to read the description, which refers to relevant standard development organizations (e.g. 3GPP, IEEE).

The goal of the connectivity object is to carry information reflecting the more up to date values of the current connection for monitoring purposes. Resources such as signal quality, signal level; cell id are retrieved during connected mode at least for cellular networks.

Object info:

| Object | Object ID | Object URN | Multiple Instances? | Mandatory? |
| --- | --- | --- | --- | --- |
| Connectivity Monitoring | 4 |  | No | No |

Resource Info:

| Resource Name | Resource ID | Supported Operations | Multiple Instances? | Mandatory? | Data Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network Bearer | 0 | R | No | Yes | Integer |  | - | Indicates the current underlying network bearer among the below network bearer list. 0~20 are Cellular Bearers 0: GSM cellular network 1: TD-SCDMA cellular network 2: WCDMA cellular network 3: CDMA2000 cellular network 4: WiMAX cellular network 5: LTE-TDD cellular network 6: LTE-FDD cellular network 7~20: Reserved for other type cellular network 21~40 are Wireless Bearers 21: WLAN network 22: Bluetooth network 23: IEEE 802.15.4 network 24~40: Reserved for other type local wireless network 41~50 are Wireline Bearers 41: Ethernet 42: DSL 43: PLC 44~50: reserved for others type wireline networks. |
| Available Network Bearer | 1 | R | Yes | Yes | Integer |  | - | Indicates list of current available network bearer. Each Resource Instance has the value among the network bearer list. |
| Radio signal strength | 2 | R | No | Yes | Integer |  | dBm | This node contains the average value of the received signal strength indication used in current network bearer in case Network Bearer Resource has Cellular Networks (RXLEV range 0…64) 0 is < 110dBm, 64 is >-48 dBm) Refers to [3GPP.44018] for more details on Network Measurement Report encoding and [3GPP 45.008] or Wireless Network refers to the appropriate wireless standard. |
| Link Quality | 3 | R | No | No | Integer |  |  | This contains received link quality (e.g., LQI for IEEE 802.15.4, (Range (0..255)), RxQual Downlink (for GSM range is 0…7) Refers to [3GPP.44018] for more details on Network Measurement Report encoding. |
| IP Addresses | 4 | R | Yes | Yes | String |  | - | The IP addresses assigned to the connectivity interface. (e.g., IPv4, IPv6, etc.) |
| Parent IP Addresses | 5 | R | Yes | No | String |  | - | The IP address of the next-hop IP router in case Network Bearer Resource has 1(Wireless). (e.g., IPv4, IPv6, etc.) Note: these IP Addresses doesn’t indicate the Server IP address |
| Link Utilization | 6 | R | No | No | Integer | 0-100 | % | The average utilization of the link to the next-hop IP router in % in case Network Bearer Resource has 1(Wireless). |
| APN | 7 | R | Yes | No | String |  | - | Access Point Name in case Network Bearer Resource is a Cellular Network. |
| Cell ID | 8 | R | No | No | Integer |  | - | Serving Cell ID in case Network Bearer Resource is a Cellular Network. As specified in TS [3GPP 23.003] and in [3GPP. 24.008]. Range (0…65535) in GSM/EDGE UTRAN Cell ID is 28 bits length Cell Identity in WCDMA/TD-SCDMA. Range: (0..268435455). LTE Cell ID. Length is 28 bits Parameter definitions in [3GPP 25.331]. |
| SMNC | 9 | R | No | No | Integer |  | - | Serving Mobile Network Code. In case Network Bearer Resource has 0(cellular network). Range (0…999) As specified in TS [3GPP 23.003]. |
| SMCC | 10 | R | No | No | Integer |  | - | Serving Mobile Country Code. . In case Network Bearer Resource has 0(cellular network). Range (0…999) As specified in TS [3GPP 23.003]. |

LWM2M Object: Firmware

Description: This LWM2M Object enables FW management of firmware which is to be updated. This Object includes installing firmware package, updating firmware, and performing actions after updating firmware

Object Info:

| Object | Object ID | Object URN | Multiple Instances? | Mandatory? |
| --- | --- | --- | --- | --- |
| Firmware | 5 |  | No | No |

Resource Info:

| Resource Name | Resource ID | Supported Operations | Multiple Instances? | Mandatory? | Data Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Package | 1 | W | No | Yes | Opaque | - | - | Firmware package |
| Package URI | 1 | W | No |  | String | 0-255 bytes | - | URI from where the device can download the firmware package by an alternative mechanism. As soon the device has received the Package URI it performs the download at the next practical opportunity. |
| Update | 2 | E | No | Yes | - | - | - | Updates firmware by using the firmware package stored in Package, or, by using the firmware downloaded from the Package URI. This Resource is only executable when the value of the State Resource is Downloaded. |
| State | 3 | R | No | Yes | Integer | 1-3 |  | Indicates current state with respect to this firmware update. This value is set by the LWM2M Client. 1: Idle (before downloading or after updating) 2: Downloading (The data sequence is on the way) 3: Downloaded If writing the firmware package to Package Resource is done, or, if the device has downloaded the firmware package from the Package URI the state changes to Downloaded. If writing an empty string to Package Resource is done, the state changes to Idle. If performing the Update Resource failed, the state remains at Downloaded. If performing the Update Resource was successful, the state changes from Downloaded to Idle. |
| UpdateSupportedObjects | 4 | R, W | No | No | Boolean |  |  | If this value is true, the LWM2M Client MUST inform the registered LWM2M Servers of Objects and Object Instances parameter by sending an Update or Registration message after the firmware update operation at the next practical opportunity if supported Objects in the Client have changed, in order for the Servers to promptly manage newly installed Objects. If false, Objects and Object Instances parameter MUST be reported at the next periodic Update message. The default value is false. |
| Update Result | 5 | R | No | Yes | Integer | 0-6 | - | 0: Default value. Once the updating process is initiated, this Resource SHOULD be reset to default value. 1: Firmware updated successfully, 2: Not enough storage for the new firmware package. 3. Out of memory during downloading process. 4: Connection lost during downloading process. 5: CRC check failure for new downloaded package. 6: Unsupported package type. 7: Invalid URI This resource MAY be reported by sending Observe operation. |

Firmware Update Consideration

If some Objects are not supported after firmware update, the Client MUST delete all the Object Instances of the Objects that are not supported.

LWM2M Object: Location

Description: This LWM2M objects provide a range of device related information which can be queried by the LWM2M server, and a device reboot and factory reset function.

Object info:

| Object | Object ID | Object URN | Multiple Instances? | Mandatory? |
| --- | --- | --- | --- | --- |
| Location | 6 |  | No | No |

Resource Info:

| Resource Name | Resource ID | Supported Operations | Multiple Instances? | Mandatory? | Data Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitude | 0 | R | No | Yes | String |  | Deg | The decimal notation of latitude, e.g. -43.5723 [World Geodetic System 1984] |
| Longitude | 1 | R | No | Yes | String |  | Deg | The decimal notation of longitude, e.g. 153.21760 [World Geodetic System 1984] |
| Altitude | 2 | R | No | No | String |  | m | The decimal notation of altitude in meters above sea level. |
| Uncertainty | 3 | R | No | No | String |  | m | The accuracy of the position in meters. |
| Velocity | 4 | R | No | No | Opaque |  | Refers to 3GPP GAD specs | The velocity of the device as defined in 3GPP 23.032 GAD specification. This set of values may not be available if the device is static. |
| Timestamp | 5 | R | No | Yes | Time |  |  | The timestamp of when the location measurement was performed. |

LWM2M Object: Connectivity Statistics

Description: This LWM2M objects enables client to collect statistical information and enables the server to retrieve these information, set the collection duration and reset the statistical parameters.

Object info:

| Object | Object ID | Object URN | Multiple Instances? | Mandatory? |
| --- | --- | --- | --- | --- |
| Connectivity Statistics | 7 |  | No | No |

Resource Info:

| Resource Name | Resource ID | Supported Operations | Multiple Instances? | Mandatory? | Data Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SMS Tx Counter | 0 | R | No | No | Integer |  |  | Indicate the total number of SMS successfully transmitted during the collection period. |
| SMS Rx Counter | 1 | R | No | No | Integer |  |  | Indicate the total number of SMS successfully received during the collection period. |
| Tx Data | 2 | R | No | No | Integer |  | Kilo-Bytes | Indicate the total amount of data transmitted during the collection period. |
| Rx Data | 3 | R | No | No | Integer |  | Kilo-Bytes | Indicate the total amount of data received during the collection period. |
| Max Message Size | 4 | R | No | No | Integer |  | Byte | The maximum message size that is used during the collection period. |
| Average Message Size | 5 | R | No | No | Integer |  | Byte | The average message size that is used during the collection period. |
| StartOrReset | 6 | E | No | Yes |  |  |  | Start to collect information or reset all other resources to zeros in this object. For example, the first time this resource is executed, the client starts to collect information. The second time this resource is executed, the values of resource 0~5 are reset to 0. |

Example LWM2M Client (Informative)

This section defines an example LWM2M Client for a simple imaginary device with a Cellular interface including instantiated Objects and their values, which is used throughout this specification in examples. The example has the Endpoint Name “example-client”. The example device has two Server Objects (it is configured to register with two different LWM2M Servers), two accompanying ACL Objects for those servers, a Device Object and a Connectivity Monitoring Object for a Cellular interface. The first Server controls the access control rights for both servers.

# Table 22: Object instances of the example

| Object | Object ID | Object Instance ID |
| --- | --- | --- |
| LWM2M Server Access Security Object[0] | 0 | 0 |
| LWM2M Server Access Security Object[1] | 0 | 1 |
| LWM2M Server Access Security Object[2] | 0 | 2 |
| LWM2M Server Object [1] | 1 | 1 |
| LWM2M Server Object [2] | 1 | 2 |
| Access Control Object [0] | 2 | 0 |
| Access Control Object [1] | 2 | 1 |
| Access Control Object [2] | 2 | 2 |
| Device Object | 3 | - |
| Connectivity Monitoring Object | 4 | - |

# Table 23: LWM2M Server Access Security Object [0]

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Server URI | 0 |  | coap://bootstrap.example.com | Example Bootstrap Server |
| Bootstrap Server | 1 |  | true |  |
| Security Mode | 2 |  | 0 | PSK mode |
| Public Key or Identity | 3 |  | [identity string] | PSK Identity |
| Secret Key | 4 |  | [secret key data] | AES key |
| Short Server ID | 5 |  | 0 | unused |
| Client Hold Off Time | 6 |  | 3600 |  |

# Table 24: LWM2M Server Access Security Object [1]

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Server URI | 0 |  |  | unused |
| Bootstrap Server | 1 |  | false |  |
| Security Mode | 2 |  | 0 | PSK mode |
| Public Key or Identity | 3 |  | [identity string] | PSK Identity |
| Secret Key | 4 |  | [secret key data] | AES key |
| Short Server ID | 5 |  | 101 |  |
| Client Hold Off Time | 6 |  | 0 | unused |

# Table 25: LWM2M Server Access Security Object [2]

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Server URI | 0 |  |  | unused |
| Bootstrap Server | 1 |  | false |  |
| Security Mode | 2 |  | 0 | PSK mode |
| Public Key or Identity | 3 |  | [identity string] | PSK Identity |
| Secret Key | 4 |  | [secret key data] | AES key |
| Short Server ID | 5 |  | 102 |  |
| Client Hold Off Time | 6 |  | 0 | unused |

# Table 26: LWM2M Server Object [1]

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| LWM2M Server URI | 0 |  | coap://server1.example.com | Example LWM2M Server 1 |
| Lifetime | 1 |  | 86400 |  |
| Short Server ID | 2 |  | 101 |  |
| Default Minimum Period | 3 |  | 300 |  |
| Default Maximum Period | 4 |  | 6000 |  |
| DisableTimeout | 6 |  | 86400 |  |
| Notification Storing When Disabled or Offline | 7 |  | True |  |
| Binding Preference | 8 |  | U | UDP binding preference |

# Table 27: LWM2M Server Object [2]

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| LWM2M Server URI | 0 |  | coap://server2.example.com | Example LWM2M Server 2 |
| Lifetime | 1 |  | 86400 |  |
| Short Server ID | 2 |  | 102 |  |
| Default Minimum Period | 3 |  | 60 |  |
| Default Maximum Period | 4 |  | 6000 |  |
| DisableTimeout | 6 |  | 86400 |  |
| Notification Storing When Disabled or Offline | 7 |  | False |  |
| Binding Preference | 8 |  | UQ | UDP with Queuing binding preference |

# Table 28: Access Control Object [0] (for the Device Object)

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Object ID | 0 |  | 3 | Device Object |
| Object Instance ID | 1 |  | - |  |
| ACL | 2 | 101 | R. W, E | Server 1 has all access rights. Note that the Resource Instance ID indicates the Server ID. |
| ACL | 2 | 102 | R | Server 2 has read-only access rights. Note that the Resource Instance ID indicates the Server ID. |
| Access Control Owner | 3 |  | 1 | Server 1 controls this Object’s access rights. |

# Table 29: Access Control Object [1] (for the Connectivity Monitoring Object)

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Object ID | 0 |  | 4 | Connectivity Monitoring Object |
| Object Instance ID | 1 |  | - |  |
| ACL | 2 | 101 | R | Server 1 has read-only access rights. Note that the Resource Instance ID indicates the Server ID. |
| ACL | 2 | 0 | R | The other Servers except Server 1 have read-only access rights. Note that this Resource Instance ID indicates the default Server ID. |
| Access Control Owner | 3 |  | 1 | Server 1 controls this Object’s access rights. |

# Table 30: Access Control Object [2] (for the Firmware Object)

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Object ID | 0 |  | 5 | Firmware Object |
| Object Instance ID | 1 |  | - |  |
| ACL | 2 | 101 | C | Server 1 can create Firmware Object Instance |
| Access Control Owner | 3 |  | - | This Object Instance must be managed by Bootstrap Interface |

# Table 31: Device Object

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Manufacturer | 0 |  | Open Mobile Alliance |  |
| Model Number | 1 |  | Lightweight M2M Client |  |
| Serial Number | 2 |  | 345000123 |  |
| Firmware version | 3 |  | 1.0 |  |
| Available Power Sources | 6 | 0 | 1 | Internal Battery |
| Available Power Sources | 6 | 1 | 5 | USB |
| Power Source Voltage | 7 | 0 | 3800 | 3.8V battery |
| Power Source Voltage | 7 | 1 | 5000 | USB VBUS |
| Power Source Current | 8 | 0 | 125 | 125mA |
| Power Source Current | 8 | 1 | 900 | USB 900mA |
| Battery level | 9 |  | 100 |  |
| Memory free | 10 |  | 15 | 15 kB of free memory |
| Error code | 11 | 0 | 0 | No errors |
| Current Time | 13 |  | 1367491215 | May 2nd, 2013 at 11:42 AM GMT |
| UTC Offset | 14 |  | +02:00 | UTC+2 (CET) |
| Supported Binding and Modes | 15 |  | U | UDP binding |

# Table 32: Connectivity Monitoring Object

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Network Bearer | 0 |  | 0 | GSM Bearer |
| Available Network Bearer | 1 |  | 0 | GSM Bearer |
| Radio signal strength | 2 |  | 92 | RSSI in dBm |
| Link Quality | 3 |  | 2 | RxQual Downlink |
| IP Addresses | 4 | 0 | 192.168.0.100 |  |
| Parent IP Addresses | 5 | 0 | 192.168.1.1 |  |
| Link Utilization | 6 |  | 5 | % |
| APN | 7 | 0 | internet |  |

Storage of LWM2M Bootstrap Information on the Smartcard (Normative)

This section aims at specifying the storage mechanism of Bootstrap Information on UICC SmartCard platform type [ETSI TS 102.221] activated in 3G mode.

Note: There is no rational to equip LWM2M device with 2G-only Smart Card.

File structure

The information format is based on [PKCS#15] specification. The Bootstrap data is located under the PKCS#15 directory allowing the card issuer to decide the identifiers and the file locations. The smartcard operations that are relevant include:

- Application selection

- Cardholder verification

- File access (select file, read, write)

File access (select file, read, write)The [PKCS#15] specification defines a set of files. Within the PKCS#15 application, the starting point to access these files is the Object Directory File (ODF). The EF(ODF) contains pointers to other directory files. These directory files contain information on different types of objects (authentication objects, data objects, etc).  For the purpose of Bootstrap data, EF (ODF) SHALL contain the EF Record describing the DODF-bootstrap. The EF(ODF) is described in section F.3.1 and [PKCS#15].

EF(ODF) contains pointers to one or more Data Object Directory Files (DODF) in priority order (i.e. the first DODF has the highest priority). Each DODF is regarded as the directory of data objects known to the PKCS#15 application. For the purposes of LWM2M bootstrapping, EF(DODF-bootstrap) contains pointer to the Bootstrap data, namely LWM2M_Bootstrap File. The EF(DODF-bootstrap) is described in section F.3.2 and [PKCS#15].

The provisioning files are stored as PKCS#15 opaque data objects.

The support of smartcard Bootstrap data will be indicated by the presence in the EF DIR (see [ETSI TS 102.221]) of an application template as defined here after.

The RECOMMENDED format of EF(DIR) is a linear fixed record in order to be in line with  [ETSI TS 102.221].

EF (DIR) SHALL contain the application template used for a PKCS#15 application as defined in [PKCS#15]. Application template SHALL consist of Application identifier (tag 0x4F) and Path (tag 0x51) information.

The EF(ODF) and EF(DODF-bootstrap) SHALL be used by the Device to determine the path of the LWM2M_Bootstrap file.

UICC SmartCard platforms can support two modes of activation:  2G and 3G. In the context of LWM2M, for Device simplification, UICC SHALL be activated in 3G Mode

UICC smartcard platform activated in a 3G mode has the physical and logical characteristics according to [ETSI TS 102.221]. In that case, smartcard operations for accessing the Bootstrap data are specified in section F2.

Bootstrap Information on UICC (Activated in 3G Mode)

Access to the file structure

To select the PKCS#15 application, the Device:

- SHALL  evaluate the PKCS#15 application template – i.e. PKCS#15 AID - present in the EF (DIR),

- SHALL  open a logical channel using UICC Command MANAGE CHANNEL as specified in [ETSI TS 102.221],

- SHALL select the PKCS#15 ADF using the PKCS#15 AID as parameter of the UICC Command SELECT, using direct application selection as defined in [ETSI TS 102.221].

LWM2M_Bootstrap file will be located under the PKCS#15 ADF.

Files Overview

# Figure 24: File structure for Bootstrap Message on 3G UICC

Access Method

UICC Commands Read Binary and Update Binary, as defined in [ETSI TS 102.221], are used to access bootstrap data.

Access Conditions

The Device is informed of the access conditions of provisioning files by evaluating the “private” and “modifiable” flags in the corresponding DODF-bootstrap files structure.

In the case where one of the above mentioned flag is set, cardholder verification is required. The Device must evaluate the PIN references that must be verified as defined in [ETSI TS 102.221] (ie evaluate FCP)

Requirements on the 3G UICC

To retrieve the Bootstrap Information from the 3G UICC, the Device SHALL perform the following steps:

- Select PKCS#15 file structure as specified in F.2.1.

- Read ODF to locate the DODF-bootstrap,

- Read DODF-bootstrap to locate the LWM2M_Bootstrap file,

- Read the LWM2M_Bootstrap file

Files Description

All files defined are binary files as defined in [ETSI TS 102.221]. These files are read and updated using 3G UICC Commands related to the application they belong to.

Object Directory File, EF ODF

The mandatory Object Directory File (ODF) ([PKCS#15], section 5.5.1) contains pointers to other EFs, each one containing a directory of PKCS#15 objects of a particular class (e.g. DODF-bootstrap). The File ID is specified in [PKCS#15]. The card issuer decides the file size. The EF (ODF) can be read but it SHALL NOT be modifiable by the user.

The EF (ODF) is described below:

| Identifier: default 0x5031, see [PKCS#15] | Structure: Binary |  | Mandatory |
| --- | --- | --- | --- |
| File size: decided by the card issuer |  | Update activity: low |  |
| Access Conditions: READ ALW UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |
| Description |  |  |  |
| See [PKCS#15] |  |  |  |

Bootstrap Data Object Directory File, EF DODF-bootstrap

This Data Object Directory File provisioning contains directories of provisioning data objects ([PKCS#15], section 6.7) known to the PKCS#15 application.

The File ID is described in the EF (ODF). The file size depends on the number of provisioning objects stored in the smartcard. Thus, the card issuer decides the file size.

| Identifier: 0x6430, See ODF | Structure: Binary |  | Mandatory |
| --- | --- | --- | --- |
| File size: decided by the card issuer |  | Update activity: low |  |
| Access Conditions: READ ALW or Universal / application / Local PIN (UICC, See section C.2) UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |
| Description |  |  |  |
| See hereafter and [PKCS#15] |  |  |  |

The EF (DODF-bootstrap) SHALL contain information on provisioning objects:

- Readable label describing the provisioning document (CommonObjectAttributes.label). The ME could display this label to the user.

- Flags indicating whether the provisioning document is private (i.e., is protected with a PIN) and/or modifiable (CommonObjectAttributes.flags). The card issuer decides whether or not a file is private (it does not need to be if it does not contain any sensitive information)

- Object identifier indicating a LWM2M boostrap object and the type of the provisioning object (CommonDataObjectAttributes.applicationOID)

- Pointer to the contents of the provisioning document (Path.path)

EF  LWM2M_Bootstrap

Only the card issuer can modify EF  LWM2M_Bootstrap

.

| Identifier: See DODF | Structure: Binary |  | Optional |
| --- | --- | --- | --- |
| File size: decided by the card issuer |  | Update activity: low |  |
| Access Conditions: READ ALW or Universal / application / Local PIN (UICC, See section C.2) UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |
| Description |  |  |  |
| Contains Bootstrap data (encapsulated LWM2M Objects) |  |  |  |

This file size is limited to 32KB; the effective file size, in Bytes, is accessible from the File header.

In this file, the Bootstrap data relies on LWM2M TLV Data format specification.

The LWM2M specification already describes the TLV format for coding multiples instances and resources of a given object (§6.3.3)., this section will only detailled how storing a collection of LWM2M objects in this file; each object being coded as a simple TLV with LWM2M Object ID as the tag, a LWM2M-TLV coding the object instances as the TLV payload, and the TLV length being the size of the payload (LWM2M-TLV of the object instances).

Additionally, this Bootstrap data will have a 2 Byte header indicating the number of objects contained in that file and another 2 Bytes for indicating the size of the payload (size of the collection of LWM2M Objects).

Using a BNF-like description:

<bootstrap_data>              ::= <number of objects> <size> <collection_of_lwm2m_objects>

<number of Objects>           ::= HWORD

<size>                       ::= HWORD

<collection_of_lwm2m_objects> ::= <single_lwm2m_object>*

<single_lwm2m_object>  ::=  <lwm2m_object_ID> <length_of_object> <lwm2m_object_instances>

<lwm2m_object_ID>             ::=  HWORD

<length_of_object>            ::=  HWORD

<lwm2m_object_instances>      ::= TLV data format as described in §6.3.3

HWORD                         ::= %x00-FFFF

In reading and processing the data of this file, the LWM2M Client is then able to be configured with the Bootstrap Information and thus to access the LWM2M Server(s)

Secure channel between Smartcard and LWM2M Device Storage for secure Bootstrap Data provisioning (Normative)

During LWM2M Bootstrap procedure, sensitive data have to be provisioned in LWM2M Device.

When Bootstrap information comes from Smartcard, a secure channel SHOULD be established.  When required this secure channel SHALL follow the following procedure based on [GLOBALPLATFORM][GP SCP03]  which is illustrated below. The Bootstrap information will be retrieved from Smartcard as described in Appendix F of this document but in including the channel securisation.

Pre-requisite :  the Smartcard and the LWM2M device have to share the same static Keys KEY_ENC, KEY_MAC, KEY_DEK as specified in  [GLOBALPLATFORM] [GP SCP03]

These keys are provisioned in the devices in using out-of-band methods.

The steps for the secure transfer are the following and are illustrated below (figure 25):

- The PKSC#15 application used for transferring the Bootstrap information is selected

- Secure channel (mutual authentication) is established

- PKCS#15 flow as described in Appendix F takes place for selecting and transferring the Bootstrap file from Smartcard to the device: the sensitive Bootstrap data are transferred crypted.

Figure 25: Bootstrap Infromation transfer from Smartcard to LWM2M Device using Secure channel according to [GLOBALPLATFORM] [GP SCP03] [GP AMD_A]

Note 1: The INITIALIZE_UPDATE specifies the logical channel to use (CLA: 80H / 83H)

Note 2: The security level  (P1) of the EXTERNAL_AUTH  command is C-DECRYPTION, R-ENCRYPTION, C-MAC and R-MAC (P1=0x33)

Static Conformance Requirements	(Normative)

The notation used in this appendix is specified in [IOPPROC].

SCR for LWM2M Client

| Item | Function | Reference | Status | Requirement |
| --- | --- | --- | --- | --- |
| XYZ-C-001 | Something mandatory | Section x.y | M | (XYZ-C-001 OR XYZ-C-003) AND XYZ-C-002 |
| XYZ-C-002 | Something optional | Section x.y | O |  |
| XYZ-C-003 | Dependencies on ZYX | Section x.y | M | ZYX:MCF |
| XYZ-C-004 | Dependencies on ZYX | Section x.y | O | ZYX:OCF |

SCR for LWM2M Server

| Item | Function | Reference | Status | Requirement |
| --- | --- | --- | --- | --- |
| XYZ-S-001 | Something mandatory | Section x.y | M | XYZ-S-001 OR XYZ-S-002 OR XYZ-S-003 |
| XYZ-S-002 | Something optional | Section x.y | O |  |
| XYZ-S-003 | Dependencies on ZYX | Section x.y | M | ZYX:MSF |
| XYZ-S-004 | Dependencies on ZYX | Section x.y | O | ZYX:OSF |