| Enabler Test Specification for RCS-e |  |  |
| --- | --- | --- |
| Draft Version 1.2 – 19 Jul 2012 |  |  |
| Open Mobile Alliance |  |  |
| OMA-ETS-RCS-e-V1_2-20120719-D |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2012 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

Contents

1.	Scope	6

2.	References	7

2.1	Normative References	7

2.2	Informative References	7

3.	Terminology and Conventions	8

3.1	Conventions	8

3.2	Definitions	8

3.3	Abbreviations	8

4.	Introduction	9

5.	RCS-e Conformance Test Cases	10

5.1	RCS-e-1.2-con-0001 First-time unsuccessful configuration: subscriber unauthorized	10

5.2	RCS-e-1.2-con-0002 Configuration triggered by a reboot: No need for update	10

5.3	RCS-e-1.2-con-0003 Configuration triggered by a reboot: Update needed	11

5.4	RCS-e-1.2-con-0004 Configuration triggered by a reboot: No reconfiguration as the parameters are still valid	11

5.5	RCS-e-1.2-con-0005 Configuration triggered by a reboot: Update needed	12

5.6	RCS-e-1.2-con-0006 User discovery in addressbook: Discovery of users via checking capabilities (Non-RCS-e user)	13

5.7	RCS-e-1.2-con-0007 Remove a user from the RCS-e list	13

5.8	RCS-e-1.2-con-0008 Successful file transfer from FT/gallery app (one-to-one)	14

5.9	RCS-e-1.2-con-0009 Successful multiple file transfer from chat application (one-to-one)	14

5.10	RCS-e-1.2-con-0010 Chat inactivity timeout	15

5.11	RCS-e-1.2-con-0011 Re-Opening a chat / conversation history	15

5.12	RCS-e-1.2-con-0012 SIP and MSRP protocol transport over PS	16

5.13	RCS-e-1.2-con-0101 Mobile Initiated Capability Discovery - Successful	16

5.14	RCS-e-1.2-con-0102 Mobile Initiated Capability Discovery – Unsuccessful – 480 Temporary Unavailable	17

5.15	RCS-e-1.2-con-0103 Mobile Initiated Capability Discovery – Unsuccessful –408  Request Timeout	18

5.16	RCS-e-1.2-con-0104 Mobile initiated Capability discovery – Unsuccessful – 404 Not Found	18

5.17	RCS-e-1.2-con-0105 Mobile Initiated Capability Discovery – Successful – Multiple Identities	19

5.18	RCS-e-1.2-con-0107 Mobile Terminated Capability Discovery – Correct Presentation of Remote Capabilities	20

5.19	RCS-e-1.2-con-0108 Mobile Terminated Capability Discovery – Learning Unknown New Users RCS-e Capabilities	20

5.20	Capability update during a MO call for video /image sharing	21

5.21	Capability update during a MT call for video /image sharing	22

5.22	Capability update during a MO voice call fails as other end has no capability	23

5.23	MO video sharing call establishment - successful	23

5.24	MT video sharing call establishment - successful	24

6.	RCS-e Interoperability Test Cases	26

6.1	RCS-e-1.2-int-0101- First-time successful configuration	26

6.2	RCS-e-1.2-int-0301- User discovery in addressbook: Initial check of capabilities	26

6.3	RCS-e-1.2-int-0302- User discovery in addressbook: Adding a RCS-e contact when registered	27

6.4	RCS-e-1.2-int-0303- User discovery in addressbook: Discovery of users via checking capabilities (RCS-e user)	27

6.5	RCS-e-1.2-int-0401- Capability query during a call	28

6.6	RCS-e-1.2-int-0501- Capability query in addressbook	28

6.7	RCS-e-1.2-int-0502- Capability query in call-log	29

6.8	RCS-e-1.2-int-0503- Capability query in 1-2-1 chat (file transfer supported)	29

6.9	RCS-e-1.2-int-0701- Successful file transfer from addressbook (one-to-one)	30

6.10	RCS-e-1.2-int-0702- Successful file transfer from chat application (one-to-one)	31

6.11	RCS-e-1.2-int-0703- Successful multiple file transfer from FT/gallery app (one-to-many)	31

6.12	RCS-e-1.2-int-0704- File transfer rejected by the destination	32

6.13	RCS-e-1.2-int-0901- Video share: sending video – caller	33

6.14	RCS-e-1.2-int-0902- Video share: sending video – receiver	33

6.15	RCS-e-1.2-int-0903- Video share: reject	34

6.16	RCS-e-1.2-int-1001- Share a picture taken using the camera	34

6.17	RCS-e-1.2-int-1002- Share a stored image	35

6.18	RCS-e-1.2-int-1003- Share a stored image. User B rejects	35

6.19	RCS-e-1.2-int-1101- Chat 1 to 1. Start from address book	36

6.20	RCS-e-1.2-int-1102- Chat 1 to 1. Start from IM/Chat application	36

6.21	RCS-e-1.2-int-1103– Composing chat window/leaving background	37

6.22	RCS-e-1.2-int-1104– “Is composing” Notification /Typing info	37

6.23	RCS-e-1.2-int-1105- Close chat window	38

6.24	RCS-e-1.2-int-1106– Change chat 1 to 1 -> one to many	39

Appendix A.	Change History (Informative)	41

A.1	Approved Version History	41

A.2	Draft/Candidate Version 1.0 History	41

Appendix B.	<<Optional>>Conformance Test Case applicability	42

B.1	Introduction	42

B.2	Test Cases testing only mandatory features	42

B.3	Applicability	42

B.3.1	Client ICS	42

B.3.2	Client IXIT	42

B.3.3	Server ICS	42

B.3.4	Server IXIT	43

B.4	ICS to test case mapping	43

Appendix C.	<Additional Information>	44

C.1	App Headers	44

C.1.1	More Headers	44

Figures

Figure 1: Example Figure	44

Tables

Table 1: Test Information for 0101 Interoperability Test	26

Table 2: Test Information for 0301 Interoperability Test	27

Table 3: Test Information for 0302 Interoperability Test	27

Table 4: Test Information for 0303 Interoperability Test	28

Table 5: Test Information for 0401 Interoperability Test	28

Table 6: Test Information for 0501 Interoperability Test	29

Table 7: Test Information for 0502	 Interoperability Test	29

Table 8: Test Information for 0503	 Interoperability Test	30

Table 9: Test Information for 0701 Interoperability Test	31

Table 10: Test Information for 0702 Interoperability Test	31

Table 11: Test Information for 0703 Interoperability Test	32

Table 12: Test Information for 0704 Interoperability Test	33

Table 13: Test Information for 0901 Interoperability Test	33

Table 14: Test Information for 0902 Interoperability Test	34

Table 15: Test Information for 0903 Interoperability Test	34

Table 16: Test Information for 1001 Interoperability Test	35

Table 17: Test Information for 1002 Interoperability Test	35

Table 18: Test Information for 1003 Interoperability Test	36

Table 19: Test Information for 1101 Interoperability Test	36

Table 20: Test Information for 1102 Interoperability Test	37

Table 21: Test Information for 1103 Interoperability Test	37

Table 22: Test Information for 1104 Interoperability Test	38

Table 23: Test Information for 1105 Interoperability Test	39

Table 24: Test Information for 1106 Interoperability Test	40

# Scope

This document describes in detail available test cases for the deployment suite of RCS-e 1.2.2, http://www.gsma.com/rcs/wp-content/uploads/2012/03/rcs-e_advanced_comms_specification_v1_2_2_approved.pdf.

The test cases are split in two categories, conformance and interoperability test cases.

The conformance test cases are aimed to verify the adherence to normative requirements described in the technical specifications.

The interoperability test cases are aimed to verify that implementations of the specifications work satisfactory.

If either conformance or interoperability tests do not exist at the creation of the test specification this part should be marked unavailable.

# References

## Normative References

| [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |
| --- | --- |
| [OMA_IM_TS] | “Instant Messaging Requirements”, Version 1.0, Open Mobile Alliance™, OMA-RD-IM-V1_0, URL: http://www.openmobilealliance.org/ |
| [OMA_IM_XDM] | “IM XDM Specification”, Version 1.0, Open Mobile Alliance™, OMA-TS-IM_XDM-V1_0, URL: http://www.openmobilealliance.org/ |

## Informative References

| [OMADICT] | “Dictionary for OMA Specifications”, Version x.y, Open Mobile Alliance™, OMA-ORG-Dictionary-Vx_y, URL:http://www.openmobilealliance.org/ |
| --- | --- |
| [RCS-e] | RCS-e Specifications http://www.gsma.com/rcs/wp-content/uploads/2012/03/rcs-e_advanced_comms_specification_v1_2_2_approved.pdf |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope”, are normative, unless they are explicitly indicated to be informative.

The following numbering scheme is used:

xxx-y.z-con-number where:
xxx	Name of enabler, e.g. MMS or Browsing
y.z	Version of enabler release, e.g. 1.2 or 1.2.1
’con’	Indicating this test is a conformance test case
number	Leap number for the test case

Or

xxx-y.z-int-number where:
xxx	Name of enabler, e.g. MMS or Browsing
y.z	Version of enabler release, e.g. 1.2 or 1.2.1
’int’	Indicating this test is a interoperability test case
number	Leap number for the test case

## Definitions

## Abbreviations

| RCS-e | Rich Communications Suite-enhanced |
| --- | --- |

# Introduction

The purpose of this document is to provide test cases for RCS-e version 1.2.2.

The implementation of some features is optional for the Clients and/or the Servers in the RCS-e Enabler.  The tests associated with these optional features are marked as "(Includes Optional Features)" in the test specification.

# RCS-e Conformance Test Cases

## RCS-e-1.2-con-0001 First-time unsuccessful configuration: subscriber unauthorized

| Test Case Id | RCS-e-1.2-con-0001 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | First-time successful configuration |
| Specification Reference | RCS-e 2.2.2.1.2 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | Autoconfiguration server ready User A is not IMS registered for RCS-e RCS-e services have not been previously configured on the phone/SIM pair |
| Test Procedure | 1) Handset is powered on 2) The handset should contact the autoconfiguration server 3) The autoconfiguration server validates the SIM/phone, and generates a response with the XML but both validity and version set to 0 4) Following a reboot the handset retries the configuration |
| Pass-Criteria | 1) Configuration takes places seamlessly to the user 2) RCS-e UX elements not enabled (vanilla behaviour) as the configuration is not successful |

## RCS-e-1.2-con-0002 Configuration triggered by a reboot: No need for update

| Test Case Id | RCS-e-1.2-con-0002 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Configuration triggered by a reboot: No need for update |
| Specification Reference | RCS-e 2.2.2.1.2 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | Autoconfiguration server ready RCS-e services have been successfully configured on the phone/SIM pair The validity of the previous configuration is now expired |
| Test Procedure | 1) Handset is powered on 2) The handset should contact the autoconfiguration server 3) The autoconfiguration server validates the SIM/phone and provides a response without configuration data except from the version (same as the one in the phone) and the validity (set to the default period configured in the server) 4) The configuration is transparent, the RCS-e services are available just after boot is completed |
| Pass-Criteria | 1) Configuration takes places seamlessly to the user 2) The RCS-e UX and services remain in the same condition as prior to the configuration |

## RCS-e-1.2-con-0003 Configuration triggered by a reboot: Update needed

| Test Case Id | RCS-e-1.2-con-0003 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Configuration triggered by a reboot: Update needed |
| Specification Reference | RCS-e ID_RCSE_1_2_3 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | Autoconfiguration server ready RCS-e services have been successfully configured on the phone/SIM pair The validity of the previous configuration is now expired |
| Test Procedure | 1) Handset is powered on 2) The handset should contact the autoconfiguration server 3) The autoconfiguration server validates the SIM/phone and sends a valid configuration to the phone with a new version 4) Once the configuration is successfully sent, it is possible to access RCS-e services (test a successful chat to verify behaviour) |
| Pass-Criteria | 1) Configuration takes places seamlessly to the user 2) RCS-e UX elements are enabled and the user can perform a successful chat after the successful configuration and first-time registration with the new configuration |

## RCS-e-1.2-con-0004 Configuration triggered by a reboot: No reconfiguration as the parameters are still valid

| Test Case Id | RCS-e-1.2-con-0004 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Configuration triggered by a reboot: No reconfiguration as the parameters are still valid |
| Specification Reference | RCS-e ID_RCSE_1_2_4 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | OMA-DM server ready to fulfil the requirements described in section 2.2.2.1.2 of the specification and Annex A RCS-e services have been successfully configured on the phone/SIM pair The validity of the previous configuration is NOT expired |
| Test Procedure | 1) Handset is powered on 2) The handset/client should NOT contact the OMA-DM server as the configuration is still valid 3) The configuration is transparent, the RCS-e services are available just after boot is completed |
| Pass-Criteria | 1) The RCS-e UX and services remain in the same condition as prior to the configuration |

## RCS-e-1.2-con-0005 Configuration triggered by a reboot: Update needed

| Test Case Id | RCS-e-1.2-con-0005 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Configuration triggered by a reboot: Update needed |
| Specification Reference | RCS-e ID_RCSE_1_2_5 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | OMA-DM server ready to fulfil the requirements described in section 2.2.2.1.2 of the specification and Annex A RCS-e services have been successfully configured on the phone/SIM pair The validity of the previous configuration is now expired |
| Test Procedure | 1) Handset is powered on 2) The handset should contact the OMA-DM server 3) The OMA-DM server validates the SIM/phone and sends a valid configuration to the phone with a new version 4) Once the configuration is successfully sent, it is possible to access |
| Pass-Criteria | 1) Configuration takes places seamlessly to the user 2) RCS-e UX elements are enabled and the user can perform a successful chat after the successful configuration and first-time registration with the new configuration |

## RCS-e-1.2-con-0006 User discovery in addressbook: Discovery of users via checking capabilities (Non-RCS-e user)

| Test Case Id | RCS-e-1.2-con-0006 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | User discovery in addressbook: Discovery of users via checking capabilities (Non-RCS-e user) |
| Specification Reference | RCS-e ID_RCSE_2_1_6 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | RCS-e services have been previously configured successfully on the phone/SIM pair The phone/client is registered and the registration timer is far from expiration |
| Test Procedure | 1) The user selects user A who is in the list of known RCS-e users, however, the result of the capability query shows that the user is no longer a RCS-e user (It receives an error response (404) or a 200 OK response without, the RCS-e IM capability tag) 2) The list can be verified from the IM application (available users to initiate a chat) |
| Pass-Criteria | 1) User A is removed from the list of RCS-e users |

## RCS-e-1.2-con-0007 Remove a user from the RCS-e list

| Test Case Id | RCS-e-1.2-con-0007 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Remove a user from the RCS-e list |
| Specification Reference | RCS-e ID_RCSE_2_1_7 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | RCS-e services have been previously configured successfully on the phone/SIM pair The phone/client is registered and the registration timer is far from expiration |
| Test Procedure | 1) User A is a known RCS-e user (in the RCS-e users list) 2) The capabilities are queried by selecting the contact from the addressbook 3) A 404 response or 200 OK without the IM tag is received, therefore user A is not longer an RCS-e user 4) The list can be verified from the IM application (available users to initiate a chat) |
| Pass-Criteria | 1) User A is removed from the list of RCS-e users |

## RCS-e-1.2-con-0008 Successful file transfer from FT/gallery app (one-to-one)

| Test Case Id | RCS-e-1.2-con-0008 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Successful file transfer from FT/gallery app (one-to-one) |
| Specification Reference | RCS-e ID_RCSE_4_2_1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A has RCS-e contacts User B is an RCS-e user who is online User A is on Wi-Fi coverage, user B is on EDGE/3G/HSPA coverage All users have enough free storage space to receive file/s |
| Test Procedure | 1. User A browses his/her files. 2. User A selects a file 3. User A selects file transfer option 4. User A selects one contact from his/her RCS-e contacts list and press file sharing option 5. User B accepts the invitation and opens the file |
| Pass-Criteria | 1) User B receives the invitation stating the file type and the size 2) If it is a supported file type, user B should visualize the relevant MIME icon or mini-preview should be displayed within the chat window (all FT take place on a chat window) 3) User A can progress the transfer in the notification bar (the transfer does not block the UI) 4) User B receives file 5) User B can open the file properly |

## RCS-e-1.2-con-0009 Successful multiple file transfer from chat application (one-to-one)

| Test Case Id | RCS-e-1.2-con-0009 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Successful multiple file transfer from chat application (one-to-one) |
| Specification Reference | RCS-e ID_RCSE_4_3_2 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A has RCS-e contacts User B is an RCS-e contact of User A who is online User A is on Wi-Fi coverage, user B is on EDGE/3G/HSPA coverage All users have enough free storage space to receive file/s |
| Test Procedure | 1. User A starts an IM/Chat with User B. 2. User A selects file transfer option 3. User A selects two files from the internal memory, and two files from the external memory list, and select share option 4. User B accepts the invitation and opens the files |
| Pass-Criteria | For each user/file, the following steps can be verified: 1) User B receives the invitation stating the file type and the size 2) If it is a supported file type, user B should visualize the relevant MIME icon or mini-preview should be displayed within the chat window (all FT take place on a chat window) 3) User A can progress the transfer in the notification bar (the transfer does not block the UI) 4) User B receives file 5) User B can open the file properly |

## RCS-e-1.2-con-0010 Chat inactivity timeout

| Test Case Id | RCS-e-1.2-con-0010 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Chat inactivity timeout |
| Specification Reference | RCS-e ID_RCSE_6_1_14 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | Terminals are registered and able to access IMS/RCS-e core network and relevant servers. Supported by network: 2G, 3G, HSPA, Wi-Fi |
| Test Procedure | 1. Remain chat inactive till timeout (configurable period of time) expires. |
| Pass-Criteria | 1) The network will close the IM establish session when no activity in the chat is detected. 2) The user does not get any indication |

## RCS-e-1.2-con-0011 Re-Opening a chat / conversation history

| Test Case Id | RCS-e-1.2-con-0011 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Re-Opening a chat / conversation history |
| Specification Reference | RCS-e ID_RCSE_6_1_16 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | Terminals are registered and able to access IMS/RCS-e core network and relevant servers. Supported by network: 2G, 3G, HSPA, Wi-Fi |
| Test Procedure | 1) User A selects a previous chat history with user B from the IM/chat application. |
| Pass-Criteria | 1) The previous stored conversation are displayed 2) User A can continue the conversation with user B from the point when chat was released 3) IM session will be established only when a new message is sent and user B accepts the chat by opening the chat window |

## RCS-e-1.2-con-0012 SIP and MSRP protocol transport over PS

| Test Case Id | RCS-e-1.2-con-0012 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | SIP and MSRP protocol transport over PS |
| Specification Reference | RCS-e ID_RCSE_7_1_1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A and B is registered and able to access IMS/RCS-e core network and relevant servers. Supported by network: 2G, 3G, HSPA |
| Test Procedure | 1) User A starts a chat from the addressbook with User B 2) User B sees the notification and opens the chat window 3) User B writes a new message |
| Pass-Criteria | 1) User B gets the first message from user A 2) User A gets the message back from user B 3) User A and B get the delivery and displayed notifications for both messages |

## RCS-e-1.2-con-0101 Mobile Initiated Capability Discovery - Successful

| Test Case Id | RCS-e-1.2-con-0101 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Mobile Initiated Capability Discovery - Successful |
| Specification Reference | RCS-e 2.3.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e |
| Test Procedure | 1. User A triggers a capability discovery procedure towards User B at User A’s device. This can be done by the following: a. Adding new User B entry new to User A’s address book b. Refreshing User B’s status manually 2. User A’s RCS-e client sends a SIP OPTIONS message towards User B. 3. The test tool responds with a 200 OK towards User A’s RCS-e client. 4. Check that User B is shown as available for RCS-e on User A’s RCS-e client |
| Pass-Criteria | At step 2 the SIP OPTION message contains the following information: - Request-URI is set to User-B’s IMS identity (e.g. SIP-URI or TEL-URI/MSISDN) - The Contact header includes all tags according to ICS - The Accept-Contact header includes all tags according to ICS At step 4 User A’s RCS-e client shows User B as available. |

## RCS-e-1.2-con-0102 Mobile Initiated Capability Discovery – Unsuccessful – 480 Temporary Unavailable

| Test Case Id | RCS-e-1.2-con-0102 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Mobile Initiated Capability Discovery – Unsuccessful – 480 Temporary Unavailable |
| Specification Reference | RCS-e 2.3.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e |
| Test Procedure | 1. User A triggers a capability discovery procedure towards User B on User A’s device. This can be done by the following: a. Adding new User B entry new to User A’s address book b. Refreshing User B’s status manually 2. User A’s RCS-e client sends a SIP OPTIONS message towards User B. 3. The test tool responds with a 480 TEMPORARILY UNAVAILABLE towards User A’s RCS-e client. 4. Check that User B is shown as not available for RCS-e on User A’s RCS-e client |
| Pass-Criteria | At step 2 the SIP OPTION message contains the following information: - Request-URI is set to User-B’s IMS identity (e.g. SIP-URI or TEL-URI/MSISDN) - The Contact header includes all tags according to ICS - The Accept-Contact header includes all tags according to ICS At step 4 User A’s RCS-e client shows User B as not available. |

## RCS-e-1.2-con-0103 Mobile Initiated Capability Discovery – Unsuccessful –408  Request Timeout

| Test Case Id | RCS-e-1.2-con-0103 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Mobile Initiated Capability Discovery – Unsuccessful –408 Request Timeout |
| Specification Reference | RCS-e 2.3.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e |
| Test Procedure | 1. User A triggers a capability discovery procedure towards User B on User A’s device. This can be done by the following: a. Adding new User B entry to User A’s address book b. Refreshing User B’s status manually 2. User A’s RCS-e client sends a SIP OPTIONS message towards User B. 3. The test tool responds with a 408 REQUEST TIMEOUT towards the RCS-e client. 4. Check that User B is shown as not available for RCS-e on User A’s RCS-e client |
| Pass-Criteria | At step 2 the SIP OPTION message contains the following information: - Request-URI is set to User-B’s IMS identity (e.g. SIP-URI or TEL-URI/MSISDN) - The Contact header includes all tags according to ICS - The Accept-Contact header includes all tags according to ICS At step 4 User A’s RCS-e client shows User B as not available. |

## RCS-e-1.2-con-0104 Mobile initiated Capability discovery – Unsuccessful – 404 Not Found

| Test Case Id | RCS-e-1.2-con-0104 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Mobile initiated Capability discovery – Unsuccessful – 404 Not Found |
| Specification Reference | RCS-e 2.3.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e |
| Test Procedure | 1. User A triggers a capability discovery procedure towards User B on User A’s device. This can be done by the following: a. Adding new User B entry to User A’s address book b. Refreshing User B’s status manually 2. User A’s RCS-e client sends a SIP OPTIONS message towards User B. 3. The test tool responds with a 404 NOT FOUND towards User A’s RCS-e client. 4. Check that User B is shown as not available for RCS-e on User A’s RCS-e client |
| Pass-Criteria | At step 2 the SIP OPTION message contains the following information: - Request-URI is set to User-B’s IMS identity (e.g. SIP-URI or TEL-URI/MSISDN) - The Contact header includes all tags according to ICS - The Accept-Contact header includes all tags according to ICS At step 4 User A’s RCS-e client shows User B as not available. |

## RCS-e-1.2-con-0105 Mobile Initiated Capability Discovery – Successful – Multiple Identities

| Test Case Id | RCS-e-1.2-con-0105 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Mobile Initiated Capability Discovery – Successful – Multiple Identities |
| Specification Reference | RCS-e 2.3.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e User A’s address book contains an entry for User C with a SIP-URI as contact and a phone number |
| Test Procedure | 1. User A triggers a capability discovery procedure towards User C on User A’s device. This can be done by the following: a. Refreshing User C’s status manually 2. User A’s RCS-e client sends two SIP OPTIONS messages towards User C with the following difference: a. Using the SIP-URI as identity. b. Using the phone number as identity 3. The test tool responds with 200 OK towards User A’s RCS-e client for each OPTIONS message. 4. Check that User C’s two identities are shown as available for RCS-e on User A’s RCS-e client |
| Pass-Criteria | At step 2 the SIP OPTION messages contains the following information: - In one message Request-URI is set to User-C’s SIP-URI - In the other message Request-URI is set to User-C’s phone number (TEL-URI) - The Contact header includes all tags according to ICS - The Accept-Contact header includes all tags according to ICS At step 4 User A’s RCS-e client shows User C’s two identities as available. |

## RCS-e-1.2-con-0107 Mobile Terminated Capability Discovery – Correct Presentation of Remote Capabilities

| Test Case Id | RCS-e-1.2-con-0107 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Mobile Terminated Capability Discovery – Correct Presentation of Remote Capabilities |
| Specification Reference | RCS-e 2.4.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e User B is stored in User A’s address book |
| Test Procedure | 1. The test tool sends a SIP OPTIONS message with the RCS-e IM service tag in the Contact header towards User A’s RCS-e client. 2. User A’s RCS-e client responds with a 200 OK towards the test tool. 3. Check that User B is shown as available for RCS-e on User A’s RCS-e client. |
| Pass-Criteria | At step 2 the 200 OK message contains the following information: - The Contact header includes all tags according to ICS - The Accept-Contact header includes all tags according to ICS At step 3 User A’s RCS-e client shows User B as available for RCS-e. |

## RCS-e-1.2-con-0108 Mobile Terminated Capability Discovery – Learning Unknown New Users RCS-e Capabilities

| Test Case Id | RCS-e-1.2-con-0108 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Mobile Terminated Capability Discovery – Learning Unknown New Users RCS-e Capabilities |
| Specification Reference | RCS-e 2.4.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | - User A is IMS registered for RCS-e - User A’s RCS-e device is provisioned for RCS-e - User X is not in User A’s address book |
| Test Procedure | 1. The test tool sends a SIP OPTIONS message with the below condition towards User A’s RCS-e client: a. The RCS-e IM service tag in the Contact header b. User X as originator 2. User A’s RCS-e client responds with a 200 OK towards the test tool. 3. Check that User X is shown as temporarily available for RCS-e on User A’s RCS-e client. |
| Pass-Criteria | At step 2 the 200 OK message contains the following information: - The Contact header includes all tags according to ICS - The Accept-Contact header includes all tags according to ICS At step 3 User A’s RCS-e client shows User X as temporarily available for RCS-e. |

## Capability update during a MO call for video /image sharing

| Test Case Id | RCS-e-1.2-con-0109 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | When a voice call is established the participants have to update their capabilities |
| Specification Reference | RCS-e 3.3.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e User A’s RCS-e device as 3G, HSPA or Wifi connectivity |
| Test Procedure | 1. User A initiates a voice call to User B at User A’s device. 2. User A’s RCS-e client sends a SIP OPTIONS message towards User B to indicate video and/or image share capability.. 3. The test tool responds with a 200 OK towards User A’s RCS-e client indicating video and image share capability and a SDP body. 4. Check that User B is shown as available for video and image sharing on User A’s RCS-e client if supported by User A’s RCS-e client. |
| Pass-Criteria | At step 2 the SIP OPTION message contains the following information: - Request-URI is set to User-B’s IMS identity (e.g. SIP-URI or TEL-URI/MSISDN) - The Contact header includes at least the video and/or image share tag according to ICS. - The Accept-Contact header includes at least the video and/or image share tags according to ICS. At step 4 User A’s RCS-e client shows User B as available for video / image sharing if supported by User A’s RCS-e client. |

## Capability update during a MT call for video /image sharing

| Test Case Id | RCS-e-1.2-con-0110 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | When a voice call is established the participants have to update their capabilities |
| Specification Reference | RCS-e 3.3.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e User A’s RCS-e device as 3G, HSPA or Wifi connectivity |
| Test Procedure | 1. User A accepts a voice call from User B at User A’s device. 2. The test tool sends a SIP OPTIONS message towards User A to indicate video and image share capability of User B. 3. User A’s RCS-e client responds with a 200 OK towards User B’s RCS-e client indicating image and/or video share capability and a SDP body. 4. Check that User B is shown as available for video and/or image sharing on User A’s RCS-e client according to ICS. |
| Pass-Criteria | At step 3 the 200 OK message contains the following information: - The Contact header includes at least the video and/or image share tag according to ICS. - The Accept-Contact header includes at least the video and/or image share tags according to ICS. - The SDP body should indicate at least support for “H.264/MPEG-4 Part 10 // AVC“ and “H.263-2000” according to RCS-e 2.7.3 if video sharing is supported by User A’s RCS-e client. At step 4 User A’s RCS-e client shows User B as available for video and/or image sharing according to ICS. |

## Capability update during a MO voice call fails as other end has no capability

| Test Case Id | RCS-e-1.2-con-0111 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | When a voice call is established the participants have to update their capabilities. The callee should not be shown as available for video / image share if the corresponding tags are not included in the 200 OK response |
| Specification Reference | RCS-e 3.3.1 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e User A’s RCS-e device as 3G, HSPA or Wifi connectivity |
| Test Procedure | 1. User A initiates a voice call to User B at User A’s device. 2. User A’s RCS-e client sends a SIP OPTIONS message towards User B to indicate video and/or image share capability according to ICS. 3. The test tool responds with a 200 OK towards User A’s RCS-e client indicating NO video and image share capability and no SDP body. 4. Check that User B is shown as not available for video and image sharing on User A’s RCS-e client. |
| Pass-Criteria | At step 2 the SIP OPTION message contains the following information: - Request-URI is set to User-B’s IMS identity (e.g. SIP-URI or TEL-URI/MSISDN) - The Contact header includes at least the video and/or image share tag according to ICS. - The Accept-Contact header includes at least the video and/or image share tags according to ICS. At step 4 User A’s RCS-e client shows User B as not available for video andimage sharing. |

## MO video sharing call establishment - successful

| Test Case Id | RCS-e-1.2-con-0112 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Success full mobile originated video sharing call establishment |
| Specification Reference | RCS-e 3.3.3 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | RCS-e device is configured for IETF mode User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e User A has an established voice call to User B OPTIONS capability exchange happened successful. |
| Test Procedure | 1. User A initiates a video share call to User B at User A’s device. 2. User A’s RCS-e client sends a SIP INVITE message towards User B to start the video share call. 3. The test tool responds with a 180 RINGING towards User A’s RCS-e client. 4. The test tool responds with a 200 OK towards User A’s RCS-e client. 5. User A’s RCS-e client sends a SIP ACK message towards User B. 6. Test tool receives a RTP video stream from User A’s RCS-e client. |
| Pass-Criteria | At step 2 the SIP INVITE message contains the following information: - Request-URI is set to User-B’s IMS identity (e.g. SIP-URI or TEL-URI/MSISDN) - The Accept-Contact header includes at least the video share tag. - The supported header does not have the tag “preconditions”. - The SDP body includes a line “a=sendonly”. - The SDP body includes the supported video codecs. At step 6 the test tool receives RTP packets from User A’s RCS-e client. |

## MT video sharing call establishment - successful

| Test Case Id | RCS-e-1.2-con-0113 |
| --- | --- |
| Test Object | RCS-e Client |
| Test Case Description | Success full mobile originated video sharing call establishment |
| Specification Reference | RCS-e 3.3.3 |
| SCR Reference |  |
| Tool |  |
| Test code | Verified TTCN-3 code |
| Preconditions | RCS-e device is configured for IETF mode User A is IMS registered for RCS-e User A’s RCS-e device is provisioned for RCS-e User A has an established voice call to User B OPTIONS capability exchange happened successful. |
| Test Procedure | 1. The test tool sends a SIP INVITE message towards User A’s RCS-e client to start the video share call. 2. User A’s RCS-e client responds with a 180 RINGING. 3. User A accepts the incoming video share request. 4. User A’s RCS-e client responds with a 200 OK. 5. The test tool sends a SIP ACK message. 6. User A’s RCS-e device presents a RTP video stream from the test tool. |
| Pass-Criteria | At step 4 the SIP 200 OK response contains the following information: - The Contact header includes at least the video share tag. - The SDP body includes a line “a=recvonly”. - The SDP body includes one of the offered video codecs. At step 6 on User A’s RCS-e client a video stream can be seen. |

# RCS-e Interoperability Test Cases

## RCS-e-1.2-int-0101- First-time successful configuration

| Test Case Id | RCS-e-1.2-int-0101 |
| --- | --- |
| Test Object | Client |
| Test Case Description | First-time registration |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - Autoconfiguration server ready - RCS-e services have not been previously configured |
| Test Procedure | - Client is powered on - The client contacts the autoconfiguration server - The autoconfiguration server validates the SIM/client, provisions the user and sends a valid configuration to the client - Once the configuration is successfully sent by the autoconfiguration server and processed by client, it is possible to access RCS-e services - Add a contact which is known to be a RCSe user - Verify OPTIONs exchange is working - Test a successful chat to verify behaviour |
| Pass-Criteria | Configuration takes places seamlessly to the client If applicable, the welcome message is displayed and the user accepts RCS-e UX elements are enabled and the user can perform a successful chat after the successful configuration and first-time registration |

Table 1: Test Information for 0101 Interoperability Test

## RCS-e-1.2-int-0301- User discovery in addressbook: Initial check of capabilities

| Test Case Id | RCS-e-1.2-int-0301 |
| --- | --- |
| Test Object | Client |
| Test Case Description | User discovery in addressbook: Initial check of capabilities |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - The handset/client is registered for the first time following the first-time configuration |
| Test Procedure | - 1) Following a first-time registration a capability exchange is made with the contacts available in the addressbook. - 2) Those contacts who are RCS-e customers who are currently registered (i.e. a 200 OK response containing the RCS-e IM tag) should be added to the list of RCS-e users - 3) The list can be verified from the IM application (available users to initiate a chat) |
| Pass-Criteria | 1) The RCS-e user list is populated 2) The background activity does not impact UX |

Table 2: Test Information for 0301 Interoperability Test

## RCS-e-1.2-int-0302- User discovery in addressbook: Adding a RCS-e contact when registered

| Test Case Id | RCS-e-1.2-int-0302 |
| --- | --- |
| Test Object | Client |
| Test Case Description | User discovery in addressbook: Adding a RCS-e contact when registered |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - RCS-e services have been successfully configured on the client/SIM pair - The client is registered and the registration timer is far from expiration |
| Test Procedure | - The user adds user A with a valid number - An capability exchange (OPTIONS message) is made with the new contact. If he is an RCS-e customer and is currently registered (i.e. a 200 OK response containing the RCS-e IM tag) he should be added to the list of RCS-e users - The list can be verified from the IM application (available users to initiate a chat) |
| Pass-Criteria | User A is added to the list of RCS-e user |

Table 3: Test Information for 0302 Interoperability Test

## RCS-e-1.2-int-0303- User discovery in addressbook: Discovery of users via checking capabilities (RCS-e user)

| Test Case Id | RCS-e-1.2-int-0303 |
| --- | --- |
| Test Object | Client |
| Test Case Description | User discovery in addressbook: Discovery of users via checking capabilities (RCS-e user) |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - RCS-e services have been successfully configured on the phone/SIM pair - The client is registered and the registration timer is far from expiration |
| Test Procedure | - The user selects user A who is not in the list of known RCS-e users. - The capabilities are queried and the result shows that User A is a RCS-e user (It receives a 200 OK response with, at least, the RCS-e IM capability tag) - The list can be verified from the IM application (available users to initiate a chat) |
| Pass-Criteria | User A is added to the list of RCS-e user |

Table 4: Test Information for 0303 Interoperability Test

## RCS-e-1.2-int-0401- Capability query during a call

| Test Case Id | RCS-e-1.2-int-0401 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Successful capability query during a call |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code |  |
| Preconditions | - Both clients are RCS-e registered - Both clients are on 3G, HSPA or Wi-Fi data coverage |
| Test Procedure | - User A initiates a voice call to user B. - Exchange of capabilities takes place just after the call is started |
| Pass-Criteria | Both users are able to see each other’s supported capabilities, e.g. video share, image share. |

Table 5: Test Information for 0401 Interoperability Test

## RCS-e-1.2-int-0501- Capability query in addressbook

| Test Case Id | RCS-e-1.2-int-0501 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Capability query in addressbook (only chat available due to coverage) |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - Both clients are RCS-e registered - At least one client is only in 2G |
| Test Procedure | - 1) The user selects user B in the addressbook who is also an RCS-e user. - 2) As a result a capability exchange takes place and the right capabilities/services available are displayed |
| Pass-Criteria | The client shows supported RCS-e services as available. In this case only chat is available due to the coverage. |

Table 6: Test Information for 0501 Interoperability Test

## RCS-e-1.2-int-0502- Capability query in call-log

| Test Case Id | RCS-e-1.2-int-0502 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Capability query in call-log |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code |  |
| Preconditions | - Both clients are RCS-e registered and online |
| Test Procedure | - The user selects user B in the call-log who is also RCS-e user - As a result a capability exchange takes place. |
| Pass-Criteria | The client shows supported RCS-e services as available. |

Table 7: Test Information for 0502	 Interoperability Test

## RCS-e-1.2-int-0503- Capability query in 1-2-1 chat (file transfer supported)

| Test Case Id | RCS-e-1.2-int-0503 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Capability query in 1-2-1 chat optimization |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code |  |
| Preconditions | - User A and B clients are RCS-e registered - User A and B are under 3G/HSPA/WiFi coverage - Both users are in a RCS-e 1-2-1 chat - Both clients support file transfer. |
| Test Procedure | User A checks user B capabilities. |
| Pass-Criteria | File transfer is shown as an available service |

Table 8: Test Information for 0503	 Interoperability Test

## RCS-e-1.2-int-0701- Successful file transfer from addressbook (one-to-one)

| Test Case Id | RCS-e-1.2-int-0701 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Successful file transfer from addressbook (one-to-one) |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - User A has contacts in his/her address book with RCS services active - User B is an RCS-e user who is online - User A in on Wi-Fi coverage, user B is on EDGE/3G/HSPA coverage - All users have enough free storage space to receive file/s |
| Test Procedure | - User A selects User B from his/her address book - User A selects file transfer option - User A selects a file from the memory list and select share option - User B accepts the invitation and opens the file |
| Pass-Criteria | - User B receives the invitation stating the file type and the size If it is a supported file type, user B should visualize the relevant MIME icon or mini-preview should be displayed within the chat window (all FileTransfer take place on a chat window) - User A can progress the transfer in the notification bar (the transfer does not block the UI) - User B receives file - User B can open the file properly |

Table 9: Test Information for 0701 Interoperability Test

## RCS-e-1.2-int-0702- Successful file transfer from chat application (one-to-one)

| Test Case Id | RCS-e-1.2-int-0702 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Successful file transfer from chat application (one-to-one) |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - User A has RCS-e contacts - User B is an RCS-e user who is online - User A is on Wi-Fi coverage, user B is on EDGE/3G/HSPA coverage - All users have enough free storage space to receive file/s |
| Test Procedure | 1. User A starts an IM/Chat with User B. 2. User A selects file transfer option 3. User A selects a file from the memory list and select share option 4. User B accepts the invitation and opens the file |
| Pass-Criteria | 1) User B receives the invitation stating the file type and the size 2) If it is a supported file type, user B should visualize the relevant MIME icon or mini-preview should be displayed within the chat window (all FileTransfer take place on a chat window) 3) User A can progress the transfer in the notification bar (the transfer does not block the UI) 4) User B receives file 5) User B can open the file properly |

Table 10: Test Information for 0702 Interoperability Test

## RCS-e-1.2-int-0703- Successful multiple file transfers from FT/gallery app (one-to-many)

| Test Case Id | RCS-e-1.2-int-0703 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Successful multiple file transfers from FT/gallery app (one-to-many) |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - User A has RCS-e contacts - User B is an RCS-e user who is online - User C is an RCS-e user who is online - User A is on Wi-Fi coverage, user B is on EDGE/3G/HSPA coverage - All users have enough free storage space to receive file/s |
| Test Procedure | 1. User A browses his/her files. 2. User A selects three files 3. User A selects file transfer option 4. User A selects two contacts from his/her RCS-e contacts list and press file sharing option 5. Users B and C accept the invitation and open the files |
| Pass-Criteria | For each user/file, the following steps can be verified: 1) User B and C receive the invitation stating the file type and the size 2) If it is a supported file type, user B and C should visualize the relevant MIME icon or mini-preview should be displayed within the chat window (all FileTransfer take place on a chat window) 3) User A can progress the transfer in the notification bar (the transfer does not block the UI) 4) Users B and C receive the file 5) Users B and C can open the file properly |

Table 11: Test Information for 0703 Interoperability Test

## RCS-e-1.2-int-0704- File transfer rejected by the destination

| Test Case Id | RCS-e-1.2-int-0704 |
| --- | --- |
| Test Object | Client |
| Test Case Description | File transfer rejected by the destination |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool |  |
| Test code |  |
| Preconditions | - User A has contacts in his/her address book with RCS-e services active - User B is an RCS-e user who is online - User A in on Wi-Fi coverage, user B is on EDGE/3G/HSPA coverage - All users have enough free storage space to receive file/s |
| Test Procedure | 1. User A selects User B from his/her address book 2. User A selects file transfer option 3. User A selects a file from the memory list and selects share option 4. User B rejects file transfer |
| Pass-Criteria | User B receives file invitation User A receives error message |

Table 12: Test Information for 0704 Interoperability Test

## RCS-e-1.2-int-0901- Video share: sending video – caller

| Test Case Id | RCS-e-1.2-int-0901 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Video share: sending video – caller |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | User A and B are registered RCS-e users Both users coverage is 3G, HSPA or Wi-Fi User A and B are in an active voice call Capabilities exchange has taken place confirming video share is possible |
| Test Procedure | After the call is established, user A starts video share to user B. User B accepts the invite. User A sends video to user B. Video can be a pre-stored (stream) or live (camera[s]). |
| Pass-Criteria | User B receives the video. |

Table 13: Test Information for RCS-e-1.2-int-0901 Interoperability Test

## RCS-e-1.2-int-0902- Video share: sending video – receiver

| Test Case Id | RCS-e-1.2-int-0902 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Video share: sending video – receiver |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - User A and B are registered RCS-e users - Both users coverage is 3G, HSPA or Wi-Fi - User A and B are in an active voice call - Capabilities exchange has taken place confirming video share is possible |
| Test Procedure | - After the call is established, user B starts video share to user A. - User A accepts the invite. - User B sends video to user A. Video can be a pre-stored (stream) or live (camera[s]). |
| Pass-Criteria | User A receives the video. |

Table 14: Test Information for 0902 Interoperability Test

## RCS-e-1.2-int-0903- Video share: reject

| Test Case Id | RCS-e-1.2-int-0903 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Video share: reject |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - User A and B are registered RCS-e users - Both users coverage is 3G, HSPA or Wi-Fi - User A and B are in an active voice call - Capability exchange has taken place confirming video share is possible |
| Test Procedure | - User B rejects user A’s invite to video share. It is not possible for user A to start sharing. |
| Pass-Criteria | The call remains without video share |

Table 15: Test Information for 0903 Interoperability Test

## RCS-e-1.2-int-1001- Share a picture taken using the camera

| Test Case Id | RCS-e-1.2-int-1001 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Share a picture. A picture taken using the camera |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | None |
| Test code | None |
| Preconditions | - User A and B are registered RCS-e users - Both users coverage is 3G, HSPA or Wi-Fi - User A and B are in an active voice call - Capabilities exchange has taken place confirming image share is possible - Client A has a camera |
| Test Procedure | - User A takes a picture - User A invites user B to receive a Picture - User B accepts |
| Pass-Criteria | - Picture sharing starts and get successfully transferred - Image is displayed on user B's client |

Table 16: Test Information for 1001 Interoperability Test

## RCS-e-1.2-int-1002- Share a stored image

| Test Case Id | RCS-e-1.2-int-1002 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Share a picture. A file (“send stored image”) |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - User A and B are registered RCS-e users - Both users coverage is 3G, HSPA or Wi-Fi - User A and B are in an active voice call - Capability exchange has taken place confirming image share is possible |
| Test Procedure | - User A invites user B to receive a picture, stored file - User B accepts |
| Pass-Criteria | 1. Picture sharing starts and get successfully transferred 2. Image is displayed on user B's client |

Table 17: Test Information for 1002 Interoperability Test

## RCS-e-1.2-int-1003- Share a stored image. User B rejects

| Test Case Id | RCS-e-1.2-int-1003 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Share a stored image. User B rejects |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - User A and B are registered RCS-e users - Both users coverage is 3G, HSPA or Wi-Fi - User A and B are in an active voice call - Capability exchange has taken place confirming image share is possible |
| Test Procedure | 1. User A invites user B to receive a picture, stored picture 2. User B rejects |
| Pass-Criteria | 1. Picture sharing doesn't start |

Table 18: Test Information for 1003 Interoperability Test

## RCS-e-1.2-int-1101- Chat 1 to 1. Start from address book

| Test Case Id | RCS-e-1.2-int-1101 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Chat 1 to 1. Start from address book |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - Supported by network: 2G, 3G, HSPA, Wi-Fi - Both clients are RCS-e registered - Both clients are on 3G, HSPA or Wi-Fi data coverage |
| Test Procedure | - User A sends chat invitation to User B from address book - User B accepts invitation - User A start typing to start the chat session. |
| Pass-Criteria | 1. User B sees the invitation from user A in the notification bar 2. User A sees the message first as delivered and once user B opens the chat window as displayed |

Table 19: Test Information for 1101 Interoperability Test

## RCS-e-1.2-int-1102- Chat 1 to 1. Start from IM/Chat application

| Test Case Id | RCS-e-1.2-int-1102 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Chat 1 to 1. Start from IM/Chat application |
| Specification Reference | ID_RCSE_6_1_2 |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - Both clients are RCS-e registered - Both clients are on 3G, HSPA or Wi-Fi data coverage - Supported by network: 2G, 3G, HSPA, Wi-Fi |
| Test Procedure | 1. User A sends chat invitation to User B from IM/Chat application 2. User B accepts invitation |
| Pass-Criteria | 1. User B sees the invitation from A in the notification bar 3. User A sees the message first as delivered and once B opens the chat window as displayed |

Table 20: Test Information for 1102 Interoperability Test

## RCS-e-1.2-int-1103– Composing chat window/leaving background

| Test Case Id | RCS-e-1.2-int-1102 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Composing chat window/ leaving background |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - Terminals are registered and able to access IMS/RCS-e core network and relevant servers. - Supported by network: 2G, 3G, HSPA, Wi-Fi |
| Test Procedure | - Chat session is set up between users A&B - User A leaves the chat for managing any other functionality from the mobile home screen without closing chat window. - User B sends new messages to user A which are displayed in the user A’s notification bar - User B sees that the new messages he sent are delivered but not read |
| Pass-Criteria | - Chat window and IM session kept active in background - User A gets new messages from user B which are displayed in the notification bar - User B sees that the new messages he sent are delivered but not read - Chat session remains active |

Table 21: Test Information for 1103 Interoperability Test

## RCS-e-1.2-int-1104– “Is composing” Notification /Typing info

| Test Case Id | RCS-e-1.2-int-1104 |
| --- | --- |
| Test Object | Client |
| Test Case Description | "Is Composing" Notification / Typing info |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - Terminals are registered and able to access IMS/RCS-e core network and relevant servers. - Supported by network: 2G, 3G, HSPA, Wi-Fi |
| Test Procedure | - User A start a a chat session with User B - User A starts typing some text in a message to User B - User A sends the message to user B - User A starts typing a message, but does not send it. - User A removes the message composed so far (empty message box) |
| Pass-Criteria | An "Is Composing" notification is sent to the user B. The ‘Is Composing’ indication is removed from user B UI when the message is received by user B. Step 4, An "Is Composing" notification is sent to the user B. Step 5 - The 'Is Composing' indication is removed from the UI |

Table 22: Test Information for 1104 Interoperability Test

## RCS-e-1.2-int-1105- Close chat window

| Test Case Id | RCS-e-1.2-int-1105 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Close chat window |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - Terminals are registered and able to access IMS/RCS-e core network and relevant servers. - Supported by network: 2G, 3G, HSPA, Wi-Fi |
| Test Procedure | - A chat session is established between users A&B - User A closes the chat from the chat composing window or in the IM/chat application. - User B remains in the chat. - User B writes a message and sends it |
| Pass-Criteria | - User A's chat window is closed and the previous window is shown - A new session is established (new INVITE message is sent to the user A). |

Table 23: Test Information for 1105 Interoperability Test

## RCS-e-1.2-int-1106– Change chat 1 to 1 -> one to many

| Test Case Id | RCS-e-1.2-int-1106 |
| --- | --- |
| Test Object | Client |
| Test Case Description | Change chat 1 to 1 -> one to many |
| Specification Reference |  |
| SCR Reference |  |
| ETR Reference |  |
| Tool | n/a |
| Test code | n/a |
| Preconditions | - Terminals are registered and able to access IMS/RCS-e core network and relevant servers. - Supported by network: 2G, EDGE, 3G, HSPA, Wi-Fi |
| Test Procedure | - User A starts a chat conversation by sending a message to user B (invitation) - User B accepts invitation to start chatting (opens the chat window). User A sends another message to user B - User A selects user C from the addressbook (user C's RCS-e capabilities previously detected) and invites user C to join the chat by sending a message invitation - User C gets the notification that he has been invited to a group chat and decides to join (consuming the message/opening the chat window) - Tempt the file transfer between user A and user B - Check if delivery/display notifications are shown |
| Pass-Criteria | - User B receives the invitation from user A to start chatting - User B receives the message from user A. The chat is started between both users - User C receives the invitation from user A to start chatting - User C is added to the chat, a multichat is started between all of them - 1-2-1 session is converted into a group chat session - File transfer becomes unavailable - Delivery/Displayed notifications are not longer available |

Table 24: Test Information for 1106 Interoperability Test

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| n/a | n/a | No prior version |

Draft/Candidate Version 1.0 History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-ETS-RCS-e-V1_2 | 07 Feb 2012 | n/a | No previous version within OMA |
|  | 10 Feb 2012 | IOP | Basic set of test cases that cover main RCS-e features |
|  | 14 Feb 2012 | IOP | Basic test case list updated |
|  | 16 Apr 2012 | CON | Conformance test cases added (0019R02 and 0020R02) and 12 new conformance test cases |
|  | 19 Jul 2012 | 5 | Updated according to: OMA-IOP-MEC-2012-0071R01-INP_RCS_e_capability_exchange_for_video_image_share OMA-IOP-MEC-2012-0072-INP_RCS_e_video_share_call_establishment Re-numbering of tables in the whole document |

<<Optional>>Conformance Test Case applicability

Introduction

This section shall help implementers of the <xxx> Enabler to select appropriate Conformance test cases that are applicable to the features implemented.

This appendix lists all test cases testing only mandatory features, ICS (Implementation  Conformance Specification) , IXIT (Implementation eXtra Information)  and a mapping from ICS/IXIT to applicable test cases as defined by the Open Mobile Alliance.

Test Cases testing only mandatory features

# These Conformance test cases are independent from any applicability, are testing only mandatory SCRs and SHALL be run with every implementation.

Test Case

SUPL-1.0-con-000 - Compatible Versions

Applicability

Client ICS

| ICS | Description | SCR Reference(s) | Supported (yes/no) |
| --- | --- | --- | --- |
| ics_SLP_SessionID _Ipv4 | IPV4 in SLP session ID supported |  |  |

Client IXIT

| IXIT | Description | Unit <(Range of values)> | Value |
| --- | --- | --- | --- |
| ixit_timer_UT1 | Value of Timer UT1 | seconds (5 to 25) |  |

Server ICS

| ICS | Description | SCR Reference(s) | Supported (yes/no) |
| --- | --- | --- | --- |
| ics_SLP_SessionID _Ipv4 | IPV4 in SLP session ID supported |  |  |

Server IXIT

| IXIT | Description | Unit <(Range of values)> | Value |
| --- | --- | --- | --- |
| ixit_timer_ST1 | Value of Timer ST1 | seconds (5 to 25) |  |

ICS to test case mapping

# According to the ICS described above the applicable test cases can be derived from the following table.

| Applicability | Test Cases |
| --- | --- |
| ics_MT_SMS AND ics_3GPP | SUPL-1.0-con-014 - Incorrect MT SMS message content |

<Additional Information>

If needed, add annex to provide additional information to support the document.  In general, this information should be informative, as normative material should be contained in the primary body of the document.

Note that the styles for the headers in the appendix (App1, App2, App3) are different than the main body.  The use below is intended to validate the styles to be used.  Remove if not needed.

DELETE THIS COMMENT

App Headers

<More text>

More Headers

<More text>

Even More Headers

Figure 1: Example Figure