| # Enabler Test Specification for RCS-e interoperability |  |  |
| --- | --- | --- |
| Draft Version 1.0 – XX Oct 2012 |  |  |
| Open Mobile Alliance |  |  |
| OMA-ETS-RCS-e-INT-V1_0-201210XX-D |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2012 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

Contents

1.	Scope	4

2.	References	5

2.1	Normative References	5

2.2	Informative References	5

3.	Terminology and Conventions	6

3.1	Conventions	6

3.2	Definitions	6

3.3	Abbreviations	6

4.	Introduction	7

5.	RCS-e Conformance Test Cases	8

6.	RCS-e Interoperability Test Cases	9

6.1	RCS-e-1.2-int-001- First-time configuration	9

6.2	RCS-e-1.2-int-002- RCS-e user disabled	9

6.3	RCS-e-1.2-int-010- User discovery in address book: initial check of capabilities	10

6.4	RCS-e-1.2-int-011- User discovery in address book: adding an RCS-e contact	10

6.5	RCS-e-1.2-int-020- Capability query during a call: image and video share	11

6.6	RCS-e-1.2-int-021- Capability query in address book: coverage limitation	11

6.7	RCS-e-1.2-int-022- Capability query in 1-to-1 chat: file transfer supported	12

6.8	RCS-e-1.2-int-030- 1-to-1 Chat	12

6.9	RCS-e-1.2-int-031- Leaving chat window	13

6.10	RCS-e-1.2-int-032- Closing chat window	14

6.11	RCS-e-1.2-int-033- Change 1-to-1 chat to group chat	14

6.12	RCS-e-1.2-int-040- File transfer from address book	15

6.13	RCS-e-1.2-int-041- File transfer from chat application	15

6.14	RCS-e-1.2-int-042- File transfer: rejected	16

6.15	RCS-e-1.2-int-051- Video share: bi-directional	17

6.16	RCS-e-1.2-int-052- Video share: reject	17

Appendix A.	Change History (Informative)	19

A.1	Approved Version History	19

A.2	Draft/Candidate Version 1.0 History	19

Figures

No table of figures entries found.

Tables

No tables included.

# Scope

This document for RCSe describes in detail the interoperability test cases for RCS-e Version 1.2.2, as detailed in [RCS-e].

The test cases are split into two categories, conformance and interoperability test cases. The conformance test cases are in a separate conformance ETS.

The interoperability test cases are aimed to verify that implementations of the specifications work satisfactory.

# References

## Normative References

| [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |
| --- | --- |
| [OMA_IM_TS] | “Instant Messaging Requirements”, Version 1.0, Open Mobile Alliance™, OMA-RD-IM-V1_0, URL:http://www.openmobilealliance.org/ |
| [OMA_IM_XDM] | “IM XDM Specification”, Version 1.0, Open Mobile Alliance™, OMA-TS-IM_XDM-V1_0, URL:http://www.openmobilealliance.org/ |

## Informative References

| [OMADICT] | “Dictionary for OMA Specifications”, Version 2.9, Open Mobile Alliance™, OMA-ORG-Dictionary-V2_9, URL:http://www.openmobilealliance.org/ |
| --- | --- |
| [RCS-e] | “RCS-e – Advanced Communications: Services and Client Specification”, v1.2.2, July 2012, GSM Association, RCS_e_Specification_Document_1_2_2, URL:http://www.gsma.com/rcs/wp-content/uploads/2012/03/rcs-e_advanced_comms_specification_v1_2_2_approved.pdf |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope”, are normative, unless they are explicitly indicated to be informative.

The following numbering scheme is used:

xxx-y.z-con-number where:
xxx	Name of enabler, e.g. MMS or Browsing
y.z	Version of enabler release, e.g. 1.2 or 1.2.1
’con’	Indicating this test is a conformance test case
number	Leap number for the test case

Or

xxx-y.z-int-number where:
xxx	Name of enabler, e.g. MMS or Browsing
y.z	Version of enabler release, e.g. 1.2 or 1.2.1
’int’	Indicating this test is a interoperability test case
number	Leap number for the test case

## Definitions

| Auto-Configuration Server | A component of RCS-e functionality that provides an interoperable way to provision RCS-e configuration information from a Service Provider to an RCS-e Client. |
| --- | --- |
| Client | Uses definition from [OMADICT]. |
| Component | Uses definition from [OMADICT]. |
| RCS-e Client | A Client which adheres and conforms to GSMA RCS-e specifications and is capable to receive RCS-e service. |
| Service | Uses definition from [OMADICT]. |
| Service Provider | Uses definition from [OMADICT]. |

## Abbreviations

| 2G | Second generation (i.e. mobile/wireless telephony) |
| --- | --- |
| 3G | Third generation (i.e. 3rd Generation mobile/wireless telephony) |
| HSPA | High Speed Packet Access |
| RAN | Radio Access Network |
| RCS-e | Rich Communications Suite-enhanced |
| SIM | Subscriber Identity Module |
| USIM | Universal Subscriber Identity Module |
| UX | User eXperience |

# Introduction

The purpose of this document is to provide interoperability test cases for RCS-e version 1.2.2.

The implementation of some features is optional for the Clients and/or the Servers in the RCS-e Enabler.  The tests associated with these optional features are marked as "(Includes Optional Features)" in the test specification.

# RCS-e Conformance Test Cases

Conformance test cases can be found in the OMA-ETS-RCS-e-CON-V1_2 document.

# RCS-e Interoperability Test Cases

## RCS-e-1.2-int-001- First-time configuration

| Test Case Id | RCS-e-1.2-int-001 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful first-time configuration and registration |
| Specification Reference | RCS-e 2.2.2 |
| Preconditions | - Auto-Configuration Server ready - User A NOT IMS registered for RCS-e - RCS-e services have not been previously configured on the phone/(U)SIM pair |
| Test Procedure | - User A’s handset is powered on - The RCS-e client (running on the handset) contacts the Auto-Configuration Server - The Auto-Configuration Server validates the (U)SIM/phone, provisions User A and sends back a valid configuration to User A’s client - Once the configuration is successfully received from the Auto-Configuration Server and processed by User A’s client, User A’s RCS-e UX is enabled |
| Pass-Criteria | - Configuration takes places seamlessly to User A - At step 4, if applicable, an RCS-e welcome message is displayed and User A accepts - At step 4 RCS-e UX elements are enabled |

## RCS-e-1.2-int-002- RCS-e user disabled

| Test Case Id | RCS-e-1.2-int-002 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies correct operation when RCS-e user is disabled by the Service Provider, including auto-configuration query |
| Specification Reference | RCS-e 2.2.2.1.2 |
| Preconditions | - Auto-Configuration Server ready - User A is NOT IMS registered for RCS-e - RCS-e services previously configured on the phone/(U)SIM pair - User A’s RCS-e service is disabled by the Service Provider, including auto-configuration query |
| Test Procedure | - User A’s handset is powered on - The RCS-e Client (running on the handset) contacts the Auto-Configuration Server - The Auto-Configuration Server validates the phone/(U)SIM and sends back the RCS-e user disable response to the RCS-e client - The RCS-e client receives the response and processes it - User A’s UX no longer shows RCS-e services enabled |
| Pass-Criteria | - At step 5 RCS-e UX elements are no longer enabled (i.e. handset exhibits non RCS-e UX) |

## RCS-e-1.2-int-010- User discovery in address book: initial check of capabilities

| Test Case Id | RCS-e-1.2-int-010 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies correct operation for RCS-e users discovery in address book when the initial check of capabilities is performed. |
| Specification Reference | RCS-e 2.4 |
| Preconditions | - RCS-e capable handset initially registered following a successful RCS-e first-time configuration - Some (i.e. a subset) of the contacts in User A’s address book are IMS registered and configured for RCS-e services (e.g. User B and C) |
| Test Procedure | - Following a successful RCS-e first-time registration, the RCS-e client (running on the handset) performs a capability exchange for the contacts available in User A’s address book - Contacts are identified as RCS-e users if they are currently IMS registered (e.g. a capabilities exchange with a contact indicates support of RCS-e IM 1-to-1 chat) - The identified contacts (e.g. User B and C) are identified as RCS-e users on User A’s handset. For example, they may be added to a list of RCS-e users accessible to User A’s handset. |
| Pass-Criteria | - At step 3 the list of RCS-e users (e.g. User B and C) are identified as RCS-e users on User A’s handset - Capabilities discovery (i.e. operating as a background activity) does not impact UX |

## RCS-e-1.2-int-011- User discovery in address book: adding an RCS-e contact

| Test Case Id | RCS-e-1.2-int-011 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies correct operation for User discovery in address book: Adding an RCS-e contact who is currently RCS-e registered |
| Specification Reference | RCS-e 2.4 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair - User A and B are IMS registered for RCS-e and the registration timer for each user is far from expiration |
| Test Procedure | - User A manually enters a new contact (User B) with a valid telephone number (e.g. a TEL-URI) to his contact list - The RCS-e client on User A’s handset performs capability discovery with User B (e.g., P2P OPTIONS exchange between User A and B occurs) - As a result, the RCS-e client on User A’s handset determines that User B is RCS-e capable and identifies User B as an RCS-e user on User A’s handset |
| Pass-Criteria | - At step 3 User B is identified as an RCS-e user on User A’s handset. |

## RCS-e-1.2-int-020- Capability query during a call: image and video share

| Test Case Id | RCS-e-1.2-int-020 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful image and video share capability query during a call |
| Specification Reference | RCS-e 2.7 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair - User A and B are IMS registered and the registration timer for each user is far from expiration - User A and B handset’s both normally support image and video share capability - User A and B handset clients are operating with data coverage over the following RANs: - 3G; or - HSPA; or - Wi-Fi. |
| Test Procedure | - User A initiates a voice call to User B - An RCS-e capabilities exchange takes place on both handsets just after the voice call between User A and B starts |
| Pass-Criteria | - At step 2 both users are able to see each other’s supported RCS-e capabilities including video share and image share. |

## RCS-e-1.2-int-021- Capability query in address book: coverage limitation

| Test Case Id | RCS-e-1.2-int-021 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful capability query in address book when only 1-to-1 chat is available due to coverage |
| Specification Reference | RCS-e 2.7 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for both User A and B - User A and B are IMS registered and the registration timer for each user is far from expiration - User A and B handset’s both normally support image and video share capability - At least one handset (i.e., User A or B) is operating in 2G, the other could have 2G, 3G, HSPA or Wi-Fi |
| Test Procedure | - From User A’s address book, User A selects User B’s contact - An RCS-e capabilities exchange takes place and User B’s corresponding supported RCS-e capabilities/services are displayed and updated on User A’s handset |
| Pass-Criteria | - At step 2 User A’s RCS-e client shows supported RCS-e capabilities/services of User B and - that only 1-to-1 chat is available due to 2G coverage |

## RCS-e-1.2-int-022- Capability query in 1-to-1 chat: file transfer supported

| Test Case Id | RCS-e-1.2-int-022 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful capability query in 1-to-1 chat when file transfer is supported |
| Specification Reference | RCS-e 2.7 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for both User A and B - User A and B are IMS registered and the registration timer for each user is far from expiration - User A and B handset clients are operating with data coverage over the following RANs: - 3G; or - HSPA; or - Wi-Fi. - User A and B are currently engaged in 1-to-1 chat - On User A’s RCS-e client, User B’s File Transfer capability is not known Editor’s note: It is not clear how to set this condition up on User A’s handset, or even if it is possible. If it is not possible then this test case cannot be run and will need to be deleted. - User B’s RCS-e client is able to support File Transfer |
| Test Procedure | - User A attempts to send a file (e.g. an image) to User B - User A invokes capability discovery with User B - An RCS-e capabilities exchange takes place and User B’s corresponding supported RCS-e capabilities/services (including File Transfer) are updated on User A’s handset |
| Pass-Criteria | - At step 3 on User A’s handset, User B is shown (in RCS-e UX) as supporting file transfer |

## RCS-e-1.2-int-030- 1-to-1 Chat

| Test Case Id | RCS-e-1.2-int-030 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful 1-to-1Chat, starting from address book and including “Is Composing “ notification |
| Specification Reference | RCS-e 3.2.4.1, 3.2.4.2, 3.2.4.6 |
| Preconditions | - User A and B are IMS registered and the registration timer for each user is far from expiration - RCS-e services have been successfully configured on the phone/(U)SIM pair for both User A and B |
| Test Procedure | - User A selects User B to engage in a 1-to-1 chat - An empty message composer UX element is provided to User A - User A types a chat message to User B in the message composer - User A initiates transmission of the first 1-to-1 chat message (e.g. by pushing the ‘send’ button next to message composer UX element) - A 1-to-1chat invitation is sent from User A to B - User B sees and accepts the chat invitation from User A by opening the chat window - User A types additional chat message to User B (but not sent yet) - After 15 seconds wait, User A sends the chat message to User B - User B sees the message from User A and starts to reply back to User A (but not sent yet) - After 15 seconds, User B removes the reply text (before it is sent to User A) |
| Pass-Criteria | - At step 6 User B sees a notification from User A on his handset (e.g. notification bar) - At step 6 User A sees a notification that the first message was delivered to User B - At step 6 User B sees User A’s first chat message - At step 7 User B’s UI elements displays User A typing via the “Is Composing” notification - At step 8 User A sees the second message was delivered to User B - At step 8 User B sees User A’s second chat message - At step 9 User A’s UI elements displays User B typing via the “Is Composing” notification - At step 10 User A’s UI elements removes the “Is Composing” notification |

## RCS-e-1.2-int-031- Leaving chat window

| Test Case Id | RCS-e-1.2-int-031 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies correct operation when leaving and then returning to the chat window |
| Specification Reference | RCS-e 3.2.4.5 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for both User A and B - User A and B are IMS registered and the registration timer for each user is far from expiration - User A and B are already engaged in a 1-to-1 chat |
| Test Procedure | - User A leaves the 1-to-1 chat to manage other functionality on the handset (e.g. User A minimizes the 1-to-1 chat) - User B sends a new message to User A, which is displayed in User A’s notification bar - User A returns to his chat window and sees the original chat messages and the new one. |
| Pass-Criteria | - At step 1 User A’s chat window is kept active in the background - At step 2 User A gets new message from User B which is displayed in the notification bar - At step 2 User B sees that the new message that he sent is delivered - At step 3 the chat session remains active and all the messages from the complete chat session are displayed to User A |

## RCS-e-1.2-int-032- Closing chat window

| Test Case Id | RCS-e-1.2-int-032 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies correct operation when closing chat window |
| Specification Reference | RCS-e 3.2.4.7 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for both User A and B - User A and B are IMS registered and the registration timer for each user is far from expiration - User A and B already engaged in a 1-to-1 chat |
| Test Procedure | - User A terminates the chat (e.g., closing the chat composing window or the IM/1-to-1 chat application) - User B continues to keep his chat window open - User B composes a message and sends it to User A |
| Pass-Criteria | - At step 3 User A sees a notification from User B on his handset (e.g. notification bar) for a new 1-to-1 chat session |

## RCS-e-1.2-int-033- Change 1-to-1 chat to group chat

| Test Case Id | RCS-e-1.2-int-033 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies correct operation when changing from 1-to-1 chat to 1-to-many (group chat) |
| Specification Reference | RCS-e 3.2.4.13 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for User A, B and C - All users are IMS registered and the registration timer for each user is far from expiration - User A and B are already engaged in a 1-to-1 chat |
| Test Procedure | - User A invites User C to join the active chat session between User A and B, by sending an invitation message to User C - User C receives the notification that he has been invited to a group chat with User A and B and accepts the invitation - User A sends a chat message from the group chat session. |
| Pass-Criteria | - At step 2 User C accepts the invitation and a group chat between Users A, B and C is started. (i.e., the 1-to-1 chat session is converted into a group chat session) - At step 3 User B and C receive User A’s chat message |

## RCS-e-1.2-int-040- File transfer from address book

| Test Case Id | RCS-e-1.2-int-040 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful file transfer from address book |
| Specification Reference | RCS-e 3.2.4.14 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for User A, and User B - User A and B are IMS registered and the registration timer for each user is far from expiration - User A and B handset clients are operating with data coverage over the following RANs: - 3G; or - HSPA; or - Wi-Fi. - User A has contacts in his address book, including User B - All users have enough free storage space to receive file/s |
| Test Procedure | - User A selects User B from his address book - User A selects file transfer option - User A selects a file from storage on his handset, and selects to share it with User B - User B reviews and then accepts the invitation - The file is transferred to User B |
| Pass-Criteria | - At step 2 User B receives the file transfer invitation - At step 4, if it is a supported file type and less than the maximum size allowed (by Service Provider), User B should see: - The filename; - The relevant file-type; - An icon or thumbnail preview; - The size of the file. - At step 5 upon User B accepting the file transfer invitation, he receives the file - At step 5 User B can access the file (e.g. from storage on his handset) |

## RCS-e-1.2-int-041- File transfer from chat application

| Test Case Id | RCS-e-1.2-int-041 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful file transfer from chat application |
| Specification Reference | RCS-e 3.2.4.14 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for User A and B - All users are IMS registered and the registration timer for each user is far from expiration - User A and B handset clients are operating with data coverage over the following RANs: - 3G; or - HSPA; or - Wi-Fi. - User A and B are already engaged in a 1-to-1 chat - All users have enough free storage space to receive file/s |
| Test Procedure | - User A selects file transfer option from his chat window - User A selects a file from storage on his handset, and selects to share it with User B - User B reviews and then accepts the invitation - The file is transferred to User B |
| Pass-Criteria | - At step 2 User B receives the invitation - At step 3 If it is a supported file type and less than the maximum size allowed (by Service Provider), User B should see: - The filename; - The relevant file-type; - An icon or thumbnail preview; - The size of the file. - At step 4 upon User B accepting the file transfer invitation, he receives the file in his chat window - At step 4 User B can access the file (e.g. from storage on his handset) |

## RCS-e-1.2-int-042- File transfer: rejected

| Test Case Id | RCS-e-1.2-int-042 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies correct operation when file transfer is rejected |
| Specification Reference | RCS-e 3.4.3 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for User A and B - User A and B are IMS registered and the registration timer for each user is far from expiration - User A and B handset clients are operating with data coverage over the following RANs: - 3G; or - HSPA; or - Wi-Fi. - User A has contacts in his address book, including User B |
| Test Procedure | - User A selects User B from his address book - User A selects file transfer option - User A selects a file from storage on his handset, and selects to share it with User B - User B rejects the invitation |
| Pass-Criteria | - At step 3 User B receives the file transfer invitation - At step 4 the file is not received by User B - At step 4 User A is notified that User B has rejected the file transfer request |

## RCS-e-1.2-int-051- Video share: bi-directional

| Test Case Id | RCS-e-1.2-int-051 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful bi-directional video share |
| Specification Reference | RCS-e 3.3.3 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for User A and B - User A and B are IMS registered and the registration timer for each user is far from expiration - User A and B handset clients are operating with data coverage over the following RANs: - HSPA; or - Wi-Fi. - User A and B are already engaged in an active voice call |
| Test Procedure | - During the active voice call, User A initiates video sharing with User B - User B initiates video sharing with User A - User B accepts the video share invitation request from User A - User A accepts the video share invitation request from User B - User A begins sharing the video stream with User B (i.e. video stream may be a pre-stored video or from a live camera[s]) - User B begins sharing the video stream with User A (i.e. video stream may be a pre-stored video or from a live camera[s]) |
| Pass-Criteria | - At step 1 User B receives the video share invitation from User A - At step 2 User A receives the video share invitation from User B - At step 5 User B receives the video shared by User A - At step 6 User A receives the video shared by User B |

## RCS-e-1.2-int-052- Video share: reject

| Test Case Id | RCS-e-1.2-int-052 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies correct operation when a request for video share is rejected |
| Specification Reference | RCS-e 3.3.11 |
| Preconditions | - RCS-e services have been successfully configured on the phone/(U)SIM pair for User A and B - User A and B are IMS registered and the registration timer for each user is far from expiration - User A and B handset clients are operating with data coverage over the following RANs: - 3G; or - HSPA; or - Wi-Fi. - User A and B are in an active voice call |
| Test Procedure | - During the active voice call, User A initiates video sharing with User B - User B rejects the video share invitation request from User A |
| Pass-Criteria | - At step 1 User B receives the video share invitation - At step 2 the call remains without video share - At step 2 User A is notified that User B has rejected the video share request |

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| n/a | n/a | No prior version |

Draft/Candidate Version 1.0 History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-ETS-RCS-e-V1_0 | 07 Feb 2012 | n/a | No previous version within OMA |
|  | 10 Feb 2012 | IOP | Basic set of test cases that cover main RCS-e features |
|  | 14 Feb 2012 | IOP | Basic test case list updated |
|  | 16 Apr 2012 | CON | Conformance test cases added (0019R02 and 0020R02) and 12 new conformance test cases |
|  | 19 Jul 2012 | 5 | Updated according to: OMA-IOP-MEC-2012-0071R01-INP_RCS_e_capability_exchange_for_video_image_share OMA-IOP-MEC-2012-0072-INP_RCS_e_video_share_call_establishment Re-numbering of tables in the whole document |
|  | 28 Aug 2012 | All | Updated according to: OMA-IOP-MEC-2012-0082-CR_RCSe_conformance_fixes OMA-IOP-MEC-2012-0085-CR_RCSe_fixes |
|  | 05 Sep 2012 | 3.2, 5.25 to 5.29 (new) 6.25 (new) | Updated according to: OMA-IOP-MEC-2012-0081R02-CR_RCSe_addDefinition OMA-IOP-MEC-2012-0083R01-CR_RCSe_disable_RCSuser OMA-IOP-MEC-2012-0092-CR_RCSe_IOP_new_conformance_tests, combined with and superseding OMA-IOP-MEC-2012-0086-CR_RCSe_additional_conformance_tests |
| Draft Version OMA-ETS-RCS-e-INT-V1_0 | 19 Sep 2012 | CON | Conformance test cases were taken out to separate conformance ETS. IOP test cases were renumbered., as agreed in OMA-IOP-MEC-2012-0106R01-CR_RCS_e_INT_ETS |