Change Request

| Title: | Few Notifications Attributes Type must be fixed | Public OMA Confidential |
| --- | --- | --- |
| To: | DM LightweightM2M WG |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0-20150513-D |  |
| Submission Date: |  |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thierry GARNIER, Gemalto N.V |  |
| Replaces: | n/a |  |

Reason for Change

.  Value  type of gt, lt and stp  (Notification Attributes) is wrong

.  Some Text to improve  a bit

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and agree this CR

Detailed Change Proposal

Text  Clarification

## Attributes

### Attributes Definitions and Rules

Attributes are metadata which can be attached to an Object, an Object Instance or a Resource and may be different for each related LWM2M Server. These attributes can fulfill various roles: from just carrying information (e.g. Discover), up to containing parameters for Notification for example.

.

.

| Attribute characteristics | Description |
| --- | --- |
| Name | Attribute Name used to reference a specific Attribute in that Enabler (e.g. “Minimum Period”) |
| CoRE Link Param | the string used when this Attribute is transferred to CoAP as a CoRE link parameter (ex pmin) |
| Attachment | The Object, Object Instance or Resource, to which an Attribute applies |
| Assignation Level | The Level (Object, Object Instance, Resource) where the Attribute is specified. |
| Class | Attributes are organized according to their purpose; 2 Class of Attributes are supported in LWM2M TS 1.0 [NOTIFICATION] gather Attributes regarding Notify operations parameters [PROPERTIES] gather Attributes regarding general informations |
| Access Mode | R, W, RW: operation allowed by the LWM2M Server. |
| Applicability | Condition to fulfil for allowing to attach such an Attribute |
| Default Value | <value> ¦ - |
| Value Type | LWM2M data type |
| Value | The Value carried by this Attribute : its data type must be of “Value Type” |

Table 1: Attribute Characteristics

Some Attributes MAY be exposed to the LWM2M Server in the payload response of a “Discover” command (Section 5.4.2).

The value of some Attributes MAY be changed by the LWM2M Server in using the “Write Attributes” command (Section 5.4.4); which Attribute are concerned are marked as “W” (writable) in the table of the next Section.

Note:  A LWM2M message payload is a list of application/link-format CoRE Links [RFC6690] which will includes the LWM2M Attributes

Fix  [NOTIFICATION] Class Attribute  Type

### Attributes Classification

[PROPERTIES] Class Attributes listed table 2.

| Attribute Name | CoRE Link param | Attachment | Assignation Level | Required | Access Mode | Value Type | Default Value | Applicability | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dimension | dim | Resource | Resource Level | YES (Client) | R | Integer [0:255] | - | Multiple Resource | Number of Instantiations for a Multiple Resource |

Table 3: [PROPERTIES] Class Attributes

[NOTIFICATION] Class Attributes

The role of these R-Attributes is to provide parameters to the “Notify” operation; any readable Resource can have such R-attributes.

In the message sent by a LWM2M Client in response to an “Observe” operation, the current Resource value is reported; this event can be considered as the initial notification.

Each time a Resource notification is sent, the “Minimum Period” and “Maximum Period” timers associated to this Resource are restarted.

The notification of a Resource value will be sent when the combination of a change value condition (“Greater Than”, “Less Than”, “Step) and the “Minimun Period” timing conditions are both fulfilled for that Resource.

The LWM2M Server MUST support and LWM2M Client SHOULD support all the [NOTIFICATION] Class Attributes listed table 3

| Attribute Name | CoRE Link param |  | Attachment | Assignation Level | Required | Access Mode | Value Type | Default Value | Apply Condition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Minimum Period | pmin |  | Resource | Resource Level Object Instance Level Object Level | No | RW | Integer | 1 (sec) | Readable Resource |
| Notes: The Minimum Period Attribute indicates the minimum time in seconds the LWM2M Client MUST wait between two notifications. If a Resource value has to be notified during the specified quiet period, the notification MUST be sent as soon as this period expires. In the absence of this parameter, the Minimum Period is defined by the Default Minimum Period set in the LWM2M Server Account. |  |  |  |  |  |  |  |  |  |
| Max imum Period | pmax |  | Resource | Resource Level Object Instance Level Object Level | No | RW | Integer | - | Readable Resource |
| Notes: The Maximum Period Attribute indicates the maximum time in seconds the LWM2M Client MAY wait between two notifications. When this “Maximun Period” expires after the last notification, a new notification MUST be sent. In the absence of this parameter, the “Maximum Period” is defined by the Default Maximum Period set in the LWM2M Server Account. The maximum period parameter MUST be not smaller than the minimum period parameter. |  |  |  |  |  |  |  |  |  |
| Greater Than | gt |  | Resource | Resource Level | No | RW | Float | - | Numerical &Readable Resource |
| Notes: This “Greater Than” Attribute defines a threshold high value. When this Attributes is present, the LWM2M Client MUST notify the Server each time the Observed Resource value crosses the “Greater Than” Attribute value with respect to pmin parameter. |  |  |  |  |  |  |  |  |  |
| Less Than | lt | Resource |  | Resource Level | No | RW | Float | - | Numerical &Readable Resource |
| Notes: This “Less Than” Attribute defines a threshold low value. When this Attributes is present, the LWM2M Client MUST notify the Server each time the Observed Resource value crosses the “Less Than” Attribute value with respect to pmin parameter. |  |  |  |  |  |  |  |  |  |
| Step | stp | Resource |  | Resource Level | No | RW | Float | - | Numerical &Readable Resource |
| Notes: This “Step” Attribute defines a minimum change value between two notifications. When this Attribute is present, the change value condition will occur when the value variation since the last notification of the Observed Resource, is greater or equal to the “Step” Attribute value. When the “Step” change value condition occurs, the LWM2M Client MUST notify the Server with respect to “Period Minimum” rule. |  |  |  |  |  |  |  |  |  |

Note: the following rules MUST be respected (“lt” value + 2*”stp” values < “gt” value)

Table 4: [NOTIFICATION] class Attributes