OMA Smart Card Web Server

OMA Smart Card Web Server Enabler

OMA Smart Card Web Server (OMA SCWS) allows local communication between the Smart Card Web Server and an HTTP application (e.g. Web browser) running in the handset. This facilitates the implementation of a mobile operator’s services making them portable across handsets. OMA SCWS provides static and dynamic web pages and allows transactions between the Smart Card and an HTTP based application running in the handset for example a Web browser. OMA SCWS Enabler provides also the facilities to remotely administrate the content of the SCWS using HTTP(s) protocol.

OMA SCWS V1.1 was approved in May 2009. OMA SCWS V1.2 reached Candidate status in April of 2011 and is expected to reach final approval in end of September  2012. OMA SCWS V1.2 offers several enhancements to SCWS V1.1.

These enhancements include:

- Clarification of the usage when the smart card implements a full TCP/IP stack on top of the USB-IC (InterChip) interface.

- Improvement of the SCWS remote administration when third parties are involved and definition of memory polls where the static resources of a given third party are allocated.

- Specification of the usage of the last versions of TLS (Transport Layer Security), TLS 1.1 and TLS 1.2.

- Improvement of the security for applications in the handset accessing the SCWS content: the implementation of the Access Control Policy enforcer becomes mandatory for the handset (e.g. only granted applications have access to the SCWS).

- In order to trigger a remote administration session to update the content of the SCWS, the usage of a SIP Push message has been defined in addition to the Push SMS and reusing the OMA SIP Push Enabler.

OMA Smart Card Web Server Architecture Diagram

The Importance of Standardization

The main important for SCWS were:

- the InterOPerability between the device and the Smart Card or between the Administration server and the Smart Card.

- The portability from one device to the other.

Benefits of OMA Smart Card Web Server for Operators and Mobile Users

Operator Benefits

- OMA SCWS acts as a Web server in the Smart Card (e.g. a (U)SIM)

- OMA SCWS implements a mobile operator’s services by providing static and dynamic web pages and making it portable across handsets.

- Mobile operators’ (U)SIM applications (e.g. (U)SIM Toolkit apps) benefit from an enhanced Web “look and feel” using the browser’s multimedia capabilities

- OMA SCWS acts as a means to deploy secure services with standard Web technology leveraging on inherent Smart Card security features

- OMA SCWS is a standard platform to manage remotely web content

User Benefits

• OMA SCWS improves accessibility of the Web browser from any phone MMI

• OMA SCWS is accessible at all times

• OMA SCWS ensures secure browsing help e-commerce adoption

Basic Use Cases for OMA Smart card Web Server

Use Case 1: Quick and Simple Access to Network Operator Services

Via a regular handset browser, OMA SCWS portal gives the end-user a set of pages constituting menus that enable service discovery based on local content reachable immediately at any moment. This teaser, free of charge, provides access to remote content by setting up a call to a voice server, sending a SMS or opening a Mobile Internet session at a dedicated address.

Use Case 2: The Content Store

The mix between local and remote content is optimized to favor the on-line purchasing of music or other multimedia content. The OMA SCWS Visual-Content Store provides a local catalog of contents with a rich display. The end-user can preview and pre-listen before purchasing content. This Content Store can be periodically updated remotely thanks to the OMA SCWS remote administration protocol to reflect new content or a specific promotion—such as the top-ten of the most downloaded content of the week.

Use Case 3:  The Helpdesk

OMA SCWS can be used to display a helpdesk on the browser with local content, allowing information about handset usage and configuration, as well as the services provided by the telecom operator to be integrated. This gives the end-user an off-line help service that is always available (even when out of coverage). Additionally, SCWS will help reduce the call center costs and increase service usage by keeping the end-user informed.

Use Case 4: Rich User Interface

OMA SCWS offers a Rich Graphical User Interface to the Mobile Operator’s existing Services (TV-voting, horoscope…) or SIM applications (Phonebook synchronization, M-banking, Instant Messaging, m-NFC services user interface…)

- Example 1: wallet display for mobile NFC services. m-NFC services based on a Contactless Mobile Payment application residing in the Smart Card needs to have a friendly interface with the user (activation/deactivation or configuration of the contactless application). This User Interface can be implemented thanks to the OMA SCWS. All the elements composing the m-NFC services are thus deployed inside the Smart Card. It will ensure the portability of the service and the interoperability whatever the type of the handset.

- Example 2: TV-voting services: The ability to display local content on the card allows the OMA SCWS User Interface to provide TV-voting interactive services by displaying pictures or clips concerning each candidate and weekly updates after nominations.

- Example 3 with the SIM ToolKit based services: The OMA SCWS user interface allows the re-use of existing SIM ToolKit based services with an enhanced graphical user interface.

The OMA Architecture Smart Card Technology (ARC-SCT)) Working Group provides the primary source of expertise within the OMA on issues related to Smart Cards and other secure hardware tokens using similar technology.

Some examples in addition to the SCWS Enabler:

ARC-SCT has worked in the past on the On Board Key Generation (OBKG) Enabler including the WIM (Wireless Identity Module) Application specification.

More recently, ARC-SCT has contributed to the UICC related features of the OMA Open Connection Manager API Enabler.

For more information about other OMA Specifications and to see their definitions, please visit: http://www.openmobilealliance.org/Technical/releaseprogram.aspx