| # RESTful Network API for Alias Management |  |  |  |
| --- | --- | --- | --- |
| Draft Version 1.0 – 12 Oct 2017 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-TS-REST_NetAPI_AliasManagement-V1_0-20171012-D |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2017 Open Mobile Alliance All Rights Reserved.
Used with the permission of the Open Mobile Alliance under the terms set forth above.

Contents

1.	Scope	6

2.	References	7

2.1	Normative References	7

2.2	Informative References	7

3.	Terminology and Conventions	8

3.1	Conventions	8

3.2	Definitions	8

3.3	Abbreviations	8

4.	Introduction	10

4.1	Version 1.0	10

5.	[Functional Area] API definition	11

5.1	Resources Summary	11

5.2	Data Types	16

5.2.1	XML Namespaces	16

5.2.2	Structures	16

5.2.2.1	Type: [Type Name (without Light-weight Resources)]	17

5.2.2.2	Type: [Type Name (with Light-weight Resources)]	18

5.2.3	Enumerations	18

5.2.3.1	Enumeration: [Enumeration Name]	18

5.2.4	Values of the Link “rel” attribute	19

5.3	Sequence Diagrams	19

5.3.1	[Title of flow scenario]	19

6.	Detailed specification of the resources	22

6.1	Resource: [Description of the resource]	22

6.1.1	Request URL variables	23

6.1.1.1	Light-weight Resource relative paths	23

6.1.2	Response Codes and Error Handling	23

6.1.3	GET	24

6.1.3.1	Example 1: [Example title]  (Informative)	24

6.1.3.1.1	Request	25

6.1.3.1.2	Response	25

6.1.3.2	Example 2: [Example title]  (Informative)	25

6.1.3.2.1	Request	25

6.1.3.2.2	Response	26

6.1.4	PUT	26

6.1.4.1	Example 1: [Example title]  (Informative)	26

6.1.4.1.1	Request	26

6.1.4.1.2	Response	26

6.1.4.2	Example 2: [Example title]  (Informative)	26

6.1.4.2.1	Request	26

6.1.4.2.2	Response	26

6.1.5	POST	27

6.1.5.1	Example 1: [Example title]  (Informative)	27

6.1.5.1.1	Request	27

6.1.5.1.2	Response	27

6.1.5.2	Example 2: [Example title]  (Informative)	27

6.1.5.2.1	Request	27

6.1.5.2.2	Response	27

6.1.6	DELETE	27

6.1.6.1	Example 1: [Example title]  (Informative)	28

6.1.6.1.1	Request	28

6.1.6.1.2	Response	28

6.1.6.2	Example 2: [Example title]  (Informative)	28

6.1.6.2.1	Request	28

6.1.6.2.2	Response	28

7.	Fault definitions	29

7.1	Service Exceptions	29

7.1.1	SVC[code number]: [Text for exception header]	29

7.2	Policy Exceptions	29

7.2.1	POL[code number]: [Text for exception header]	30

7.2.1	POL1003: Refund exceeds original charge amount	30

Appendix A.	Change History (Informative)	31

A.1	Approved Version History	31

A.2	Draft/Candidate Version 1.0 History	31

Appendix B.	Static Conformance Requirements (Normative)	32

B.1	SCR for REST.FUNCAREA Server	32

B.1.1	SCR for REST.FUNCAREA.FUNCTION Server	32

Appendix C.	JSON examples  (Informative)	33

C.1	[Example Title] (section [section number cross reference])	33

Appendix D.	[Baseline specification] operations mapping (Informative)	34

Appendix E.	Light-weight Resources (Informative)	35

Appendix F.	Authorization aspects (Normative)	36

F.1	Use with OMA Authorization Framework for Network APIs	36

F.1.1	Scope values	36

F.1.1.1	Definitions	36

F.1.1.2	Downscoping	37

F.1.1.3	Mapping with resources and methods	37

F.1.2	Use of ‘acr:auth’	39

Figures

Figure 1 Resource structure defined by this specification	13

Figure 2 [Caption of this flow]	21

Figure 2 [Caption of this flow]	22

Tables

Table 1 [Baseline specification] operations mapping	40

# Scope

This specification defines a RESTful API for Alias Management using HTTP protocol bindings.

# References

## Normative References

| [Autho4API_10] |  | “Authorization Framework for Network APIs”, Open Mobile Alliance™, OMA-ER-Autho4API-V1_0, URL: http://www.openmobilealliance.org/ |  |  |
| --- | --- | --- | --- | --- |
| [REST_NetAPI_ACR] |  |  |  | “RESTful Network API for Anonymous Customer Reference Management ”, Open Mobile Alliance™, OMA-TS-REST_NetAPI_ACR-V1_0, URL: http://www.openmobilealliance.org/ |
| [REST_NetAPI_Common] |  | “Common definitions for RESTful Network APIs”, Open Mobile Alliance™, OMA-TS-REST_NetAPI_Common-V1_0, URL: http://www.openmobilealliance.org/ |  |  |
| [REST_SUP_AliasManagemet] |  | “XML schema for the RESTful Network API for Alias Management”, Open Mobile Alliance™, OMA-SUP-XSD_rest_netapi_aliasManagement-V1_0, URL: http://www.openmobilealliance.org/ |  |  |
| [RFC2119] |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL: http://tools.ietf.org/html/rfc2119.txt |  |  |
| [RFC7231] |  | “Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content, R. Fielding, Ed., J.Raschke, Ed., June 2014, URL: http://tools.ietf.org/html/rfc7231.txt |  |  |
| [RFC3966] |  | “The tel URI for Telephone Numbers”, H.Schulzrinne, December 2004, URL: http://tools.ietf.org/html/rfc3966.txt |  |  |
| [RFC3986] |  | “Uniform Resource Identifier (URI): Generic Syntax”, R. Fielding et. al, January 2005, URL: http://tools.ietf.org/html/rfc3986.txt |  |  |
| [RFC7159] |  | “The JavaScript Object Notation (JSON) Data Interchange Format”, T. Bray, Ed., March 2014, URL: http://tools.ietf.org/html/rfc7159.txt |  |  |
| [SCRRULES] |  | “SCR Rules and Procedures”, Open Mobile Alliance™, OMA-ORG-SCR_Rules_and_Procedures, URL: http://www.openmobilealliance.org/ |  |  |
| [XMLSchema1] |  | W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures, W3C Recommendation 5 April 2012, URL: http://www.w3.org/TR/xmlschema11-1/ |  |  |
| [XMLSchema2] |  | W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes, W3C Recommendation 5 April 2012, URL: http://www.w3.org/TR/xmlschema11-2/ |  |  |

## Informative References

| [OMADICT] |  | “Dictionary for OMA Specifications”, Version 2.9, Open Mobile Alliance™, OMA-ORG-Dictionary-V2_9, URL:http://www.openmobilealliance.org/ |  |
| --- | --- | --- | --- |
| [REST_WP] |  | “Guidelines for RESTful Network APIs”, Open Mobile Alliance™, OMA-WP-Guidelines_for_RESTful_Network_APIs, URL:http://www.openmobilealliance.org/ |  |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

## Definitions

For the purpose of this TS, all definitions from the OMA Dictionary apply [OMADICT].

| Chatbot | An RSC-based service provided to users whose output is presented in a conversational form and which provides some kind of value to the users. Often a piece of software interfacing with one or more users aiming to simulate intelligent human conversation |
| --- | --- |
| Chatbot Platform | A system that provides a mechanism for Chatbot developers to create and register Chatbots, which can then be exposed to the users connected to the platform through a messaging system. |

## 3.3	Abbreviations

| ACR | Anonymous Customer Reference |
| --- | --- |
| API | Application Programming Interface |
| HTTP | HyperText Transfer Protocol |
| JSON | JavaScript Object Notation |
| MIME | Multipurpose Internet Mail Extensions |
| OMA | Open Mobile Alliance |
| REST | REpresentational State Transfer |
| SCR | Static Conformance Requirements |
| SIP | Session Initiation Protocol |
| TS | Technical Specification |
| URI | Uniform Resource Identifier |
| URL | Uniform Resource Locator |
| WP | White Paper |
| XML | eXtensible Markup Language |
| XSD | XML Schema Definition |

# Introduction

The Technical Specification of the RESTful Network API for Alias Management contains HTTP protocol bindings for Alias Management, using the REST architectural style. The specification provides resource definitions, the HTTP verbs applicable for each of these resources, and the element data structures, as well as support material including flow diagrams and examples using the various supported message body formats (i.e. XML, JSON).

## Version 1.0

Version 1.0 of this specification supports the following operations:

- Retrieve current Alias setting

- Enable Alias functionality (by setting it to “on”)

- Disable Alias functionality (by setting it to “off”)

- Renew Alias value (by enabling Alias functionality operation)

- Enable Alias-MSISDN linkage report

In addition this specification provides::

- Support for scope values used with authorization framework defined in [Autho4API_10]

- Support for Anonymous Customer Reference (ACR) as an end user identifier

- Support for “acr:auth” as a reserved keyword in an ACR

# Alias Management API definition

This section is organized to support a comprehensive understanding of the Alias Management API design. It specifies the definition of all resources, definition of all data structures, and definitions of all operations permitted on the specified resources.

This Network API enables an authorized client (on behalf of the user) to control the exposure of user’s true identity (e.g. MSISDN) from online services (e.g. Chatbots). The alias which is a tokenized user identity (aka ACR) is managed by the server and the client, using this Network API, is enabled to exert control (on behalf of the user) over the aliasing functionality by turning it off and hence exposing user’s true identity or turning it on and hence hiding user’s true identity. This Network API also enables client(s) (which may be  different (e.g. Chatbot Platform) than the one requested aliasing to be turned off) to subscriber to notifications in order to be informed of the association between an alias and the user’s true identity (e.g. MSISDN) when aliasing functionality (on behalf of the user) is turned off.

Common data types, naming conventions, fault definitions and namespaces are defined in [REST_NetAPI_Common].

The remainder of this document is structured as follows:

Section 5 starts with a diagram representing the resources hierarchy followed by a table listing all the resources (and their URL) used by this API, along with the data structure and the supported HTTP verbs (section 5.1). What follows are the data structures (section 5.2). A sample of typical use cases is included in section 5.3, described as high level flow diagrams.

Section 6 contains detailed specification for each of the resources. Each such subsection defines the resource, the request URL variables that are common for all HTTP methods, and the supported HTTP verbs. For each supported HTTP verb, a description of the functionality is provided, along with an example of a request and an example of a response. For each unsupported HTTP verb, the returned HTTP error status is specified, as well as what should be returned in the Allow header.

All examples in section 6 use XML as the format for the message body. JSON examples are provided in Appendix C.

Section 7 contains fault definition details such as Service Exceptions and Policy Exceptions.

Appendix B provides the Static Conformance Requirements (SCR).

Appendix E provides a list of all Light-weight Resources, where applicable.

Appendix F defines authorization aspects to control access to the resources defined in this specification.

Note: Throughout this document client and application can be used interchangeably.

## Resources Summary

This section summarizes all the resources used by the RESTful Network API for Alias Management.

The "apiVersion" URL variable SHALL have the value "v1" to indicate that the API corresponds to this version of the specification. See [REST_NetAPI_Common] which specifies the semantics of this variable.

Figure 1 Resource structure defined by this specification

The following tables give a detailed overview of the resources defined in this specification, the data type of their representation and the allowed HTTP methods.

Purpose: To allow a client to manage Alias Function

| Resource | URL Base URL: //{serverRoot}/aliasmgmt/{apiVersion} | Data Structures | HTTP verbs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GET | PUT | POST | DELETE |
| User’s alias | /control/{chatbotId}/{userId}/alias | Empty | no | no | Turn off aliasing function and report alias-user identity mapping (i.e. aliasLink) | no |
| User’s alias status | /control/{chatbotId}/{userId}/alias/status | Status | Fetch alias’s setting | Set alias status to On/Off (i.e.aliasOn/aliasOff) Note: setting alias to “on” status results in a new alias generated even if alias is already On. (i.e. renew alias value) | no | no |

Purpose: To allow client to manage subscriptions to notifications about aliasing functionality (e.g. AliasLink)

| Resource | URL Base URL: //{serverRoot}/aliasmgmt/{apiVersion} | Data Structures | HTTP verbs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GET | PUT | POST | DELETE |
| All subscriptions to alias notifications | /report/{chatbotPlatformId/subscriptions | AliasSubscriptionList (used for GET) AliasSubscription (used for POST) common:ResourceReference (OPTIONAL alternative for POST response) | Retrieve all active alias notification subscriptions | no | Create new subscription for alias notifications | no |
| Individual subscription | /report/{chatbotPlatformId/subscriptions/{subscriptionId} | AliasSubscription (used for GET response) | Retrieve an individual subscription | no | no | Cancel subscription and stop corresponding notifications |

Purpose: To allow server to notify client about alias notifications

| Resource | URL<specified by the client> | Data Structures | HTTP verbs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GET | PUT | POST | DELETE |
| Client notification about alias-MSISDN linkage | Specified by client when subscription is created or provisioned | AliasLinkNotification | no | no | Notifies client about the linkage between an alias and user’s MSISDN | no |

## Data Types

### XML Namespaces

The XML namespace for the Alias Management data types is:

urn:oma:xml:rest:netapi:aliasManagement:1

The 'xsd' namespace prefix is used in the present document to refer to the XML Schema data types defined in XML Schema [XMLSchema1, XMLSchema2]. The 'common' namespace prefix is used in the present document to refer to the data types defined in [REST_NetAPI_Common]. The use of namespace prefixes such as 'xsd' is not semantically significant.

The XML schema for the data structures defined in the section below is given in [REST_SUP_AliasManagement].

### Structures

The subsections of this section define the data structures used in the Alias Management API.

Some of the structures can be instantiated as so-called root elements.

For structures that contain elements which describe a user identifier, the statements in section 6 regarding 'tel', 'sip' and 'acr' URI schemes apply.

#### Type: AliasSubscriptionList

This type represents a list of subscriptions to notifications regarding alias events.

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| subscription | AliasSubscription[0..unbounded] | Yes | Array of notification subscriptions. |
| resourceURL | xsd:anyURI | No | Self referring URL. |

A root element named aliasSubscriptionList of type AliasSubscriptionList is allowed in response bodies.

#### Type: AliasSubscription

This type represents a subscription to notifications regarding alias events (e.g. alias-MSISDN association (aka “AliasLink”) event).

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| callbackReference | common:CallbackReference | No | Client's Notification URL and OPTIONAL callbackData. |
| duration | xsd:unsignedInt | Yes | Period of time (in seconds) notifications are provided for. If set to “0” (zero), a default duration time, which is specified by the service policy, will be used. If the parameter is omitted, the notifications will continue until the maximum duration time, which is specified by the service policy, unless the notifications are stopped by deletion of subscription for notifications. This element MAY be given by the client during resource creation in order to signal the desired lifetime of the subscription. The server SHOULD return in this element the period of time for which the subscription will still be valid. |
| clientCorrelator | xsd:string | Yes | A correlator that the client can use to tag this particular resource representation during a request to create a resource on the server. This element MAY be present. Note: this allows the client to recover from communication failures during resource creation and therefore avoids duplicate subscriptions in such situations. In case the element is present, the server SHALL not alter its value, and SHALL provide it as part of the representation of this resource. In case the field is not present, the server SHALL NOT generate it. |
| resourceURL | xsd:anyURI | Yes | Self referring URL. The resourceURL SHALL NOT be included in POST requests by the client, but MUST be included in POST requests representing notifications by the server to the client, when a complete representation of the resource is embedded in the notification. The resourceURL MUST also be included in responses to any HTTP method that returns an entity body, and in PUT requests. |

A root element named aliasSubscription of type AliasSubscription is allowed in request and/or response bodies

#### Type: Empty

A dummy element

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| (empty) |  | No | This type is empty and used as a dummy element, to overcome some limitations, for example: - HTTP POST requires a body, but the semantics of the request/response may not need that body. - Some implementations may not be able to support non-existent (empty) HTTP body. |

A root element named empty of type Empty is allowed in request and/or response bodies.

#### Type: Status

This type represents the status of an alias

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| aliasStatus | AliasStatus | No | Status of the current aliasing functionality in the GET or PUT response. Status of the required aliasing functionality in PUT request. In response to GET operation, “On” is the default value when the User-Chatbot pair is not found. FFS: RCC.07 (Table 54: Privacy Management Commands definition) states that even if the User,Chatbot pair is not found the response to GET should be AliasOn.This doesn’t make sense in REST model where the HTTP resource Not Found (404) response is expected. Need to communicate this to GSG (GSMA). GSG needs to either the platform by default has all the users’ aliasOn until it receives a alias Mgmt command or they instruct API GW or RCS client to issue a PUT command (alias=On) after a HTTP 404 result from a fetchSetting. |
| resourceURL | xsd:anyURI | Yes | Self referring URL. The resourceURL SHALL NOT be included in POST requests by the client, but MUST be included in POST requests representing notifications by the server to the client, when a complete representation of the resource is embedded in the notification. The resourceURL MUST also be included in responses to any HTTP method that returns an entity body, and in PUT requests. |

A root element named status of type Status is allowed in request and/or response bodies.

#### Type: AliasLinkInfo

This type represents information linking user’s alias employed by a chatbot to user’s true identity (e.g. MSISDN).

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| userId | xsd:anyURI | No | The address (e.g. ‘sip’ URI,’tel’ URI, ‘acr’ URI) of the user. |
| alias | xsd:anyURI | No | The string comprising the user’s alias in the context of a given chatbot identified by the chatbotId (parameter below). |
| chatbotId | xsd:anyURI | Yes | The address (e.g. ‘sip’ URI,’tel’ URI, ‘acr’ URI) of the chatbot. |

#### Type: AliasLinkNotification

This type represents a notification about the linkage between an alias and a MSISDN.

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| callbackData | xsd:string | Yes | The ‘callbackData’ element if it was passed by the application in the ‘callbackReference’ element when creating a subscription to notifications. See [REST_NetAPI_Common] |
| aliasLinkInfo | AliasLinkInfo | No | User’s MSISDN associated with the user’s alias used for communication with a given chatbot. |
| link | common:Link [0..unbounded] | Yes | Links to other resources that are in relationship to the notification. Further, the server SHOULD include a link to the related subscription. |

A root element named aliasLinkNotification of type AliasLinkNotification is allowed in notification

### Enumerations

The subsections of this section define the enumerations used in the Alias Management API.

#### Enumeration: AliasStatus

This enumeration defines possible values to describe the status of the aliasing functionality for a user-chatbot pair.

| Enumeration | Description |
| --- | --- |
| On | Aliasing functionality is set to On. Hence user’s identity (i.e. MSISDN) is hidden from the chatbot. |
| Off | Aliasing functionality is set to Off. Hence user’s identity (i.e. MSISDN) is exposed to the chatbot |

### Values of the Link “rel” attribute

The “rel” attribute of the Link element is a free string set by the server implementation, to indicate a relationship between the current resource and an external resource. The following are possible strings (list is non-exhaustive, and can be extended):

- AliasSubscription

These values indicate the kind of resource that the link points to.

## Sequence Diagrams

The following subsections describe the resources, methods and steps involved in typical scenarios.

In a sequence diagram, a step which involves delivering a notification is labeled with “POST or NOTIFY”, where “POST” refers to delivery via the HTTP POST method, and “NOTIFY” refers to delivery using the Notification Channel [REST_NetAPI_NotificationChannel].

### Subscription to Aliasing notifications

This figure below shows a scenario for a client application (e.g. Chatbot Plaform) subscribing to Aliasing notifications (e.g. aliaslink), querying for a list of active subscriptions, querying information pertaining to a subscription and unsubscribing to Aliasing notifications .

The notification URL passed by the client during the subscription step is a Client-side Notification URL.

The resources:

- To subscribe to Aliasing notifications, create a new  resource under http://{serverRoot}/aliasmgmt/{apiVersion}/report/{chatbotPlatformId/subscriptions

- To retrieve list of active subscriptions, read the following resource http://{serverRoot}/aliasmgmt/{apiVersion}/report/{chatbotPlatformId/subscriptions

- To retrieve information about an individual subscription, read the following resource http://{serverRoot}/aliasmgmt/{apiVersion}/report/{chatbotPlatformId/subscriptions/{subscriptionId}

- To cancel subscription to Aliasing notifications delete the resource under http://{serverRoot}/aliasmgmt/{apiVersion}/report/{chatbotPlatformId/subscriptions/{subscriptionId}

Figure 2 Alias notifications subscription

Outline of the flows:

- An application (e.g. Chatbot Platform) subscribes to Aliasing notifications using the POST method to submit the AliasSubscription data structure to the resource containing all subscriptions.

- The application receives the result resource URL containing the subscriptionId.

- An application requests the list of active subscriptions, using the GET method to the resource containing all subscriptions.

- The server returns subscriptions list belonging to the application in the response.

- An application requests information pertaining to an individual subscription using the GET method to the resource.

- The server returns subscription’s data which includes duration, etc. in the response.

- The application stops receiving notifications (on a given subscription) using DELETE with the resource URL containing the subscriptionId.

- Deletion confirmation.

# Detailed specification of the resources

The following applies to all resources defined in this specification regardless of the representation format (i.e. XML, JSON):

- Reserved characters in URL variables (parts of a URL denoted below by a name in curly brackets) MUST be percent-encoded according to [RFC3986]. Note that this always applies, no matter whether the URL is used as a Request URL or inside the representation of a resource (such as in “resourceURL” and “link” elements).

- If a user identifier (e.g. address, participantAddress, etc.) of type anyURI is in the form of an MSISDN, it MUST be defined as a global number according to [RFC3966] (e.g. tel:+19585550100). The use of characters other than digits and the leading “+” sign SHOULD be avoided in order to ensure uniqueness of the resource URL. This applies regardless of whether the user identifier appears in a URL variable or in a parameter in the body of an HTTP message.

- If an equipment identifier of type anyURI is in the form of a SIP URI, it MUST be defined according to [RFC3261].

- If a user identifier (e.g. address, userId, etc) of type anyURI is in the form of an Anonymous Customer Reference (ACR), it MUST be defined according to [REST_NetAPI_ACR], i.e. it MUST include the protocol prefix 'acr:' followed by the ACR.

  - The ACR ‘auth’ is a supported reserved keyword, and MUST NOT be assigned as an ACR to any particular end user. See F.1.2 for details regarding the use of this reserved keyword.

- For requests and responses that have a body, the following applies: in the requests received, the server SHALL support JSON and XML encoding of the parameters in the body. The Server SHALL return either JSON or XML encoded parameters in the response body, according to the result of the content type negotiation as specified in [REST_NetAPI_Common]. In notifications to the Client, the server SHALL use either XML or JSON encoding, depending on which format the client has specified in the related subscription. The generation and handling of the JSON representations SHALL follow the rules for JSON encoding in HTTP Requests/Responses as specified in [REST_NetAPI_Common].

## Resource: [Description of the resource]

<< Description of the resource in the title heading should match the description of the resource from the first column of the purpose table in section 5.1 >>

The resource used is:

[resource URL]

[without Light-weight Resources usually http://{serverRoot}/funcarea/{apiVersion}/...]

[with Light-weight Resources usually http://{serverRoot}/funcarea/{apiVersion}/.../[ResourceRelPath]]

This resource is used for [descriptive explanation of the resource].

If the resource is on the server side and supports creating a subscription for notifications, and if the use of Notification Channel is supported, include/adapt this paragraph, otherwise delete it. This resource can be used in conjunction with a Client-side Notification URL, or in conjunction with a Server-side Notification URL. In this latter case, the application MUST first create a Notification Channel (see [REST_NetAPI_NotificationChannel]) before creating a subscription.

Alternatively, if the resource is a notification resource to which the server provides notifications based on a previously created subscription,   and if the use of Notification Channel is supported, include/adapt this paragraph and the following Note, otherwise delete them. This resource is a callback URL provided by the client for notification about FOO. The RESTful [Functional Area] API does not make any assumption about the structure of this URL. If this URL is a Client-side Notification URL, the server will POST notifications directly to it. If this URL is a Server-side Notification URL, the server uses it to determine the address of the Notification Server to which the notifications will subsequently be POSTed. The way the server determines the address of the Notification Server is out of scope of this specification.

Note: In the case when the client has set up a Notification Channel in order to use Long Polling to obtain the notifications, in order to retrieve the notifications, the client needs to use the Long Polling mechanism described in [REST_NetAPI_NotificationChannel], instead of the mechanism described below in section 6.x.y. 6.x.y to be replaced by the reference to the section that describes the actual POST method on THIS resource (e.g. in this case 6.1.5)

### Request URL variables

The following request URL variables are common for all HTTP methods:

| Name | Description |
| --- | --- |
| serverRoot | Server base url: hostname+port+base path. Port and base path are OPTIONAL. Example: example.com/exampleAPI |
| apiVersion | Version of the API client wants to use. The value of this variable is defined in section 5.1 |
| [ResourceRelPath] | Relative resource path for a Light-weight Resource, consisting of a relative path down to an element in the data structure. For more information about the applicable values (strings) for this variable see [section number entitled “Light-weight relative resource paths” applicable for the current resource]. [This row is only present in case the resource has Light-weight child resources] |
| << Add/Remove rows to this table as needed - DELETE This Row>> |  |

See section 6 for a statement on the escaping of reserved characters in URL variables.

<< Light-weight Resource relative paths. This subsection is only applicable if the resource allows accessing individual sub-trees in the data structure using the Light-weight Resource mechanism (i.e. [ResourceRelPath is part of the resourceURL]>>

#### Light-weight Resource relative paths

The following table describes the types of Light-weight Resources that can be accessed by using this resource, applicable methods, and links to data structures that contain values (strings) for those relative resource paths.

| Light-weight Resource type | Method supported | Description |
| --- | --- | --- |
| [Description of the type] | [list of HTTP methods, POST not allowed] | [Description and reference to the allowed values] |
| << Example - DELETE This and the following Row >> |  |  |
| Person attributes | GET, PUT, DELETE | Enables access to a single presence attribute related to a person. See data structure 5.2.2.4 for possible values for the Light-weight relative resource path. |

### Response Codes and Error Handling

For HTTP response codes, see [REST_NetAPI_Common].

For Policy Exception and Service Exception fault codes applicable to [Functional Area], see section 7.

### GET

<< This is a blueprint for GET in case it is not a valid operation>>

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [PUT/POST/DELETE]’ field in the response as per sections 6.5.5 and 7.4.1 of [RFC7231].

<< This is a blueprint for GET in case it is a valid operation>>

This operation is used for [description of operation].

<< The following table is optional and is used only if query parameters are supported in request URL for GET, otherwise it needs to be deleted >>

Supported parameters in the query string of the Request URL are:

| Name | Type/Values | Optional | Description |
| --- | --- | --- | --- |
| [Parameter name] | [Type/Values] | [Yes/No] | [Parameter description] |
| << Add/Remove rows to this table as needed - DELETE This Row>> |  |  |  |

When using query parameters the following conventions apply: >>

    - Query parameters are appended to the resource URL starting with a question mark “?” character and then followed by query parameter name – value pairs.

    - Multiple query parameter name-value pairs are separated by an ampersand "&" character. Example: ?par1=par1Val&par2=par2Val&..

    - Multiple values for the same query parameters are specified as a list of name-value pairs using the same name, separated by an ampersand “&” character. Example: ?par1=par1Val1&par1=par1Val2&...

#### Example 1: [Example title] 	(Informative)

<< Section 6.1.3.1 provides guidance w.r.t. naming conventions and validation of examples.

If there is only one example, remove the sequence number from the title heading >>

<< Prior to Candidate approval, a TS with XML examples MUST be submitted to the OMA XML validation service for validation of the examples: http://www.openmobilealliance.org/xml/. It is RECOMMENDED to do the same prior to or as part of CONR.

The examples must use real-world values. See document OMA-ARC-REST-2010-0675R01

The following conventions apply:

    - {serverRoot} http://example.com/exampleAPI also to be updated in the tables where {serverRoot} is defined in section 6.x.

    - {version} In our case this is v1 matching the TS version.

    - {userId} E-mail names: mailto:alice@example.com mailto:bob@example.com  or phone numbers: tel:+1-555-555-0100 to tel:+1-555-555-0199. In fact, only 555-0100 through 555-0199 are now specifically reserved for fictional use, with the other numbers having been released for actual assignment.

    - {deviceAddress}, {senderAddress} Typically a phone number

    - {equipmentId} Typically a manufacturer type name or serial number

    - {memberListId} Typically a group name, “friend”, “list123”

    - {contactId} Typically a person’s name, “bob”

    - {memberId} Typically a phone number or e-mail address or SIP URI

    - {subscriptionId} Typically a number or a sequence of digits and letters, “sub123”

    - {messageId} Typically a number or a sequence of digits and letters, “msg123”

    - {interactionId} Typically a number or a sequence of digits and letters, “int123”

    - {registrationId} Typically a number or a sequence of digits and letters, “reg123”

    - {requestId} Typically a number or a sequence of digits and letters, “req123”

    - {ruleId} Typically a number or a sequence of digits and letters, “rule123”>>

##### Request

[HTTP headers] [XML request (if applicable), starting with <?xml]

##### Response

[HTTP headers] [XML response (if applicable), starting with <?xml]

#### Example 2: [Example title] 	(Informative)

<< Section 6.1.3.1 provides guidance w.r.t. naming conventions and validation of examples.

If there is only one example, remove this section >>

##### Request

[HTTP headers] [XML request (if applicable), starting with <?xml]

##### Response

[HTTP headers] [XML response (if applicable), starting with <?xml]

### PUT

<< This is a blueprint for PUT in case it is not a valid operation>>

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [GET/POST/DELETE]’ field in the response as per sections 6.5.5 and 7.4.1 of [RFC7231].

<< This is a blueprint for PUT in case it is a valid operation>>

This operation is used for [description of operation].

#### Example 1: [Example title] 	(Informative)

<< Section 6.1.3.1 provides guidance w.r.t. naming conventions and validation of examples.

If there is only one example, remove the sequence number from the title heading >>

##### Request

[HTTP headers] [XML request (if applicable), starting with <?xml]

##### Response

[HTTP headers] [XML response (if applicable), starting with <?xml]

#### Example 2: [Example title] 	(Informative)

<< Section 6.1.3.1 provides guidance w.r.t. naming conventions and validation of examples.

If there is only one example, remove this section >>

##### Request

[HTTP headers] [XML request (if applicable), starting with <?xml]

##### Response

[HTTP headers] [XML response (if applicable), starting with <?xml]

### POST

<< This is a blueprint for POST in case it is not a valid operation>>

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [GET/PUT/DELETE]’ field in the response as per sections 6.5.5 and 7.4.1 of [RFC7231].

<< This is a blueprint for POST in case it is a valid operation>>

This operation is used for [description of operation].

If the resource is on the server side and it supports creating a subscription for notifications, and if the use of Notification Channel is supported, include/adapt this paragraph, otherwise delete it. The notifyURL in the callbackReference either contains the Client-side Notification URL (as defined by the client) or the Server-side Notification URL (as obtained during the creation of the Notification Channel [REST_NetAPI_NotificationChannel]).

#### Example 1: [Example title] 	(Informative)

<< Section 6.1.3.1 provides guidance w.r.t. naming conventions and validation of examples.

If there is only one example, remove the sequence number from the title heading >>

##### Request

[HTTP headers] [XML request (if applicable, starting with <?xml]

##### Response

[HTTP headers] [XML response (if applicable, starting with <?xml]

#### Example 2: [Example title] 	(Informative)

<< Section 6.1.3.1 provides guidance w.r.t. naming conventions and validation of examples.

If there is only one example, remove this section >>

##### Request

[HTTP headers] [XML request (if applicable, starting with <?xml]

##### Response

[HTTP headers] [XML response (if applicable, starting with <?xml]

### DELETE

<< This is a blueprint for DELETE in case it is not a valid operation>>

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [GET/PUT/POST]’ field in the response as per sections 6.5.5 and 7.4.1 of [RFC7231].

<< This is a blueprint for DELETE in case it is a valid operation>>

This operation is used for [description of operation].

#### Example 1: [Example title] 	(Informative)

<< Section 6.1.3.1 provides guidance w.r.t. naming conventions and validation of examples.

If there is only one example, remove the sequence number from the title heading >>

##### Request

[HTTP headers] [XML request (if applicable, starting with <?xml]

##### Response

[HTTP headers] [XML response (if applicable, starting with <?xml]

#### Example 2: [Example title] 	(Informative)

<< Section 6.1.3.1 provides guidance w.r.t. naming conventions and validation of examples.

If there is only one example, remove this section >>

##### Request

[HTTP headers] [XML request (if applicable, starting with <?xml]

##### Response

[HTTP headers] [XML response (if applicable, starting with <?xml]

# Fault definitions

## Service Exceptions

<< This section provides details about Service Exception type of faults specific for that particular API. Some APIs do have specific Service Exception fault definitions, some don’t have. Pick the right text block. Delete this comment. >>

<< If API has no specific Service Exception codes defined either in that particular API version, or in the baseline version, use the following text. Delete this comment. >>

For common Service Exceptions refer to [REST_NetAPI_Common]. There are no additional Service Exception codes defined for the RESTful [Functional Area] API.

<< If API has specific Service Exception codes defined either in that particular version, or in the baseline version, use the following text and include the codes in the table(s).

Service Exception codes consists of a prefix “SVC” followed by 4 digit code number.  
The original Service Exception codes from the baseline product (if any) are included unchanged. 
For a new Service Exception code, 4 digit code number MUST be obtained from OMNA Exception codes registry. Delete this comment >>

For common Service Exceptions refer to [REST_NetAPI_Common]. The following additional Service Exception codes are defined for the RESTful [Functional Area] API.

### SVC[code number]: [Text for exception header]

| Name | Description |
| --- | --- |
| MessageID | SVC[code number] |
| Text | [Text describing the fault with optional replacement variables marked with %n, where n is an index into the list of <Variables> elements, starting at 1] |
| Variables | [%n variables to substitute into the string, or “None”] |
| HTTP status code(s) | [HTTP status code(s) where that particular Service Exception code can be used with] |

## Policy Exceptions

<< This section provides details about Policy Exception type of faults specific for that particular API. Some APIs do have specific Policy Exception fault definitions, some don’t have. Pick the right text block. Delete this comment. >>

<< If API has no specific Policy Exception codes defined either in that particular API version, or in the baseline version, use the following text. Delete this comment. >>

For common Policy Exceptions refer to [REST_NetAPI_Common]. There are no additional Policy Exception codes defined for the RESTful [Functional Area] API.

<< If API has specific Policy Exception codes defined either in that particular version, or in the baseline version, use the following text and include the codes in the table(s).

Policy Exception code consists of a prefix “POL” followed by 4 digit code number.  
The original Policy Exception codes from the baseline product (if any) are included unchanged. 
For a new Policy Exception code, 4 digit code number MUST be obtained from OMNA Exception codes registry. Delete this comment. >>

For common Policy Exceptions refer to [REST_NetAPI_Common]. The following additional Policy Exception codes are defined for the RESTful [Functional Area] API.

### POL[code number]: [Text for exception header]

| Name | Description |
| --- | --- |
| MessageID | POL[code number] |
| Text | [Text describing the fault with optional replacement variables marked with %n, where n is an index into the list of <variables> elements, starting at 1] |
| Variables | [%n variables to substitute into the string, or “None”] |
| HTTP status code(s) | [HTTP status code(s) where that particular Policy Exception code can be used with] |

<< Example - DELETE this row and the following table. >>

### POL1003: Refund exceeds original charge amount

| Name | Description |
| --- | --- |
| MessageID | POL1003 |
| Text | The refund amount exceeds the original amount charged %1 |
| Variables | %1 – the original amount charged |
| HTTP status code(s) | 403 Forbidden |

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| n/a | n/a | No prior version |

Draft/Candidate Version 1.0 History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Version: REST_NetAPI_AliasManagement-V1_0-20170922-D | 22 Sep 2017 | All | First draft |
| Draft Version: REST_NetAPI_AliasManagement-V1_0-20171010-D | 10 Oct 2017 | 1, 2.1, 2.2, 3.2, 4, 4.1, 5, 5.1 | OMA-ARC-Alias-2017-0006-CR_TS_Intro_sections OMA-ARC-Alias-2017-0007R01-CR_TS_sec5_resource_summary OMA-ARC-Alias-2017-0008-CR_TS_sec5_API_def OMA-ARC-Alias-2017-0010-CR_Refs_Defs |
| Draft Version: REST_NetAPI_AliasManagement-V1_0-20171012-D | 12 Oct 2017 | 2.1, 5.1, 5.2 | OMA-ARC-Alias-2017-0011R03-CR_resource_tree_subscription_improvement OMA-ARC-Alias-2017-0012R01-CR_data_types |

Static Conformance Requirements	(Normative)

The notation used in this appendix is specified in [SCRRULES].

SCR for REST.FUNCAREA Server

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| REST-FUNCAREA-SUPPORT-S-001-M | Support for the RESTful [FuncArea] API | [section(s)] |  |
| REST- FUNCAREA-SUPPORT-S-002-M | Support for the XML request & response format | [section(s)] |  |
| REST- FUNCAREA-SUPPORT-S-003-M | Support for the JSON request & response format | [section(s)] |  |

SCR for REST.FUNCAREA.FUNCTION Server

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| [Item number] | [Description of the function] | [Section reference] | [Applicable items (requirements)] |
| [Item number] | [Description of the function] | [Section reference] | [Applicable items (requirements)] |

<< If an Item is MANDATORY (-M) it has no requirement.

If an Item is OPTIONAL (-O), but other OPTIONAL items are conditional on that first item i.e. MUST be implemented if the first item is implemented, then the conditional items are listed in the Requirements column of the first item, linked by “AND”

Example: optional resource with conditional GET and DELETE operations. Delete this comment and the following table.>>

| REST-CN-SUBSCR-INDCALLDIR-S-001-O | Support for access to an individual subscription to call direction notifications | 5.8 | REST-CN-SUBSCR-INDCALLDIR-S-002-O AND REST-CN-SUBSCR-INDCALLDIR-S-003-O |
| --- | --- | --- | --- |
| REST-CN-SUBSCR-INDCALLDIR-S-002-O | Retrieving an individual subscription to call direction notifications – GET | 5.8.3 |  |
| REST-CN-SUBSCR-INDCALLDIR-S-003-O | Deleting an individual subscription to call direction notifications – DELETE | 5.8.6 |  |

JSON examples 	(Informative)

JSON (JavaScript Object Notation) is a Light-weight, text-based, language-independent data interchange format. It provides a simple means to represent basic name-value pairs, arrays and objects. JSON is relatively trivial to parse and evaluate using standard JavaScript libraries, and hence is suited for REST invocations from browsers or other processors with JavaScript engines. Further information on JSON can be found at [RFC7159].

The following examples show the request and response for various operations using the JSON data format. The examples follow the XML to JSON serialization rules in [REST_NetAPI_Common]. A JSON response can be obtained by using the content type negotiation mechanism specified in [REST_NetAPI_Common].

For full details on the operations themselves please refer to the section number indicated.

[Example Title] (section [section number cross reference])

<< Example title should be copied from title heading of the section with the corresponding XML example. And that section number should be added in brackets to the end of the example title >>

Request:

[HTTP headers copied from referenced example, type specific signalling updated for JSON] [JSON example generated from the equivalent XML example using the JSON2XML utility]

Response:

[HTTP headers copied from referenced example, type specific signalling updated for JSON] [JSON example generated from the equivalent XML example using the JSON2XML utility]

[Baseline specification] operations mapping	(Informative)

<< This appendix is only needed for specifications which define REST bindings for an existing interface / API, such as Parlay X.

For ParlayREST specifications, substitute [Baseline specification] with “Parlay X”. For other baselines, reword accordingly.

In case there is no baseline, the headline is “Operations mapping to a pre-existing baseline specification	(Informative)”

Delete this comment.>>

<<If there is no baseline, use the following wording. Delete this comment. >>

As this specification does not have a baseline specification, this appendix is empty.

<<If there is a baseline, use the following wording. Delete this comment. >>

The table below illustrates the mapping between REST resources/methods defined in this specification and [Baseline specification] [[BASELINE_REF]] equivalent operations.

| REST Resource |  | REST Method | REST Section reference | [Baseline specification] equivalent operation |
| --- | --- | --- | --- | --- |
| [Resource description from first column in one of the tables in section 5.1] |  | [GET/PUT/POST/DELETE] | [section cross-reference] | [Operation name from Baseline specification] |
| << Add/Remove rows to this table as needed - DELETE This Row>> |  |  |  |

Table 1 [Baseline specification] operations mapping

Light-weight Resources	(Informative)

<< This appendix lists Light-weight Resources defined in this specification. Delete this comment>>

<<If there are no Light-weight Resource in the spec, the following wording is used. Delete this comment.>>

As this version of the specification does not define any Light-weight Resources, this appendix is empty.

<<If there are no Light-weight Resource in the spec, the following wording is used. Delete this comment.>>

The following table lists all [FuncArea] data structure elements that can be accessed individually as Light-weight Resources.

For each Light-weight Resource, the following information is provided: corresponding root element name, root element type and [ResourceRelPath] string.

| Type of Light-weight Resources (and references to data structures) |  | Element/attribute that can be accessed as Light-weight Resource | Root element name for the Light-weight Resource | Root element type for the Light-weight Resource | [ResourceRelPath] string that needs to be appended to the corresponding Heavy-weight Resource URL |
| --- | --- | --- | --- | --- | --- |
| [Resource Type] ([section ref]) |  | [child element name] | [root element name] | [root element type] | [ResourceRelPath] |
|  |  | [child element name] | [root element name] | [root element type] | [ResourceRelPath] |
| << Example below - DELETE this Row and the following table>> |  |  |  |  |  |  |  |
| Presence data (5.2.3) | person |  | person | PersonAttributes | person |
|  | service |  | service | ServiceAttributes | service/{serviceId}/{version} |
|  | device |  | device | DeviceAttributes | device/{deviceId} |

<<If [ResourceRelPath] strings in the above table include variables in curly brackets “{}” then the following paragraph shall be included also; otherwise it should be deleted. Delete this comment >>

Note: When appending [ResourceRelPath] string to its Heavy-weight Resource URL, all variables within curly brackets “{}” such as: [list of variable names from ResourceRelPath strings]  have to be replaced by their real values.

Authorization aspects	(Normative)

<< This appendix lists authorization aspects specific of the particular API, such as OAuth scope values. It is mandatory but may be empty. Delete this comment. >>

<< If there are no Authorization aspects specified for the specs, the following wording is used. Delete this comment. >>

None specified in this version of the specification.

<< If there are Authorization aspects specified for the specs, the following wording is used. Delete this comment. >>

This appendix specifies how to use the RESTful [Func Area] API in combination with some authorization frameworks.

Use with OMA Authorization Framework for Network APIs

The RESTful [FuncArea] API MAY support the authorization framework defined in [Autho4API_10].

A RESTful [FuncArea] API supporting [Autho4API_10]:

SHALL conform to section D.1 of [REST_NetAPI_Common];

SHALL conform to this section G.1.

Scope values

Definitions

In compliance with [Autho4API_10], an authorization server serving clients requests for getting authorized access to the resources exposed by the RESTful [FuncArea] API:

SHALL support the scope values defined in the table below;

MAY support scope values not defined in this specification.

| Scope value |  | Description |  | For one-time access token |  |
| --- | --- | --- | --- | --- | --- |
| [Scope value] |  | [Scope value description] |  | [No/Yes] |  |
| [Scope value] |  | [Scope value description] |  | [No/Yes] |  |
| << Example - DELETE this and next two Rows>> |  |  |  |  |  |
| oma_rest_messaging.all_{apiVersion} |  |  | Provide access to all defined operations on the resources in this version of the API. The {apiVersion} part of this identifier SHALL have the same value as the “apiVersion” URL variable which is defined in section 5.1. This scope value is the union of the other scope values listed in next rows of this table. |  | No |
| oma_rest_messaging.in_regist |  |  | Provide access to all defined operations on inbound messages using registration |  | No |

Table 2: Scope values for RESTful [FuncArea] API

Downscoping

In the case where the client requests authorization for “oma_rest_funcarea.all_{apiVersion}” scope, the authorization server and/or resource owner MAY restrict the granted scope to some of the following scope values:

[list of scope values]

Mapping with resources and methods

Tables in this section specify how the scope values defined in section G.1.1.1 for the RESTful [FuncArea] API map to the REST resources and methods of this API. In these tables, the root “oma_rest_funcarea.” of scope values is omitted for readability reasons.

<< Note: this part of the TS uses a landscape layout, started and terminated by a section break.  Delete this comment. >>

| Resource | URL Base URL: http://{serverRoot}/Functional Area/{apiVersion} |  | Section reference | HTTP verbs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | GET | PUT | POST | DELETE |
| [Description of the resource] | [URL for the resource] |  | [Section refrerence] | [supported scope value(s)] | [supported scope value(s)] | [supported scope value(s)] | [supported scope value(s)] |
| << Example below - DELETE this and the following Row>> |  |  |  |  |  |  |  |
| Inbound messages for a given registration |  | /inbound/registrations/{registrationId}/messages | 6.1 | all_{apiVersion} or in_regist | n/a | n/a | n/a |

Table 3: Required scope values for: [text describing function(s) associated with that particular scope values]

Use of ‘acr:auth’

<< Some APIs do have user identifiers in resource URL that could be a subject for ‘acr:Authirization’, some don’t have. Pick the right text block. Delete this comment. >>

<<If there are no user identifiers candidate for ‘acr:auth’, the following wording is used. Delete this comment. >>

As this version of the specification does not define any parameter that could be a candidate for ‘acr:auth’, this appendix is empty

<< The text below is a blueprint of Appendix F.1.2 if there are user identifiers candidate for ‘acr:auth’. Delete this comment. >>

This section specifies the use of ‘acr:auth’ in place of an end user identifier in a resource URL path.

An ‘acr’ URI of the form ‘acr:auth’, where ‘auth’ is a reserved keyword MAY be used to avoid exposing a real end user identifier in the resource URL path.

A client MAY use ‘acr:auth’ in a resource URL in place of a {senderAddress} replace/adapt “senderAddress” with a variable name of end user identifier which is a candidate for acr:auth. If multiple identifiers are candidate they shall be separated by comma. when the the RESTful [FuncArea] API is used in combination with [Autho4API_10].

In the case the RESTful [FuncArea] API supports [Autho4API_10], the server:

- SHALL accept ‘acr:auth’ as a valid value for the resource URL variable {senderAddress} replace/adapt “senderAddress” with a name of end user identifier which is a candidate for acr:auth. If multiple identifiers are candidate they shall be separated by comma.

- SHALL conform to [REST_NetAPI_Common] section 5.8.1.1 regarding the processing of ‘acr:auth’.