Change Request

| Title: | Required Flag Names | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA ARC |  |
| Doc to Change: | OMA-TS-REST_NetAPI_NMS-V1_0-20130716-D |  |
| Submission Date: | 06 Aug. 2013 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Shahram Mohajeri, sm7084@att.com |  |
| Replaces: | n/a |  |

Reason for Change

As stated in “Rich Communication Suite RCS API Detailed Requirements Version 2.3” document (relevant part from section 4.15 is copy/pasted below), the flags are defined in IETF RFC3501.

| UNI-NMS-004 | The NMS API SHALL support changing message status (flags) of a specific message(s) with message UID(s) | oauth_token={accesstoken} Message_UID={UID,…} Message_status={UID¦fla gs,…) …. | The returned information is a list of tuples consisting of the message UID and corresponding flags. The flags are defined in IETF RFC 3501 |
| --- | --- | --- | --- |

Additionally, “CPM Message Storage” document refers to a required minimum set of Flag names as listed below.

The following is copy & pasted from “CPM Message Storage” document, section 5.5:

“With respect to the metadata flags, the Message Storage Client and Message Storage Server SHALL support at least the following flags defined in [RFC3501], [RFC5788] and Appendix C:

•	\Seen (message has been read),

•	\Answered (message has been answered),

•	\Flagged (message is "flagged" for urgent and/or special attention),

•	\Deleted (message is "deleted" for removal by later EXPUNGE),

•	\Draft (message has not completed composition (marked as a draft),

•	\Recent (message is "recently" arrived in this mailbox),

•	$MDNSent (A disposition notification has been sent for this message),

•	$Forwarded (message has been forwarded), and

•	\read-report-sent (A read receipt has been sent for this message) as defined in Appendix C.”

According to the above list of well known flags, the objective of this CR is to provide a table (Normative) in Appendix H which lists the above CPM-defined flag names while leaving implementation with the flexibility to add additional flag names as necessary.

R01: incorporates ARC discussion on CC which took place on Aug. 06, 2013.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The ARC group is recommended to accept the proposed changes to NMS TS.

Detailed Change Proposal

For details about the proposed changes, see attached OMA-TS-REST_NetAPI_NMS-V1_0-20130716-D_CR14R01.