Change Request

| Title: | CONR_add_more_pathToId_examples | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA ARC |  |
| Doc to Change: | OMA-TS-REST_NetAPI_NMS-V1_0-20140318-D |  |
| Submission Date: | 14 Apr. 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Shahram Mohajeri, sm7084@att.com |  |
| Replaces: | n/a |  |

Reason for Change

The objective of this CR is to resolve CONRR item A234, A237, A238 & A100:

| A234 | 2014.03.07 | E | 6.8.5 | Source: Metaswitch Form: CONR-2014-0009 Comment: It would be useful to include an example of a request that includes at least one path to a non-existing object, and separately an example containing a forbidden/invalid path. Currently it’s unclear what the response would be if some of the paths match existing objects, but some do not. Proposed Change: Add the examples. | Status: Closed. Changed as proposed |
| --- | --- | --- | --- | --- | --- |
| A237 | 2014.03.07 | Q | 6.13.5 | Source: Metaswitch Form: CONR-2014-0009 Comment: What is the response if one of the paths is for a non-existing folder? It should be the same overall shape as the response to a POST request for a series of objects with the same problem. See 6.13.5.2 which may answer the question, and the technical question about 6.13.5.2 section (M442) below. Proposed Change: Include an example. | Status: Closed. Changed as proposed. |
| A238 | 2014.03.07 | T | 6.13.5.2 | Source: Metaswitch Form: CONR-2014-0009 Comment: What is an “invalid path”? Proposed Change: Clarify the meaning, e.g. is it for a path that does not match an existing folder, or a path that is invalid due to being badly formed, e.g.“//”. | Status: Closed. Changed as proposed. Further clarification added |
| A100 | 2014.02.25 | T | 5.2.2.3 | Source: AT&T Form: <OMA-CONR-2014-0012> Comment: Need to work the FFS regarding “whether ObjectReference is needed or should it be renamed” Proposed Change: provide a resolution for the FFS | Status: CLOSED Merged ObjectReferrence and FolderReferrence as both data type had the same elements. This merge would also allow the usage of the same “Response“ data type used in pathToIdResponseList for bulk object & folder pathToId operations |

R01: also added A237 & A238 as they were dealing with the same kind of issue as A234. The resolution (newly added data structures: PathToIdResponseList & Response)) for A234 would resolve A237 & A238 issues as well.

Additionally added A100 resolution in order to merge ObjectReferrence and FolderReferrence data types so that it PathToIdResponseList & Response can be used for both object and folder bulk pathToId operation.

R02: Uploading the correct attachment which was not correctly done with CR53R01.

R03: incorporates Comverse’s suggestion regarding the usage of “success” and “faiure” instead of “found” and “failed”. Also, incorporates MetaSwitch’s suggestion in using Link element in designing a merge between ObjectReferrence and FolderReferrence data types.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The ARC group is recommended to accept the proposed changes to NMS TS.

Detailed Change Proposal

For Change 1 below also see attached OMA-TS-REST_NetAPI_NMS-V1_0-20140318-D_CR53R02 which appropriately changes references to ObjectReferrence/objectReferrence and FolderReferrence/folderReferrence to NmsResourceReference/nmsResourceReference  throughout the document.

- A100 – Merge ObjectReferrence and FolderReferrence which would also allows the usage of the same “Response “ and “pathToIdResponseList” data types for bulk object & folder pathToId operations.

#### Type:  NmsResourceReference

Reference to stored object

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| link | common:Link | No | The resource URL of the referenced object or folder. The rel subelement MUST be either “object” or “folder”. |
| path | xsd:string | Yes | The location of the object or folder in the hierarchical storage. |

A root element named nmsResourceReference of type NmsResourceReference is allowed in request and/or response bodies.

#### Type: ObjectReferenceList

List of object references

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| objectReference | NmsResourceReference[0..unbounded] | Yes | A list of object references. |

A root element named objectReferenceList of type ObjectReferenceList is allowed in response bodies.

#### Type: FolderReferenceList

List of folder references

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| folderReference | NmsResourceReference[0..unbounded] | Yes | A list of folder references. |

A root element named folderReferenceList of type FolderReferenceList is allowed in response bodies.

#### Resource Table in Section 5.1 Updated

| Resource URLs of a selected set of objects in the storage | /pathToId Note: read as path-To-Id | NmsResourceReference (used for GET response) PathList (used for POST request) PathToIdResponseList (used for POST response) | Retrieve resource URL for an object, based on its pathname which is provided via query string | no | Retrieve resource URLs for a list of objects, based on their pathnames | no |
| --- | --- | --- | --- | --- | --- | --- |
| Resource URLs of a selected set of folders in the storage | /pathToId Note: read as path-To-Id | NmsResourceReference (used for GET response) PathList (used for POST request) PathToIdResponseList (used for POST response) | Retrieve resource URL for a folder, based on its pathname which is provided via query string | no | Retrieve resource URLs for a list of folders, based on their pathnames | no |

- Provide  the newly required data types allowing partial success response to  bulk pathToId operation

#### Type: Response

Status of a single path lookup in the bulk pathToId operation

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| success | NmsResourceReference | Choice | Reference to a given object’s path. This element MUST be present when a given path corresponds to an object. |
| failure | common:RequestError | Choice | The error that occurred when attempting to find the object using its path. This element MUST be present when finding the object using its path failed. It identifies the reason for the failure. |

XSD modelling uses a “choice” to select either success or failure.

#### Type: PathToIdResponseList

Response to a bulk pathToId operation

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| response | Response[1..unbounded] | No | List of pathToId responses. See section x.x.x. |

A root element named pathToIdResponseList of type PathToIdResponseList is allowed in response bodies.

FFS: investigate whether we want to have a generic BulkResponseList data type which can be used in other bulk operations (e.g. bulk creation, bulk pathToId, bulk deletion, etc. If so then the above data types can be merged into Creation/CreationList data types using a separate CR.

- A234 -New example of a request that includes at least one path to a non-existing object

#### 6.8.5.2 Example 2: Retrieve list of objects’ resource URLs based on their paths, at least one path to a non-existing object  	(Informative)

In this example there is an invalid path which does not match an existing object. Note that a malformed path (e.g. /main//funny) would also be considered an “invalid” path.

##### Request

POST /exampleAPI/nms/v1/myStore/tel%3A%2B19585550100/objects/operations/pathToId HTTP/1.1 Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903 Accept: application/xml Content-Type: application/xml Content-Length: nnnn <?xml version="1.0" encoding="UTF-8"?> <nms:pathList xmlns:nms="urn:oma:xml:rest:netapi:nms:1"> <path>/main/f81d4fae-7dec-11d0-a765-00a0c91e6bf6/oId111</path> <path>/cleanups/conversation5/oId221</path> </nms:pathList>

##### Response

HTTP/1.1 200 OK Date: Tue, 18 Feb 2014 12:09:09 GMT Content-Type: application/xml Content-Length: nnnn <?xml version="1.0" encoding="UTF-8"?> <nms:pathToIdResponseList xmlns:nms="urn:oma:xml:rest:netapi:1"> <response> <success> <link rel="object" href="http://exampleAPI/nms/v1/myStore/tel%3A%2B19585550100/objects/oId111"/> <path>>/main/f81d4fae-7dec-11d0-a765-00a0c91e6bf6/oId111</path> </success> </response> <response> <failure> <serviceException> <messageId>SVC0002</messageId> <text>Invalid input value for message part %1</text> <variables>/cleanups/conversation5/oId221</variables> </serviceException> </failure> </response> </nms:pathToIdResponseList >

- A234 -New example of a request containing an invalid path

#### 6.8.5.3 Example 3: Retrieve list of objects’ resource URLs based on their paths, at least one invalid path object  	(Informative)

In this example there is an invalid path which does not match an existing object. Note that a malformed path (e.g. /main//funny/obj123) would also be considered an “invalid” path.

##### Request

POST /exampleAPI/nms/v1/myStore/tel%3A%2B19585550100/objects/operations/pathToId HTTP/1.1 Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903 Accept: application/xml Content-Type: application/xml Content-Length: nnnn <?xml version="1.0" encoding="UTF-8"?> <nms:pathList xmlns:nms="urn:oma:xml:rest:netapi:nms:1"> <path>/main/conversation5/oId221</path> <path>/personal/oId222</path> </nms:pathList>

##### Response

HTTP/1.1 200 OK Date: Tue, 18 Feb 2014 12:09:09 GMT Content-Type: application/xml Content-Length: nnnn <?xml version="1.0" encoding="UTF-8"?> <nms:pathToIdResponseList xmlns:nms="urn:oma:xml:rest:netapi:1"> <response> <success> <link rel="object" href="http://exampleAPI/nms/v1/myStore/tel%3A%2B19585550100/objects/ oId221"/> <path>/main/conversation5/oId221</path> </success> </response> <response> <failure> <serviceException> <messageId>SVC0002</messageId> <text>Invalid input value for message part %1</text> <variables>/personal/oId222</variables> </serviceException> </failure> </response> </nms:pathToIdResponseList >

- A237 & A238 – Updated example allows partial success while reporting the invalid path

#### 6.14.5.2 Example 2: Retrieve list of folders’ resource URLs based on their paths, response failure due to an invalid path in the list 	(Informative)

In this example there is an invalid path which does not match an existing folder. Note that a malformed path (e.g. /main//funny) would also be considered an “invalid” path.

##### Request

POST /exampleAPI/nms/v1/myStore/tel%3A%2B19585550100/folders/operations/pathToId HTTP/1.1 Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903 Accept: application/xml Content-Type: application/xml Content-Length: nnnn <?xml version="1.0" encoding="UTF-8"?> <nms:pathList xmlns:nms="urn:oma:xml:rest:netapi:nms:1"> <path>/main/f81d4fae-7dec-11d0-a765-00a0c91e6bf6</path> <path>/main/conversation5</path> <path>/main/SorrentoMeeting/year2015</path> </nms:pathList>

##### Response

HTTP/1.1 400 Bad request Date: Tue, 20 Nov 2013 02:51:59 GMT Content-Type: application/xml Content-Length: nnnn <?xml version="1.0" encoding="UTF-8"?> <nms:pathToIdResponseList xmlns:nms="urn:oma:xml:rest:netapi:1"> <response> <success> <link rel="folder" href="http://exampleAPI/nms/v1/myStore/tel%3A%2B19585550100/folders /fId9"/> <path>/main/conversation5/</path> </success> </response> <response> <failure> <serviceException> <messageId>SVC0002</messageId> <text>Invalid input value for message part %1</text> <variables>/main/SorrentoMeeting/year2015</variables> </serviceException> </failure> </response> </nms:pathToIdResponseList >